- INTERTANKO TANKER CHARTERING QUESTIONNAIRE 88 	Version 4

| 1. | VESSEL DESCRIPTION |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1.1 | Date updated: |  |  |  |  |  |  |  |  |  |  | Oct 27, 2015 |  |  |
| 1.2 | Vessel’s name (IMO number): |  |  |  |  |  |  |  |  |  |  | Genmar Zeus (9411032) |  |  |
| 1.3 | Vessel’s previous name(s) and date(s) of change: |  |  |  |  |  |  |  |  |  |  | Crudemed (Jul 02, 2010) |  |  |
| 1.4 | Date delivered / Builder (where built): |  |  |  |  |  |  |  |  |  |  | Feb 04, 2010 / HYUNDAI HEAVY INDUSTRIES Co. LTD |  |  |
| 1.5 | Flag / Port of Registry: |  |  |  |  |  |  |  |  |  |  | Marshall Island / Majuro |  |  |
| 1.6 | Call sign / MMSI: |  |  |  |  |  |  |  |  |  |  | V7AE4 / 53800295 |  |  |
| 1.7 | Vessel’s contact details (satcom/fax/email etc.): |  |  |  |  |  |  |  |  |  |  | Tel: +870 773 207 343 Fax: NA Email: master.gzeus@gmmfleet.com |  |  |
| 1.8 | Type of vessel (as described in Form A or Form B Q1.11 of the IOPPC): |  |  |  |  |  |  |  |  |  |  | Oil Tanker |  |  |
| 1.9 | Type of hull: |  |  |  |  |  |  |  |  |  |  | Double Hull |  |  |
| Classification |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1.10 | Classification society: |  |  |  |  |  |  |  |  |  |  | Det Norske Veritas |  |  |
| 1.11 | Class notation: |  |  |  |  |  |  |  |  |  |  | +1A1,CSR TANKER FOR OIL, ESP, E0, VCS- 2,BIS, TMON |  |  |
| 1.12 | Is the vessel subject to any conditions of class, class extensions, outstanding memorandums or class recommendations? If yes, give details: |  |  |  |  |  |  |  |  |  |  | No No |  |  |
| 1.13 | If classification society changed, name of previous and date of change: |  |  |  |  |  |  |  |  |  |  | N/A, Not Applicable |  |  |
| 1.14 | IMO type, if applicable: |  |  |  |  |  |  |  |  |  |  | N/A |  |  |
| 1.15 | Does the vessel have ice class? If yes, state what level: |  |  |  |  |  |  |  |  |  |  | N/A, None |  |  |
| 1.16 | Date / place of last dry-dock: |  |  |  |  |  |  |  |  |  |  | Feb 09, 2015 / |  |  |
| 1.17 | Date next dry dock due / next annual survey due: |  |  |  |  |  |  |  |  |  |  | Feb 04, 2020 |  | Feb 04, 2016 |
| 1.18 | Date of last special survey / next special survey due: |  |  |  |  |  |  |  |  |  |  | Feb 09, 2015 |  | Feb 04, 2020 |
| 1.19 | If ship has Condition Assessment Program (CAP), what is the latest overall rating: |  |  |  |  |  |  |  |  |  |  | No, |  |  |
| 1.20 | Does the vessel have a statement of compliance issued under the provisions of the Condition Assessment Scheme (CAS): If yes, what is the expiry date? |  |  |  |  |  |  |  |  |  |  | N/A |  |  |
| Dimensions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1.21 | Length overall (LOA): |  |  |  |  |  |  |  |  |  |  | 333.13 Metres |  |  |
| 1.22 | Length between perpendiculars (LBP): |  |  |  |  |  |  |  |  |  |  | 319.00 Metres |  |  |
| 1.23 | Extreme breadth (Beam): |  |  |  |  |  |  |  |  |  |  | 60.05 Metres |  |  |
| 1.24 | Moulded depth: |  |  |  |  |  |  |  |  |  |  | 30.40 Metres |  |  |
| 1.25 | Keel to masthead (KTM)/ Keel to masthead (KTM) in collapsed condition, if applicable: |  |  |  |  |  |  |  |  |  |  | 64.73 Metres |  |  |
| 1.26 | Bow to center manifold (BCM) / Stern to center manifold (SCM): |  |  |  |  |  |  |  |  |  |  | 165.98 Metres |  | 167.15 Metres |
| 1.27 | Distance bridge front to center of manifold: |  |  |  |  |  |  |  |  |  |  | 113.10 Metres |  |  |
| 1.28 | Parallel body distances |  |  |  |  | Lightship |  |  |  |  |  | Normal Ballast |  | Summer Dwt |
|  | # Forward to mid-point manifold: |  |  |  |  | 61.30 Metres |  |  |  |  |  | 76.56 Metres |  | 76.56 Metres |
|  | # Aft to mid-point manifold: |  |  |  |  | 24.65 Metres |  |  |  |  |  | 54.15 Metres |  | 81.01 Metres |
|  | # Parallel body length: |  |  |  |  | 85.95 Metres |  |  |  |  |  | 130.70 Metres |  | 157.57 Metres |
| # 1.29 | # FWA/TPC at summer draft: |  |  |  |  |  |  |  |  |  |  | 512 Millimetres |  | 178.10 Metric Tonnes |
| 1.30 | Constant (excluding fresh water): |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1.31 | What is the company guidelines for Under Keel Clearance (UKC) for this vessel? |  |  |  |  |  |  |  |  |  |  | Open Sea 50pct of deepest hydrostatic draft, confined waters 10pct of deepest hydrostatic draft, coastal waters 15 pct of depest hydrostatic draft Malacca Straits min 3.5m. |  |  |
| 1.32 | What is the max height of mast above waterline (air draft) |  |  |  |  |  |  |  |  |  |  | Full Mast |  | Collapsed Mast |
|  | Lightship: |  |  |  |  |  |  |  |  |  |  | 61.22 Metres |  | 0 Metres |
|  | Normal ballast: |  |  |  |  |  |  |  |  |  |  | 58.83 Metres |  | 0 Metres |
|  | At loaded summer deadweight: |  |  |  |  |  |  |  |  |  |  | 42.105 Metres |  | 0 Metres |
| Tonnages |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1.33 | Net Tonnage: |  |  |  |  |  |  |  |  |  |  | 110,402.00 |  |  |
| 1.34 | Gross Tonnage / Reduced Gross Tonnage (if applicable): |  |  |  |  |  |  |  |  |  |  | 161,939.64 |  | 129,635 |
| 1.35 | Suez Canal Tonnage - Gross (SCGT) / Net (SCNT): |  |  |  |  |  |  |  |  |  |  | 161,939.64 |  | 153,241.98 |
| # 1.36 | Panama Canal Net Tonnage (PCNT): |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Ownership and Operation |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1.37 | Registered owner - Full style: |  |  |  |  |  | GMR Zeus LLC P.O. Box 1404, Majuro, Marshall Islands, MH96960 Tel: +1 212 763 5669 Fax: Not Applicable Telex: Not Applicable Email: mg@gmmllc.com Company IMO#: 1039244 |  |  |  |  |  |  |  |
| 1.38 | Technical operator - Full style: |  |  |  |  |  | NORTHERN MARINE MANAGEMENT Ltd Alba House, 2 Central Avenue, Clydebank, G81 2QR Tel: +44 141 876 3000 Fax: +44 141 941 2791 Telex: 779797 NTHN G Email: nmmvetting@stena.com Web: www.nmm-stena.com Company IMO#: 1039244 |  |  |  |  |  |  |  |
| 1.39 | Commercial operator - Full style: |  |  |  |  |  | VL8 Pool Inc. C/O Navig8 Pte. Ltd. No. 3 Temasek Ave, #25-01, Centennial Tower, Singapore 039190 Tel: +65 - 6622 0088 Fax: +65 - 6622 0099 Telex: Not Applicable Email: ops@navig8group.com |  |  |  |  |  |  |  |
| 1.40 | Disponent owner - Full style: |  |  |  |  |  | VL8 POOL INC Trust Company Complex, Ajeltake Road, Ajeltake Island, Majuro, Marshall Islands MH 96960 |  |  |  |  |  |  |  |
| 2. | CERTIFICATION |  |  |  |  | Issued |  |  |  |  |  | Last Annual |  | Expires |
| 2.1 | Safety Equipment Certificate (SEC): |  |  |  |  | Feb 09, 2015 |  |  |  |  |  | Feb 09, 2015 |  | Feb 04, 2020 |
| 2.2 | Safety Radio Certificate (SRC): |  |  |  |  | Feb 09, 2015 |  |  |  |  |  | Feb 09, 2015 |  | Feb 04, 2020 |
| 2.3 | Safety Construction Certificate (SCC): |  |  |  |  | Feb 09, 2015 |  |  |  |  |  | Feb 09, 2015 |  | Feb 04, 2020 |
| 2.4 | International Loadline Certificate (ILC): |  |  |  |  | Feb 09, 2015 |  |  |  |  |  | Feb 09, 2015 |  | Feb 04, 2020 |
| 2.5 | International Oil Pollution Prevention Certificate (IOPPC): |  |  |  |  | Jul 16, 2015 |  |  |  |  |  | Not Applicable |  | Feb 04, 2020 |
| 2.6 | ISM Safety Management Certificate (SMC): |  |  |  |  | Sep 26, 2015 |  |  |  |  |  | Not Applicable |  | Feb 25, 2016 |
| # 2.7 | # Document of Compliance (DOC): |  |  |  |  | # Jun 04, 2014 |  |  |  |  |  | May 21, 2015 |  | Jun 22, 2019 |
| 2.8 | USCG Certificate of Compliance (COC): |  |  |  |  | Feb 12, 2014 |  |  |  |  |  |  |  | Feb 12, 2016 |
| 2.9 | Civil Liability Convention (CLC) 1992 Certificate: |  |  |  |  | Feb 20, 2015 |  |  |  |  |  | Not Applicable |  | Feb 20, 2016 |
| 2.10 | Civil Liability for Bunker Oil Pollution Damage Convention (CLBC) Certificate: |  |  |  |  | Feb 20, 2015 |  |  |  |  |  | Not Applicable |  | Feb 20, 2016 |
| 2.11 | Ship Sanitation Control (SSCC)/Ship Sanitation Control Exemption (SSCE) Certificate: |  |  |  |  | Oct 13, 2015 |  |  |  |  |  | Not Applicable |  | Apr 13, 2016 |
| 2.12 | U.S. Certificate of Financial Responsibility (COFR): |  |  |  |  | Aug 25, 2017 |  |  |  |  |  | Not Applicable |  | Aug 25, 2017 |
| 2.13 | Certificate of Class (COC): |  |  |  |  | Feb 09, 2015 |  |  |  |  |  | Feb 09, 2015 |  | Feb 04, 2020 |
| # 2.14 | # International Sewage Pollution Prevention Certificate (ISPPC): |  |  |  |  | # Feb 09, 2015 |  |  |  |  |  | # Not Applicable |  | Feb 04, 2020 |
| # 2.15 | # Certificate of Fitness (COF): |  |  |  |  | # Not Applicable |  |  |  |  |  |  |  |  |
| # 2.16 | # International Energy Efficiency Certificate (IEEC): |  |  |  |  | # Jan 31, 2013 |  |  |  |  |  | Not Applicable |  | Not Applicable |
| # 2.17 | # International Ship Security Certificate (ISSC): |  |  |  |  | # Sep 26, 2015 |  |  |  |  |  |  |  | Feb 25, 2016 |
| # 2.18 | # International Air Pollution Prevention Certificate (IAPPC): |  |  |  |  | # Feb 09, 2015 |  |  |  |  |  | Feb 09, 2015 |  | Feb 04, 2020 |
| # 2.19 | # Maritime Labour Certificate (MLC): |  |  |  |  | # Sep 26, 2015 |  |  |  |  |  | Not Applicable |  | Feb 25, 2016 |
| Documentation |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2.20 | Owner warrant that vessel is member of ITOPF and will remain so for the entire duration of this voyage/contract: |  |  |  |  |  |  |  |  |  |  | Yes |  |  |
| 2.21 | Does vessel have in place a Drug and Alcohol Policy complying with OCIMF guidelines for Control of Drugs and Alcohol Onboard Ship? |  |  |  |  |  |  |  |  |  |  | Yes |  |  |
| 2.22 | Is the ITF Special Agreement on board (if applicable)? |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2.23 | ITF Blue Card expiry date: |  |  |  |  |  |  |  |  |  |  | Dec 31, 2017 |  |  |
| 3. | CREW |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3.1 | Nationality of Master: |  |  |  |  |  |  |  |  |  |  | Russian |  |  |
| 3.2 | Number and Nationality of Officers: |  |  |  |  |  |  |  |  |  |  | 9 Russian, Bulgarian, Croatian, British |  |  |
| 3.3 | Number and Nationality of Crew: |  |  |  |  |  |  |  |  |  |  | 14 Indian |  |  |
| 3.4 | What is the common working language onboard: |  |  |  |  |  |  |  |  |  |  | English |  |  |
| 3.5 | Do officers speak and understand English? |  |  |  |  |  |  |  |  |  |  | Yes |  |  |
| 3.6 | If Officers/Crew employed by a Manning Agency - Full style: |  |  |  |  |  | Officers: Stena Marine Management PTE LTD 78 Shenton Way 12/01 079120 Singapore Tel: +65 6323 2066 Fax: +65 6323 2766 Telex: Not Applicable Email: admin@austen.com.sg Crew: Northern Marine Management Pte Ltd Alba House, 2 Central Avenue, Clyderbank, G81 2Qr, United Kingdom Tel: +44 141 876 3230 Fax: +44 141 876 3240 Telex: Not Applicable Email: nmmpersonneltankerfleet1@stena.com |  |  |  |  |  |  |  |
| 4. | FOR USA CALLS |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4.1 | Has the vessel Operator submitted a Vessel Spill Response Plan to the US Coast Guard which has been approved by official USCG letter? |  |  |  |  |  |  |  |  |  |  |  | Yes |  |
| 4.2 | Qualified individual (QI) - Full style: |  |  |  |  |  | O' Brien's Response Management Inc. 103 Morgan Lane, suite 103 Plainsboro, New Jersy 086536-3339 Tel: +1-985-781-0804 Fax: +1-985-781-0580 Email: commandcenter@obriensrm.com |  |  |  |  |  |  |  |
| 4.3 | Oil Spill Response Organization (OSRO) - Full style: |  |  |  |  |  | National Response Corporation 3500 Sunrise Highway, suite T 103 Great River, NY 11739 USA Tel: +1-631-224-9141 Fax: +1-631-224-9086 Email: iocdo@nrcc.com |  |  |  |  |  |  |  |
| 5. | CARGO AND BALLAST HANDLING |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Double Hull Vessels |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.1 | Is vessel fitted with centerline bulkhead in all cargo tanks? If Yes, solid or perforated: |  |  |  |  |  |  |  |  |  |  | Yes, Solid |  |  |
| # Loadline Information |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| # 5.2 | # Loadline |  | # Freeboard |  |  | # Draft |  |  |  |  | # Deadweight |  |  | # Displacement |
|  | # Summer: |  | 5.05 Metres |  |  | 22.63 Metres |  |  |  |  | # 318,325.31 Metric Tonnes |  |  | 364,452.01 Metric Tonnes |
|  | # Winter: |  | 5.52 Metres |  |  | 22.15 Metres |  |  |  |  | 309,940.10 Metric Tonnes |  |  | 356,067.51 Metric Tonnes |
|  | # Tropical: |  | 4.58 Metres |  |  | 23.10 Metres |  |  |  |  | 326,717.11 Metric Tonnes |  |  | 372,843.81 Metric Tonnes |
|  | # Lightship: |  | 24.16 Metres |  |  | 3.51 Metres |  |  |  |  | Not Applicable |  |  | 46,126.70 Metric Tonnes |
|  | # Normal Ballast Condition: |  | 18.12 Metres |  |  | 9.55 Metres |  |  |  |  | 94,707.70 Metric Tonnes |  |  | 140,834.40 Metric Tonnes |
| # 5.3 | # Does vessel have multiple SDWT? If yes, please provide all assigned loadlines: |  |  |  |  |  |  |  |  |  | No |  |  |  |
| Cargo Tank Capacities |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.4 | Number of cargo tanks and total cubic capacity (98%): |  |  |  |  |  |  |  |  |  |  | 15 + 2 SLOPS |  | 338,670.70 Cu. Metres |
| 5.5 | Capacity (98%) of each natural segregation with double valve (specify tanks): |  |  |  |  |  |  |  |  |  |  | Seg#1: 119059.0 m3 (1C-2C-4C-5W) Seg#2: 112486.0 m3 (3C-1W-4W-SlopTanks) Seg#3: 113295.0 m3 (5C-2W-3W) |  |  |
| 5.6 | Number of slop tanks and total cubic capacity (98%): |  |  |  |  |  |  |  |  |  |  |  |  | 7,044.60 Cu. Metres |
| 5.7 | Specify segregations which slops tanks belong to and their capacity with double valve: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.8 | Residual/Retention oil tank(s) capacity (98%), if applicable: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.9 | Does vessel have Segregated Ballast Tanks (SBT) or Clean Ballast Tanks (CBT): |  |  |  |  |  |  |  |  |  |  | SBT |  |  |
| SBT Vessels |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.10 | What is total SBT capacity and percentage of SDWT vessel can maintain? |  |  |  |  |  |  |  |  |  |  | 98,242.40 Cu. Metres |  | 31.63 % |
| 5.11 | Does vessel meet the requirements of MARPOL Annex I Reg 18.2: |  |  |  |  |  |  |  |  |  |  | Yes |  |  |
| Cargo Handling and Pumping Systems |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.12 | How many grades/products can vessel load/discharge with double valve segregation: |  |  |  |  |  |  |  |  |  |  | 3 |  |  |
| 5.13 | Are there any cargo tank filling restrictions? If yes, specify number of slack tanks, max s.g., ullage restrictions etc.: |  |  |  |  |  |  |  |  |  |  | Yes 98% |  |  |
| 5.14 | Pumps |  |  |  | No. | Type |  |  |  |  |  | Capacity |  | At What Head (sg=1.0) |
|  | Cargo Pumps: |  |  |  | 3 | Centrifugal |  |  |  |  |  | 5000 M3/HR |  | 150 Meters 150 Meters 150 Meters |
|  | Cargo Eductors: |  |  |  | 2 | liquid/water jet |  |  |  |  |  | 600 Cu. Metres/Hour |  | 30 Metres |
|  | Stripping: |  |  |  | 1 | Reciprocating |  |  |  |  |  | 400 Cu. Metres/Hour |  | 150 Metres |
|  | Ballast Pumps: |  |  |  | 2 | Centrifugal |  |  |  |  |  | 3,000 Cu. Metres/Hour |  | 35 Metres |
|  | Ballast Eductors: |  |  |  | 1 | Water jet |  |  |  |  |  | 200 Cu. Metres/Hour |  | 2.50 Metres |
| 5.15 | Max loading rate for homogenous cargo per manifold connection: |  |  |  |  |  |  |  |  |  |  | 6,830 Cu. Metres/Hour |  |  |
| 5.16 | Max loading rate for homogenous cargo loaded simultaneously through all manifolds: |  |  |  |  |  |  |  |  |  |  | 20,500.00 Cu. Metres/Hour |  |  |
| 5.17 | How many cargo pumps can be run simultaneously at full capacity: |  |  |  |  |  |  |  |  |  |  | 3 |  |  |
| Cargo Control Room |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.18 | Is ship fitted with a Cargo Control Room (CCR)? |  |  |  |  |  |  |  |  |  |  | Yes |  |  |
| 5.19 | Can tank innage / ullage be read from the CCR? |  |  |  |  |  |  |  |  |  |  | Yes |  |  |
| Gauging and Sampling |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.20 | Can cargo be transferred under closed loading conditions in accordance with ISGOTT 11.1.6.6? |  |  |  |  |  |  |  |  |  |  | Yes |  |  |
| 5.21 | What type of fixed closed tank gauging system is fitted: |  |  |  |  |  |  |  |  |  |  | Radar |  |  |
| 5.22 | Number of portable gauging units (example- MMC) on board: |  |  |  |  |  |  |  |  |  |  | 4 |  |  |
| 5.23 | Are overfill (high) alarms fitted? If Yes, indicate whether to all tanks or partial: |  |  |  |  |  |  |  |  |  |  | Yes, All |  |  |
| 5.24 | Are cargo tanks fitted with multipoint gauging? If yes, specify type and locations: |  |  |  |  |  |  |  |  |  |  | Yes, |  |  |
| 5.25 | Is gauging system certified and calibrated? If no, specify which ones are not calibrated: |  |  |  |  |  |  |  |  |  |  | Yes, |  |  |
| Vapor Emission Control System (VECS) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.26 | Is a Vapour Emission Control System (VECS) fitted? |  |  |  |  |  |  |  |  |  |  | Yes |  |  |
| 5.27 | Number/size of VECS manifolds (per side): |  |  |  |  |  |  |  |  |  |  | 2 |  | 400 Millimetres |
| 5.28 | Number / size / type of VECS reducers: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Venting |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.29 | State what type of venting system is fitted: |  |  |  |  |  |  |  |  |  |  | Mast Riser / PV Valves / PV Breaker / VapourLine |  |  |
| Cargo Manifolds and Reducers |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.30 | Does vessel comply with the latest edition of the OCIMF 'Recommendations for Oil Tanker Manifolds and Associated Equipment’? |  |  |  |  |  |  |  |  |  |  | Yes |  |  |
| 5.31 | Total number / size of cargo manifold connections on each side: |  |  |  |  |  |  |  |  |  |  | 2 / 500.00 Millimetres |  |  |
| 5.32 | What type of valves are fitted at manifold: |  |  |  |  |  |  |  |  |  |  | Butterfly |  |  |
| 5.33 | What is the material/rating of the manifold: |  |  |  |  |  |  |  |  |  |  | Steel / |  |  |
| 5.34 | Does the vessel have a Common Line Manifold connection? If yes, describe: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.35 | Distance between cargo manifold centers: |  |  |  |  |  |  |  |  |  |  | 3,000 Millimetres |  |  |
| 5.36 | Distance ships rail to manifold: |  |  |  |  |  |  |  |  |  |  | 4,395 Millimetres |  |  |
| 5.37 | Distance manifold to ships side: |  |  |  |  |  |  |  |  |  |  | 4,600 Millimetres |  |  |
| 5.38 | Top of rail to center of manifold: |  |  |  |  |  |  |  |  |  |  | 670 Millimetres |  |  |
| 5.39 | Distance main deck to center of manifold: |  |  |  |  |  |  |  |  |  |  | 2,100 Millimetres |  |  |
| 5.40 | Spill tank grating to center of manifold: |  |  |  |  |  |  |  |  |  |  | 900 Millimetres |  |  |
| 5.41 | Manifold height above the waterline in normal ballast / at SDWT condition: |  |  |  |  |  |  |  |  |  |  | 22.95 Metres |  | 9.90 Metres |
| 5.42 | Number / size / type of reducers: |  |  |  |  |  |  |  |  |  |  | 6 x 650/500mm (26/20") 3 x 650/600mm (26/24") 6 x 650/400mm (26/16") 3 x 650/300mm (26/12") 2 x 600/300mm (24/12") ANSI |  |  |
| 5.43 | Is vessel fitted with a stern manifold? If yes, state size: |  |  |  |  |  |  |  |  |  |  | No, |  |  |
| Heating |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.44 | Cargo / slop tanks fitted with a cargo heating system? |  |  |  |  |  |  | Type |  |  |  | Coiled |  | Material |
|  | Cargo Tanks: |  |  |  |  |  |  | None |  |  |  | No |  |  |
|  | Slop Tanks: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.45 | Maximum temperature cargo can be loaded / maintained: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.46 | Minimum temperature cargo can be loaded / maintained: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Coating / Anodes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.47 | Tank Coating |  |  | Coated |  | Type |  |  |  |  |  | To What Extent |  | Anodes |
|  | Cargo tanks: |  |  | Yes |  | Pure Epoxy |  |  |  |  |  | Bottom & up to 0.15 m upward / overhead & down to 2 m downward. Whole Slop Tanks coated. |  | No |
|  | Ballast tanks: |  |  | Yes |  | Pure epoxy |  |  |  |  |  | Whole |  | Yes |
|  | Slop tanks: |  |  | Yes |  | Modified epoxy |  |  |  |  |  | Whole Tank |  |  |
| 6. | INERT GAS AND CRUDE OIL WASHING |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6.1 | Is a Crude Oil Washing (COW) installation fitted / operational? |  |  |  |  |  |  |  |  |  |  | Yes / Yes |  |  |
| 6.2 | Is an Inert Gas System (IGS) fitted / operational? |  |  |  |  |  |  |  |  |  |  | Yes / Yes |  |  |
| 6.3 | Is IGS supplied by flue gas, inert gas (IG) generator and/or nitrogen: |  |  |  |  |  |  |  |  |  |  | Flue Gas |  |  |
| 7. | MOORING |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7.1 | Wires (on drums) | No. |  | Diameter |  | Material |  |  |  |  |  | Length |  | Breaking Strength |
|  | Forecastle: | 4 |  | 42 Millimetres |  | steel |  |  |  |  |  | 275 Metres |  | 115 Metric Tonnes |
|  | Main deck fwd: | 6 |  | 42 Millimetres |  | steel |  |  |  |  |  | 275 Metres |  | 115 Metric Tonnes |
|  | Main deck aft: | 4 |  | 42 Millimetres |  | steel |  |  |  |  |  | 275 Metres |  | 115 Metric Tonnes |
|  | Poop deck: | 6 |  | 42 Millimetres |  | steel |  |  |  |  |  | 275 Metres |  | 115 Metric Tonnes |
| 7.2 | Wire tails | No. |  | Diameter |  | Material |  |  |  |  |  | Length |  | Breaking Strength |
|  | Forecastle: | 4 |  | 100 Millimetres |  | nylon |  |  |  |  |  | 11 Metres |  | 158 Metric Tonnes |
|  | Main deck fwd: | 6 |  | 100 Millimetres |  | nylon |  |  |  |  |  | 11 Metres |  | 158 Metric Tonnes |
|  | Main deck aft: | 4 |  | 100 Millimetres |  | nylon |  |  |  |  |  | 11 Metres |  | 158 Metric Tonnes |
|  | Poop deck: | 6 |  | 100 Millimetres |  | nylon |  |  |  |  |  | 11 Metres |  | 158 Metric Tonnes |
| 7.3 | Ropes (on drums) | No. |  | Diameter |  | Material |  |  |  |  |  | Length |  | Breaking Strength |
|  | Forecastle: |  |  |  |  | Not Applicable |  |  |  |  |  |  |  |  |
|  | Main deck fwd: |  |  |  |  | Not Applicable |  |  |  |  |  |  |  |  |
|  | Main deck aft: |  |  |  |  | Not Applicable |  |  |  |  |  |  |  |  |
|  | Poop deck: |  |  |  |  | Not Applicable |  |  |  |  |  |  |  |  |
| 7.4 | Other lines | No. |  | Diameter |  | Material |  |  |  |  |  | Length |  | Breaking Strength |
|  | Forecastle: | 4 |  | 84 Millimetres |  | polymix |  |  |  |  |  | 275 Metres |  | 140 Metric Tonnes |
|  | Main deck fwd: |  |  |  |  | Not Applicable |  |  |  |  |  |  |  |  |
|  | Main deck aft: |  |  |  |  | Not Applicable |  |  |  |  |  |  |  |  |
|  | Poop deck: | 2 |  | 84 Millimetres |  | polymix |  |  |  |  |  | 275 Metres |  | 140 Metric Tonnes |
| 7.5 | Winches | No. |  | No. Drums |  | Motive Power |  |  |  |  |  | Brake Capacity |  | Type of Brake |
|  | Forecastle: | 2 |  | Double |  | Hydraulic |  |  |  |  |  | 69 Metric Tonnes |  | mannual |
|  | Main deck fwd: | 3 |  | Double |  | Hydraulic |  |  |  |  |  | 69 Metric Tonnes |  | mannua |
|  | Main deck aft: | 2 |  | Double |  | Hydraulic |  |  |  |  |  | 69 Metric Tonnes |  | mannua |
|  | Poop deck: | 3 |  | Double |  | Hydraulic |  |  |  |  |  | 69 Metric Tonnes |  | mannua |
| 7.6 | Bitts, closed chocks/fairleads |  |  | No. Bitts |  | SWL Bitts |  |  |  |  |  | No. Closed Chocks |  | SWL Closed Chocks |
|  | Forecastle: |  |  | 3 |  | 92 Metric Tonnes |  |  |  |  |  | 10 |  | 92 Metric Tonnes |
|  | Main deck fwd: |  |  | 9 |  | 92 Metric Tonnes |  |  |  |  |  | 21 |  | 92 Metric Tonnes |
|  | Main deck aft: |  |  | 6 |  | 92 Metric Tonnes |  |  |  |  |  | 14 |  | 92 Metric Tonnes |
|  | Poop deck: |  |  | 6 |  | 92 Metric Tonnes |  |  |  |  |  | 15 |  | 92 Metric Tonnes |
| Anchors/Emergency Towing System |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7.7 | Number of shackles on port / starboard cable: |  |  |  |  |  |  |  |  |  |  | 14 / 14 |  |  |
| 7.8 | Type / SWL of Emergency Towing system forward: |  |  |  |  |  |  |  |  |  |  | System, Type 4000A-SJEmergency TowingSystem, Type 4000F-SJ |  | 200 Metric Tonnes |
| 7.9 | Type / SWL of Emergency Towing system aft: |  |  |  |  |  |  |  |  |  |  | Emergency TowingSystem, Type 4000A-SJ |  | 200 Metric Tonnes |
| Escort Tug |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7.10 | What is size / SWL of closed chock and/or fairleads of enclosed type on stern: |  |  |  |  |  |  |  |  |  |  | 600 mm x 450 mm |  | 200 Metric Tonnes |
| 7.11 | What is SWL of bollard on poop deck suitable for escort tug: |  |  |  |  |  |  |  |  |  |  | 200 Metric Tonnes |  |  |
| Bow/Stern Thruster |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7.12 | What is brake horse power of bow thruster (if fitted): |  |  |  |  |  |  |  |  |  |  | No, |  |  |
| 7.13 | What is brake horse power of stern thruster (if fitted): |  |  |  |  |  |  |  |  |  |  | No, |  |  |
| Single Point Mooring (SPM) Equipment |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7.14 | Does the vessel meet the recommendations in the latest edition of OCIMF 'Recommendations for Equipment Employed in the Bow Mooring of Conventional Tankers at Single Point Moorings (SPM)’? |  |  |  |  |  |  |  |  |  |  | Yes |  |  |
| 7.15 | If fitted, how many chain stoppers: |  |  |  |  |  |  |  |  |  |  | 2 |  |  |
| 7.16 | State type / SWL of chain stopper(s): |  |  |  |  |  |  |  |  |  |  | tongue type |  | 350.00 Metric Tonnes |
| 7.17 | What is the maximum size chain diameter the bow stopper(s) can handle: |  |  |  |  |  |  |  |  |  |  | 76.00 Millimetres |  |  |
| 7.18 | Distance between the bow fairlead and chain stopper/bracket: |  |  |  |  |  |  |  |  |  |  | 2,700 Millimetres |  |  |
| 7.19 | Is bow chock and/or fairlead of enclosed type of OCIMF recommended size (600mm x 450mm)? If not, give details of size: |  |  |  |  |  |  |  |  |  |  | Yes Not Applicable |  |  |
| Lifting Equipment |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7.20 | Derrick / Crane description (Number, SWL and location): |  |  |  |  |  |  |  |  |  |  | Cranes: 2 x 20 Tonnes port/starboard |  |  |
| 7.21 | What is maximum outreach of cranes / derricks outboard of the ship’s side: |  |  |  |  |  |  |  |  |  |  | 8.95 Metres |  |  |
| Ship To Ship Transfer (STS) / Helicopter Operations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7.22 | Does vessel comply with recommendations contained in OCIMF/ICS Ship To Ship Transfer Guide (Petroleum, Chemicals or Liquified Gas, as applicable)? |  |  |  |  |  |  |  |  |  |  | Yes |  |  |
| 7.23 | Can the ship comply with the ICS Helicopter Guidelines? If Yes, state whether winching or landing area provided and diameter of the circle provided: |  |  |  |  |  |  |  |  |  |  | Yes, Landing 16.00 Metres |  |  |
| 8. | MISCELLANEOUS |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Engine |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.1 | Speed |  |  |  |  |  |  |  |  |  |  | Maximum |  | Economic |
|  | Ballast speed: |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Laden speed: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.2 | What type of fuel is used for main propulsion / generating plant: |  |  |  |  |  |  |  |  |  |  | hfo 380 cst |  | HFO / MGO |
| 8.3 | Type / Capacity of bunker tanks: |  |  |  |  |  |  |  |  |  |  | Fuel Oil: 8,163 Cu. Metres Diesel Oil: 0 Cu. Metres Gas Oil: 433.50 Cu. Metres |  |  |
| 8.4 | Is vessel fitted with fixed or controllable pitch propeller(s): |  |  |  |  |  |  |  |  |  |  | Fixed |  |  |
| 8.5 | Engines |  |  |  |  |  |  |  |  | No |  | Capacity |  | Make/Type |
|  | Main engine: |  |  |  |  |  |  |  |  |  |  |  |  | Hyundai Wartsila TRT-Flex 82 T |
|  | Aux engine: |  |  |  |  |  |  |  |  | 3 |  |  |  |  |
|  | Power packs: |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Boilers: |  |  |  |  |  |  |  |  | 2 |  | 45.00 Metric Tonnes/Hour |  |  |
| Emissions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.6 | Main engine IMO NOx emission standard: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.7 | Energy Efficiency Design Index (EEDI) rating number: |  |  |  |  |  |  |  |  |  |  | 73484 |  |  |
| Insurance |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.8 | P & I Club - Full Style: |  |  |  |  |  |  |  | SKULD SKULD -Mutual Protection & indemnity Association (Bermuda) Ltd. PO Box-1376 Vika, N-0114, OSLO , Norway |  |  |  |  |  |
| 8.9 | P & I Club pollution liability coverage / expiration date: |  |  |  |  |  |  |  |  |  |  | 1,000,000,000 US$ |  | Feb 20, 2016 |
| 8.10 | Hull & Machinery insured by - Full Style: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.11 | Hull & Machinery insured value / expiration date: |  |  |  |  |  |  |  |  |  |  | 87,990,000 US$ |  | Apr 15, 2016 |
| Recent Operational History |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.12 | Date and place of last Port State Control inspection: |  |  |  |  |  |  |  |  |  |  | Jul 10, 2015 / Augusta, Italy |  |  |
| 8.13 | Any outstanding deficiencies as reported by any Port State Control? If yes, provide details: |  |  |  |  |  |  |  |  |  |  | No None |  |  |
| 8.14 | Has vessel been involved in a pollution, grounding, serious casualty or collision incident during the past 12 months? If yes, full description: |  |  |  |  |  |  |  |  |  |  | Pollution: No, Grounding: No, Casualty: No, Collision: No, |  |  |
| 8.15 | Last three cargoes / charterers / voyages (Last / 2nd Last / 3rd Last): |  |  |  |  |  |  |  |  |  |  | 1st Last: Arab Light, Arab Medium / Bahri / Ras Tanura to Ain Sukhna 2nd Last: Arab Light, Arab Medium, Arab Heavy / Bahri / Ras Tanura to Ain Sukhna 3rd Last: Qua Iboe Crude Oil / IOC / Qua Iboe to Paradip |  |  |
| 8.16 | Date/place of last STS operation: |  |  |  |  |  |  |  |  |  |  | Bunkering off Port Gentil October 12, 2013 |  |  |
| Vetting |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.17 | Date of last SIRE inspection: |  |  |  |  |  |  |  |  |  |  | Oct 14, 2015 |  |  |
| 8.18 | Date of last CDI inspection: |  |  |  |  |  |  |  |  |  |  | N/A |  |  |
| 8.19 | Recent Oil company inspections/screenings (To the best of owners knowledge and without guarantee of acceptance for future business)*: * "Approvals" are not given by Oil Majors and ships are accepted for the voyage on a case by case basis. |  |  |  |  |  |  |  |  |  |  | PORT STATE, LUKOIL, SHELL, FLAG STATE, STATOIL, PHILLIPS66, CHEVRON, TESORO |  |  |
| Additional Information |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.20 | Additional information relating to features of the ship or operational characteristics: |  |  |  |  |  |  |  |  |  |  | None |  |  |

Rev 2015 (INTERTANKO / Q88.com)

Form completed on http://www.q88.com/integration.aspx  Please email support@q88.com an updated copy if this is not the latest version.