- INTERTANKO TANKER CHARTERING QUESTIONNAIRE 88 	Version 4

| 1. | VESSEL DESCRIPTION |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1.1 | Date updated: |  |  |  |  |  |  |  |  |  |  |  |  | 9 May 2016 |  |  |  |
| 1.2 | Vessel’s name (IMO number): |  |  |  |  |  |  |  |  |  |  |  |  | Cap Philippe / 9321718 |  |  |  |
| 1.3 | Vessel’s previous name(s) and date(s) of change: |  |  |  |  |  |  |  |  |  |  |  |  | N/A / N/A |  |  |  |
| 1.4 | Date delivered / Builder (where built): |  |  |  |  |  |  |  |  |  |  |  |  | 14 July 2006 / Yard: Samsung H.I. Place: Korea |  |  |  |
| 1.5 | Flag / Port of Registry: |  |  |  |  |  |  |  |  |  |  |  |  | GREECE Port of Registry: PIRAEUS |  |  |  |
| 1.6 | Call sign / MMSI: |  |  |  |  |  |  |  |  |  |  |  |  | SWLF / 240546000 |  |  |  |
| 1.7 | Vessel’s contact details (satcom/fax/email etc.): |  |  |  |  |  |  |  |  |  |  |  |  | Phone: 764608961 / 62 Fax: 764608963 Telex: 424054610 / 424054611 E-Mail: capphilippe@euronav.eu |  |  |  |
| 1.8 | Type of vessel (as described in Form A or Form B Q1.11 of the IOPPC): |  |  |  |  |  |  |  |  |  |  |  |  | Oil Tanker |  |  |  |
| 1.9 | Type of hull: |  |  |  |  |  |  |  |  |  |  |  |  | Double hull |  |  |  |
| Classification |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1.10 | Classification society: |  |  |  |  |  |  |  |  |  |  |  |  | American Bureau of Shipping |  |  |  |
| 1.11 | Class notation: |  |  |  |  |  |  |  |  |  |  |  |  | + A1(E), Oil Carries ESP, ICE CALSS 1C, +AMS, +ACCU, SH, VEC(-L), SH, SPM |  |  |  |
| 1.12 | Is the vessel subject to any conditions of class, class extensions, outstanding memorandums or class recommendations? If yes, give details: |  |  |  |  |  |  |  |  |  |  |  |  | No |  |  |  |
| 1.13 | If classification society changed, name of previous and date of change: |  |  |  |  |  |  |  |  |  |  |  |  | Previous Class: N/A Date: N/A |  |  |  |
| 1.14 | IMO type, if applicable: |  |  |  |  |  |  |  |  |  |  |  |  | Not Applicable. |  |  |  |
| 1.15 | Does the vessel have ice class? If yes, state what level: |  |  |  |  |  |  |  |  |  |  |  |  | Yes/ABS/Ice Class IC |  |  |  |
| 1.16 | Date / place of last dry-dock: |  |  |  |  |  |  |  |  |  |  |  |  | Date: 29-Jun-2011 Place: SETUBAL |  |  |  |
| 1.17 | Date next dry dock due / next annual survey due: |  |  |  |  |  |  |  |  |  |  |  |  | 13-Jul-2016 |  |  | 13/07/2016 |
| 1.18 | Date of last special survey / next special survey due: |  |  |  |  |  |  |  |  |  |  |  |  | 29-Jun-2011 |  |  | 13-Jul-2016 |
| 1.19 | If ship has Condition Assessment Program (CAP), what is the latest overall rating: |  |  |  |  |  |  |  |  |  |  |  |  | 0 |  |  |  |
| 1.20 | Does the vessel have a statement of compliance issued under the provisions of the Condition Assessment Scheme (CAS): If yes, what is the expiry date? |  |  |  |  |  |  |  |  |  |  |  |  | No |  |  |  |
| Dimensions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1.21 | Length overall (LOA): |  |  |  |  |  |  |  |  |  |  |  |  | 274.292 m |  |  |  |
| 1.22 | Length between perpendiculars (LBP): |  |  |  |  |  |  |  |  |  |  |  |  | 264.00 m |  |  |  |
| 1.23 | Extreme breadth (Beam): |  |  |  |  |  |  |  |  |  |  |  |  | 48.000 m |  |  |  |
| 1.24 | Moulded depth: |  |  |  |  |  |  |  |  |  |  |  |  | 23.20 m |  |  |  |
| 1.25 | Keel to masthead (KTM)/ Keel to masthead (KTM) in collapsed condition, if applicable: |  |  |  |  |  |  |  |  |  |  |  |  | 50.20 m |  |  | 50.02 |
| 1.26 | Bow to center manifold (BCM) / Stern to center manifold (SCM): |  |  |  |  |  |  |  |  |  |  |  |  | 137.73 m |  |  | 136.56 m |
| 1.27 | Distance bridge front to center of manifold: |  |  |  |  |  |  |  |  |  |  |  |  | 94.40 m |  |  |  |
| 1.28 | Parallel body distances |  |  |  |  |  |  | Lightship |  |  |  |  |  | Normal Ballast |  |  | Summer Dwt |
|  | # Forward to mid-point manifold: |  |  |  |  |  |  | 69.00 m |  |  |  |  |  | 77.00 m |  |  | 77.00 m |
|  | # Aft to mid-point manifold: |  |  |  |  |  |  | 29.00 m |  |  |  |  |  | 49.00 m |  |  | 70.00 m |
|  | # Parallel body length: |  |  |  |  |  |  | 98.00 m |  |  |  |  |  | 126.00 m |  |  | 147.00 m |
| # 1.29 | # FWA/TPC at summer draft: |  |  |  |  |  |  |  |  |  |  |  |  | 384 mm |  |  | 118.9 mt |
| 1.30 | Constant (excluding fresh water): |  |  |  |  |  |  |  |  |  |  |  |  | 300 |  |  |  |
| 1.31 | What is the company guidelines for Under Keel Clearance (UKC) for this vessel? |  |  |  |  |  |  |  |  |  |  |  |  | a) Ocean passages - 20% of the deepest draught; b) Fairways - 15% of the deepest draught; c) Alongside - 1.5% of the vessel beam or 0.3 meter whichever is the greater; d) Alongside an SBM - 10% of deepest draught; e) Rivers & Canals - 10% of the deepest draught; f) Shallow waters - 10% of the deepest draught; |  |  |  |
| 1.32 | What is the max height of mast above waterline (air draft) |  |  |  |  |  |  |  |  |  |  |  |  | Full Mast |  |  | Collapsed Mast |
|  | Lightship: |  |  |  |  |  |  |  |  |  |  |  |  | 45.77 m |  |  | 46.48 |
|  | Normal ballast: |  |  |  |  |  |  |  |  |  |  |  |  | 42.00 m |  |  | 42.02 |
|  | At loaded summer deadweight: |  |  |  |  |  |  |  |  |  |  |  |  | 36.92 |  |  | 36.92 |
| Tonnages |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1.33 | Net Tonnage: |  |  |  |  |  |  |  |  |  |  |  |  | 51,277 mt |  |  |  |
| 1.34 | Gross Tonnage / Reduced Gross Tonnage (if applicable): |  |  |  |  |  |  |  |  |  |  |  |  | 81,324 mt |  |  | 64376 mt |
| 1.35 | Suez Canal Tonnage - Gross (SCGT) / Net (SCNT): |  |  |  |  |  |  |  |  |  |  |  |  | 83,135.83 mt |  |  | 77,904.50 mt |
| # 1.36 | Panama Canal Net Tonnage (PCNT): |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |
| Ownership and Operation |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1.37 | Registered owner - Full style: |  |  |  |  |  |  |  | EURONAV N.V. Address: DE GERLACHE KAAI 20,2000 ANTWERPEN Phone:+ 32 3 247 4411 Fax:+32 3 247 44 80 E mail: operations@euronav.com |  |  |  |  |  |  |  |  |
| 1.38 | Technical operator - Full style: |  |  |  |  |  |  |  | EURONAV SHIP MANAGEMENT (HELLAS) LTD Address:69 AKTI MIAOULI STREET,185 37 PIRAEUS,GREECE Phone:+30 210 455 8000 Fax:+ 30 210 4558050 E mail: hsqe.smh@euronav.com |  |  |  |  |  |  |  |  |
| 1.39 | Commercial operator - Full style: |  |  |  |  |  |  |  | EURONAV N.V. Address:DE GERLACHE KAAI 20,2000 ANTWERPEN Phone:+ 32 3 247 4411 Fax:+32 3 247 44 80 E mail: operations@euronav.com |  |  |  |  |  |  |  |  |
| 1.40 | Disponent owner - Full style: |  |  |  |  |  |  |  | N/A Address: N/A Phone:N/A Fax:N/A E mail: N/A |  |  |  |  |  |  |  |  |
| 2. | CERTIFICATION |  |  |  |  |  |  | Issued |  |  |  |  |  | Last Annual |  |  | Expires |
| 2.1 | Safety Equipment Certificate (SEC): |  |  |  |  |  |  | 29-Jul-2011 |  |  |  |  |  | 13-Oct-2015 |  |  | 13-Jul-2016 |
| 2.2 | Safety Radio Certificate (SRC): |  |  |  |  |  |  | 29-Jul-2011 |  |  |  |  |  | 13-Oct-2015 |  |  | 13-Jul-2016 |
| 2.3 | Safety Construction Certificate (SCC): |  |  |  |  |  |  | 29-Jul-2011 |  |  |  |  |  | 13-Oct-2015 |  |  | 13-Jul-2016 |
| 2.4 | International Loadline Certificate (ILC): |  |  |  |  |  |  | 29-Jun-2011 |  |  |  |  |  | 10-May-2015 |  |  | 13-Jul-2016 |
| 2.5 | International Oil Pollution Prevention Certificate (IOPPC): |  |  |  |  |  |  | 29-Jun-2011 |  |  |  |  |  | 13-Oct-2015 |  |  | 13-Jul-2016 |
| 2.6 | ISM Safety Management Certificate (SMC): |  |  |  |  |  |  | 15-Dec-2011 |  |  |  |  |  | 04-Nov-2014 |  |  | 25-Dec-2016 |
| # 2.7 | # Document of Compliance (DOC): |  |  |  |  |  |  | # 19-Jan-2016 |  |  |  |  |  | N/A |  |  | 09-Mar-2021 |
| 2.8 | USCG Certificate of Compliance (COC): |  |  |  |  |  |  | 03/08/2016 |  |  |  |  |  | NA |  |  | 03/08/2018 |
| 2.9 | Civil Liability Convention (CLC) 1992 Certificate: |  |  |  |  |  |  | 20-Feb-2016 |  |  |  |  |  | Not Applicable |  |  | 20-Feb-2017 |
| 2.10 | Civil Liability for Bunker Oil Pollution Damage Convention (CLBC) Certificate: |  |  |  |  |  |  | 28-Jan-2015 |  |  |  |  |  | Not Applicable |  |  | 20-Feb-2017 |
| 2.11 | Ship Sanitation Control (SSCC)/Ship Sanitation Control Exemption (SSCE) Certificate: |  |  |  |  |  |  | 11/30/2015 |  |  |  |  |  | Not Applicable |  |  | 06/30/2016 |
| 2.12 | U.S. Certificate of Financial Responsibility (COFR): |  |  |  |  |  |  | 14-Jul-2015 |  |  |  |  |  | Not Applicable |  |  | 14-Jul-2018 |
| 2.13 | Certificate of Class (COC): |  |  |  |  |  |  | 08-Nov-2011 |  |  |  |  |  | 13/10/2015 |  |  | 13-Jul-2016 |
| # 2.14 | # International Sewage Pollution Prevention Certificate (ISPPC): |  |  |  |  |  |  | # 29-Jun-2011 |  |  |  |  |  | # N/A |  |  | 13-Jul-2016 |
| # 2.15 | # Certificate of Fitness (COF): |  |  |  |  |  |  | # NA |  |  |  |  |  | NA |  |  | NA |
| # 2.16 | # International Energy Efficiency Certificate (IEEC): |  |  |  |  |  |  | # 29/06/2011 |  |  |  |  |  | Not Applicable |  |  | Not Applicable |
| # 2.17 | # International Ship Security Certificate (ISSC): |  |  |  |  |  |  | # 15-Dec-2011 |  |  |  |  |  | # 04/11/2014 |  |  | 25-Dec-2016 |
| # 2.18 | # International Air Pollution Prevention Certificate (IAPPC): |  |  |  |  |  |  | # 27-Jun-2011 |  |  |  |  |  | 13-Oct-2015 |  |  | 27-Jun-2016 |
| # 2.19 | # Maritime Labour Certificate (MLC): |  |  |  |  |  |  | # 21-Jan-2014 |  |  |  |  |  | N/A |  |  | 20-Jan-2019 |
| Documentation |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2.20 | Owner warrant that vessel is member of ITOPF and will remain so for the entire duration of this voyage/contract: |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |
| 2.21 | Does vessel have in place a Drug and Alcohol Policy complying with OCIMF guidelines for Control of Drugs and Alcohol Onboard Ship? |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |
| 2.22 | Is the ITF Special Agreement on board (if applicable)? |  |  |  |  |  |  |  |  |  |  |  |  | No |  |  |  |
| 2.23 | ITF Blue Card expiry date: |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |
| 3. | CREW |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3.1 | Nationality of Master: |  |  |  |  |  |  |  |  |  |  |  |  | Greek |  |  |  |
| 3.2 | Number and Nationality of Officers: |  |  |  |  |  |  |  |  |  |  |  |  | Nbr of Officers: 9 Nationalities: Greek-Indian-Romanian-Ukrainian |  |  |  |
| 3.3 | Number and Nationality of Crew: |  |  |  |  |  |  |  |  |  |  |  |  | Nbr of Crew: 15 Nationalities: PHIL, SALV, HOND, PAN |  |  |  |
| 3.4 | What is the common working language onboard: |  |  |  |  |  |  |  |  |  |  |  |  | ENGLISH |  |  |  |
| 3.5 | Do officers speak and understand English? |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |
| 3.6 | If Officers/Crew employed by a Manning Agency - Full style: |  |  |  |  |  |  |  | Officers: UNISEA PHILIPPINE INC Address: 2F AND 3F PHILCOX BUILDING,172 SALCEDO STREET, KEGAPSI VILLAGE ,MAKATI CITY 1229 PHILIPPINES Phone: +6328940851-55 Fax: +6328934240 E mail: unisea@unisea.com.ph Crew: EURONAV CENTRAL AMERICA RATINGS Address: San Salvador,El Salvador Centro Financiero Gigante 63 Avenida Sur y Alameda Franklin Roosevelt Torre "B", nivel 8, Local 2 San Salvador, El Salvador Phone: + 503 2223 5185 VoIP: 8105 Mobile: + 503 7862 5511 Puerto Cortes, Honduras office Buenos Aires 15 calle entre 3rd y 4rd avenida Edificio Beige, Segundo nivel, Local 3 Puerto Cortes, Honduras Phone: +504 2630-5448 VOIP: 8107 Mobile: +504 3188-4811 Phone: 503 2223 5185 / +504 2630-5448 / Fax: 30 210 455 8040 E Mail: crew.sv@euronav.com, Crew.hn@euronav.com, |  |  |  |  |  |  |  |  |
| 4. | FOR USA CALLS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4.1 | Has the vessel Operator submitted a Vessel Spill Response Plan to the US Coast Guard which has been approved by official USCG letter? |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |
| 4.2 | Qualified individual (QI) - Full style: |  |  |  |  |  |  |  | Gallagher Marine Services INC., QIS 100 Century Parkway,Suite 130 Mt. Laurel, NJ 08054 Phone:+1 856 642 2091 Fax: +1 856 642 3945 E mail: gmsi@chgms.com / info@chgms.com 24 hrs emergency number: 24h: +1 703 683 4700 Alt 24h: +1 215 492 5473 |  |  |  |  |  |  |  |  |
| 4.3 | Oil Spill Response Organization (OSRO) - Full style: |  |  |  |  |  |  |  | MARINE SPILL RESPONSE CORPORATION. (MSRC) Phone: + 1 703 326 5600 Fax: +1 703 326 5660 E mail: notifications@msrc.org 24 hrs Emergency number: +1 732 417 0175 |  |  |  |  |  |  |  |  |
| 5. | CARGO AND BALLAST HANDLING |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Double Hull Vessels |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.1 | Is vessel fitted with centerline bulkhead in all cargo tanks? If Yes, solid or perforated: |  |  |  |  |  |  |  |  |  |  |  |  | Yes / Solid |  |  |  |
| # Loadline Information |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| # 5.2 | # Loadline |  |  | # Freeboard |  |  |  | # Draft |  |  |  |  |  | # Deadweight |  |  | # Displacement |
|  | # Summer: |  |  | 6.970 m |  |  |  | 17.024 m |  |  |  |  |  | # 158,920 mt |  |  | 182,680.00 mt |
|  | # Winter: |  |  | 7.300 m |  |  |  | 16.670 m |  |  |  |  |  | 154721.4 mt |  |  | 178481 mt |
|  | # Tropical: |  |  | 6.630 m |  |  |  | 17.378 m |  |  |  |  |  | 163139.1 mt |  |  | 186898.7 mt |
|  | # Lightship: |  |  | 20.490 m |  |  |  | 2.750 m |  |  |  |  |  | Not Applicable |  |  | 23759.6 mt |
|  | # Normal Ballast Condition: |  |  | 15.580 m |  |  |  | 7.660 m |  |  |  |  |  | 51347.4 mt |  |  | 75107 mt |
| # 5.3 | # Does vessel have multiple SDWT? If yes, please provide all assigned loadlines: |  |  |  |  |  |  |  |  |  |  |  |  | 158920-149999-124999 |  |  |  |
| Cargo Tank Capacities |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.4 | Number of cargo tanks and total cubic capacity (98%): |  |  |  |  |  |  |  |  |  |  |  |  | 14 |  |  | 176724 m3 |
| 5.5 | Capacity (98%) of each natural segregation with double valve (specify tanks): |  |  |  |  |  |  |  |  |  |  |  |  | - 54,894.6 m3 - 58,221.4 m3 - 58,876.0 m3 Tanks - 1,4 P+S ,Slp S - 2,5 P+S - 3,5 P+S Slop P |  |  |  |
| 5.6 | Number of slop tanks and total cubic capacity (98%): |  |  |  |  |  |  |  |  |  |  |  |  | 2 |  |  | 4,730 m3 |
| 5.7 | Specify segregations which slops tanks belong to and their capacity with double valve: |  |  |  |  |  |  |  |  |  |  |  |  | As per 5.5 |  |  |  |
| 5.8 | Residual/Retention oil tank(s) capacity (98%), if applicable: |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |
| 5.9 | Does vessel have Segregated Ballast Tanks (SBT) or Clean Ballast Tanks (CBT): |  |  |  |  |  |  |  |  |  |  |  |  | SBT |  |  |  |
| SBT Vessels |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.10 | What is total SBT capacity and percentage of SDWT vessel can maintain? |  |  |  |  |  |  |  |  |  |  |  |  | 54619 m3 |  |  | 34.70% |
| 5.11 | Does vessel meet the requirements of MARPOL Annex I Reg 18.2: |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |
| Cargo Handling and Pumping Systems |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.12 | How many grades/products can vessel load/discharge with double valve segregation: |  |  |  |  |  |  |  |  |  |  |  |  | 3 |  |  |  |
| 5.13 | Are there any cargo tank filling restrictions? If yes, specify number of slack tanks, max s.g., ullage restrictions etc.: |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |
| 5.14 | Pumps |  |  |  |  | No. |  | Type |  |  |  |  |  | Capacity |  |  | At What Head (sg=1.0) |
|  | Cargo Pumps: |  |  |  |  | 3 |  | Cenrifugal |  |  |  |  |  | 4000 m3/hr |  |  | 135 m |
|  | Cargo Eductors: |  |  |  |  | 1 |  | Venturi |  |  |  |  |  | 700 m3/hr |  |  | 30 m |
|  | Stripping: |  |  |  |  | 1 |  | Reciprocating |  |  |  |  |  | 300 m3/hr |  |  | 135 m |
|  | Ballast Pumps: |  |  |  |  | 2 |  | CETRIFUGAL |  |  |  |  |  | 2500 m3/hr |  |  | 30 m |
|  | Ballast Eductors: |  |  |  |  | 2 |  | eductor |  |  |  |  |  | 700 m3/hr |  |  | 30 m |
| 5.15 | Max loading rate for homogenous cargo per manifold connection: |  |  |  |  |  |  |  |  |  |  |  |  | 5600 |  |  |  |
| 5.16 | Max loading rate for homogenous cargo loaded simultaneously through all manifolds: |  |  |  |  |  |  |  |  |  |  |  |  | 16800 m3/hr |  |  |  |
| 5.17 | How many cargo pumps can be run simultaneously at full capacity: |  |  |  |  |  |  |  |  |  |  |  |  | 3 |  |  |  |
| Cargo Control Room |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.18 | Is ship fitted with a Cargo Control Room (CCR)? |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |
| 5.19 | Can tank innage / ullage be read from the CCR? |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |
| Gauging and Sampling |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.20 | Can cargo be transferred under closed loading conditions in accordance with ISGOTT 11.1.6.6? |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |
| 5.21 | What type of fixed closed tank gauging system is fitted: |  |  |  |  |  |  |  |  |  |  |  |  | Radar |  |  |  |
| 5.22 | Number of portable gauging units (example- MMC) on board: |  |  |  |  |  |  |  |  |  |  |  |  | 3 |  |  |  |
| 5.23 | Are overfill (high) alarms fitted? If Yes, indicate whether to all tanks or partial: |  |  |  |  |  |  |  |  |  |  |  |  | Yes/ All |  |  |  |
| 5.24 | Are cargo tanks fitted with multipoint gauging? If yes, specify type and locations: |  |  |  |  |  |  |  |  |  |  |  |  | Yes / ENRAF TANKSYSTEM SA/ mid tank |  |  |  |
| 5.25 | Is gauging system certified and calibrated? If no, specify which ones are not calibrated: |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |
| Vapor Emission Control System (VECS) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.26 | Is a Vapour Emission Control System (VECS) fitted? |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |
| 5.27 | Number/size of VECS manifolds (per side): |  |  |  |  |  |  |  |  |  |  |  |  | 3 |  |  | 16" |
| 5.28 | Number / size / type of VECS reducers: |  |  |  |  |  |  |  |  |  |  |  |  | 1x(16"x10"), 1x(16"x12") |  |  |  |
| Venting |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.29 | State what type of venting system is fitted: |  |  |  |  |  |  |  |  |  |  |  |  | Free flow vent riser |  |  |  |
| Cargo Manifolds and Reducers |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.30 | Does vessel comply with the latest edition of the OCIMF 'Recommendations for Oil Tanker Manifolds and Associated Equipment’? |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |
| 5.31 | Total number / size of cargo manifold connections on each side: |  |  |  |  |  |  |  |  |  |  |  |  | 3 / 406 mm |  |  |  |
| 5.32 | What type of valves are fitted at manifold: |  |  |  |  |  |  |  |  |  |  |  |  | 406 mm |  |  |  |
| 5.33 | What is the material/rating of the manifold: |  |  |  |  |  |  |  |  |  |  |  |  | Arc welded carbon steel STPY 400 - ANSI B16.5 150 LB / 166 |  |  |  |
| 5.34 | Does the vessel have a Common Line Manifold connection? If yes, describe: |  |  |  |  |  |  |  |  |  |  |  |  | yes |  |  |  |
| 5.35 | Distance between cargo manifold centers: |  |  |  |  |  |  |  |  |  |  |  |  | 2500 mm |  |  |  |
| 5.36 | Distance ships rail to manifold: |  |  |  |  |  |  |  |  |  |  |  |  | 4600 mm |  |  |  |
| 5.37 | Distance manifold to ships side: |  |  |  |  |  |  |  |  |  |  |  |  | 4600 mm |  |  |  |
| 5.38 | Top of rail to center of manifold: |  |  |  |  |  |  |  |  |  |  |  |  | 1400 mm |  |  |  |
| 5.39 | Distance main deck to center of manifold: |  |  |  |  |  |  |  |  |  |  |  |  | 2100 mm |  |  |  |
| 5.40 | Spill tank grating to center of manifold: |  |  |  |  |  |  |  |  |  |  |  |  | 920 mm |  |  |  |
| 5.41 | Manifold height above the waterline in normal ballast / at SDWT condition: |  |  |  |  |  |  |  |  |  |  |  |  | 17.68 m |  |  | 8.32 m |
| 5.42 | Number / size / type of reducers: |  |  |  |  |  |  |  |  |  |  |  |  | Number:5 Size:400 mm x 300 mm Type: ANSI Number:3 Size:400 mm x 200 mm Number:4 Size: 400 mm x 250 mm Number:3 Size:200 mm x 100 mm Number:1 Size:200 mm x 300 mm Number: N/A Size: N/A x N/A |  |  |  |
| 5.43 | Is vessel fitted with a stern manifold? If yes, state size: |  |  |  |  |  |  |  |  |  |  |  |  | No |  |  |  |
| Heating |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.44 | Cargo / slop tanks fitted with a cargo heating system? |  |  |  |  |  |  |  |  | Type |  |  |  | Coiled |  |  | Material |
|  | Cargo Tanks: |  |  |  |  |  |  |  |  | Heating coils |  |  |  | Yes |  |  | Other |
|  | Slop Tanks: |  |  |  |  |  |  |  |  | steam |  |  |  | Yes |  |  | Other |
| 5.45 | Maximum temperature cargo can be loaded / maintained: |  |  |  |  |  |  |  |  |  |  |  |  | 70°C |  |  | 60°C |
| 5.46 | Minimum temperature cargo can be loaded / maintained: |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  | N/A |
| Coating / Anodes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.47 | Tank Coating |  |  |  | Coated |  |  | Type |  |  |  |  |  | To What Extent |  |  | Anodes |
|  | Cargo tanks: |  |  |  | Yes |  |  | Two component multipourpose anticorrosive epoxy system, Sigmaprime K |  |  |  |  |  | Tank top and 1.7 metres down parallel to upper deck line. Bottom and 1.5 metres up. Slop tanks fully coated. |  |  | No |
|  | Ballast tanks: |  |  |  | Yes |  |  | Two component multipourpose anticorrosive epoxy system, Sigmaprime K |  |  |  |  |  | Fullly coated |  |  | Yes |
|  | Slop tanks: |  |  |  | Yes |  |  | Sigma prime series red brown |  |  |  |  |  | Full |  |  | No |
| 6. | INERT GAS AND CRUDE OIL WASHING |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6.1 | Is a Crude Oil Washing (COW) installation fitted / operational? |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |
| 6.2 | Is an Inert Gas System (IGS) fitted / operational? |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |
| 6.3 | Is IGS supplied by flue gas, inert gas (IG) generator and/or nitrogen: |  |  |  |  |  |  |  |  |  |  |  |  | Flue Gas |  |  |  |
| 7. | MOORING |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7.1 | Wires (on drums) |  | No. |  | Diameter |  |  | Material |  |  |  |  |  | Length |  |  | Breaking Strength |
|  | Forecastle: |  | 4 |  | 38 mm |  |  | IWRC 6X37 |  |  |  |  |  | 280 m |  |  | 86 mt |
|  | Main deck fwd: |  | <4 |  | 38 mm |  |  | IWRC 6X37 |  |  |  |  |  | 280 m |  |  | 86 mt |
|  | Main deck aft: |  | 4 |  | 38 mm |  |  | IWRC 6X37 |  |  |  |  |  | 280 m |  |  | 86 mt |
|  | Poop deck: |  | 6 |  | 38 mm |  |  | IWRC 6X37 |  |  |  |  |  | 280 m |  |  | 86 mt |
| 7.2 | Wire tails |  | No. |  | Diameter |  |  | Material |  |  |  |  |  | Length |  |  | Breaking Strength |
|  | Forecastle: |  | 4 |  | 80 mm |  |  | Nylon |  |  |  |  |  | 11 m |  |  | 135 mt |
|  | Main deck fwd: |  | 4 |  | 80 mm |  |  | Nylon |  |  |  |  |  | 11 m |  |  | 135 mt |
|  | Main deck aft: |  | 4 |  | 80 mm |  |  | Nylon |  |  |  |  |  | 11 m |  |  | 135 mt |
|  | Poop deck: |  | 6 |  | 80 mm |  |  | Nylon |  |  |  |  |  | 11 m |  |  | 135 mt |
| 7.3 | Ropes (on drums) |  | No. |  | Diameter |  |  | Material |  |  |  |  |  | Length |  |  | Breaking Strength |
|  | Forecastle: |  | N/A |  | 0 mm |  |  | 0 |  |  |  |  |  | 0 m |  |  | 0 mt |
|  | Main deck fwd: |  | 0 |  | 0 mm |  |  | 0 |  |  |  |  |  | 0 m |  |  | 0 mt |
|  | Main deck aft: |  | 0 |  | 0 mm |  |  | 0 |  |  |  |  |  | 0 m |  |  | 0 mt |
|  | Poop deck: |  | 0 |  | 0 mm |  |  | 0 |  |  |  |  |  | 0 m |  |  | 0 mt |
| 7.4 | Other lines |  | No. |  | Diameter |  |  | Material |  |  |  |  |  | Length |  |  | Breaking Strength |
|  | Forecastle: |  | 2 |  | 72 mm |  |  | 8-STRAND D-FLEX ROPE |  |  |  |  |  | 220 m |  |  | 94 mt |
|  | Main deck fwd: |  | 0 |  | 0 mm |  |  | D-FLEX |  |  |  |  |  | 0 m |  |  | 0 mt |
|  | Main deck aft: |  | 2 |  | 72 mm |  |  | 8-STRAND D-FLEX ROPE |  |  |  |  |  | 220 m |  |  | 94 mt |
|  | Poop deck: |  | 2 |  | 72 mm |  |  | 8-STRAND D-FLEX ROPE |  |  |  |  |  | 220 m |  |  | 94 mt |
| 7.5 | Winches |  | No. |  | No. Drums |  |  | Motive Power |  |  |  |  |  | Brake Capacity |  |  | Type of Brake |
|  | Forecastle: |  | 2 |  | double drum |  |  | hyrdaulic |  |  |  |  |  | 69 mt |  |  | Manual with band |
|  | Main deck fwd: |  | 2 |  | double drum |  |  | Hydraulic |  |  |  |  |  | 69 mt |  |  | Manual with band |
|  | Main deck aft: |  | 2 |  | double drum |  |  | Hydraulic |  |  |  |  |  | 69 mt |  |  | Manual with band |
|  | Poop deck: |  | 3 |  | double drum |  |  | hydraulic |  |  |  |  |  | 69 mt |  |  | Manual with band |
| 7.6 | Bitts, closed chocks/fairleads |  |  |  | No. Bitts |  |  | SWL Bitts |  |  |  |  |  | No. Closed Chocks |  |  | SWL Closed Chocks |
|  | Forecastle: |  |  |  | 4 |  |  | 69 mt |  |  |  |  |  | 18 |  |  | 200-69 |
|  | Main deck fwd: |  |  |  | 4 |  |  | 69 mt |  |  |  |  |  | 7 |  |  | 92-86-69 |
|  | Main deck aft: |  |  |  | 4 |  |  | 69 mt |  |  |  |  |  | 12 |  |  | 92-86-69 |
|  | Poop deck: |  |  |  | 6 |  |  | 69.0 mt |  |  |  |  |  | 16 |  |  | 200-69 |
| Anchors/Emergency Towing System |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7.7 | Number of shackles on port / starboard cable: |  |  |  |  |  |  |  |  |  |  |  |  | 14/ 13 |  |  |  |
| 7.8 | Type / SWL of Emergency Towing system forward: |  |  |  |  |  |  |  |  |  |  |  |  | TANK TECH KETA-45F |  |  | 200 mt |
| 7.9 | Type / SWL of Emergency Towing system aft: |  |  |  |  |  |  |  |  |  |  |  |  | TANK TECH KETASP-40A |  |  | 200 mt |
| Escort Tug |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7.10 | What is size / SWL of closed chock and/or fairleads of enclosed type on stern: |  |  |  |  |  |  |  |  |  |  |  |  | 600 x 450 mm |  |  | 200 mt |
| 7.11 | What is SWL of bollard on poop deck suitable for escort tug: |  |  |  |  |  |  |  |  |  |  |  |  | 200 mt |  |  |  |
| Bow/Stern Thruster |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7.12 | What is brake horse power of bow thruster (if fitted): |  |  |  |  |  |  |  |  |  |  |  |  | NA |  |  |  |
| 7.13 | What is brake horse power of stern thruster (if fitted): |  |  |  |  |  |  |  |  |  |  |  |  | NA |  |  |  |
| Single Point Mooring (SPM) Equipment |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7.14 | Does the vessel meet the recommendations in the latest edition of OCIMF 'Recommendations for Equipment Employed in the Bow Mooring of Conventional Tankers at Single Point Moorings (SPM)’? |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |
| 7.15 | If fitted, how many chain stoppers: |  |  |  |  |  |  |  |  |  |  |  |  | 2 |  |  |  |
| 7.16 | State type / SWL of chain stopper(s): |  |  |  |  |  |  |  |  |  |  |  |  | Tongue Type |  |  | 200 mt |
| 7.17 | What is the maximum size chain diameter the bow stopper(s) can handle: |  |  |  |  |  |  |  |  |  |  |  |  | 97 mm |  |  |  |
| 7.18 | Distance between the bow fairlead and chain stopper/bracket: |  |  |  |  |  |  |  |  |  |  |  |  | 3 mm |  |  |  |
| 7.19 | Is bow chock and/or fairlead of enclosed type of OCIMF recommended size (600mm x 450mm)? If not, give details of size: |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |
| Lifting Equipment |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7.20 | Derrick / Crane description (Number, SWL and location): |  |  |  |  |  |  |  |  |  |  |  |  | Cranes. 2 / 15 mt Location: Midship section near cargo manifolds. |  |  |  |
| 7.21 | What is maximum outreach of cranes / derricks outboard of the ship’s side: |  |  |  |  |  |  |  |  |  |  |  |  | 7.5 m |  |  |  |
| Ship To Ship Transfer (STS) / Helicopter Operations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7.22 | Does vessel comply with recommendations contained in OCIMF/ICS Ship To Ship Transfer Guide (Petroleum, Chemicals or Liquified Gas, as applicable)? |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |
| 7.23 | Can the ship comply with the ICS Helicopter Guidelines? If Yes, state whether winching or landing area provided and diameter of the circle provided: |  |  |  |  |  |  |  |  |  |  |  |  | Yes / Landing / 13 m |  |  |  |
| 8. | MISCELLANEOUS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Engine |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.1 | Speed |  |  |  |  |  |  |  |  |  |  |  |  | Maximum |  |  | Economic |
|  | Ballast speed: |  |  |  |  |  |  |  |  |  |  |  |  | 15.5 |  |  | 7.5 |
|  | Laden speed: |  |  |  |  |  |  |  |  |  |  |  |  | 15.0 |  |  | 13.3 |
| 8.2 | What type of fuel is used for main propulsion / generating plant: |  |  |  |  |  |  |  |  |  |  |  |  | HFO 380 cst |  |  | HFO380 cst |
| 8.3 | Type / Capacity of bunker tanks: |  |  |  |  |  |  |  |  |  |  |  |  | IFO: 4248 M3 MGO: 239 |  |  |  |
| 8.4 | Is vessel fitted with fixed or controllable pitch propeller(s): |  |  |  |  |  |  |  |  |  |  |  |  | Fixed |  |  |  |
| 8.5 | Engines |  |  |  |  |  |  |  |  |  |  | No |  | Capacity |  |  | Make/Type |
|  | Main engine: |  |  |  |  |  |  |  |  |  |  | 1 |  | 22920 |  |  | B&W 6S70MC-C |
|  | Aux engine: |  |  |  |  |  |  |  |  |  |  | 3 |  | 950 |  |  | Yanmar 8N21L-GV |
|  | Power packs: |  |  |  |  |  |  |  |  |  |  | NA |  | NA |  |  | NA |
|  | Boilers: |  |  |  |  |  |  |  |  |  |  | 2 |  | 70000 |  |  | MHI / Mac 35B |
| Emissions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.6 | Main engine IMO NOx emission standard: |  |  |  |  |  |  |  |  |  |  |  |  | Tier I |  |  |  |
| 8.7 | Energy Efficiency Design Index (EEDI) rating number: |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |
| Insurance |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.8 | P & I Club - Full Style: |  |  |  |  |  |  |  |  |  | Gard As Kittelsbuktveien 31, NO-4836 Arendal Norway |  |  |  |  |  |  |
| 8.9 | P & I Club pollution liability coverage / expiration date: |  |  |  |  |  |  |  |  |  |  |  |  | 1,000,000,000 |  |  | 20/02/2017 |
| 8.10 | Hull & Machinery insured by - Full Style: |  |  |  |  |  |  |  |  |  | belgibo |  |  |  |  |  |  |
| 8.11 | Hull & Machinery insured value / expiration date: |  |  |  |  |  |  |  |  |  |  |  |  | US$55,000,000.00 |  |  | 31/12/2016 |
| Recent Operational History |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.12 | Date and place of last Port State Control inspection: |  |  |  |  |  |  |  |  |  |  |  |  | 14-Mar-2016 / Covenas, Colombia |  |  |  |
| 8.13 | Any outstanding deficiencies as reported by any Port State Control? If yes, provide details: |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |
| 8.14 | Has vessel been involved in a pollution, grounding, serious casualty or collision incident during the past 12 months? If yes, full description: |  |  |  |  |  |  |  |  |  |  |  |  | No |  |  |  |
| 8.15 | Last | Cargo |  |  |  |  | Charterer |  |  |  |  |  | Load Port |  |  | Disport |  |
|  | 1st | Castilla C.O. / Vasconia C.O. |  |  |  |  | Phillips 66 |  |  |  |  |  | Covenas |  |  | Long Beach |  |
|  | 2nd | Fuel Oil |  |  |  |  | KOCH |  |  |  |  |  | Ust Luga |  |  | St Eustatius- Yabucoa |  |
|  | 3rd | Arabian Light C.O. |  |  |  |  | TOTAL/CSSA |  |  |  |  |  | Ras Tanura |  |  | Le Havre |  |
| 8.16 | Date/place of last STS operation: |  |  |  |  |  |  |  |  |  |  |  |  | 22 Oct 2015 //PAL |  |  |  |
| Vetting |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.17 | Date of last SIRE inspection: |  |  |  |  |  |  |  |  |  |  |  |  | 23-Jan-2016/ Le Havre, France |  |  |  |
| 8.18 | Date of last CDI inspection: |  |  |  |  |  |  |  |  |  |  |  |  | NA |  |  |  |
| 8.19 | Recent Oil company inspections/screenings (To the best of owners knowledge and without guarantee of acceptance for future business)*: * "Approvals" are not given by Oil Majors and ships are accepted for the voyage on a case by case basis. |  |  |  |  |  |  |  |  |  |  |  |  | Total, Repsol |  |  |  |
| Additional Information |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.20 | Additional information relating to features of the ship or operational characteristics: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Rev 2015 (INTERTANKO / Q88.com)