| 1. | VESSEL DESCRIPTION |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 1.1 | Date updated: |  |  | 12TH APR. 2016 |  |
| 1.2 | Vessel’s name: |  |  | HANKUK CHEMI |  |
| 1.3 | IMO number: |  |  | 9232369 |  |
| 1.4 | Vessel’s previous name(s) and date(s) of change: |  |  | CHEMTRANS MABUHAY (Aug 25, 2015) CHEMICAL SUPPLIER (Dec 15, 2010) GOLDEN TOMO (Oct 09, 2003) |  |
| 1.5 | Date delivered: |  |  | Nov 28, 2000 |  |
| 1.6 | Builder (where built): |  |  | FUKUOKA SHIPBUILDING CO LTD., Japan |  |
| 1.7 | Flag: |  |  | KOREA |  |
| 1.8 | Port of Registry: |  |  | JEJU |  |
| 1.9 | Call sign: |  |  | D7BY |  |
| 1.10 | Vessel’s sat com phone number: |  |  | 870 773111149 |  |
|  | Vessel’s fax number: |  |  | 870 783112431 |  |
|  | Vessel’s telex number: |  |  |  |  |
|  | Vessel’s email address: |  |  | hankukchemi@sea-one.com |  |
| 1.11 | Type of vessel: |  |  | Chemical |  |
| 1.12 | Type of hull: |  |  | Double Hull |  |
| Classification |  |  |  |  |  |
| 1.13 | Classification society: |  |  | Korea Register |  |
| 1.14 | Class notation: |  |  | KRS1 - Oil/Chemical Tanker(double hull) *E8P(FBC) product II &III 2G/1.50SG(IBC) ERS, ENV(IAFS,IOPP,ISPP,IAPP,IEE) CHA LI |  |
| 1.15 | If Classification society changed, name of previous society: |  |  | American Bureau of Shipping |  |
| 1.16 | If Classification society changed, date of change: |  |  | 28th Aug. 2015 |  |
| 1.17 | IMO type, if applicable: |  |  | II |  |
| 1.18 | Does the vessel have ice class? If yes, state what level: |  |  | NA |  |
| 1.19 | Date / place of last dry-dock: |  |  | 28th Aug 2015 | Yeosu, Korea |
| 1.20 | Date next dry dock due |  |  | 28th Aug 2018 |  |
| 1.21 | Date of last special survey / next survey due: |  |  | 28th Aug 2015 | 27th Aug 2020 |
| 1.22 | Date of last annual survey: |  |  | Nov 28, 2014 |  |
| 1.23 | If ship has Condition Assessment Program (CAP), what is the latest overall rating: |  |  | N/A |  |
| 1.24 | Does the vessel have a statement of compliance issued under the provisions of the Condition Assessment Scheme (CAS): If yes, what is the expiry date? |  |  | N/A |  |
| Dimensions |  |  |  |  |  |
| 1.25 | Length Over All (LOA): |  |  | 139.70 m |  |
| 1.26 | Length Between Perpendiculars (LBP): |  |  | 132.80 m |  |
| 1.27 | Extreme breadth (Beam): |  |  | 22.83 m |  |
| 1.28 | Moulded depth: |  |  | 12.00 m |  |
| 1.29 | Keel to Masthead (KTM) / KTM in collapsed condition (if applicable): |  |  | 34.83 m | 34.826 m |
| 1.30 | Bow to Center Manifold (BCM) / Stern to Center Manifold (SCM): |  |  | 68.38 m | 71.33 m |
| 1.31 | Distance bridge front to center of manifold: |  |  | 44.58 m |  |
| 1.32 | Parallel body distances: |  | Lightship | Normal ballast | Summer load |
|  | # Forward to mid-point manifold: |  | 24.70 m | 29.18 m | 32.73 m |
|  | # Aft to mid-point manifold: |  | 19.25 m | 27.83 m | 35.13 m |
|  | # Parallel body length: |  | 43.95 m | 57.00 m | 67.85 m |
| # 1.33 | # FWA at summer draft / TPC immersion at summer draft: |  |  | 201.00 mm | 27.50 MT |
| 1.34 | What is the max height of mast above waterline (air draft) |  |  | Full Mast | Collapsed Mast |
|  | Lightship: |  |  | 32.56 m | 32.556 m |
|  | Normal ballast: |  |  | 29.51 m | 29.516 m |
|  | At loaded summer deadweight: |  |  | 25.61 m | 25.606 m |
| Tonnages |  |  |  |  |  |
| 1.35 | Net Tonnage: |  |  | 5339.00 |  |
| 1.36 | Gross Tonnage / Reduced Gross Tonnage (if applicable): |  |  | 9797.00 |  |
| 1.37 | Suez Canal Tonnage – Gross (SCGT) / Net (SCNT): |  |  | 10275.19 | 8890.18 |
| # 1.38 | Panama Canal Net Tonnage (PCNT): |  |  | 8263.00 |  |
| # Loadline Information |  |  |  |  |  |
| # 1.39 | # Loadline | # Freeboard | # Draft | # Deadweight | # Displacement |
|  | # Summer: | 2.81 m | 9.22 m | 17427.00 MT | 22183.76 MT |
|  | # Winter: | 3.01 m | 9.02 m | 16899.93 MT | 21657.01 MT |
|  | # Tropical: | 2.62 m | 9.41 m | 17955.81 MT | 22712.89 MT |
|  | # Lightship: | 9.76 m | 2.27 m |  | 4757.08 MT |
|  | # Normal Ballast Condition: | 4.72 m | 5.31 m | 7278.34 MT | 12035.42 MT |
| # 1.40 | # Does vessel have multiple SDWT? |  |  | Yes |  |
| 1.41 | # If yes, what is the maximum assigned deadweight? |  |  | 17427.00 MT |  |
| Ownership and Operation |  |  |  |  |  |
| 1.42 | Registered owner - Full style: |  |  | KT CAPITAL CO.,LTD 12F, Golden Tower, 551, Samsung-ro, Gangnam-gu, Seoul,Korea |  |
| 1.43 | Technical operator - Full style: |  |  | EASTERN TANKER CO., LTD (Choryang-dong, MYONGJIN B/D), 9F, 7-2, Jungang-daero 320beon-gil, Dong-gu, Busan, Korea (601-836) T:+82-51-463-0866 /F:+82-51-463-0867 E-MAIL: est@easterntanker.com |  |
| 1.44 | Commercial operator - Full style: |  |  | DAELIM CORPORATION KCCI B/D, 11th Floor 45-4ga, Mandaemun-ro, Jung-gu, Seoul, Korea T:+82-2-3708-3325 /F:+82-2-753-2527 E-MAIL: chemical@daelimcorp.co.kr |  |
| 1.45 | Deponent owner - Full style: |  |  | DM SHIPPING CO.,LTD (U-dong, Byuksan e Centumclassone 2- cha), 1211, 71, Centumdong-ro, Haeundae-gu, Busan, South Korea. T:+82-51-413-7740/F:+82-51-413-7742 E-MAIL:dm@dmshipping.co.kr |  |
| 2. | CERTIFICATION |  | Issued | Last Annual or Intermediate | Expires |
| 2.1 | Safety Equipment Certificate: |  | 23rd Sep 2015 | N/A | 27th Aug 2020 |
| 2.2 | Safety Radio Certificate: |  | 28th Aug 2015 | N/A | 27th Aug 2020 |
| 2.3 | Safety Construction Certificate: |  | 28th Aug 2015 | N/A | 27th Aug 2020 |
| 2.4 | Load line Certificate: |  | 23rd Sep 2015 | N/A | 27th Aug 2020 |
| 2.5 | International Oil Pollution Prevention Certificate (IOPPC): |  | 20th Oct 2015 | N/A | 27th Aug 2020 |
| 2.6 | Safety Management Certificate (SMC): |  | 21st Dec 2015 | N/A | 20th Dec 2020 |
| # 2.7 | # Document of Compliance (DOC): |  | # 27th May 2015 | 05th Oct 2015 | 23rd Jul 2017 |
| 2.8 | USCG (specify: COC, LOC or COI): |  | N/A | N/A | N/A |
| 2.9 | Civil Liability Convention Certificate (CLC): |  | 26th Jan 2016 |  | 20th Feb 2017 |
| 2.10 | Civil Liability for Bunker Oil Pollution Damage Convention Certificate (CLBC): |  | 27th Jan 2016 |  | 20th Feb 2017 |
| 2.11 | U.S. Certificate of Financial Responsibility (COFR): |  | N/A | N/A | N/A |
| # 2.12 | # Certificate of Fitness (Chemicals): |  | 20th Oct 2015 | N/A | 27th Aug 2020 |
| 2.13 | Certificate of Fitness (Gas): |  | N/A | N/A | N/A |
| 2.14 | Certificate of Class: |  | 27th Jan 2016 | N/A | 27th Aug 2020 |
| # 2.15 | # International Ship Security Certificate (ISSC): |  | 04th Jan 2016 | N/A | 22nd Dec 2020 |
| # 2.16 | # International Sewage Pollution Prevention Certificate (ISPPC) |  | 28th Aug 2015 | N/A | 27th Aug 2020 |
| # 2.17 | # International Air Pollution Prevention Certificate (IAPP): |  | 28th Aug 2015 | N/A | 27th Aug 2020 |
| Documentation |  |  |  |  |  |
| 2.18 | Does vessel have all updated publications as listed in the Vessel Inspection Questionnaire, Chapter 2- Question 2.24, as applicable: |  |  | Yes |  |
| 2.19 | Owner warrant that vessel is member of ITOPF and will remain so for the entire duration of this voyage/contract: |  |  | Yes |  |
| 3. | CREW MANAGEMENT |  |  |  |  |
| 3.1 | Nationality of Master: |  |  | Korean |  |
| 3.2 | Nationality of Officers: |  |  | Korean / Indonesian |  |
| 3.3 | Nationality of Crew: |  |  | Korean / Myanmarese / Bangladeshi |  |
| 3.4 | If Officers/Crew employed by a Manning Agency - Full style: |  |  | PIA SHIPMANAGEMENT CO.,LTD (Rm:202) 16-2, Jungang-daero 180beon-gil, Dong-gu, Busan. Korea. (Zip code : 601-839) TEL:+82-51-441-0028 FAX:+82-51-441-0083 E-mail: piasm@piasm.com |  |
| 3.5 | What is the common working language onboard: |  |  | ENGLISH |  |
| 3.6 | Do officers speak and understand English: |  |  | Yes |  |
| 3.7 | In case of Flag Of Convenience, is the ITF Special Agreement on board: |  |  | Yes |  |
| 4. | HELICOPTERS |  |  |  |  |
| 4.1 | Can the ship comply with the ICS Helicopter Guidelines: |  |  | No |  |
| 4.2 | If Yes, state whether winching or landing area provided: |  |  |  |  |
| 5. | FOR USA CALLS |  |  |  |  |
| 5.1 | Has the vessel Operator submitted a Vessel Spill Response Plan to the US Coast Guard which has been approved by official USCG letter: |  |  | N/A |  |
| 5.2 | Qualified individual (QI) - Full style: |  |  | N/A |  |
| 5.3 | Oil Spill Response Organization (OSRO) -Full style: |  |  | N/A |  |
| 5.4 | Has technical operator signed the SCIA / C-TPAT agreement with US customs concerning drug smuggling: |  |  | N/A |  |

| 6. | CARGO AND BALLAST HANDLING |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Double Hull Vessels |  |  |  |  |  |  |  |  |  |
| 6.1 | Is vessel fitted with centerline bulkhead in all cargo tanks: |  |  |  |  |  | Yes |  |  |
| 6.2 | If Yes, is bulkhead solid or perforated: |  |  |  |  |  | Solid |  |  |
| Cargo Tank Capacities |  |  |  |  |  |  |  |  |  |
| 6.3 | Capacity (98%) of each natural segregation with double valve (specify tanks): |  |  |  |  |  | Seg#1: 658.79 m3 (Wings (P & S Combined) Number 1) Seg#2: 1203.4 m3 (Wings (P & S Combined) Number 2) Seg#3: 1483.78 m3 (Wings (P & S Combined) Number 3) Seg#4: 1697.49 m3 (Wings (P & S Combined) Number 4) Seg#5: 571.24 m3 (Wings (P & S Combined) Number 5) Seg#6: 2287.89 m3 (Wings (P & S Combined) Number 6) Seg#7: 2431.35 m3 (Wings (P & S Combined) Number 7) Seg#8: 570.38 m3 (Wings (P & S Combined) Number 8) Seg#9: 1858.36 m3 (Wings (P & S Combined) Number 9) Seg#10: 1284.8 m3 (Wings (P & S Combined) Number 10) Seg#11: 1420.76 m3 (Wings (P & S Combined) Number 11) Seg#13: 2214.11 m3 (Wings (P & S Combined) Number 13) |  |  |
| 6.4 | Total cubic capacity (98%, excluding slop tanks): |  |  |  |  |  | 17682.35 m3 |  |  |
| 6.5 | Slop tank(s) capacity (98%): |  |  |  |  |  | 553.811 m3 |  |  |
| 6.6 | Residual/Retention oil tank(s) capacity (98%), if applicable: |  |  |  |  |  |  |  |  |
| 6.7 | Does vessel have Segregated Ballast Tanks (SBT) or Clean Ballast Tanks (CBT): |  |  |  |  |  | SBT |  |  |
| SBT Vessels |  |  |  |  |  |  |  |  |  |
| 6.8 | What is total capacity of SBT? |  |  |  |  |  | 5583.48 m3 |  |  |
| 6.9 | What percentage of SDWT can vessel maintain with SBT only: |  |  |  |  |  | 32.80 % |  |  |
| 6.10 | Does vessel meet the requirements of MARPOL Annex I Reg 18.2: (previously Reg 13.2) |  |  |  |  |  | Yes |  |  |
| Cargo Handling |  |  |  |  |  |  |  |  |  |
| 6.11 | How many grades/products can vessel load/discharge with double valve segregation: |  |  |  |  |  | 26 |  |  |
| 6.12 | Maximum loading rate for homogenous cargo per manifold connection: |  |  |  |  |  | 400 m3/hr |  |  |
| 6.13 | Maximum loading rate for homogenous cargo loaded simultaneously through all manifolds: |  |  |  |  |  | 1600.00 m3/hr |  |  |
| 6.14 | Are there any cargo tank filling restrictions. If yes, please specify: |  |  |  |  |  | Yes CARGO DENSITY: 1.5T/cum |  |  |
| Pumping Systems |  |  |  |  |  |  |  |  |  |
| 6.15 | Pumps: |  |  |  |  | No. | Type | Capacity |  |
|  | Cargo: |  |  |  |  | 10 16 | SD150 SD125 | 300 m3/hr 200 m3/hr |  |
|  | Stripping: |  |  |  |  |  | N/A | 0 m3/hr |  |
|  | Eductors: |  |  |  |  |  | N/A | 0 m3/hr |  |
|  | Ballast: |  |  |  |  | 1 | HDE-250MC | 400 m3/hr |  |
| 6.16 | How many cargo pumps can be run simultaneously at full capacity: |  |  |  |  |  | 4 |  |  |
| Cargo Control Room |  |  |  |  |  |  |  |  |  |
| 6.17 | Is ship fitted with a Cargo Control Room (CCR): |  |  |  |  |  | Yes |  |  |
| 6.18 | Can tank innage / ullage be read from the CCR: |  |  |  |  |  | Yes |  |  |
| Gauging and Sampling |  |  |  |  |  |  |  |  |  |
| 6.19 | Can ship operate under closed conditions in accordance with ISGOTT: |  |  |  |  |  | Yes |  |  |
| 6.20 | What type of fixed closed tank gauging system is fitted: |  |  |  |  |  | Floating |  |  |
| 6.21 | Are overfill (high-high) alarms fitted? If Yes, indicate whether to all tanks or partial: |  |  |  |  |  | Yes(All tanks) |  |  |
| Vapor Emission Control |  |  |  |  |  |  |  |  |  |
| 6.22 | Is a vapor return system (VRS) fitted: |  |  |  |  |  | Yes |  |  |
| 6.23 | Number/size of VRS manifolds (per side): |  |  |  |  |  | 2 |  | 150 mm |
| Venting |  |  |  |  |  |  |  |  |  |
| 6.24 | State what type of venting system is fitted: |  |  |  |  |  | Independent/High velocity vents |  |  |
| Cargo Manifolds |  |  |  |  |  |  |  |  |  |
| 6.25 | Does vessel comply with the latest edition of the OCIMF 'Recommendations for Oil Tanker Manifolds and Associated Equipment’: |  |  |  |  |  | Yes |  |  |
| 6.26 | What is the number of cargo connections per side: |  |  |  |  |  | 26 |  |  |
| 6.27 | What is the size of cargo connections: |  |  |  |  |  | 150.00 mm |  |  |
| 6.28 | What is the material of the manifold: |  |  |  |  |  | SUS316L |  |  |
| Manifold Arrangement |  |  |  |  |  |  |  |  |  |
| 6.29 | Distance between cargo manifold centers: |  |  |  |  |  | 400.00 mm |  |  |
| 6.30 | Distance ships rail to manifold: |  |  |  |  |  | 4430.00 mm |  |  |
| 6.31 | Distance manifold to ships side: |  |  |  |  |  | 4600.00 mm |  |  |
| 6.32 | Top of rail to center of manifold: |  |  |  |  |  | 822.00 mm |  |  |
| 6.33 | Distance main deck to center of manifold: |  |  |  |  |  | 2550.00 mm |  |  |
| 6.34 | Manifold height above the waterline in normal ballast / at SDWT condition: |  |  |  |  |  | 9.27 m |  | 5.34 m |
| 6.35 | Number / size reducers: |  |  |  |  |  | 1 x 150/250mm (6/10") 2 x 150/200mm (6/8") 2 x 150/150mm (6/6") 2 x 150/125mm (6/5") 2 x 150/100mm (6/4") |  |  |
| Stern Manifold |  |  |  |  |  |  |  |  |  |
| 6.36 | Is vessel fitted with a stern manifold: |  |  |  |  |  | No |  |  |
| 6.37 | If stern manifold fitted, state size: |  |  |  |  |  | mm |  |  |
| Cargo Heating |  |  |  |  |  |  |  |  |  |
| 6.38 | Type of cargo heating system? |  |  |  |  |  | Coils |  |  |
| 6.39 | If fitted, are all tanks coiled? |  |  |  |  |  | Yes |  |  |
| 6.40 | If fitted, what is the material of the heating coils: |  |  |  |  |  | Other |  |  |
| 6.41 | Maximum temperature cargo can be loaded/maintained: |  |  |  |  |  | 70.0 °C / 158.0 °F |  | 70 °C / 158 °F |
| Tank Coating |  |  |  |  |  |  |  |  |  |
| 6.42 | Are cargo, ballast and slop tanks coated? |  |  |  | Coated |  | Type |  | To What Extent |
|  | Cargo tanks: |  |  |  | No |  | SUS316L |  | Whole Tank |
|  | Ballast tanks: |  |  |  | Yes |  | Epoxy |  | Whole Tank |
|  | Slop tanks: |  |  |  |  |  | SUS316L |  | Whole Tank |
| 6.43 | If fitted, what type of anodes are used: |  |  |  |  |  | ZINC (Only for Ballast Tank) |  |  |
| 7. | INERT GAS AND CRUDE OIL WASHING |  |  |  |  |  |  |  |  |
| 7.1 | Is an Inert Gas System (IGS) fitted: |  |  |  |  |  | Yes |  |  |
| 7.2 | Is IGS supplied by flue gas, inert gas (IG) generator and/or nitrogen: |  |  |  |  |  | Nitrogen Inerting system |  |  |
| 7.3 | Is a Crude Oil Washing (COW) installation fitted: |  |  |  |  |  | No |  |  |
| 8. | MOORING |  |  |  |  |  |  |  |  |
| 8.1 | Mooring wires (on drums) | No. | Diameter | Material |  |  | Length |  | Breaking Strength |
|  | Forecastle: | 0 | 0.00 mm | Not Applicable |  |  | 0.00 mm |  | 0.00 MT |
|  | Main deck fwd: | 0 | 0.00 mm |  |  |  | 0.00 mm |  | 0.00 MT |
|  | Main deck aft: | 0 | 0.00 mm |  |  |  | 0.00 mm |  | 0.00 MT |
|  | Poop deck: | 0 | 0.00 mm |  |  |  | 0.00 mm |  | 0.00 MT |
| 8.2 | Wire tails | No. | Diameter | Material |  |  | Length |  | Breaking Strength |
|  | Forecastle: | 0 | 0.00 mm |  |  |  | 0.00 mm |  | 0.00 MT |
|  | Main deck fwd: |  | 0.00 mm |  |  |  | 0.00 mm |  | 0.00 MT |
|  | Main deck aft: |  | 0.00 mm |  |  |  | 0.00 mm |  | 0.00 MT |
|  | Poop deck: |  | 0.00 mm |  |  |  | 0.00 mm |  | 0.00 MT |
| 8.3 | Mooring ropes (on drums) | No. | Diameter | Material |  |  | Length |  | Breaking Strength |
|  | Forecastle: | 4 | 56.00 mm | 40% POLYESTER / 60% DANLINE CORE |  |  | 220.00 m |  | 63.00 MT |
|  | Main deck fwd: | 0 | 0.00 mm | Not Applicable |  |  | 0.00 m |  | 0.00 MT |
|  | Main deck aft: |  | 0.00 mm | Not Applicable |  |  | 0.00 m |  | 0.00 MT |
|  | Poop deck: | 4 | 56.00 mm | 40% POLYESTER / 60% DANLINE CORE |  |  | 220.00 m |  | 63.00 MT |
| 8.4 | Other mooring lines | No. | Diameter | Material |  |  | Length |  | Breaking Strength |
|  | Forecastle: | 3 | 65.00 mm | 40% POLYESTER / 60% DANLINE CORE |  |  | 220.00 m |  | 62.00 MT |
|  | Main deck fwd: |  | 0.00 mm | Not Applicable |  |  | 0.00 m |  | 0.00 MT |
|  | Main deck aft: |  | 0.00 mm | Not Applicable |  |  | 0.00 m |  | 0.00 MT |
|  | Poop deck: | 3 | 65.00 mm | 40% POLYESTER / 60% DANLINE CORE |  |  | 220.00 m |  | 76.00 MT |
| 8.5 | Mooring winches |  |  | No. |  |  | # Drums |  | Brake Capacity |
|  | Forecastle: |  |  | 2 |  |  | Double Drums |  | 26.00 MT |
|  | Main deck fwd: |  |  | 0 |  |  | N/A |  | 0.00 MT |
|  | Main deck aft: |  |  | 0 |  |  | N/A |  | 0.00 MT |
|  | Poop deck: |  |  | 2 |  |  | Double |  | 26.00 MT |
| 8.6 | Mooring bitts |  |  |  |  |  | No. |  | SWL |
|  | Forecastle: |  |  |  |  |  | 6 |  | MT |
|  | Main deck fwd: |  |  |  |  |  | 6 |  | MT |
|  | Main deck aft: |  |  |  |  |  | 6 |  | MT |
|  | Poop deck: |  |  |  |  |  | 6 |  | MT |
| 8.7 | Closed chocks and/or fairleads of enclosed type |  |  |  |  |  | No. |  | SWL |
|  | Forecastle: |  |  |  |  |  |  |  | MT |
|  | Main deck fwd: |  |  |  |  |  |  |  | MT |
|  | Main deck aft: |  |  |  |  |  |  |  | MT |
|  | Poop deck: |  |  |  |  |  |  |  | MT |
| Emergency Towing System |  |  |  |  |  |  |  |  |  |
| 8.8 | Type / SWL of Emergency Towing system forward: |  |  |  |  |  | Not Applicable |  | 0 MT |
| 8.9 | Type / SWL of Emergency Towing system aft: |  |  |  |  |  | Not Applicable |  | 0 MT |
| Anchors |  |  |  |  |  |  |  |  |  |
| 8.10 | Number of shackles on port cable: |  |  |  |  |  | 10 |  |  |
| 8.11 | Number of shackles on starboard cable: |  |  |  |  |  | 10 |  |  |
| Escort Tug |  |  |  |  |  |  |  |  |  |
| 8.12 | What is SWL and size of closed chock and/or fairleads of enclosed type on stern: |  |  |  |  |  | 47.00 MT |  | Not Applicable |
| 8.13 | What is SWL of bollard on poop deck suitable for escort tug: |  |  |  |  |  | 72.00 MT |  |  |
| Bow/Stern Thruster |  |  |  |  |  |  |  |  |  |
| 8.14 | What is brake horse power of bow thruster (if fitted): |  |  |  |  |  | 900.00 bhp |  | 671.13 Kw |
| 8.15 | What is brake horse power of stern thruster (if fitted): |  |  |  |  |  | 0.00 bhp |  | 0 Kw |
| Single Point Mooring (SPM) Equipment |  |  |  |  |  |  |  |  |  |
| 8.16 | Does vessel comply with the latest edition of OCIMF 'Recommendations for Equipment Employed in the Mooring of Vessels at Single Point Moorings (SPM)’: |  |  |  |  |  | No |  |  |
| 8.17 | Is vessel fitted with chain stopper(s): |  |  |  |  |  | No |  |  |
| 8.18 | How many chain stopper(s) are fitted: |  |  |  |  |  | 0 |  |  |
| 8.19 | State type of chain stopper(s) fitted: |  |  |  |  |  |  |  |  |
| 8.20 | Safe Working Load (SWL) of chain stopper(s): |  |  |  |  |  | 0.00 MT |  |  |
| 8.21 | What is the maximum size chain diameter the bow stopper(s) can handle: |  |  |  |  |  | 0.00 mm |  |  |
| 8.22 | Distance between the bow fairlead and chain stopper/bracket: |  |  |  |  |  | 0 mm |  |  |
| 8.23 | Is bow chock and/or fairlead of enclosed type of OCIMF recommended size (600mm x 450mm)? If not, give details of size: |  |  |  |  |  | Yes |  |  |
| Lifting Equipment |  |  |  |  |  |  |  |  |  |
| 8.24 | Derrick / Crane description (Number, SWL and location): |  |  |  |  |  | Cranes: 1x5.00 Tonnes center |  |  |
| 8.25 | What is maximum outreach of cranes / derricks outboard of the ship’s side: |  |  |  |  |  | 3.00 m |  |  |
| Ship To Ship Transfer (STS) |  |  |  |  |  |  |  |  |  |
| 8.26 | Does vessel comply with recommendations contained in OCIMF/ICS Ship To Ship Transfer Guide (Petroleum or Liquified Gas, as applicable): |  |  |  |  |  | Yes |  |  |
| 9. | MISCELLANEOUS |  |  |  |  |  |  |  |  |
| Engine Room |  |  |  |  |  |  |  |  |  |
| 9.1 | What type of fuel is used for main propulsion? |  |  |  |  |  | IFO 180 CST |  |  |
| 9.2 | What type of fuel is used in the generating plant? |  |  |  |  |  | MDO |  |  |
| 9.3 | Capacity of bunker tanks - IFO and MDO/MGO: |  |  |  |  |  | 993.3 m3 |  | 147.53 m3 0 m3 |
| 9.4 | Is vessel fitted with fixed or controllable pitch propeller(s)? |  |  |  |  |  | Fixed Pitch |  |  |
| Insurance |  |  |  |  |  |  |  |  |  |
| 9.5 | P & I Club - Full Style: |  |  |  |  |  | JAPAN P& I |  |  |
| 9.6 | P & I Club coverage - pollution liability coverage: |  |  |  |  |  | US$ 1 BILLION |  |  |
| Port State Control |  |  |  |  |  |  |  |  |  |
| 9.7 | Date and place of last Port State Control inspection: |  |  |  |  |  | Feb 25, 2016 / Penang, Malaysia |  |  |
| 9.8 | Any outstanding deficiencies as reported by any Port State Control: |  |  |  |  |  | No |  |  |
| 9.9 | If yes, provide details: |  |  |  |  |  | Nil |  |  |
| Recent Operational History |  |  |  |  |  |  |  |  |  |
| 9.10 | Has vessel been involved in a pollution, grounding, serious casualty or collision incident during the past 12 months? If yes, full description: |  |  |  |  |  | Pollution: No , Grounding: N/A , Serious casualty: No , Collision: No , |  |  |
| 9.11 | Last three cargoes / charterers / voyages (Last / 2nd Last / 3rd Last): |  |  |  |  |  | Contact owner for details |  |  |
| Vetting |  |  |  |  |  |  |  |  |  |
| 9.12 | Date/Place of last SIRE Inspection: |  |  |  |  |  | SHELL – 05th Dec 2015 / Ningbo, China. TESORO - 18th Sep 2015 / Pyeongtaek, Korea |  |  |
| 9.13 | Date/Place of last CDI Inspection: |  |  |  |  |  | 3rd Sep 2015 / Ulsan, Korea |  |  |
| 9.14 | Recent Oil company inspections/screenings (To the best of owners knowledge and without guarantee of acceptance for future business)*: * Blanket "approvals" are no longer given by Oil Majors and ships are accepted for the voyage on a case by case basis. |  |  |  |  |  | Contact owner for details. |  |  |

Version 3 (INTERTANKO / Q88.com)