- INTERTANKO TANKER CHARTERING QUESTIONNAIRE 88	Version 4

| 1. | VESSEL DESCRIPTION |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1.1 | Date updated: |  |  |  |  |  |  |  |  |  |  | 03.May.2016 |  |  |
| 1.2 | Vessel’s name (IMO number): |  |  |  |  |  |  |  |  |  |  | Nordic Star (9748679) |  |  |
| 1.3 | Vessel’s previous name(s) and date(s) of change: |  |  |  |  |  |  |  |  |  |  | NA |  |  |
| 1.4 | Date delivered / Builder (where built): |  |  |  |  |  |  |  |  |  |  | (Expected delivery) August 31, 2016/ Sungdong Shipyard and Marine Engineering (South Korea) |  |  |
| 1.5 | Flag / Port of Registry: |  |  |  |  |  |  |  |  |  |  | Cayman Islands / George Town |  |  |
| 1.6 | Call sign / MMSI: |  |  |  |  |  |  |  |  |  |  | ZGFO8 / 319095100 |  |  |
| 1.7 | Vessel’s contact details (satcom/fax/email etc.): |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1.8 | Type of vessel (as described in Form A or Form B Q1.11 of the IOPPC): |  |  |  |  |  |  |  |  |  |  | Crude Oil Tanker |  |  |
| 1.9 | Type of hull: |  |  |  |  |  |  |  |  |  |  | Monohull (Double skin) |  |  |
| Classification |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1.10 | Classification society: |  |  |  |  |  |  |  |  |  |  | DNVGL |  |  |
| 1.11 | Class notation: |  |  |  |  |  |  |  |  |  |  | +1A1, Tanker for Oil, ESP, CSR, E0, BIS, COAT-PSPC(B, C), SPM, BWM-T, BWM-E(S), TMON, VCS-2, ERS |  |  |
| 1.12 | Is the vessel subject to any conditions of class, class extensions, outstanding memorandums or class recommendations? If yes, give details: |  |  |  |  |  |  |  |  |  |  | Newbuilding with clean papers |  |  |
| 1.13 | If classification society changed, name of previous and date of change: |  |  |  |  |  |  |  |  |  |  | NA |  |  |
| 1.14 | IMO type, if applicable: |  |  |  |  |  |  |  |  |  |  | NA |  |  |
| 1.15 | Does the vessel have ice class? If yes, state what level: |  |  |  |  |  |  |  |  |  |  | No |  |  |
| 1.16 | Date / place of last dry-dock: |  |  |  |  |  |  |  |  |  |  | Newbuilding |  |  |
| 1.17 | Date next dry dock due / next annual survey due: |  |  |  |  |  |  |  |  |  |  | Newbuilding |  | Newbuilding |
| 1.18 | Date of last special survey / next special survey due: |  |  |  |  |  |  |  |  |  |  | Newbuilding |  | Newbuilding |
| 1.19 | If ship has Condition Assessment Program (CAP), what is the latest overall rating: |  |  |  |  |  |  |  |  |  |  | NA |  |  |
| 1.20 | Does the vessel have a statement of compliance issued under the provisions of the Condition Assessment Scheme (CAS): If yes, what is the expiry date? |  |  |  |  |  |  |  |  |  |  | NA |  |  |
| Dimensions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1.21 | Length overall (LOA): |  |  |  |  |  |  |  |  |  |  | 277.00 meters (design) |  |  |
| 1.22 | Length between perpendiculars (LBP): |  |  |  |  |  |  |  |  |  |  | 267.00 meters (design) |  |  |
| 1.23 | Extreme breadth (Beam): |  |  |  |  |  |  |  |  |  |  | 48.00 meters (design) |  |  |
| 1.24 | Moulded depth: |  |  |  |  |  |  |  |  |  |  | 23.10 meters (design) |  |  |
| 1.25 | Keel to masthead (KTM)/ Keel to masthead (KTM) in collapsed condition, if applicable: |  |  |  |  |  |  |  |  |  |  | ABT. 54.2meters |  |  |
| 1.26 | Bow to center manifold (BCM) / Stern to center manifold (SCM): |  |  |  |  |  |  |  |  |  |  | 141.20 meters |  | 135.80 meters |
| 1.27 | Distance bridge front to center of manifold: |  |  |  |  |  |  |  |  |  |  | 89.67 meters |  |  |
| 1.28 | Parallel body distances |  |  |  |  | Lightship |  |  |  |  |  | Normal ballast |  | Scantling |
|  | # Forward to mid-point manifold: |  |  |  |  | 68.327 meters |  |  |  |  |  | 69.200 meters |  | 69.200 meters |
|  | # Aft to mid-point manifold: |  |  |  |  | 32.396 meters |  |  |  |  |  | 47.800 meters |  | 67.432 meters |
|  | # Parallel body length: |  |  |  |  | 100.723 meters |  |  |  |  |  | 117.000 meters |  | 136.632 meters |
| # 1.29 | # FWA/TPC at summer draft: |  |  |  |  |  |  |  |  |  |  | 384mm/ 119.4 t |  |  |
| 1.30 | Constant (excluding fresh water): |  |  |  |  |  |  |  |  |  |  | DEADWEIGHT CONSTANT: 198.6 ton |  |  |
| 1.31 | What is the company guidelines for Under Keel Clearance (UKC) for this vessel? |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1.32 | What is the max height of mast above waterline (air draft) |  |  |  |  |  |  |  |  |  |  | Full Mast |  | Collapsed Mast |
|  | Lightship: |  |  |  |  |  |  |  |  |  |  | 49.1 meters |  | NA |
|  | Normal ballast: |  |  |  |  |  |  |  |  |  |  | 44.7 meters |  | NA |
|  | At loaded summer deadweight: |  |  |  |  |  |  |  |  |  |  | 37.05 meters |  | NA |
| Tonnages |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1.33 | Net Tonnage: |  |  |  |  |  |  |  |  |  |  | Not yet |  |  |
| 1.34 | Gross Tonnage / Reduced Gross Tonnage (if applicable): |  |  |  |  |  |  |  |  |  |  | 81800 |  |  |
| 1.35 | Suez Canal Tonnage - Gross (SCGT) / Net (SCNT): |  |  |  |  |  |  |  |  |  |  | Not yet |  |  |
| # 1.36 | Panama Canal Net Tonnage (PCNT): |  |  |  |  |  |  |  |  |  |  | Not yet |  |  |
| Ownership and Operation |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1.37 | Registered owner - Full style: |  |  |  |  |  | NORDIC AMERICAN TANKERS LIMITED LOM BUILDING, 27 REID STR., HAMILTON HM 11 BERMUDA c/o Scandic American Shipping Ltd P.O. Box 56 3201 Sandefjord, Norway Tel: +47 33 42 73 00 Email: sas@scandicamerican.com Web: www.nat.bm Company IMO#: 4037590 |  |  |  |  |  |  |  |
| 1.38 | Technical operator - Full style: |  |  |  |  |  | Columbia Shipmanagement (Deutschland) GmbH Grosse Elbstrasse 275, 22767 Hamburg, P.O. Box 261213, D-20502 Hamburg, Germany Tel: +49 40 3613040 Fax: +49 40 361304 550 Telex: Not Applicable Email: vetting@csm-d.com Web: www.csm-d.com Company IMO#: 1898811 |  |  |  |  |  |  |  |
| 1.39 | Commercial operator - Full style: |  |  |  |  |  | V SHIPS UK LTD V SHIPS UK LTD on behalf of ORION TANKERS LTD As Agents Only THE SKYPARK 8, ELLIOT PLACE, GLASGOW G3 8EP, SCOTLAND Tel: +44 141 243 2435 Fax: +44 141 243 2436 Telex: 776311 Email: ORIONOPS@VSHIPS.COM Web: N/A |  |  |  |  |  |  |  |
| 1.40 | Disponent owner - Full style: |  |  |  |  |  | ORION TANKERS LTD AS AGENTS ONLY TO NORDIC AMERICAN TANKERS LIMITED C/O ORION TANKERS AS FRIDTJOF NANSENS PLASS 7, N-0160 OSLO, NORWAY Tel: +47 2369 6900 Fax: n/a Telex: n/a Email: CHARTERING@ORIONTANKERS.COM Web: n/a |  |  |  |  |  |  |  |
| 2. | CERTIFICATION (Please refer to attached Building Spec 0.6) |  |  |  |  | Issued (Yes/No) |  |  |  |  |  | Last Annual |  | Expires |
| 2.1 | Safety Equipment Certificate (SEC): |  |  |  |  | Yes |  |  |  |  |  |  |  |  |
| 2.2 | Safety Radio Certificate (SRC): |  |  |  |  | Yes |  |  |  |  |  |  |  |  |
| 2.3 | Safety Construction Certificate (SCC): |  |  |  |  | Yes |  |  |  |  |  |  |  |  |
| 2.4 | International Loadline Certificate (ILC): |  |  |  |  | Yes |  |  |  |  |  |  |  |  |
| 2.5 | International Oil Pollution Prevention Certificate (IOPPC): |  |  |  |  | Yes |  |  |  |  |  |  |  |  |
| 2.6 | ISM Safety Management Certificate (SMC): |  |  |  |  | To be issued after ISM audit & delivery. |  |  |  |  |  |  |  |  |
| # 2.7 | # Document of Compliance (DOC): |  |  |  |  | - |  |  |  |  |  |  |  |  |
| 2.8 | USCG Certificate of Compliance (COC): |  |  |  |  | Yes |  |  |  |  |  |  |  |  |
| 2.9 | Civil Liability Convention (CLC) 1992 Certificate: |  |  |  |  | - |  |  |  |  |  |  |  |  |
| 2.10 | Civil Liability for Bunker Oil Pollution Damage Convention (CLBC) Certificate: |  |  |  |  | - |  |  |  |  |  |  |  |  |
| 2.11 | Ship Sanitation Control (SSCC)/Ship Sanitation Control Exemption (SSCE) Certificate: |  |  |  |  | Yes |  |  |  |  |  |  |  |  |
| 2.12 | U.S. Certificate of Financial Responsibility (COFR): |  |  |  |  | - |  |  |  |  |  |  |  |  |
| 2.13 | Certificate of Class (COC): |  |  |  |  | Yes |  |  |  |  |  |  |  |  |
| # 2.14 | # International Sewage Pollution Prevention Certificate (ISPPC): |  |  |  |  | Yes |  |  |  |  |  |  |  |  |
| # 2.15 | # Certificate of Fitness (COF): |  |  |  |  | - |  |  |  |  |  |  |  |  |
| # 2.16 | # International Energy Efficiency Certificate (IEEC): |  |  |  |  | Yes |  |  |  |  |  |  |  |  |
| # 2.17 | # International Ship Security Certificate (ISSC): |  |  |  |  | - |  |  |  |  |  |  |  |  |
| # 2.18 | # International Air Pollution Prevention Certificate (IAPPC): |  |  |  |  | Yes |  |  |  |  |  |  |  |  |
| # 2.19 | # Maritime Labour Certificate (MLC): |  |  |  |  | Yes |  |  |  |  |  |  |  |  |
| Documentation |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2.20 | Owner warrant that vessel is member of ITOPF and will remain so for the entire duration of this voyage/contract: |  |  |  |  |  |  |  |  |  |  | Yes |  |  |
| 2.21 | Does vessel have in place a Drug and Alcohol Policy complying with OCIMF guidelines for Control of Drugs and Alcohol Onboard Ship? |  |  |  |  |  |  |  |  |  |  | Yes |  |  |
| 2.22 | Is the ITF Special Agreement on board (if applicable)? |  |  |  |  |  |  |  |  |  |  | Yes |  |  |
| 2.23 | ITF Blue Card expiry date: |  |  |  |  |  |  |  |  |  |  | Not Applicable |  |  |
| 3. | CREW |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3.1 | Nationality of Master: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3.2 | Number and Nationality of Officers: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3.3 | Number and Nationality of Crew: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3.4 | What is the common working language onboard: |  |  |  |  |  |  |  |  |  |  | ENGLISH |  |  |
| 3.5 | Do officers speak and understand English? |  |  |  |  |  |  |  |  |  |  | Yes |  |  |
| 3.6 | If Officers/Crew employed by a Manning Agency - Full style: |  |  |  |  |  | Officers: Columbia Shipmanagement (Deutschland) GMBH Grosse Elbstrasse 275, 22767 Hamburg, P.O. Box 261213, D-20502 Hamburg, Germany. Tel: +49 40 - 361304 - 0 Fax: +49 40 - 361304 - 55 Telex: Not Applicable Email: csm@csm-d.com Web: www.columbia-shipmanagement.com Crew: Columbia Shipmanagement (Deutschland) GMBH Grosse Elbstrasse 275, 22767 Hamburg, P.O. Box 261213, D-20502 Hamburg, Germany. Tel: +49 40 - 361304 - 0 Fax: +49 40 - 361304 - 55 Telex: Not Applicable Email: csm@csm-d.com Web: www.columbia-shipmanagement.com |  |  |  |  |  |  |  |
| 4. | FOR USA CALLS |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4.1 | Has the vessel Operator submitted a Vessel Spill Response Plan to the US Coast Guard which has been approved by official USCG letter? |  |  |  |  |  |  |  |  |  |  |  | Yes |  |
| 4.2 | Qualified individual (QI) - Full style: |  |  |  |  |  | O' Brien's Tel - +1 985 781 0804 |  |  |  |  |  |  |  |
| 4.3 | Oil Spill Response Organization (OSRO) - Full style: |  |  |  |  |  | O' Brien's Tel - +1 985 781 0804 |  |  |  |  |  |  |  |
| 5. | CARGO AND BALLAST HANDLING |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Double Hull Vessels |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.1 | Is vessel fitted with centerline bulkhead in all cargo tanks? If Yes, solid or perforated: |  |  |  |  |  |  |  |  |  |  | YES, Solid |  |  |
| # Loadline Information |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| # 5.2 | # Loadline |  | # Freeboard |  |  | # Draft |  |  |  |  | # Deadweight |  |  | # Displacement |
|  | # Summer: |  | 5.972 m |  |  | 17.174 m |  |  |  |  | # 157660.9 ton |  |  | 183260.9 ton |
|  | # Winter: |  | 6.329 m |  |  | 16.817 m |  |  |  |  | 153403.6 ton |  |  | 179003.6 ton |
|  | # Tropical: |  | 5.615 m |  |  | 17.531 m |  |  |  |  | 161926.3 ton |  |  | 187526.3 ton |
|  | # Lightship: |  | 20.326 m |  |  | 2.820 m |  |  |  |  | 0 |  |  | 25600.0 ton |
|  | # Normal Ballast Condition: |  | 15.143 m |  |  | 8.003 m |  |  |  |  | 53416.2 ton |  |  | 79016.2 ton |
| # 5.3 | # Does vessel have multiple SDWT? If yes, please provide all assigned loadlines: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Cargo Tank Capacities |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.4 | Number of cargo tanks and total cubic capacity (98%): |  |  |  |  |  |  |  |  |  |  | 12 |  | 162,576 Cu. Meters |
| 5.5 | Capacity (98%) of each natural segregation with double valve (specify tanks): |  |  |  |  |  |  |  |  |  |  | No.1: 50787.6 m3 COTs 1 and 4 No.2: 58745.4 m3 COTs 2 and 5 No.3: 53043.0 m3 COTs 3 and 6 |  |  |
| 5.6 | Number of slop tanks and total cubic capacity (98%): |  |  |  |  |  |  |  |  |  |  | 2 |  | 3982.1 Cu. Meters |
| 5.7 | Specify segregations which slops tanks belong to and their capacity with double valve: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.8 | Residual/Retention oil tank(s) capacity (98%), if applicable: |  |  |  |  |  |  |  |  |  |  | 422.5 Cu. Meters |  |  |
| 5.9 | Does vessel have Segregated Ballast Tanks (SBT) or Clean Ballast Tanks (CBT): |  |  |  |  |  |  |  |  |  |  | SBT |  |  |
| SBT Vessels |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.10 | What is total SBT capacity and percentage of SDWT vessel can maintain? |  |  |  |  |  |  |  |  |  |  | 35.3 % |  |  |
| 5.11 | Does vessel meet the requirements of MARPOL Annex I Reg 18.2: |  |  |  |  |  |  |  |  |  |  | Yes |  |  |
| Cargo Handling and Pumping Systems |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.12 | How many grades/products can vessel load/discharge with double valve segregation: |  |  |  |  |  |  |  |  |  |  | Three (3) segregations |  |  |
| 5.13 | Are there any cargo tank filling restrictions? If yes, specify number of slack tanks, max s.g., ullage restrictions etc.: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.14 | Pumps |  |  |  | No. | Type |  |  |  |  |  | Capacity |  | At What Head (sg=1.0) |
|  | Cargo Pumps: |  |  |  | 3 | Vertical centrifugal single stage |  |  |  |  |  | 4,000 m3/h X 135mTH |  | 1.025 S.G |
|  | Cargo Eductors: |  |  |  | 1 | Driving liquid from cargo pumps |  |  |  |  |  | 600 m3/h |  | 1.025 S.G |
|  | Stripping: |  |  |  | 1 | Steam driven reciprocating |  |  |  |  |  | 200 m3/h X 135mTH |  | 1.025 S.G |
|  | Ballast Pumps: |  |  |  | 2 | Vertical centrifugal single stage |  |  |  |  |  | 2,000 m3/h X 37mTH |  | 1.025 S.G |
|  | Ballast Eductors: |  |  |  | 1 | Driving water from ballast pumps |  |  |  |  |  | 200 m3/h |  | 1.025 S.G |
| 5.15 | Max loading rate for homogenous cargo per manifold connection: |  |  |  |  |  |  |  |  |  |  | 5,200 m3/h |  |  |
| 5.16 | Max loading rate for homogenous cargo loaded simultaneously through all manifolds: |  |  |  |  |  |  |  |  |  |  | 15,600 m3/h |  |  |
| 5.17 | How many cargo pumps can be run simultaneously at full capacity: |  |  |  |  |  |  |  |  |  |  | 3ea |  |  |
| Cargo Control Room |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.18 | Is ship fitted with a Cargo Control Room (CCR)? |  |  |  |  |  |  |  |  |  |  | Yes |  |  |
| 5.19 | Can tank innage / ullage be read from the CCR? |  |  |  |  |  |  |  |  |  |  | Yes |  |  |
| Gauging and Sampling |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.20 | Can cargo be transferred under closed loading conditions in accordance with ISGOTT 11.1.6.6? |  |  |  |  |  |  |  |  |  |  | Yes |  |  |
| 5.21 | What type of fixed closed tank gauging system is fitted: |  |  |  |  |  |  |  |  |  |  | Radar beam type level transmitter |  |  |
| 5.22 | Number of portable gauging units (example- MMC) on board: |  |  |  |  |  |  |  |  |  |  | 2 sets |  |  |
| 5.23 | Are overfill (high) alarms fitted? If Yes, indicate whether to all tanks or partial: |  |  |  |  |  |  |  |  |  |  | All tanks |  |  |
| 5.24 | Are cargo tanks fitted with multipoint gauging? If yes, specify type and locations: |  |  |  |  |  |  |  |  |  |  | No |  |  |
| 5.25 | Is gauging system certified and calibrated? If no, specify which ones are not calibrated: |  |  |  |  |  |  |  |  |  |  | YES |  |  |
| Vapor Emission Control System (VECS) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.26 | Is a Vapour Emission Control System (VECS) fitted? |  |  |  |  |  |  |  |  |  |  | Yes |  |  |
| 5.27 | Number/size of VECS manifolds (per side): |  |  |  |  |  |  |  |  |  |  | 2 |  | 500A |
| 5.28 | Number / size / type of VECS reducers: |  |  |  |  |  |  |  |  |  |  | Size; Number. 500A(20”) x 250A(10”); 1. 500A(20”) x 300A(12”); 2. 500A(20”) x 400A(16”); 4. |
| Venting |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.29 | State what type of venting system is fitted: |  |  |  |  |  |  |  |  |  |  | ONE COMMON VENT RISER, ONE INDEPENDENT HIGH VELOCITY PRESSURE/VACCUM VALVE & ONE INERT GAS MAIN LINE |  |  |
| Cargo Manifolds and Reducers |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.30 | Does vessel comply with the latest edition of the OCIMF 'Recommendations for Oil Tanker Manifolds and Associated Equipment’? |  |  |  |  |  |  |  |  |  |  | OCIMF, Recommendations for Oil Tanker Manifolds and Associated Equipment, 1991 |  |  |
| 5.31 | Total number / size of cargo manifold connections on each side: |  |  |  |  |  |  |  |  |  |  | Size; Number. 600A(24”); 3. |
| 5.32 | What type of valves are fitted at manifold: |  |  |  |  |  |  |  |  |  |  | Butterfly valve |  |  |
| 5.33 | What is the material/rating of the manifold: |  |  |  |  |  |  |  |  |  |  | STPG 370 ERW 12.7 / 16K |  |  |
| 5.34 | Does the vessel have a Common Line Manifold connection? If yes, describe: |  |  |  |  |  |  |  |  |  |  | No. |  |  |
| 5.35 | Distance between cargo manifold centers: |  |  |  |  |  |  |  |  |  |  | 2500mm |  |  |
| 5.36 | Distance ships rail to manifold: |  |  |  |  |  |  |  |  |  |  | 4600mm |  |  |
| 5.37 | Distance manifold to ships side: |  |  |  |  |  |  |  |  |  |  | 4600mm |  |  |
| 5.38 | Top of rail to center of manifold: |  |  |  |  |  |  |  |  |  |  | 700mm |  |  |
| 5.39 | Distance main deck to center of manifold: |  |  |  |  |  |  |  |  |  |  | 2100mm |  |  |
| 5.40 | Spill tank grating to center of manifold: |  |  |  |  |  |  |  |  |  |  | 900mm |  |  |
| 5.41 | Manifold height above the waterline in normal ballast / at SDWT condition: |  |  |  |  |  |  |  |  |  |  | 17.101 meters |  | 8.05 meters |
| 5.42 | Number / size / type of reducers: |  |  |  |  |  |  |  |  |  |  | Size; Number. 600A(24”) x 200A(8”); 3. 600A(24”) x 250A(10”); 3. 600A(24”) x 300A(12”); 3. 600A(24”) x 400A(16”); 6. |
| 5.43 | Is vessel fitted with a stern manifold? If yes, state size: |  |  |  |  |  |  |  |  |  |  | No |  |  |
| Heating |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.44 | Cargo / slop tanks fitted with a cargo heating system? |  |  |  |  |  |  | Type |  |  |  | Coiled |  | Material |
|  | Cargo Tanks: |  |  |  |  |  |  | Heating coil |  |  |  | Yes |  | STPG370 SMLS #80 |
|  | Slop Tanks: |  |  |  |  |  |  | Heating coil |  |  |  | Yes |  | STPG370 SMLS #80 |
| 5.45 | Maximum temperature cargo can be loaded / maintained: |  |  |  |  |  |  |  |  |  |  | 66.0 °C |  | 66.0 °C |
| 5.46 | Minimum temperature cargo can be loaded / maintained: |  |  |  |  |  |  |  |  |  |  | 44.0 °C |  | 44.0 °C |
| Coating / Anodes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.47 | Tank Coating |  |  | Coated |  | Type |  |  |  |  |  | To What Extent |  | Anodes |
|  | Cargo tanks: |  |  | Yes |  | Pure epoxy |  |  |  |  |  | TANK CEILING AND 1.8 M BELOW + TANK BOTTOM AND 0.5 M UP |  | No |
|  | Ballast tanks: |  |  | Yes |  | Pure Epoxy |  |  |  |  |  | Whole tank |  | Yes |
|  | Slop tanks: |  |  | Yes |  | Pure Epoxy |  |  |  |  |  | Whole tank |  | No |
| 6. | INERT GAS AND CRUDE OIL WASHING |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6.1 | Is a Crude Oil Washing (COW) installation fitted / operational? |  |  |  |  |  |  |  |  |  |  | Yes |  |  |
| 6.2 | Is an Inert Gas System (IGS) fitted / operational? |  |  |  |  |  |  |  |  |  |  | Yes |  |  |
| 6.3 | Is IGS supplied by flue gas, inert gas (IG) generator and/or nitrogen: |  |  |  |  |  |  |  |  |  |  | By flue gas |  |  |
| 7. | MOORING |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7.1 | Wires (on drums) | No. |  | Diameter |  | Material |  |  |  |  |  | Length |  | Breaking Strength |
|  | Forecastle: | 4 |  | 34 Millimetres |  | GALVANIZED STEEL WIRE |  |  |  |  |  | 250 Metres |  | 71ton |
|  | Main deck fwd: | 4 |  | 34 Millimetres |  | GALVANIZED STEEL WIRE |  |  |  |  |  | 250 Metres |  | 71ton |
|  | Main deck aft: | 2 |  | 34 Millimetres |  | GALVANIZED STEEL WIRE |  |  |  |  |  | 250 Metres |  | 71ton |
|  | Poop deck: | 6 |  | 34 Millimetres |  | GALVANIZED STEEL WIRE |  |  |  |  |  | 250 Metres |  | 71ton |
| 7.2 | Wire tails | No. |  | Diameter |  | Material |  |  |  |  |  | Length |  | Breaking Strength |
|  | Forecastle: | 4 |  | 75 Millimetres |  | Nylon |  |  |  |  |  | 11 meters |  | 101 ton |
|  | Main deck fwd: | 4 |  | 75 Millimetres |  | Nylon |  |  |  |  |  | 11 meters |  | 101 ton |
|  | Main deck aft: | 2 |  | 75 Millimetres |  | Nylon |  |  |  |  |  | 11 meters |  | 101 ton |
|  | Poop deck: | 6 |  | 75 Millimetres |  | Nylon |  |  |  |  |  | 11 meters |  | 101 ton |
| 7.3 | Ropes (on drums) | No. |  | Diameter |  | Material |  |  |  |  |  | Length |  | Breaking Strength |
|  | Forecastle: | NA |  | NA |  | NA |  |  |  |  |  | NA |  | NA |
|  | Main deck fwd: | NA |  | NA |  | NA |  |  |  |  |  | NA |  | NA |
|  | Main deck aft: | NA |  | NA |  | NA |  |  |  |  |  | NA |  | NA |
|  | Poop deck: | NA |  | NA |  | NA |  |  |  |  |  | NA |  | NA |
| 7.4 | Other lines | No. |  | Diameter |  | Material |  |  |  |  |  | Length |  | Breaking Strength |
|  | Forecastle: | NA |  | NA |  | NA |  |  |  |  |  | NA |  | NA |
|  | Main deck fwd: | 1 |  | 38 Millimetres |  | GALVANIZED STEEL WIRE |  |  |  |  |  | 60 Metres |  | 100 ton |
|  | Main deck aft: | 1 |  | 38 Millimetres |  | GALVANIZED STEEL WIRE |  |  |  |  |  | 60 Metres |  | 100 ton |
|  | Poop deck: | NA |  | NA |  | NA |  |  |  |  |  | NA |  | NA |
| 7.5 | Winches | No. |  | No. Drums |  | Motive Power |  |  |  |  |  | Brake Capacity |  | Type of Brake |
|  | Forecastle: | 2 |  | 2 |  | 20 ton |  |  |  |  |  | 59.1 ton |  | Spring loaded fail-safe manual type |
|  | Main deck fwd: | 2 |  | 4 |  | 20 ton |  |  |  |  |  | 59.1 ton |  | Spring loaded fail-safe manual type |
|  | Main deck aft: | 1 |  | 2 |  | 20 ton |  |  |  |  |  | 59.1 ton |  | Spring loaded fail-safe manual type |
|  | Poop deck: | 3 |  | 6 |  | 20 ton |  |  |  |  |  | 59.1 ton |  | Spring loaded fail-safe manual type |
| 7.6 | Bitts, closed chocks/fairleads |  |  | No. Bitts |  | SWL Bitts |  |  |  |  |  | No. Closed Chocks |  | SWL Closed Chocks |
|  | Forecastle: |  |  | 2 |  | 74 Metric Tones |  |  |  |  |  | 8 |  | 74 Metric Tones |
|  | Main deck fwd: |  |  | 4 |  | 74 Metric Tones |  |  |  |  |  | 12 |  | 74 Metric Tones |
|  | Main deck aft: |  |  | 2 |  | 74 Metric Tones |  |  |  |  |  | 6 |  | 74 Metric Tones |
|  | Poop deck: |  |  | 4 |  | 74 Metric Tones |  |  |  |  |  | 12 |  | 74 Metric Tones |
| Anchors/Emergency Towing System |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7.7 | Number of shackles on port / starboard cable: |  |  |  |  |  |  |  |  |  |  | Port : 13 ea / Stb’d : 14 ea |  |  |
| 7.8 | Type / SWL of Emergency Towing system forward: |  |  |  |  |  |  |  |  |  |  | Tongue type |  | 350 Metric Tones |
| 7.9 | Type / SWL of Emergency Towing system aft: |  |  |  |  |  |  |  |  |  |  | Stopper type |  | 200 Metric Tones |
| Escort Tug |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7.10 | What is size / SWL of closed chock and/or fairleads of enclosed type on stern: |  |  |  |  |  |  |  |  |  |  | 600x500mm |  | 204 Metric Tones |
| 7.11 | What is SWL of bollard on poop deck suitable for escort tug: |  |  |  |  |  |  |  |  |  |  | 74 Metric Tones |  |  |
| Bow/Stern Thruster |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7.12 | What is brake horse power of bow thruster (if fitted): |  |  |  |  |  |  |  |  |  |  | NA |  |  |
| 7.13 | What is brake horse power of stern thruster (if fitted): |  |  |  |  |  |  |  |  |  |  | NA |  |  |
| Single Point Mooring (SPM) Equipment |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7.14 | Does the vessel meet the recommendations in the latest edition of OCIMF 'Recommendations for Equipment Employed in the Bow Mooring of Conventional Tankers at Single Point Moorings (SPM)’? |  |  |  |  |  |  |  |  |  |  | Yes |  |  |
| 7.15 | If fitted, how many chain stoppers: |  |  |  |  |  |  |  |  |  |  | 2 |  |  |
| 7.16 | State type / SWL of chain stopper(s): |  |  |  |  |  |  |  |  |  |  | Tongue type |  | 350 Metric Tones |
| 7.17 | What is the maximum size chain diameter the bow stopper(s) can handle: |  |  |  |  |  |  |  |  |  |  | 76 mm |  |  |
| 7.18 | Distance between the bow fairlead and chain stopper/bracket: |  |  |  |  |  |  |  |  |  |  | 5085 mm |  |  |
| 7.19 | Is bow chock and/or fairlead of enclosed type of OCIMF recommended size (600mm x 450mm)? If not, give details of size: |  |  |  |  |  |  |  |  |  |  | Yes |  |  |
| Lifting Equipment |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7.20 | Derrick / Crane description (Number, SWL and location): |  |  |  |  |  |  |  |  |  |  | Hose handling crane |  | 2set, 15ton, Main deck |
|  |  |  |  |  |  |  |  |  |  |  |  | Provision crane |  | 2set, (Port: 8ton / STB’D: 2ton), Accom. |
|  |  |  |  |  |  |  |  |  |  |  |  | Suez search light davit |  | 1ea, 0.5ton, F’cle. |
| 7.21 | What is maximum outreach of cranes / derricks outboard of the ship’s side: |  |  |  |  |  |  |  |  |  |  | 6.85 meters |  |  |
| Ship To Ship Transfer (STS) / Helicopter Operations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7.22 | Does vessel comply with recommendations contained in OCIMF/ICS Ship To Ship Transfer Guide (Petroleum, Chemicals or Liquified Gas, as applicable)? |  |  |  |  |  |  |  |  |  |  | Yes |  |  |
| 7.23 | Can the ship comply with the ICS Helicopter Guidelines? If Yes, state whether winching or landing area provided and diameter of the circle provided: |  |  |  |  |  |  |  |  |  |  | Yes, Port side; D=13 meters |  |  |
| 8. | MISCELLANEOUS |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Engine |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.1 | Speed |  |  |  |  |  |  |  |  |  |  | Maximum |  | Economic |
|  | Ballast speed: based on estimation of Model Test |  |  |  |  |  |  |  |  |  |  | 16.7 knots |  | 15.3 knots |
|  | Laden speed: based on estimation of Model Test |  |  |  |  |  |  |  |  |  |  | 15.8 knots |  | 14.2 knots |
| 8.2 | What type of fuel is used for main propulsion / generating plant: |  |  |  |  |  |  |  |  |  |  | H.F.O. having a viscosity up to 700 cSt at 50℃ M.G.O. having a viscosity of Min. 3 cSt at 40℃ |  |  |
| 8.3 | Type / Capacity of bunker tanks: Capacity of H.F.O. / D.O. tanks are 100% Full volume. |  |  |  |  |  |  |  |  |  |  | HFO: 3,955 m3 / DO: 864 m3 |  |  |
| 8.4 | Is vessel fitted with fixed or controllable pitch propeller(s): |  |  |  |  |  |  |  |  |  |  | Fixed |  |  |
| 8.5 | Engines |  |  |  |  |  |  |  |  | No |  | Capacity |  | Make/Type |
|  | Main engine: |  |  |  |  |  |  |  |  | 1 |  | 21840 Kilowatt |  | Hyundai Heavy Industry, Co., Ltd Man B&W 6G70ME-C9.5 / 6 cyls |
|  | Aux engine: |  |  |  |  |  |  |  |  | 3 |  | 900 Kilowatt |  | Hyundai Heavy Industry, Co., Ltd Himsen 5H21/32 4-stroke engine |
|  | Power packs: |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Boilers: |  |  |  |  |  |  |  |  | 2 |  | AUX. BOILER - 35.00 Metric Tonnes/Hour COMPOSITE BOILER -2.48 Metric Tonnes/Hour |  | AUX. BOILER - KANGRIM (PB0601A518) COMPOSITE BOILER - KANGRIM (PC0201P31) |
| Emissions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.6 | Main engine IMO NOx emission standard: |  |  |  |  |  |  |  |  |  |  | Tier II |  |  |
| 8.7 | Energy Efficiency Design Index (EEDI) rating number: EEDI Phase 2 is attained based on Preliminary EEDI Technical File. |  |  |  |  |  |  |  |  |  |  | Phase 2 |  |  |
| Insurance |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.8 | P & I Club - Full Style: |  |  |  |  |  |  |  | GARD Gard AS As agent only for Gard P.&I. (Bermuda) Ltd., Norwegian Branch Kittelbuktveien 31, No-4836, Arendal, Norway Tel: Tel. +47 37 01 91 00 Fax: Fax: +47 37 02 48 10 Web: www.gard.no |  |  |  |  |  |
| 8.9 | P & I Club pollution liability coverage / expiration date: |  |  |  |  |  |  |  |  |  |  | 1,000,000,000 US$ |  |  |
| 8.10 | Hull & Machinery insured by - Full Style: |  |  |  |  |  |  |  | Willis AS P.O. Box 334 Skøyen Tel: (+47) 23 29 60 00 Fax: (+47) 24 12 63 21 |  |  |  |  |  |
| 8.11 | Hull & Machinery insured value / expiration date: |  |  |  |  |  |  |  |  |  |  | TBA |  |  |
| Recent Operational History |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.12 | Date and place of last Port State Control inspection: |  |  |  |  |  |  |  |  |  |  | NA |  |  |
| 8.13 | Any outstanding deficiencies as reported by any Port State Control? If yes, provide details: |  |  |  |  |  |  |  |  |  |  | NA |  |  |
| 8.14 | Has vessel been involved in a pollution, grounding, serious casualty or collision incident during the past 12 months? If yes, full description: |  |  |  |  |  |  |  |  |  |  | NA |  |  |
| 8.15 | Last three cargoes / charterers / voyages (Last / 2nd Last / 3rd Last): |  |  |  |  |  |  |  |  |  |  | NA |  |  |
| 8.16 | Date/place of last STS operation: |  |  |  |  |  |  |  |  |  |  | NA |  |  |
| Vetting |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.17 | Date of last SIRE inspection: |  |  |  |  |  |  |  |  |  |  | NA |  |  |
| 8.18 | Date of last CDI inspection: |  |  |  |  |  |  |  |  |  |  | NA |  |  |
| 8.19 | Recent Oil company inspections/screenings (To the best of owners knowledge and without guarantee of acceptance for future business)*: * "Approvals" are not given by Oil Majors and ships are accepted for the voyage on a case by case basis. |  |  |  |  |  |  |  |  |  |  | NA |  |  |
| Additional Information |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.20 | Additional information relating to features of the ship or operational characteristics: |  |  |  |  |  |  |  |  |  |  | NA |  |  |

Rev 2015 (INTERTANKO / Q88.com)

Form completed on http://www.q88.com/integration.aspx  Please email support@q88.com an updated copy if this is not the latest version.