- INTERTANKO TANKER CHARTERING QUESTIONNAIRE 88 	Version 4

| 1. | VESSEL DESCRIPTION |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1.1 | Date updated: |  |  |  |  |  |  |  |  |  |  | May 31, 2016 |  |  |
| 1.2 | Vessel’s name (IMO number): |  |  |  |  |  |  |  |  |  |  | Tenacity (9416408) |  |  |
| 1.3 | Vessel’s previous name(s) and date(s) of change: |  |  |  |  |  |  |  |  |  |  | Not Applicable |  |  |
| 1.4 | Date delivered / Builder (where built): |  |  |  |  |  |  |  |  |  |  | Jul 07, 2014 / Guangzhou Shipyard International Co., Ltd |  |  |
| 1.5 | Flag / Port of Registry: |  |  |  |  |  |  |  |  |  |  | Liberia / MONROVIA |  |  |
| 1.6 | Call sign / MMSI: |  |  |  |  |  |  |  |  |  |  | D5DN7 / 636015928 |  |  |
| 1.7 | Vessel’s contact details (satcom/fax/email etc.): |  |  |  |  |  |  |  |  |  |  | Tel: +870773141019-18 Fax: Email: tenacity@sea-pioneer.gr |  |  |
| 1.8 | Type of vessel (as described in Form A or Form B Q1.11 of the IOPPC): |  |  |  |  |  |  |  |  |  |  | Other |  |  |
| 1.9 | Type of hull: |  |  |  |  |  |  |  |  |  |  | Double Hull |  |  |
| Classification |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1.10 | Classification society: |  |  |  |  |  |  |  |  |  |  | Det Norske Veritas |  |  |
| 1.11 | Class notation: |  |  |  |  |  |  |  |  |  |  | +1A1 CSR Tanker for Chemicals and Oil Products ESP SPM E0 VCS-2 TMON CLEAN BIS |  |  |
| 1.12 | Is the vessel subject to any conditions of class, class extensions, outstanding memorandums or class recommendations? If yes, give details: |  |  |  |  |  |  |  |  |  |  | No N/A |  |  |
| 1.13 | If classification society changed, name of previous and date of change: |  |  |  |  |  |  |  |  |  |  | NOT APPLICABLE, Not Applicable |  |  |
| 1.14 | IMO type, if applicable: |  |  |  |  |  |  |  |  |  |  | 2,3 |  |  |
| 1.15 | Does the vessel have ice class? If yes, state what level: |  |  |  |  |  |  |  |  |  |  | No, N/A |  |  |
| 1.16 | Date / place of last dry-dock: |  |  |  |  |  |  |  |  |  |  | Jul 07, 2014 / New building |  |  |
| 1.17 | Date next dry dock due / next annual survey due: |  |  |  |  |  |  |  |  |  |  | Jul 07, 2017 |  | Jul 07, 2016 |
| 1.18 | Date of last special survey / next special survey due: |  |  |  |  |  |  |  |  |  |  | Jul 07, 2014 |  | Jul 07, 2019 |
| 1.19 | If ship has Condition Assessment Program (CAP), what is the latest overall rating: |  |  |  |  |  |  |  |  |  |  | No, NOT APPLICABLE |  |  |
| 1.20 | Does the vessel have a statement of compliance issued under the provisions of the Condition Assessment Scheme (CAS): If yes, what is the expiry date? |  |  |  |  |  |  |  |  |  |  | N/A Not Applicable |  |  |
| Dimensions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1.21 | Length overall (LOA): |  |  |  |  |  |  |  |  |  |  | 183.20 Metres |  |  |
| 1.22 | Length between perpendiculars (LBP): |  |  |  |  |  |  |  |  |  |  | 178.50 Metres |  |  |
| 1.23 | Extreme breadth (Beam): |  |  |  |  |  |  |  |  |  |  | 32.31 Metres |  |  |
| 1.24 | Moulded depth: |  |  |  |  |  |  |  |  |  |  | 18.20 Metres |  |  |
| 1.25 | Keel to masthead (KTM)/ Keel to masthead (KTM) in collapsed condition, if applicable: |  |  |  |  |  |  |  |  |  |  | 45.30 Metres |  |  |
| 1.26 | Bow to center manifold (BCM) / Stern to center manifold (SCM): |  |  |  |  |  |  |  |  |  |  | 89.60 Metres |  | 93.60 Metres |
| 1.27 | Distance bridge front to center of manifold: |  |  |  |  |  |  |  |  |  |  | 56 Metres |  |  |
| 1.28 | Parallel body distances |  |  |  |  | Lightship |  |  |  |  |  | Normal Ballast |  | Summer Dwt |
|  | # Forward to mid-point manifold: |  |  |  |  | 25.94 Metres |  |  |  |  |  | 30.42 Metres |  | 30.42 Metres |
|  | # Aft to mid-point manifold: |  |  |  |  | 33.54 Metres |  |  |  |  |  | 59.08 Metres |  | 78.85 Metres |
|  | # Parallel body length: |  |  |  |  | 59.48 Metres |  |  |  |  |  | 89.50 Metres |  | 109.27 Metres |
| # 1.29 | # FWA/TPC at summer draft: |  |  |  |  |  |  |  |  |  |  | 278 Millimetres |  | 55.20 Metric Tonnes |
| 1.30 | Constant (excluding fresh water): |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1.31 | What is the company guidelines for Under Keel Clearance (UKC) for this vessel? |  |  |  |  |  |  |  |  |  |  | 15% of draft at open sea 10%of draft at confined /coastal waters 1,5% of beam at berth |  |  |
| 1.32 | What is the max height of mast above waterline (air draft) |  |  |  |  |  |  |  |  |  |  | Full Mast |  | Collapsed Mast |
|  | Lightship: |  |  |  |  |  |  |  |  |  |  | 39.30 Metres |  | 0 Metres |
|  | Normal ballast: |  |  |  |  |  |  |  |  |  |  | 38.10 Metres |  | 0 Metres |
|  | At loaded summer deadweight: |  |  |  |  |  |  |  |  |  |  | 32.40 Metres |  | 0 Metres |
| Tonnages |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1.33 | Net Tonnage: |  |  |  |  |  |  |  |  |  |  | 14,589 |  |  |
| 1.34 | Gross Tonnage / Reduced Gross Tonnage (if applicable): |  |  |  |  |  |  |  |  |  |  | 29,715 |  |  |
| 1.35 | Suez Canal Tonnage - Gross (SCGT) / Net (SCNT): |  |  |  |  |  |  |  |  |  |  | 30,480 |  | 26,207 |
| # 1.36 | Panama Canal Net Tonnage (PCNT): |  |  |  |  |  |  |  |  |  |  | 23,619 |  |  |
| Ownership and Operation |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1.37 | Registered owner - Full style: |  |  |  |  |  | TENACITY MARINE.INC 80 BROAD STREET MONROVIA LIBERIA Tel: +302106811574 Fax: +302106811436 Telex: 210988 SEA GR Email: info@sea-pioneer.gr Company IMO#: 1599727 |  |  |  |  |  |  |  |
| 1.38 | Technical operator - Full style: |  |  |  |  |  | Sea Pioneer Shipping Corporation 284 Kifisias Avenue, 15232 Halandri, Athens, Greece Tel: +30 210 6811574 Fax: +30 2106811436 Telex: 210988 SEA GR Email: info@sea-pioneer.gr Web: www.sea-pioneer.com Company IMO#: 1599727 |  |  |  |  |  |  |  |
| 1.39 | Commercial operator - Full style: |  |  |  |  |  | Sea Pioneer Shipping Corporation 284 Kifisias Avenue, 15232 Halandri, Athens, Greece Web: www.sea-pioneer.com Tel: +30 210 6811574 Fax: +30 210 6811436 Telex: 210988 SEA GR Email: info@sea-pioneer.gr Web: www.sea-pioneer.com |  |  |  |  |  |  |  |
| 1.40 | Disponent owner - Full style: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2. | CERTIFICATION |  |  |  |  | Issued |  |  |  |  |  | Last Annual |  | Expires |
| 2.1 | Safety Equipment Certificate (SEC): |  |  |  |  | Aug 26, 2014 |  |  |  |  |  | Aug 20, 2015 |  | Jul 07, 2019 |
| 2.2 | Safety Radio Certificate (SRC): |  |  |  |  | Aug 26, 2014 |  |  |  |  |  | Aug 20, 2015 |  | Jul 07, 2019 |
| 2.3 | Safety Construction Certificate (SCC): |  |  |  |  | Aug 26, 2014 |  |  |  |  |  | Aug 20, 2015 |  | Jul 07, 2019 |
| 2.4 | International Loadline Certificate (ILC): |  |  |  |  | Oct 14, 2015 |  |  |  |  |  | Aug 20, 2015 |  | Jul 07, 2019 |
| 2.5 | International Oil Pollution Prevention Certificate (IOPPC): |  |  |  |  | Aug 26, 2014 |  |  |  |  |  | Aug 20, 2015 |  | Jul 07, 2019 |
| 2.6 | ISM Safety Management Certificate (SMC): |  |  |  |  | Apr 02, 2015 |  |  |  |  |  | Dec 11, 2014 |  | Dec 10, 2019 |
| # 2.7 | # Document of Compliance (DOC): |  |  |  |  | # Apr 30, 2012 |  |  |  |  |  | Jul 30, 2015 |  | May 20, 2017 |
| 2.8 | USCG Certificate of Compliance (COC): |  |  |  |  | Dec 08, 2014 |  |  |  |  |  | Dec 28, 2015 |  | Dec 08, 2016 |
| 2.9 | Civil Liability Convention (CLC) 1992 Certificate: |  |  |  |  | Feb 20, 2016 |  |  |  |  |  | Not Applicable |  | Feb 20, 2017 |
| 2.10 | Civil Liability for Bunker Oil Pollution Damage Convention (CLBC) Certificate: |  |  |  |  | Feb 20, 2016 |  |  |  |  |  | Not Applicable |  | Feb 20, 2017 |
| 2.11 | Ship Sanitation Control (SSCC)/Ship Sanitation Control Exemption (SSCE) Certificate: |  |  |  |  |  |  |  |  |  |  | Not Applicable |  |  |
| 2.12 | U.S. Certificate of Financial Responsibility (COFR): |  |  |  |  | Jul 23, 2016 |  |  |  |  |  | Not Applicable |  | Jul 23, 2016 |
| 2.13 | Certificate of Class (COC): |  |  |  |  | Aug 26, 2014 |  |  |  |  |  | Aug 20, 2015 |  | Jul 07, 2019 |
| # 2.14 | # International Sewage Pollution Prevention Certificate (ISPPC): |  |  |  |  | # Aug 20, 2015 |  |  |  |  |  | # Not Applicable |  | Jul 07, 2019 |
| # 2.15 | # Certificate of Fitness (COF): |  |  |  |  | # Jul 07, 2019 |  |  |  |  |  | Aug 20, 2015 |  | Jul 07, 2019 |
| # 2.16 | # International Energy Efficiency Certificate (IEEC): |  |  |  |  | # Not Applicable |  |  |  |  |  | Not Applicable |  | Not Applicable |
| # 2.17 | # International Ship Security Certificate (ISSC): |  |  |  |  | # Apr 02, 2015 |  |  |  |  |  | # Dec 11, 2014 |  | Dec 10, 2019 |
| # 2.18 | # International Air Pollution Prevention Certificate (IAPPC): |  |  |  |  | # Aug 26, 2014 |  |  |  |  |  | Aug 20, 2015 |  | Jul 07, 2019 |
| # 2.19 | # Maritime Labour Certificate (MLC): |  |  |  |  | # Dec 15, 2014 |  |  |  |  |  | Not Applicable |  | Dec 07, 2019 |
| Documentation |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2.20 | Owner warrant that vessel is member of ITOPF and will remain so for the entire duration of this voyage/contract: |  |  |  |  |  |  |  |  |  |  | Yes |  |  |
| 2.21 | Does vessel have in place a Drug and Alcohol Policy complying with OCIMF guidelines for Control of Drugs and Alcohol Onboard Ship? |  |  |  |  |  |  |  |  |  |  | Yes |  |  |
| 2.22 | Is the ITF Special Agreement on board (if applicable)? |  |  |  |  |  |  |  |  |  |  | N/A |  |  |
| 2.23 | ITF Blue Card expiry date: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3. | CREW |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3.1 | Nationality of Master: |  |  |  |  |  |  |  |  |  |  | Filipino |  |  |
| 3.2 | Number and Nationality of Officers: |  |  |  |  |  |  |  |  |  |  | 9 Filipino |  |  |
| 3.3 | Number and Nationality of Crew: |  |  |  |  |  |  |  |  |  |  | 14 PHILLIPINO |  |  |
| 3.4 | What is the common working language onboard: |  |  |  |  |  |  |  |  |  |  | ENGLISH |  |  |
| 3.5 | Do officers speak and understand English? |  |  |  |  |  |  |  |  |  |  | Yes |  |  |
| 3.6 | If Officers/Crew employed by a Manning Agency - Full style: |  |  |  |  |  | Officers: Adamson Phil Inc 11F Sage House, 110 V. A. Rufino St.,Legaspi Village, MCPO Box 1707 Makati City, Philippines 1229 Tel: +63 2 816 2316/0794 Fax: +63 2 815 4572 Email: adamsonphil@eastern.com.ph Web: www.manship.com Crew: Adamson Phil Inc 11F Sage House, 110 V. A. Rufino St.,Legaspi Village, MCPO Box 1707 Makati City, Philippines 1229 Tel: +63 2 816 2316/0794 Fax: +63 2 815 4572 Email: adamsonphil@eastern.com.ph Web: www.adamsonphil.com |  |  |  |  |  |  |  |
| 4. | FOR USA CALLS |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4.1 | Has the vessel Operator submitted a Vessel Spill Response Plan to the US Coast Guard which has been approved by official USCG letter? |  |  |  |  |  |  |  |  |  |  |  | Yes |  |
| 4.2 | Qualified individual (QI) - Full style: |  |  |  |  |  | Hudson Marine Management Service 2 Aquarium Drive, New Jersey, USA, 08103 Tel: +1 856 342 7500 Fax: +1 856 342 8888 Email: info@hudsonmarine.com Web: www.hmms-usa.com |  |  |  |  |  |  |  |
| 4.3 | Oil Spill Response Organization (OSRO) - Full style: |  |  |  |  |  | Marine Spill Response Corporation Marine Spill Response Corporation 220 Spring St, Suite 500 Herndon, VA 20170 Tel: (703) 326-5617 Fax: N/A Email: roos@msrc.org |  |  |  |  |  |  |  |
| 5. | CARGO AND BALLAST HANDLING |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Double Hull Vessels |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.1 | Is vessel fitted with centerline bulkhead in all cargo tanks? If Yes, solid or perforated: |  |  |  |  |  |  |  |  |  |  | Yes, Solid |  |  |
| # Loadline Information |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| # 5.2 | # Loadline |  | # Freeboard |  |  | # Draft |  |  |  |  | # Deadweight |  |  | # Displacement |
|  | # Summer: |  | 5.313 Metres |  |  | 12.90 Metres |  |  |  |  | # 50,413 Metric Tonnes |  |  | 61,331 Metric Tonnes |
|  | # Winter: |  | 5,582 Metres |  |  | 12.63 Metres |  |  |  |  | 48,928 Metric Tonnes |  |  | 59,846 Metric Tonnes |
|  | # Tropical: |  | 5,044 Metres |  |  | 13.17 Metres |  |  |  |  | 51,898 Metric Tonnes |  |  | 62,816 Metric Tonnes |
|  | # Lightship: |  | 15,173 Metres |  |  | 3.04 Metres |  |  |  |  | Not Applicable |  |  | 10,918 Metric Tonnes |
|  | # Normal Ballast Condition: |  | 10.313 Metres |  |  | 7.90 Metres |  |  |  |  | 23,530 Metric Tonnes |  |  | 34,448 Metric Tonnes |
| # 5.3 | # Does vessel have multiple SDWT? If yes, please provide all assigned loadlines: |  |  |  |  |  |  |  |  |  | No |  |  |  |
| Cargo Tank Capacities |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.4 | Number of cargo tanks and total cubic capacity (98%): |  |  |  |  |  |  |  |  |  |  | 10 |  | 46,316.098 Cu. Metres |
| 5.5 | Capacity (98%) of each natural segregation with double valve (specify tanks): |  |  |  |  |  |  |  |  |  |  | Seg#1: 3502.160 m3 (COT1 P) Seg#2: 3513.996 m3 (COT1 S) Seg#3: 4814.094 m3 (COT2 P) Seg#4: 4819.800 m3 (COT2 S) Seg#5: 4966.896 m3 (COT3 P) Seg#6: 4968.394 m3 (COT3 S) Seg#7: 4964.815 m3 (COT4 P) Seg#8: 4963.349 m3 (COT4 S) Seg#9: 4895.828 m3 (COT5 P) Seg#10: 4906.766 m3 (COT5 P) Seg#11: 3540.618 m3 (SLOP (P)) Seg#12: 3446.163 m3 (SLOP (S)) |  |  |
| 5.6 | Number of slop tanks and total cubic capacity (98%): |  |  |  |  |  |  |  |  |  |  | 2 |  | 6,986.761 Cu. Metres |
| 5.7 | Specify segregations which slops tanks belong to and their capacity with double valve: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.8 | Residual/Retention oil tank(s) capacity (98%), if applicable: |  |  |  |  |  |  |  |  |  |  | 508.388 Cu. Metres |  |  |
| 5.9 | Does vessel have Segregated Ballast Tanks (SBT) or Clean Ballast Tanks (CBT): |  |  |  |  |  |  |  |  |  |  | SBT |  |  |
| SBT Vessels |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.10 | What is total SBT capacity and percentage of SDWT vessel can maintain? |  |  |  |  |  |  |  |  |  |  | 21,216.57 Cu. Metres |  | 42.08 % |
| 5.11 | Does vessel meet the requirements of MARPOL Annex I Reg 18.2: |  |  |  |  |  |  |  |  |  |  | Yes |  |  |
| Cargo Handling and Pumping Systems |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.12 | How many grades/products can vessel load/discharge with double valve segregation: |  |  |  |  |  |  |  |  |  |  | 6 |  |  |
| 5.13 | Are there any cargo tank filling restrictions? If yes, specify number of slack tanks, max s.g., ullage restrictions etc.: |  |  |  |  |  |  |  |  |  |  | Yes 550m3/h,98% |  |  |
| 5.14 | Pumps |  |  |  | No. | Type |  |  |  |  |  | Capacity |  | At What Head (sg=1.0) |
|  | Cargo Pumps: |  |  |  | 10 | vertical centrif electric deepwell pp |  |  |  |  |  | 550 M3/HR |  |  |
|  | Cargo Eductors: |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Stripping: |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Ballast Pumps: |  |  |  | 2 | Electrically driven, , centrifugal, deep well type |  |  |  |  |  |  |  | 2.80 Metres |
|  | Ballast Eductors: |  |  |  | 1 | Water driven |  |  |  |  |  |  |  |  |
| 5.15 | Max loading rate for homogenous cargo per manifold connection: |  |  |  |  |  |  |  |  |  |  | 1,580 Cu. Metres/Hour |  |  |
| 5.16 | Max loading rate for homogenous cargo loaded simultaneously through all manifolds: |  |  |  |  |  |  |  |  |  |  | 3,300 Cu. Metres/Hour |  |  |
| 5.17 | How many cargo pumps can be run simultaneously at full capacity: |  |  |  |  |  |  |  |  |  |  | 6 |  |  |
| Cargo Control Room |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.18 | Is ship fitted with a Cargo Control Room (CCR)? |  |  |  |  |  |  |  |  |  |  | Yes |  |  |
| 5.19 | Can tank innage / ullage be read from the CCR? |  |  |  |  |  |  |  |  |  |  | Yes |  |  |
| Gauging and Sampling |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.20 | Can cargo be transferred under closed loading conditions in accordance with ISGOTT 11.1.6.6? |  |  |  |  |  |  |  |  |  |  | Yes |  |  |
| 5.21 | What type of fixed closed tank gauging system is fitted: |  |  |  |  |  |  |  |  |  |  | TANK RADAR |  |  |
| 5.22 | Number of portable gauging units (example- MMC) on board: |  |  |  |  |  |  |  |  |  |  | 4 |  |  |
| 5.23 | Are overfill (high) alarms fitted? If Yes, indicate whether to all tanks or partial: |  |  |  |  |  |  |  |  |  |  | Yes, All |  |  |
| 5.24 | Are cargo tanks fitted with multipoint gauging? If yes, specify type and locations: |  |  |  |  |  |  |  |  |  |  | Yes, |  |  |
| 5.25 | Is gauging system certified and calibrated? If no, specify which ones are not calibrated: |  |  |  |  |  |  |  |  |  |  | Yes, |  |  |
| Vapor Emission Control System (VECS) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.26 | Is a Vapour Emission Control System (VECS) fitted? |  |  |  |  |  |  |  |  |  |  | Yes |  |  |
| 5.27 | Number/size of VECS manifolds (per side): |  |  |  |  |  |  |  |  |  |  | 2 |  | 300 Millimetres |
| 5.28 | Number / size / type of VECS reducers: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Venting |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.29 | State what type of venting system is fitted: |  |  |  |  |  |  |  |  |  |  | P/V VALVES |  |  |
| Cargo Manifolds and Reducers |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.30 | Does vessel comply with the latest edition of the OCIMF 'Recommendations for Oil Tanker Manifolds and Associated Equipment’? |  |  |  |  |  |  |  |  |  |  | Yes |  |  |
| 5.31 | Total number / size of cargo manifold connections on each side: |  |  |  |  |  |  |  |  |  |  | 6 / 350 Millimetres |  |  |
| 5.32 | What type of valves are fitted at manifold: |  |  |  |  |  |  |  |  |  |  | BUTTERFLY VALVE |  |  |
| 5.33 | What is the material/rating of the manifold: |  |  |  |  |  |  |  |  |  |  | CARBON STEEL / |  |  |
| 5.34 | Does the vessel have a Common Line Manifold connection? If yes, describe: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.35 | Distance between cargo manifold centers: |  |  |  |  |  |  |  |  |  |  | 2,000 Millimetres |  |  |
| 5.36 | Distance ships rail to manifold: |  |  |  |  |  |  |  |  |  |  | 4,600 Millimetres |  |  |
| 5.37 | Distance manifold to ships side: |  |  |  |  |  |  |  |  |  |  | 5,100 Millimetres |  |  |
| 5.38 | Top of rail to center of manifold: |  |  |  |  |  |  |  |  |  |  | 1,050 Millimetres |  |  |
| 5.39 | Distance main deck to center of manifold: |  |  |  |  |  |  |  |  |  |  | 2,100 Millimetres |  |  |
| 5.40 | Spill tank grating to center of manifold: |  |  |  |  |  |  |  |  |  |  | 900 Millimetres |  |  |
| 5.41 | Manifold height above the waterline in normal ballast / at SDWT condition: |  |  |  |  |  |  |  |  |  |  | 12.41 Metres |  | 7.40 Metres |
| 5.42 | Number / size / type of reducers: |  |  |  |  |  |  |  |  |  |  | 10 x 355/406mm (14/16") 4 x 355/305mm (14/12") 4 x 355/254mm (14/10") 4 x 355/203mm (14/8") 2 x 254/406mm (10/16") |  |  |
| 5.43 | Is vessel fitted with a stern manifold? If yes, state size: |  |  |  |  |  |  |  |  |  |  | No, |  |  |
| Heating |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.44 | Cargo / slop tanks fitted with a cargo heating system? |  |  |  |  |  |  | Type |  |  |  | Coiled |  | Material |
|  | Cargo Tanks: |  |  |  |  |  |  | steam deck heaters except slop tanks with coils |  |  |  | No |  | SS |
|  | Slop Tanks: |  |  |  |  |  |  | heating coils |  |  |  | Yes |  |  |
| 5.45 | Maximum temperature cargo can be loaded / maintained: |  |  |  |  |  |  |  |  |  |  | 65.0 °C / 149.0 °F |  | 60 °C / 140 °F |
| 5.46 | Minimum temperature cargo can be loaded / maintained: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Coating / Anodes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.47 | Tank Coating |  |  | Coated |  | Type |  |  |  |  |  | To What Extent |  | Anodes |
|  | Cargo tanks: |  |  | Yes |  | PHENOLIC EPOXY |  |  |  |  |  | ALL |  | No |
|  | Ballast tanks: |  |  | Yes |  | EPOXY |  |  |  |  |  | ALL |  | Yes |
|  | Slop tanks: |  |  | Yes |  | EPOXY PHENOLIC |  |  |  |  |  | Whole Tank |  | No |
| 6. | INERT GAS AND CRUDE OIL WASHING |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6.1 | Is a Crude Oil Washing (COW) installation fitted / operational? |  |  |  |  |  |  |  |  |  |  | Yes / |  |  |
| 6.2 | Is an Inert Gas System (IGS) fitted / operational? |  |  |  |  |  |  |  |  |  |  | Yes / Yes |  |  |
| 6.3 | Is IGS supplied by flue gas, inert gas (IG) generator and/or nitrogen: |  |  |  |  |  |  |  |  |  |  | IG Generator |  |  |
| 7. | MOORING |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7.1 | Wires (on drums) | No. |  | Diameter |  | Material |  |  |  |  |  | Length |  | Breaking Strength |
|  | Forecastle: |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Main deck fwd: |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Main deck aft: |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Poop deck: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7.2 | Wire tails | No. |  | Diameter |  | Material |  |  |  |  |  | Length |  | Breaking Strength |
|  | Forecastle: |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Main deck fwd: |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Main deck aft: |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Poop deck: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7.3 | Ropes (on drums) | No. |  | Diameter |  | Material |  |  |  |  |  | Length |  | Breaking Strength |
|  | Forecastle: | 4 |  | 56 Millimetres |  | 24 STRAND BRAIDED IMPROVED NIKASTEEL(EXTERNALLY MIXWD POLYESTER (40%) NIKASTEEL (60%) |  |  |  |  |  | 300 Metres |  | 53 Metric Tonnes |
|  | Main deck fwd: | 2 |  | 56 Millimetres |  | 24 STRAND BRAIDED IMPROVED NIKASTEEL(EXTERNALLY MIXWD POLYESTER (40%) NIKASTEEL (60%) |  |  |  |  |  | 300 Metres |  | 53 Metric Tonnes |
|  | Main deck aft: | 2 |  | 56 Millimetres |  | 24 STRAND BRAIDED IMPROVED NIKASTEEL(EXTERNALLY MIXWD POLYESTER (40%) NIKASTEEL (60%) |  |  |  |  |  | 300 Metres |  | 53 Metric Tonnes |
|  | Poop deck: | 4 |  | 56 Millimetres |  | 24 STRAND BRAIDED IMPROVED NIKASTEEL(EXTERNALLY MIXWD POLYESTER (40%) NIKASTEEL (60%) |  |  |  |  |  | 300 Metres |  | 53 Metric Tonnes |
| 7.4 | Other lines | No. |  | Diameter |  | Material |  |  |  |  |  | Length |  | Breaking Strength |
|  | Forecastle: | 1 |  | 64 Millimetres |  | 24 STRAND BRAIDED IMPROVED NIKASTEEL(EXTERNALLY MIXWD POLYESTER (40%) NIKASTEEL (60%) |  |  |  |  |  | 220 Metres |  | 63 Metric Tonnes |
|  | Main deck fwd: | 4 |  | 56 Millimetres |  | 24 strand polyester 40 % nikassteel 60% |  |  |  |  |  | 300 Metres |  | 53 Metric Tonnes |
|  | Main deck aft: | 1 |  | 64 Millimetres |  | 24 STRAND BRAIDED IMPROVED NIKASTEEL(EXTERNALLY MIXWD POLYESTER (40%) NIKASTEEL (60%) |  |  |  |  |  | 220 Metres |  | 63 Metric Tonnes |
|  | Poop deck: | 1 |  | 64 Millimetres |  | 24 strand polyester 40% nikasteel 60% |  |  |  |  |  | 220 Metres |  | 63 Metric Tonnes |
| 7.5 | Winches | No. |  | No. Drums |  | Motive Power |  |  |  |  |  | Brake Capacity |  | Type of Brake |
|  | Forecastle: | 2 |  | dbl drum |  | Hydraulic |  |  |  |  |  | 42.40 Metric Tonnes |  | manual |
|  | Main deck fwd: | 1 |  | dbl drum |  | Hydraulic |  |  |  |  |  | 42.40 Metric Tonnes |  | manual |
|  | Main deck aft: | 1 |  | dbl drum |  | Hydraulic |  |  |  |  |  | 42.40 Metric Tonnes |  | manual |
|  | Poop deck: | 2 |  | dbl drum |  | Hydraulic |  |  |  |  |  | 42.40 Metric Tonnes |  | manual |
| 7.6 | Bitts, closed chocks/fairleads |  |  | No. Bitts |  | SWL Bitts |  |  |  |  |  | No. Closed Chocks |  | SWL Closed Chocks |
|  | Forecastle: |  |  | 4 |  | 64 Metric Tonnes |  |  |  |  |  | 13 |  | 64 Metric Tonnes |
|  | Main deck fwd: |  |  | 2 |  | 64 Metric Tonnes |  |  |  |  |  | 6 |  | 53 Metric Tonnes |
|  | Main deck aft: |  |  | 2 |  | 64 Metric Tonnes |  |  |  |  |  | 6 |  | 53 Metric Tonnes |
|  | Poop deck: |  |  | 8 |  | 64 Metric Tonnes |  |  |  |  |  | 13 |  | 64 Metric Tonnes |
| Anchors/Emergency Towing System |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7.7 | Number of shackles on port / starboard cable: |  |  |  |  |  |  |  |  |  |  | 12 / 11 |  |  |
| 7.8 | Type / SWL of Emergency Towing system forward: |  |  |  |  |  |  |  |  |  |  | TONGUE TYPE |  | 200 Metric Tonnes |
| 7.9 | Type / SWL of Emergency Towing system aft: |  |  |  |  |  |  |  |  |  |  | FAIRLEAD STRONG POINT |  | 200 Metric Tonnes |
| Escort Tug |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7.10 | What is size / SWL of closed chock and/or fairleads of enclosed type on stern: |  |  |  |  |  |  |  |  |  |  | 400x260 |  | 64 Metric Tonnes |
| 7.11 | What is SWL of bollard on poop deck suitable for escort tug: |  |  |  |  |  |  |  |  |  |  | 64 Metric Tonnes |  |  |
| Bow/Stern Thruster |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7.12 | What is brake horse power of bow thruster (if fitted): |  |  |  |  |  |  |  |  |  |  | No, |  |  |
| 7.13 | What is brake horse power of stern thruster (if fitted): |  |  |  |  |  |  |  |  |  |  | N/A, |  |  |
| Single Point Mooring (SPM) Equipment |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7.14 | Does the vessel meet the recommendations in the latest edition of OCIMF 'Recommendations for Equipment Employed in the Bow Mooring of Conventional Tankers at Single Point Moorings (SPM)’? |  |  |  |  |  |  |  |  |  |  | Yes |  |  |
| 7.15 | If fitted, how many chain stoppers: |  |  |  |  |  |  |  |  |  |  | 1 |  |  |
| 7.16 | State type / SWL of chain stopper(s): |  |  |  |  |  |  |  |  |  |  | tongue |  | 200 Metric Tonnes |
| 7.17 | What is the maximum size chain diameter the bow stopper(s) can handle: |  |  |  |  |  |  |  |  |  |  | 76 Millimetres |  |  |
| 7.18 | Distance between the bow fairlead and chain stopper/bracket: |  |  |  |  |  |  |  |  |  |  | 3,600 Millimetres |  |  |
| 7.19 | Is bow chock and/or fairlead of enclosed type of OCIMF recommended size (600mm x 450mm)? If not, give details of size: |  |  |  |  |  |  |  |  |  |  | Yes |  |  |
| Lifting Equipment |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7.20 | Derrick / Crane description (Number, SWL and location): |  |  |  |  |  |  |  |  |  |  | Cranes: 1 x 10 Tonnes CENTER |  |  |
| 7.21 | What is maximum outreach of cranes / derricks outboard of the ship’s side: |  |  |  |  |  |  |  |  |  |  | 6 Metres (22-5.9) |  |  |
| Ship To Ship Transfer (STS) / Helicopter Operations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7.22 | Does vessel comply with recommendations contained in OCIMF/ICS Ship To Ship Transfer Guide (Petroleum, Chemicals or Liquified Gas, as applicable)? |  |  |  |  |  |  |  |  |  |  | Yes |  |  |
| 7.23 | Can the ship comply with the ICS Helicopter Guidelines? If Yes, state whether winching or landing area provided and diameter of the circle provided: |  |  |  |  |  |  |  |  |  |  | Yes, Winching 11.90 Metres |  |  |
| 8. | MISCELLANEOUS |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Engine |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.1 | Speed |  |  |  |  |  |  |  |  |  |  | Maximum |  | Economic |
|  | Ballast speed: |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Laden speed: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.2 | What type of fuel is used for main propulsion / generating plant: |  |  |  |  |  |  |  |  |  |  | HFO.DO.MGO.LSHFO. |  | HFO.DO.MGO.LSHFO |
| 8.3 | Type / Capacity of bunker tanks: |  |  |  |  |  |  |  |  |  |  | Fuel Oil: 1,473.351 Cu. Metres Diesel Oil: 289.261 Cu. Metres Gas Oil: 290.083 Cu. Metres |  |  |
| 8.4 | Is vessel fitted with fixed or controllable pitch propeller(s): |  |  |  |  |  |  |  |  |  |  | Fixed |  |  |
| 8.5 | Engines |  |  |  |  |  |  |  |  | No |  | Capacity |  | Make/Type |
|  | Main engine: |  |  |  |  |  |  |  |  | 1 |  |  |  |  |
|  | Aux engine: |  |  |  |  |  |  |  |  | 3 |  |  |  |  |
|  | Power packs: |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Boilers: |  |  |  |  |  |  |  |  | 1 |  | 25 Metric Tonnes/Hour |  |  |
| Emissions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.6 | Main engine IMO NOx emission standard: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.7 | Energy Efficiency Design Index (EEDI) rating number: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Insurance |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.8 | P & I Club - Full Style: |  |  |  |  |  |  |  | WEST OF ENGLAND WEST OF ENGLAND INSURANCE SERVICES TOWER BRIDGE COURT 226 TOWER BRIDGE ROAD LONDON SE1 2 UP Tel: +(44)(0)2077166000 Fax: +(44)(0)2077166100 Email: mail@westpandi.com Web: http://www.westpandi.com/ |  |  |  |  |  |
| 8.9 | P & I Club pollution liability coverage / expiration date: |  |  |  |  |  |  |  |  |  |  | 1,000,000,000 US$ |  | Feb 20, 2017 |
| 8.10 | Hull & Machinery insured by - Full Style: |  |  |  |  |  |  |  | HULL LLOYDS CONSORTIUM |  |  |  |  |  |
| 8.11 | Hull & Machinery insured value / expiration date: |  |  |  |  |  |  |  |  |  |  | 45,000,000 US$ |  | Oct 26, 2016 |
| Recent Operational History |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.12 | Date and place of last Port State Control inspection: |  |  |  |  |  |  |  |  |  |  | Dec 28, 2015 / texas city |  |  |
| 8.13 | Any outstanding deficiencies as reported by any Port State Control? If yes, provide details: |  |  |  |  |  |  |  |  |  |  | No NO |  |  |
| 8.14 | Has vessel been involved in a pollution, grounding, serious casualty or collision incident during the past 12 months? If yes, full description: |  |  |  |  |  |  |  |  |  |  | Pollution: No, Grounding: No, Casualty: No, NOT APPLICABLE Collision: No, NOT APPLICABLE |  |  |
| 8.15 | Last three cargoes / charterers / voyages (Last / 2nd Last / 3rd Last): |  |  |  |  |  |  |  |  |  |  | last Gasoil from Vladivostok Rusia To Yosu S.korea 2nd Last : Ethanol from Texas City to Dalian and Yizheng china 3rd Last ULSD /PMI load Valero Port arthur Discharge Pajaritos and Madero Mexico discharge Touxpan MEXICO |  |  |
| 8.16 | Date/place of last STS operation: |  |  |  |  |  |  |  |  |  |  | GOLA USA JAN2016 |  |  |
| Vetting |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.17 | Date of last SIRE inspection: |  |  |  |  |  |  |  |  |  |  | Feb 19, 2016 |  |  |
| 8.18 | Date of last CDI inspection: |  |  |  |  |  |  |  |  |  |  | N/A |  |  |
| 8.19 | Recent Oil company inspections/screenings (To the best of owners knowledge and without guarantee of acceptance for future business)*: * "Approvals" are not given by Oil Majors and ships are accepted for the voyage on a case by case basis. |  |  |  |  |  |  |  |  |  |  | EXXON ,CHEVRON,STATOIL,Phillips66, LUKOIL, REPSOL, OMV, KOCH, |  |  |
| Additional Information |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.20 | Additional information relating to features of the ship or operational characteristics: |  |  |  |  |  |  |  |  |  |  |  |  |  |

Rev 2015 (INTERTANKO / Q88.com)

Form completed on http://www.q88.com/integration.aspx  Please email support@q88.com an updated copy if this is not the latest version.