- INTERTANKO TANKER CHARTERING QUESTIONNAIRE 88 	Version 4

| 1. | VESSEL DESCRIPTION |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1.1 | Date updated: |  |  |  |  |  |  |  |  |  |  | Jul 22, 2016 |  |  |
| 1.2 | Vessel’s name (IMO number): |  |  |  |  |  |  |  |  |  |  | Nissos Kythnos (9592252) |  |  |
| 1.3 | Vessel’s previous name(s) and date(s) of change: |  |  |  |  |  |  |  |  |  |  | Not Applicable |  |  |
| 1.4 | Date delivered / Builder (where built): |  |  |  |  |  |  |  |  |  |  | Sep 23, 2011 / SHI (Samsung Heavy Industries), Korea |  |  |
| 1.5 | Flag / Port of Registry: |  |  |  |  |  |  |  |  |  |  | Greece / Piraeus |  |  |
| 1.6 | Call sign / MMSI: |  |  |  |  |  |  |  |  |  |  | SVBI9 / 241132000 |  |  |
| 1.7 | Vessel’s contact details (satcom/fax/email etc.): |  |  |  |  |  |  |  |  |  |  | Tel: +870773230217 Fax: +870783150632 Email: nkythnos@kykmar.gr |  |  |
| 1.8 | Type of vessel (as described in Form A or Form B Q1.11 of the IOPPC): |  |  |  |  |  |  |  |  |  |  | Oil Tanker |  |  |
| 1.9 | Type of hull: |  |  |  |  |  |  |  |  |  |  | Double Hull |  |  |
| Classification |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1.10 | Classification society: |  |  |  |  |  |  |  |  |  |  | American Bureau of Shipping |  |  |
| 1.11 | Class notation: |  |  |  |  |  |  |  |  |  |  | +A1, Oil Carrier, E, +AMS, +ACCU, VEC, TCM, AB-CM, CSR, GP, POT, PMA, RRDA, ESP, UWILD, CPS, CRC, RW. |  |  |
| 1.12 | Is the vessel subject to any conditions of class, class extensions, outstanding memorandums or class recommendations? If yes, give details: |  |  |  |  |  |  |  |  |  |  | No N/A |  |  |
| 1.13 | If classification society changed, name of previous and date of change: |  |  |  |  |  |  |  |  |  |  | , Not Applicable |  |  |
| 1.14 | IMO type, if applicable: |  |  |  |  |  |  |  |  |  |  | N/A |  |  |
| 1.15 | Does the vessel have ice class? If yes, state what level: |  |  |  |  |  |  |  |  |  |  | No, N/A |  |  |
| 1.16 | Date / place of last dry-dock: |  |  |  |  |  |  |  |  |  |  | Jul 02, 2016 / N/A |  |  |
| 1.17 | Date next dry dock due / next annual survey due: |  |  |  |  |  |  |  |  |  |  | Sep 22, 2021 |  | Sep 22, 2017 |
| 1.18 | Date of last special survey / next special survey due: |  |  |  |  |  |  |  |  |  |  | Jul 02, 2016 |  | Sep 22, 2021 |
| 1.19 | If ship has Condition Assessment Program (CAP), what is the latest overall rating: |  |  |  |  |  |  |  |  |  |  | No, |  |  |
| 1.20 | Does the vessel have a statement of compliance issued under the provisions of the Condition Assessment Scheme (CAS): If yes, what is the expiry date? |  |  |  |  |  |  |  |  |  |  | N/A Not Applicable |  |  |
| Dimensions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1.21 | Length overall (LOA): |  |  |  |  |  |  |  |  |  |  | 248.97 Metres |  |  |
| 1.22 | Length between perpendiculars (LBP): |  |  |  |  |  |  |  |  |  |  | 239.00 Metres |  |  |
| 1.23 | Extreme breadth (Beam): |  |  |  |  |  |  |  |  |  |  | 43.80 Metres |  |  |
| 1.24 | Moulded depth: |  |  |  |  |  |  |  |  |  |  | 21.00 Metres |  |  |
| 1.25 | Keel to masthead (KTM)/ Keel to masthead (KTM) in collapsed condition, if applicable: |  |  |  |  |  |  |  |  |  |  | 48.30 Metres |  |  |
| 1.26 | Bow to center manifold (BCM) / Stern to center manifold (SCM): |  |  |  |  |  |  |  |  |  |  | 125.30 Metres |  | 123.50 Metres |
| 1.27 | Distance bridge front to center of manifold: |  |  |  |  |  |  |  |  |  |  | 82.10 Metres |  |  |
| 1.28 | Parallel body distances |  |  |  |  | Lightship |  |  |  |  |  | Normal Ballast |  | Summer Dwt |
|  | # Forward to mid-point manifold: |  |  |  |  | 39.10 Metres |  |  |  |  |  | 62.20 Metres |  | 62.20 Metres |
|  | # Aft to mid-point manifold: |  |  |  |  | 29.50 Metres |  |  |  |  |  | 54.50 Metres |  | 74.60 Metres |
|  | # Parallel body length: |  |  |  |  | 68.60 Metres |  |  |  |  |  | 116.70 Metres |  | 136.80 Metres |
| # 1.29 | # FWA/TPC at summer draft: |  |  |  |  |  |  |  |  |  |  | 339.00 Millimetres |  | 99.10 Metric Tonnes |
| 1.30 | Constant (excluding fresh water): |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1.31 | What is the company guidelines for Under Keel Clearance (UKC) for this vessel? |  |  |  |  |  |  |  |  |  |  | 1.00m in approaches, 0.50m alongside, 10.0m in open sea navigation, SPM or CBM systems. |  |  |
| 1.32 | What is the max height of mast above waterline (air draft) |  |  |  |  |  |  |  |  |  |  | Full Mast |  | Collapsed Mast |
|  | Lightship: |  |  |  |  |  |  |  |  |  |  | 45.87 Metres |  | 0 Metres |
|  | Normal ballast: |  |  |  |  |  |  |  |  |  |  | 41.29 Metres |  | 0 Metres |
|  | At loaded summer deadweight: |  |  |  |  |  |  |  |  |  |  | 33.28 Metres |  | 0 Metres |
| Tonnages |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1.33 | Net Tonnage: |  |  |  |  |  |  |  |  |  |  | 35,877.00 |  |  |
| 1.34 | Gross Tonnage / Reduced Gross Tonnage (if applicable): |  |  |  |  |  |  |  |  |  |  | 61,242.00 |  | 48,962.00 |
| 1.35 | Suez Canal Tonnage - Gross (SCGT) / Net (SCNT): |  |  |  |  |  |  |  |  |  |  | 62,872.42 |  | 56,982.14 |
| # 1.36 | Panama Canal Net Tonnage (PCNT): |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Ownership and Operation |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1.37 | Registered owner - Full style: |  |  |  |  |  | KYTHNOS MARINE CORP 80 Broad Str., Monrovia, Liberia Tel: C/O Kyklades Maritim Fax: C/O Kyklades Maritim Telex: Not Applicable Email: C/O Kyklades Maritime Corporation Web: C/O Kyklades Maritime Corporation Company IMO#: 4162871 |  |  |  |  |  |  |  |
| 1.38 | Technical operator - Full style: |  |  |  |  |  | Kyklades Maritime Corporation Ethnarchou Makariou Av. and 2, D. Falireos str., Neo Faliro, Piraeus, 18547, Greece. Tel: +302104804200 Fax: +302104818210/+30210 Telex: Not Applicable Email: ihsqe@kykmar.gr Web: www.kykmar.gr Company IMO#: 1153018 |  |  |  |  |  |  |  |
| 1.39 | Commercial operator - Full style: |  |  |  |  |  | Kyklades Maritime Corporation Ethnarchou Makariou Av. and 2, D. Falireos str., Neo Faliro, Piraeus, 18547, Greece. Tel: +302104804200 Fax: +302104818210 Telex: Not Applicable Email: operation@kykmar.gr Web: www.kykmar.gr |  |  |  |  |  |  |  |
| 1.40 | Disponent owner - Full style: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2. | CERTIFICATION |  |  |  |  | Issued |  |  |  |  |  | Last Annual |  | Expires |
| 2.1 | Safety Equipment Certificate (SEC): |  |  |  |  | Jul 02, 2016 |  |  |  |  |  | Jul 02, 2016 |  | Sep 22, 2021 |
| 2.2 | Safety Radio Certificate (SRC): |  |  |  |  | Jul 02, 2016 |  |  |  |  |  | Jul 02, 2016 |  | Sep 22, 2021 |
| 2.3 | Safety Construction Certificate (SCC): |  |  |  |  | Jul 02, 2016 |  |  |  |  |  | Jul 02, 2016 |  | Sep 22, 2021 |
| 2.4 | International Loadline Certificate (ILC): |  |  |  |  | Jul 02, 2016 |  |  |  |  |  | Jul 02, 2016 |  | Sep 22, 2021 |
| 2.5 | International Oil Pollution Prevention Certificate (IOPPC): |  |  |  |  | Jul 02, 2016 |  |  |  |  |  | Jul 02, 2016 |  | Sep 22, 2021 |
| 2.6 | ISM Safety Management Certificate (SMC): |  |  |  |  | Jan 25, 2012 |  |  |  |  |  | Nov 26, 2014 |  | Jan 25, 2017 |
| # 2.7 | # Document of Compliance (DOC): |  |  |  |  | # Nov 29, 2012 |  |  |  |  |  | Apr 15, 2016 |  | Jan 31, 2018 |
| 2.8 | USCG Certificate of Compliance (COC): |  |  |  |  | Oct 28, 2015 |  |  |  |  |  | Not Applicable |  | Oct 28, 2017 |
| 2.9 | Civil Liability Convention (CLC) 1992 Certificate: |  |  |  |  | Feb 20, 2016 |  |  |  |  |  | Not Applicable |  | Feb 20, 2017 |
| 2.10 | Civil Liability for Bunker Oil Pollution Damage Convention (CLBC) Certificate: |  |  |  |  | Feb 20, 2016 |  |  |  |  |  | Not Applicable |  | Feb 20, 2017 |
| 2.11 | Ship Sanitation Control (SSCC)/Ship Sanitation Control Exemption (SSCE) Certificate: |  |  |  |  | Jun 30, 2016 |  |  |  |  |  | Not Applicable |  | Dec 30, 2016 |
| 2.12 | U.S. Certificate of Financial Responsibility (COFR): |  |  |  |  | Aug 25, 2014 |  |  |  |  |  | Not Applicable |  | Aug 25, 2017 |
| 2.13 | Certificate of Class (COC): |  |  |  |  | Jul 30, 2012 |  |  |  |  |  | Jul 02, 2016 |  | Sep 22, 2016 |
| # 2.14 | # International Sewage Pollution Prevention Certificate (ISPPC): |  |  |  |  | # Jul 02, 2016 |  |  |  |  |  | # Not Applicable |  | Sep 22, 2021 |
| # 2.15 | # Certificate of Fitness (COF): |  |  |  |  |  |  |  |  |  |  | Not Applicable |  | Not Applicable |
| # 2.16 | # International Energy Efficiency Certificate (IEEC): |  |  |  |  | # May 26, 2014 |  |  |  |  |  | Not Applicable |  | Not Applicable |
| # 2.17 | # International Ship Security Certificate (ISSC): |  |  |  |  | # Jan 25, 2012 |  |  |  |  |  | # Nov 26, 2014 |  | Jan 25, 2017 |
| # 2.18 | # International Air Pollution Prevention Certificate (IAPPC): |  |  |  |  | # Jul 02, 2016 |  |  |  |  |  | Jul 02, 2016 |  | Sep 22, 2021 |
| # 2.19 | # Maritime Labour Certificate (MLC): |  |  |  |  | # May 30, 2013 |  |  |  |  |  | Not Applicable |  | May 30, 2018 |
| Documentation |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2.20 | Owner warrant that vessel is member of ITOPF and will remain so for the entire duration of this voyage/contract: |  |  |  |  |  |  |  |  |  |  | Yes |  |  |
| 2.21 | Does vessel have in place a Drug and Alcohol Policy complying with OCIMF guidelines for Control of Drugs and Alcohol Onboard Ship? |  |  |  |  |  |  |  |  |  |  | Yes |  |  |
| 2.22 | Is the ITF Special Agreement on board (if applicable)? |  |  |  |  |  |  |  |  |  |  | N/A |  |  |
| 2.23 | ITF Blue Card expiry date: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3. | CREW |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3.1 | Nationality of Master: |  |  |  |  |  |  |  |  |  |  | Greek |  |  |
| 3.2 | Number and Nationality of Officers: |  |  |  |  |  |  |  |  |  |  | 9 Greek, Filipino |  |  |
| 3.3 | Number and Nationality of Crew: |  |  |  |  |  |  |  |  |  |  | 16 Filipino, Greek, Romanian |  |  |
| 3.4 | What is the common working language onboard: |  |  |  |  |  |  |  |  |  |  | ENGLISH |  |  |
| 3.5 | Do officers speak and understand English? |  |  |  |  |  |  |  |  |  |  | Yes |  |  |
| 3.6 | If Officers/Crew employed by a Manning Agency - Full style: |  |  |  |  |  | Officers: Bright Maritime Corporation 3rd Floor Emerald Building 24, Emerald Ave., Ortigas Center, Pasig City, PHILIPPINES Tel: +6326872577-81 Fax: +6326872583 Telex: Not Applicable Email: bright@philonline.com Crew: Bright Maritime Corporation 3rd Floor Emerald Building 24, Emerald Ave., Ortigas Center, Pasig City, PHILIPPINES Tel: +6326872577-81 Fax: +6326872583 Telex: Not Applicable Email: bright@philonline.com |  |  |  |  |  |  |  |
| 4. | FOR USA CALLS |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4.1 | Has the vessel Operator submitted a Vessel Spill Response Plan to the US Coast Guard which has been approved by official USCG letter? |  |  |  |  |  |  |  |  |  |  |  | Yes |  |
| 4.2 | Qualified individual (QI) - Full style: |  |  |  |  |  | O'Brien's Response Management, Inc 186 PrincetoHightstown Rd.,Bldg.3B West Windsor, NJ 08550 USA Tel: +1 985 781 0804 Fax: +1 985 781 0580 Telex: 49617361 OOPS UI Email: commandcenter@obriensrm.com |  |  |  |  |  |  |  |
| 4.3 | Oil Spill Response Organization (OSRO) - Full style: |  |  |  |  |  | National Response Corporation (NRC) 3500 Sunrise Highway, Suite T103, Great River, NY 11739, USA Tel: +1 631 224 9141 Fax: +1 631 224 9086 Email: iocdo@nrcc.com |  |  |  |  |  |  |  |
| 5. | CARGO AND BALLAST HANDLING |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Double Hull Vessels |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.1 | Is vessel fitted with centerline bulkhead in all cargo tanks? If Yes, solid or perforated: |  |  |  |  |  |  |  |  |  |  | Yes, Solid |  |  |
| # Loadline Information |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| # 5.2 | # Loadline |  | # Freeboard |  |  | # Draft |  |  |  |  | # Deadweight |  |  | # Displacement |
|  | # Summer: |  | 6.02 Metres |  |  | 15.02 Metres |  |  |  |  | # 115,674.40 Metric Tonnes |  |  | 134,346.60 Metric Tonnes |
|  | # Winter: |  | 6.33 Metres |  |  | 14.71 Metres |  |  |  |  | 112,574.80 Metric Tonnes |  |  | 131,247.00 Metric Tonnes |
|  | # Tropical: |  | 5.71 Metres |  |  | 15.34 Metres |  |  |  |  | 118,777.60 Metric Tonnes |  |  | 137,449.80 Metric Tonnes |
|  | # Lightship: |  | 18.61 Metres |  |  | 2.43 Metres |  |  |  |  | Not Applicable |  |  | 18,672.20 Metric Tonnes |
|  | # Normal Ballast Condition: |  | 13.99 Metres |  |  | 7.04 Metres |  |  |  |  | 39,181.70 Metric Tonnes |  |  | 57,781.70 Metric Tonnes |
| # 5.3 | # Does vessel have multiple SDWT? If yes, please provide all assigned loadlines: |  |  |  |  |  |  |  |  |  | Yes 115674.40 106990.00 99998.00 89995.00 |  |  |  |
| Cargo Tank Capacities |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.4 | Number of cargo tanks and total cubic capacity (98%): |  |  |  |  |  |  |  |  |  |  | 12 |  | 123,646.194 Cu. Metres (98%) |
| 5.5 | Capacity (98%) of each natural segregation with double valve (specify tanks): |  |  |  |  |  |  |  |  |  |  | Seg#1: 41705.076 m3 (1W+4W+SLOPS) Seg#2: 43227.408 m3 (2W+5W) Seg#3: 42582.176 m3 (3W+6W) (98%) |  |  |
| 5.6 | Number of slop tanks and total cubic capacity (98%): |  |  |  |  |  |  |  |  |  |  | 2 |  | 3,868.40 Cu. Metres |
| 5.7 | Specify segregations which slops tanks belong to and their capacity with double valve: |  |  |  |  |  |  |  |  |  |  | Seg#1: 3868.4 Cu. Metres |  |  |
| 5.8 | Residual/Retention oil tank(s) capacity (98%), if applicable: |  |  |  |  |  |  |  |  |  |  | 0 Cu. Metres |  |  |
| 5.9 | Does vessel have Segregated Ballast Tanks (SBT) or Clean Ballast Tanks (CBT): |  |  |  |  |  |  |  |  |  |  | SBT |  |  |
| SBT Vessels |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.10 | What is total SBT capacity and percentage of SDWT vessel can maintain? |  |  |  |  |  |  |  |  |  |  | 39,975.10 Cu. Metres |  | 35.42 % |
| 5.11 | Does vessel meet the requirements of MARPOL Annex I Reg 18.2: |  |  |  |  |  |  |  |  |  |  | Yes |  |  |
| Cargo Handling and Pumping Systems |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.12 | How many grades/products can vessel load/discharge with double valve segregation: |  |  |  |  |  |  |  |  |  |  | 3 |  |  |
| 5.13 | Are there any cargo tank filling restrictions? If yes, specify number of slack tanks, max s.g., ullage restrictions etc.: |  |  |  |  |  |  |  |  |  |  | N/A |  |  |
| 5.14 | Pumps |  |  |  | No. | Type |  |  |  |  |  | Capacity |  | At What Head (sg=1.0) |
|  | Cargo Pumps: |  |  |  | 3 | Vertical, Single stage, Double suction, Centrifugal |  |  |  |  |  | 2800 M3/HR |  | 130 Meters 130 Meters 130 Meters |
|  | Cargo Eductors: |  |  |  | 1 | High Pressure |  |  |  |  |  | 600.00 Cu. Metres/Hour |  | 135 Metres |
|  | Stripping: |  |  |  | 1 | Vertical Duplex Double Acting Reciprocating Pump. |  |  |  |  |  | 200.00 Cu. Metres/Hour |  | 130 Metres |
|  | Ballast Pumps: |  |  |  | 2 | Vertical, Single Stage, Double Suction, Centrifugal |  |  |  |  |  | 1,500 Cu. Metres/Hour |  | 25 Metres |
|  | Ballast Eductors: |  |  |  | 1 | Eductor |  |  |  |  |  | 400 Cu. Metres/Hour |  | 6 Metres |
| 5.15 | Max loading rate for homogenous cargo per manifold connection: |  |  |  |  |  |  |  |  |  |  | 3,300.00 Cu. Metres/Hour |  |  |
| 5.16 | Max loading rate for homogenous cargo loaded simultaneously through all manifolds: |  |  |  |  |  |  |  |  |  |  | 10,000.00 Cu. Metres/Hour |  |  |
| 5.17 | How many cargo pumps can be run simultaneously at full capacity: |  |  |  |  |  |  |  |  |  |  | 3 |  |  |
| Cargo Control Room |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.18 | Is ship fitted with a Cargo Control Room (CCR)? |  |  |  |  |  |  |  |  |  |  | Yes |  |  |
| 5.19 | Can tank innage / ullage be read from the CCR? |  |  |  |  |  |  |  |  |  |  | Yes |  |  |
| Gauging and Sampling |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.20 | Can cargo be transferred under closed loading conditions in accordance with ISGOTT 11.1.6.6? |  |  |  |  |  |  |  |  |  |  | Yes |  |  |
| 5.21 | What type of fixed closed tank gauging system is fitted: |  |  |  |  |  |  |  |  |  |  | Radar |  |  |
| 5.22 | Number of portable gauging units (example- MMC) on board: |  |  |  |  |  |  |  |  |  |  | 4 |  |  |
| 5.23 | Are overfill (high) alarms fitted? If Yes, indicate whether to all tanks or partial: |  |  |  |  |  |  |  |  |  |  | Yes, All |  |  |
| 5.24 | Are cargo tanks fitted with multipoint gauging? If yes, specify type and locations: |  |  |  |  |  |  |  |  |  |  | Yes, MMC Vapor locks |  |  |
| 5.25 | Is gauging system certified and calibrated? If no, specify which ones are not calibrated: |  |  |  |  |  |  |  |  |  |  | Yes, N/A |  |  |
| Vapor Emission Control System (VECS) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.26 | Is a Vapour Emission Control System (VECS) fitted? |  |  |  |  |  |  |  |  |  |  | Yes |  |  |
| 5.27 | Number/size of VECS manifolds (per side): |  |  |  |  |  |  |  |  |  |  | 2 |  | 400.00 Millimetres |
| 5.28 | Number / size / type of VECS reducers: |  |  |  |  |  |  |  |  |  |  | 2 x 400/300mm (16/12") 2 x 400/250mm (16/10") |  |  |
| Venting |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.29 | State what type of venting system is fitted: |  |  |  |  |  |  |  |  |  |  | INDEPENDENT HV P/V VALVES + COMMON FULL FLOW MAST RISER |  |  |
| Cargo Manifolds and Reducers |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.30 | Does vessel comply with the latest edition of the OCIMF 'Recommendations for Oil Tanker Manifolds and Associated Equipment’? |  |  |  |  |  |  |  |  |  |  | Yes |  |  |
| 5.31 | Total number / size of cargo manifold connections on each side: |  |  |  |  |  |  |  |  |  |  | 3 / 406.40 Millimetres |  |  |
| 5.32 | What type of valves are fitted at manifold: |  |  |  |  |  |  |  |  |  |  | Butterfly |  |  |
| 5.33 | What is the material/rating of the manifold: |  |  |  |  |  |  |  |  |  |  | STEEL / 400 |  |  |
| 5.34 | Does the vessel have a Common Line Manifold connection? If yes, describe: |  |  |  |  |  |  |  |  |  |  | N/A |  |  |
| 5.35 | Distance between cargo manifold centers: |  |  |  |  |  |  |  |  |  |  | 2,500.00 Millimetres |  |  |
| 5.36 | Distance ships rail to manifold: |  |  |  |  |  |  |  |  |  |  | 4,600.00 Millimetres |  |  |
| 5.37 | Distance manifold to ships side: |  |  |  |  |  |  |  |  |  |  | 4,600.00 Millimetres |  |  |
| 5.38 | Top of rail to center of manifold: |  |  |  |  |  |  |  |  |  |  | 700.00 Millimetres |  |  |
| 5.39 | Distance main deck to center of manifold: |  |  |  |  |  |  |  |  |  |  | 2,100.00 Millimetres |  |  |
| 5.40 | Spill tank grating to center of manifold: |  |  |  |  |  |  |  |  |  |  | 900.00 Millimetres |  |  |
| 5.41 | Manifold height above the waterline in normal ballast / at SDWT condition: |  |  |  |  |  |  |  |  |  |  | 16.06 Metres |  | 8.10 Metres |
| 5.42 | Number / size / type of reducers: |  |  |  |  |  |  |  |  |  |  | 3 x 400/300mm (16/12") 3 x 400/250mm (16/10") 3 x 400/200mm (16/8") ANSI |  |  |
| 5.43 | Is vessel fitted with a stern manifold? If yes, state size: |  |  |  |  |  |  |  |  |  |  | No, |  |  |
| Heating |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.44 | Cargo / slop tanks fitted with a cargo heating system? |  |  |  |  |  |  | Type |  |  |  | Coiled |  | Material |
|  | Cargo Tanks: |  |  |  |  |  |  | Steam |  |  |  | Yes |  | Mildsteel |
|  | Slop Tanks: |  |  |  |  |  |  | Steam |  |  |  | Yes |  | Galvanized Mildsteel |
| 5.45 | Maximum temperature cargo can be loaded / maintained: |  |  |  |  |  |  |  |  |  |  | 66.0 °C / 150.8 °F |  | 57 °C / 134.6 °F |
| 5.46 | Minimum temperature cargo can be loaded / maintained: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Coating / Anodes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.47 | Tank Coating |  |  | Coated |  | Type |  |  |  |  |  | To What Extent |  | Anodes |
|  | Cargo tanks: |  |  | Yes |  | Tar free epoxy |  |  |  |  |  | Deckhead and 0.5 meters below. Bottom and 0.5 meters above |  | No |
|  | Ballast tanks: |  |  | Yes |  | Tar Free Modified Epoxy |  |  |  |  |  | Whole Tank |  | Yes |
|  | Slop tanks: |  |  | Yes |  | Epoxy |  |  |  |  |  | Whole Tank |  | No |
| 6. | INERT GAS AND CRUDE OIL WASHING |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6.1 | Is a Crude Oil Washing (COW) installation fitted / operational? |  |  |  |  |  |  |  |  |  |  | Yes / Yes |  |  |
| 6.2 | Is an Inert Gas System (IGS) fitted / operational? |  |  |  |  |  |  |  |  |  |  | Yes / Yes |  |  |
| 6.3 | Is IGS supplied by flue gas, inert gas (IG) generator and/or nitrogen: |  |  |  |  |  |  |  |  |  |  | Flue Gas |  |  |
| 7. | MOORING |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7.1 | Wires (on drums) | No. |  | Diameter |  | Material |  |  |  |  |  | Length |  | Breaking Strength |
|  | Forecastle: | 4 |  | 34.00 Millimetres |  | IWRC GALVANIZED |  |  |  |  |  | 220.00 Metres |  | 74.40 Metric Tonnes |
|  | Main deck fwd: | 4 |  | 34.00 Millimetres |  | IWRC GALVANIZED |  |  |  |  |  | 220.00 Metres |  | 74.40 Metric Tonnes |
|  | Main deck aft: | 2 |  | 34.00 Millimetres |  | IWRC GALVANIZED |  |  |  |  |  | 275.00 Metres |  | 74.40 Metric Tonnes |
|  | Poop deck: | 6 |  | 34.00 Millimetres |  | IWRC Galvanized (2x220m/4x275m) |  |  |  |  |  | 275.00 Metres |  | 74.40 Metric Tonnes |
| 7.2 | Wire tails | No. |  | Diameter |  | Material |  |  |  |  |  | Length |  | Breaking Strength |
|  | Forecastle: | 4 |  | 72.00 Millimetres |  | NYLON |  |  |  |  |  | 11.00 Metres |  | 102.00 Metric Tonnes |
|  | Main deck fwd: | 4 |  | 72.00 Millimetres |  | NYLON |  |  |  |  |  | 11.00 Metres |  | 102.00 Metric Tonnes |
|  | Main deck aft: | 2 |  | 72.00 Millimetres |  | NYLON |  |  |  |  |  | 11.00 Metres |  | 102.00 Metric Tonnes |
|  | Poop deck: | 6 |  | 72.00 Millimetres |  | NYLON |  |  |  |  |  | 11.00 Metres |  | 102.00 Metric Tonnes |
| 7.3 | Ropes (on drums) | No. |  | Diameter |  | Material |  |  |  |  |  | Length |  | Breaking Strength |
|  | Forecastle: |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Main deck fwd: |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Main deck aft: |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Poop deck: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7.4 | Other lines | No. |  | Diameter |  | Material |  |  |  |  |  | Length |  | Breaking Strength |
|  | Forecastle: | 5 |  | 72.00 Millimetres |  | MIXED |  |  |  |  |  | 220.00 Metres |  | 98.00 Metric Tonnes |
|  | Main deck fwd: |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Main deck aft: |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Poop deck: | 5 |  | 72.00 Millimetres |  | MIXED |  |  |  |  |  | 220.00 Metres |  | 98.00 Metric Tonnes |
| 7.5 | Winches | No. |  | No. Drums |  | Motive Power |  |  |  |  |  | Brake Capacity |  | Type of Brake |
|  | Forecastle: | 2 |  | Double Drums |  | Hydraulic |  |  |  |  |  | 44.50 Metric Tonnes |  | SS Brake Bands |
|  | Main deck fwd: | 2 |  | Double Drums |  | Hydraulic |  |  |  |  |  | 44.50 Metric Tonnes |  | SS Brake Bands |
|  | Main deck aft: | 1 |  | Double Drums |  | Hydraulic |  |  |  |  |  | 44.50 Metric Tonnes |  | SS Brake Bands |
|  | Poop deck: | 3 |  | Double Drums |  | Hydraulic |  |  |  |  |  | 44.50 Metric Tonnes |  | SS Brake Bands |
| 7.6 | Bitts, closed chocks/fairleads |  |  | No. Bitts |  | SWL Bitts |  |  |  |  |  | No. Closed Chocks |  | SWL Closed Chocks |
|  | Forecastle: |  |  | 2 |  | 76.00 Metric Tonnes |  |  |  |  |  | 6 |  | 76.00 Metric Tonnes |
|  | Main deck fwd: |  |  | 6 |  | 76.00 Metric Tonnes (4x76T / 2x100T (plus 2x92T Fwd of Manifolds)) |  |  |  |  |  | 14 |  | 76.00 Metric Tonnes (10x76T / 2x100T / 2x92T (plus 2x92T at Manifolds)) |
|  | Main deck aft: |  |  | 4 |  | 76.00 Metric Tonnes (2x76T / 2x100T) |  |  |  |  |  | 8 |  | 76.00 Metric Tonnes (6x76T / 2x100T) |
|  | Poop deck: |  |  | 4 |  | 76.00 Metric Tonnes |  |  |  |  |  | 12 |  | 76.00 Metric Tonnes |
| Anchors/Emergency Towing System |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7.7 | Number of shackles on port / starboard cable: |  |  |  |  |  |  |  |  |  |  | 13 / 13 |  |  |
| 7.8 | Type / SWL of Emergency Towing system forward: |  |  |  |  |  |  |  |  |  |  | 2 x CLOSED CHOCKS 600x450, TONGUE TYPE, SWL:250T, A TOWING BRACKET, CHAFING CHAIN (DIA76MM x 8M APPROX) |  | 250 Metric Tonnes |
| 7.9 | Type / SWL of Emergency Towing system aft: |  |  |  |  |  |  |  |  |  |  | PICK UP GEAR, TOWING WIRE, AIR MOTOR, STORAGE DRUM, SWL:200T |  | 200 Metric Tonnes |
| Escort Tug |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7.10 | What is size / SWL of closed chock and/or fairleads of enclosed type on stern: |  |  |  |  |  |  |  |  |  |  | 450 x 600 |  | 200.00 Metric Tonnes |
| 7.11 | What is SWL of bollard on poop deck suitable for escort tug: |  |  |  |  |  |  |  |  |  |  | 200.00 Metric Tonnes |  |  |
| Bow/Stern Thruster |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7.12 | What is brake horse power of bow thruster (if fitted): |  |  |  |  |  |  |  |  |  |  | No, |  |  |
| 7.13 | What is brake horse power of stern thruster (if fitted): |  |  |  |  |  |  |  |  |  |  | No, |  |  |
| Single Point Mooring (SPM) Equipment |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7.14 | Does the vessel meet the recommendations in the latest edition of OCIMF 'Recommendations for Equipment Employed in the Bow Mooring of Conventional Tankers at Single Point Moorings (SPM)’? |  |  |  |  |  |  |  |  |  |  | Yes |  |  |
| 7.15 | If fitted, how many chain stoppers: |  |  |  |  |  |  |  |  |  |  | 2 |  |  |
| 7.16 | State type / SWL of chain stopper(s): |  |  |  |  |  |  |  |  |  |  | TONGUE TYPE |  | 250.00 Metric Tonnes |
| 7.17 | What is the maximum size chain diameter the bow stopper(s) can handle: |  |  |  |  |  |  |  |  |  |  | 76.00 Millimetres |  |  |
| 7.18 | Distance between the bow fairlead and chain stopper/bracket: |  |  |  |  |  |  |  |  |  |  | 3,000.00 Millimetres |  |  |
| 7.19 | Is bow chock and/or fairlead of enclosed type of OCIMF recommended size (600mm x 450mm)? If not, give details of size: |  |  |  |  |  |  |  |  |  |  | Yes |  |  |
| Lifting Equipment |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7.20 | Derrick / Crane description (Number, SWL and location): |  |  |  |  |  |  |  |  |  |  | Cranes: 1 x 15.00 Tonnes Center / Amidship |  |  |
| 7.21 | What is maximum outreach of cranes / derricks outboard of the ship’s side: |  |  |  |  |  |  |  |  |  |  | 7.00 Metres |  |  |
| Ship To Ship Transfer (STS) / Helicopter Operations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7.22 | Does vessel comply with recommendations contained in OCIMF/ICS Ship To Ship Transfer Guide (Petroleum, Chemicals or Liquified Gas, as applicable)? |  |  |  |  |  |  |  |  |  |  | Yes |  |  |
| 7.23 | Can the ship comply with the ICS Helicopter Guidelines? If Yes, state whether winching or landing area provided and diameter of the circle provided: |  |  |  |  |  |  |  |  |  |  | Yes, Winching 32.10 Metres |  |  |
| 8. | MISCELLANEOUS |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Engine |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.1 | Speed |  |  |  |  |  |  |  |  |  |  | Maximum |  | Economic |
|  | Ballast speed: |  |  |  |  |  |  |  |  |  |  | 15.00 Knots (WSNP) |  | 10.50 Knots (WSNP) |
|  | Laden speed: |  |  |  |  |  |  |  |  |  |  | 14.00 Knots (WSNP) |  | 10.50 Knots (WSNP) |
| 8.2 | What type of fuel is used for main propulsion / generating plant: |  |  |  |  |  |  |  |  |  |  | H.F.O. up to 700 cSt at 50 deg C |  | HFO or DMA 2cSt at 40 deg C to UFO up to 700cSt at 50 deg C |
| 8.3 | Type / Capacity of bunker tanks: |  |  |  |  |  |  |  |  |  |  | Fuel Oil: 2,861.20 Cu. Metres Diesel Oil: 278 Cu. Metres Gas Oil: 42.50 Cu. Metres |  |  |
| 8.4 | Is vessel fitted with fixed or controllable pitch propeller(s): |  |  |  |  |  |  |  |  |  |  | Fixed |  |  |
| 8.5 | Engines |  |  |  |  |  |  |  |  | No |  | Capacity |  | Make/Type |
|  | Main engine: |  |  |  |  |  |  |  |  | 1 |  | 18,035 Kilowatt |  | DOOSAN MAN B&W / 6S60MC-C Mk8 |
|  | Aux engine: |  |  |  |  |  |  |  |  | 3 |  | 800 Kilowatt |  | YANMAR / 6N21ALSW |
|  | Power packs: |  |  |  |  |  |  |  |  | N/A |  |  |  | N/A |
|  | Boilers: |  |  |  |  |  |  |  |  | 2 |  | 25.00 Metric Tonnes/Hour |  | AALBORG - 25000 Kg/Hr |
| Emissions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.6 | Main engine IMO NOx emission standard: |  |  |  |  |  |  |  |  |  |  | Tier II |  |  |
| 8.7 | Energy Efficiency Design Index (EEDI) rating number: |  |  |  |  |  |  |  |  |  |  | 10.9 |  |  |
| Insurance |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.8 | P & I Club - Full Style: |  |  |  |  |  |  |  | GARD Tel: +47 37 019 100 Fax: +47 37 019 100 Web: www.gard.no |  |  |  |  |  |
| 8.9 | P & I Club pollution liability coverage / expiration date: |  |  |  |  |  |  |  |  |  |  | 1,000,000,000 US$ |  | Feb 20, 2017 |
| 8.10 | Hull & Machinery insured by - Full Style: |  |  |  |  |  |  |  | NHC |  |  |  |  |  |
| 8.11 | Hull & Machinery insured value / expiration date: |  |  |  |  |  |  |  |  |  |  | 55,000,000 US$ |  | Mar 31, 2017 |
| Recent Operational History |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.12 | Date and place of last Port State Control inspection: |  |  |  |  |  |  |  |  |  |  | Oct 28, 2015 / San Francisco |  |  |
| 8.13 | Any outstanding deficiencies as reported by any Port State Control? If yes, provide details: |  |  |  |  |  |  |  |  |  |  | No N/A |  |  |
| 8.14 | Has vessel been involved in a pollution, grounding, serious casualty or collision incident during the past 12 months? If yes, full description: |  |  |  |  |  |  |  |  |  |  | Pollution: No, N/A Grounding: No, N/A Casualty: No, N/A Collision: No, N/A |  |  |
| 8.15 | Last three cargoes / charterers / voyages (Last / 2nd Last / 3rd Last): |  |  |  |  |  |  |  |  |  |  | 1. ESPO C.O/SIETCO/Kosmino --> Port Dickson 2. FUEL OIL/CLEARLAKE /Singapore --> T. Pelepas & Singapore 3. FUEL OIL/CLEARLAKE / T. Pelepas & Singapore --> Singapore |  |  |
| 8.16 | Date/place of last STS operation: |  |  |  |  |  |  |  |  |  |  | 05-06-2016 / Singapore |  |  |
| Vetting |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.17 | Date of last SIRE inspection: |  |  |  |  |  |  |  |  |  |  | May 13, 2016 |  |  |
| 8.18 | Date of last CDI inspection: |  |  |  |  |  |  |  |  |  |  | N/A |  |  |
| 8.19 | Recent Oil company inspections/screenings (To the best of owners knowledge and without guarantee of acceptance for future business)*: * "Approvals" are not given by Oil Majors and ships are accepted for the voyage on a case by case basis. |  |  |  |  |  |  |  |  |  |  | SHELL, TESORO |  |  |
| Additional Information |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.20 | Additional information relating to features of the ship or operational characteristics: |  |  |  |  |  |  |  |  |  |  |  |  |  |

Rev 2015 (INTERTANKO / Q88.com)

Form completed on http://www.q88.com/integration.aspx  Please email support@q88.com an updated copy if this is not the latest version.

To the best of owners knowledge all information is true and given without any guarantee.