- INTERTANKO TANKER CHARTERING QUESTIONNAIRE 88 	Version 4

| 1. | VESSEL DESCRIPTION |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1.1 | Date updated: |  |  |  |  |  |  |  |  |  |  | 18TH OCT 2016 |  |  |
| 1.2 | Vessel’s name (IMO number): |  |  |  |  |  |  |  |  |  |  | M/T CHIOS / IMO: 9772113 |  |  |
| 1.3 | Vessel’s previous name(s) and date(s) of change: |  |  |  |  |  |  |  |  |  |  | Not Applicable |  |  |
| 1.4 | Date delivered / Builder (where built): |  |  |  |  |  |  |  |  |  |  | 30th June 2016 / HHI, S.KOREA |  |  |
| 1.5 | Flag / Port of Registry: |  |  |  |  |  |  |  |  |  |  | BAHAMAS / NASSAU |  |  |
| 1.6 | Call sign / MMSI: |  |  |  |  |  |  |  |  |  |  | C6CQ8 / 311 000 546 |  |  |
| 1.7 | Vessel’s contact details (satcom/fax/email etc.): |  |  |  |  |  |  |  |  |  |  | Tel: +47 2240 7824 +47 2240 7825 +870 773 925 529 Email: chios@sunenterprises.gr |  |  |
| 1.8 | Type of vessel (as described in Form A or Form B Q1.11 of the IOPPC): |  |  |  |  |  |  |  |  |  |  | Oil Tanker |  |  |
| 1.9 | Type of hull: |  |  |  |  |  |  |  |  |  |  | Double Hull |  |  |
| Classification |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1.10 | Classification society: |  |  |  |  |  |  |  |  |  |  | American Bureau of Shipping |  |  |
| 1.11 | Class notation: |  |  |  |  |  |  |  |  |  |  | ABS+A1(E), OIL CARRIER, ESP, CSR, SAFESHIP-CM, +AMS, +ACCU, VEC, RW, ENVIRO, UWILD, POT, TCM, CPP, SPM, BWE, CPS, PMA,GP |  |  |
| 1.12 | Is the vessel subject to any conditions of class, class extensions, outstanding memorandums or class recommendations? If yes, give details: |  |  |  |  |  |  |  |  |  |  | No |  |  |
| 1.13 | If classification society changed, name of previous and date of change: |  |  |  |  |  |  |  |  |  |  | N/A |  |  |
| 1.14 | IMO type, if applicable: |  |  |  |  |  |  |  |  |  |  | N/A |  |  |
| 1.15 | Does the vessel have ice class? If yes, state what level: |  |  |  |  |  |  |  |  |  |  | No |  |  |
| 1.16 | Date / place of last dry-dock: |  |  |  |  |  |  |  |  |  |  | JUN 2016 / H.H.I, S.KOREA |  |  |
| 1.17 | Date next dry dock due / next annual survey due: |  |  |  |  |  |  |  |  |  |  | 30 JUN 2021 |  | 30 JUN 2021 |
| 1.18 | Date of last special survey / next special survey due: |  |  |  |  |  |  |  |  |  |  | 30 JUN 2016 |  | 30 JUN 2016 |
| 1.19 | If ship has Condition Assessment Program (CAP), what is the latest overall rating: |  |  |  |  |  |  |  |  |  |  | N/A |  |  |
| 1.20 | Does the vessel have a statement of compliance issued under the provisions of the Condition Assessment Scheme (CAS): If yes, what is the expiry date? |  |  |  |  |  |  |  |  |  |  | N/A |  |  |
| Dimensions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1.21 | Length overall (LOA): |  |  |  |  |  |  |  |  |  |  | 274.22 M |  |  |
| 1.22 | Length between perpendiculars (LBP): |  |  |  |  |  |  |  |  |  |  | 263.14 M |  |  |
| 1.23 | Extreme breadth (Beam): |  |  |  |  |  |  |  |  |  |  | 48 M |  |  |
| 1.24 | Moulded depth: |  |  |  |  |  |  |  |  |  |  | 23.1 M |  |  |
| 1.25 | Keel to masthead (KTM)/ Keel to masthead (KTM) in collapsed condition, if applicable: |  |  |  |  |  |  |  |  |  |  | 50 M |  | 50 M |
| 1.26 | Bow to center manifold (BCM) / Stern to center manifold (SCM): |  |  |  |  |  |  |  |  |  |  | 137.29 M |  | 136.93 M |
| 1.27 | Distance bridge front to center of manifold: |  |  |  |  |  |  |  |  |  |  | 91.43 M |  |  |
| 1.28 | Parallel body distances |  |  |  |  | Lightship |  |  |  |  |  | Normal Ballast |  | Summer Dwt |
|  | # Forward to mid-point manifold: |  |  |  |  | 50.34 M |  |  |  |  |  | 56.52 M |  | 56.52 M |
|  | # Aft to mid-point manifold: |  |  |  |  | 37.24 M |  |  |  |  |  | 40.56 M |  | 40.56 M |
|  | # Parallel body length: |  |  |  |  | 85.83 M |  |  |  |  |  | 97.08 M |  | 97.08 M |
| # 1.29 | # FWA/TPC at summer draft: |  |  |  |  |  |  |  |  |  |  | 370 MM |  | 118.33 |
| 1.30 | Constant (excluding fresh water): |  |  |  |  |  |  |  |  |  |  | 96 |  |  |
| 1.31 | What is the company guidelines for Under Keel Clearance (UKC) for this vessel? |  |  |  |  |  |  |  |  |  |  | Open Seas: 10% of the ship's deepest draft but not less than 2.0 m Confined/restricted waters: 0.6 m Alongside: 1% of ship's beam but not less than 0.3 m Where the above minimum Under Keel Clearance can not be maintained during any part of the voyage, the Master must inform the Operator well in advance for consultation. |  |  |
| 1.32 | What is the max height of mast above waterline (air draft) |  |  |  |  |  |  |  |  |  |  | Full Mast |  | Collapsed Mast |
|  | Lightship: |  |  |  |  |  |  |  |  |  |  | 48.356 M |  | 47.306 M |
|  | Normal ballast: |  |  |  |  |  |  |  |  |  |  | 41.65 M |  | 40.6 M |
|  | At loaded summer deadweight: |  |  |  |  |  |  |  |  |  |  | 34.63 M |  | 33.58 M |
| Tonnages |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1.33 | Net Tonnage: |  |  |  |  |  |  |  |  |  |  | 52014 |  |  |
| 1.34 | Gross Tonnage / Reduced Gross Tonnage (if applicable): |  |  |  |  |  |  |  |  |  |  | 81315 |  |  |
| 1.35 | Suez Canal Tonnage - Gross (SCGT) / Net (SCNT): |  |  |  |  |  |  |  |  |  |  | 82690.5 |  | 77668.73 |
| # 1.36 | Panama Canal Net Tonnage (PCNT): |  |  |  |  |  |  |  |  |  |  | 72559.86 |  |  |
| Ownership and Operation |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1.37 | Registered owner - Full style: |  |  |  |  |  | Aetna Shipping Co. Ltd |  |  |  |  |  |  |  |
| 1.38 | Technical operator - Full style: |  |  |  |  |  | SUN ENTERPRISES LTD 47-49 Akti Miaouli, 18536, Piraeus, Greece Tel: +30-210-4292910 Fax: +30-210-4292523 Telex: +30-210-4292910 Email: email@sunenterprises.gr Company IMO#: 0510075 |  |  |  |  |  |  |  |
| 1.39 | Commercial operator - Full style: |  |  |  |  |  | Sun Enterprises ltd 47-49 Akti Miaouli, 18536, Piraeus, Greece Tel: +30-210-4292910 Fax: +30-210-4292523 Telex: 212491/211422/212996 Email: email@sunenterprises.gr |  |  |  |  |  |  |  |
| 1.40 | Disponent owner - Full style: |  |  |  |  |  | N/A |  |  |  |  |  |  |  |
| 2. | CERTIFICATION |  |  |  |  | Issued |  |  |  |  |  | Last Annual |  | Expires |
| 2.1 | Safety Equipment Certificate (SEC): |  |  |  |  | 30/06/2016 |  |  |  |  |  | - |  | 29/06/2021 |
| 2.2 | Safety Radio Certificate (SRC): |  |  |  |  | 30/06/2016 |  |  |  |  |  | - |  | 29/06/2021 |
| 2.3 | Safety Construction Certificate (SCC): |  |  |  |  | 30/06/2016 |  |  |  |  |  | - |  | 29/06/2021 |
| 2.4 | International Loadline Certificate (ILC): |  |  |  |  | 30/06/2016 |  |  |  |  |  | - |  | 29/11/2016 |
| 2.5 | International Oil Pollution Prevention Certificate (IOPPC): |  |  |  |  | 30/06/2016 |  |  |  |  |  | - |  | 29/06/2021 |
| 2.6 | ISM Safety Management Certificate (SMC): |  |  |  |  | 30/06/2016 |  |  |  |  |  | - |  | 29/12/2016 |
| # 2.7 | # Document of Compliance (DOC): |  |  |  |  | # 17/02/2016 |  |  |  |  |  | - |  | 26/02/2018 |
| 2.8 | USCG Certificate of Compliance (COC): |  |  |  |  | # - |  |  |  |  |  | - |  | - |
| 2.9 | Civil Liability Convention (CLC) 1992 Certificate: |  |  |  |  | 30/06/2016 |  |  |  |  |  | Not Applicable |  | 20/02/2017 |
| 2.10 | Civil Liability for Bunker Oil Pollution Damage Convention (CLBC) Certificate: |  |  |  |  | 30/06/2016 |  |  |  |  |  | Not Applicable |  | 20/02/2017 |
| 2.11 | Ship Sanitation Control (SSCC)/Ship Sanitation Control Exemption (SSCE) Certificate: |  |  |  |  | 23/06/2016 |  |  |  |  |  | Not Applicable |  | 23/12/2016 |
| 2.12 | U.S. Certificate of Financial Responsibility (COFR): |  |  |  |  | 15/07/2016 |  |  |  |  |  | Not Applicable |  | 15/07/2019 |
| 2.13 | Certificate of Class (COC): |  |  |  |  | 30/06/2016 |  |  |  |  |  | - |  | 29/11/2016 |
| # 2.14 | # International Sewage Pollution Prevention Certificate (ISPPC): |  |  |  |  | 30/06/2016 |  |  |  |  |  | Not Applicable |  | 29/06/2021 |
| # 2.15 | # Certificate of Fitness (COF): |  |  |  |  | # Not Applicable |  |  |  |  |  | Not Applicable |  | Not Applicable |
| # 2.16 | # International Energy Efficiency Certificate (IEEC): |  |  |  |  | # 30/06/2016 |  |  |  |  |  | Not Applicable |  | 29/06/2021 |
| # 2.17 | # International Ship Security Certificate (ISSC): |  |  |  |  | 30/06/2016 |  |  |  |  |  | - |  | 29/12/2016 |
| # 2.18 | # International Air Pollution Prevention Certificate (IAPPC): |  |  |  |  | 30/06/2016 |  |  |  |  |  | Not Applicable |  | 29/06/2021 |
| # 2.19 | # Maritime Labour Certificate (MLC): |  |  |  |  | 30/06/2016 |  |  |  |  |  | - |  | 29/12/2016 |
| Documentation |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2.20 | Owner warrant that vessel is member of ITOPF and will remain so for the entire duration of this voyage/contract: |  |  |  |  |  |  |  |  |  |  | Yes |  |  |
| 2.21 | Does vessel have in place a Drug and Alcohol Policy complying with OCIMF guidelines for Control of Drugs and Alcohol Onboard Ship? |  |  |  |  |  |  |  |  |  |  | Yes |  |  |
| 2.22 | Is the ITF Special Agreement on board (if applicable)? |  |  |  |  |  |  |  |  |  |  | Yes |  |  |
| 2.23 | ITF Blue Card expiry date: |  |  |  |  |  |  |  |  |  |  | 27/06/2017 |  |  |
| 3. | CREW |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3.1 | Nationality of Master: |  |  |  |  |  |  |  |  |  |  | Greek |  |  |
| 3.2 | Number and Nationality of Officers: |  |  |  |  |  |  |  |  |  |  | 9 / GREEK & PHILIPPINO |  |  |
| 3.3 | Number and Nationality of Crew: |  |  |  |  |  |  |  |  |  |  | 16 PHILIPINO |  |  |
| 3.4 | What is the common working language onboard: |  |  |  |  |  |  |  |  |  |  | ENGLISH |  |  |
| 3.5 | Do officers speak and understand English? |  |  |  |  |  |  |  |  |  |  | Yes |  |  |
| 3.6 | If Officers/Crew employed by a Manning Agency - Full style: |  |  |  |  |  | Greek Officers + Crew: S.LIVANOS HELLAS 6 Skouze str GR 18536 PIRAEUS GREECE Tel: +30210-4292730 Fax: +30210-4292523 Telex: 212496 Email: email@sunenterprises.gr Philippino Officers + Crew: ELVICTOR MANAGEMENT LTD 79 VAS.KONSTANTINOU STR 16672 VARKIZA ATHENS GREECE Tel: 0030 4224220 Fax: 0030 2104224229 Telex: 211770 Email: crew@elvictor.com |  |  |  |  |  |  |  |
| 4. | FOR USA CALLS |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4.1 | Has the vessel Operator submitted a Vessel Spill Response Plan to the US Coast Guard which has been approved by official USCG letter? |  |  |  |  |  |  |  |  |  |  |  | Yes |  |
| 4.2 | Qualified individual (QI) - Full style: |  |  |  |  |  | Gallagher Marine Systems Inc, 200 Century Parkway, Suite D, Mount Laurel, NJ 08054-1150, USA Tel: +1 215 492 5473 Fax: +1 856 642 3945 |  |  |  |  |  |  |  |
| 4.3 | Oil Spill Response Organization (OSRO) - Full style: |  |  |  |  |  | Marine Spill Response Corporation 220 Spring Street, Suite 500, Herndon VA 20170, USA Tel: 703 326 5600 Fax: 703 326 5600 |  |  |  |  |  |  |  |
| 5. | CARGO AND BALLAST HANDLING |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Double Hull Vessels |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.1 | Is vessel fitted with centerline bulkhead in all cargo tanks? If Yes, solid or perforated: |  |  |  |  |  |  |  |  |  |  | Yes, Solid |  |  |
| # Loadline Information |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| # 5.2 | # Loadline |  | # Freeboard |  |  | # Draft |  |  |  |  | # Deadweight |  |  | # Displacement |
|  | # Summer: |  | 5.969 M |  |  | 17.168 M |  |  |  |  | # 158,902 MT |  |  | 183,853 MT |
|  | # Winter: |  | 6.326M |  |  | 16.811 M |  |  |  |  | 154,667 MT |  |  | 179,618MT |
|  | # Tropical: |  | 5.612 M |  |  | 17,525 M |  |  |  |  | 163,141 MT |  |  | 188,092 MT |
|  | # Lightship: |  | 20.406 M |  |  | 2.694 M |  |  |  |  | Not Applicable |  |  | 25,000 MT |
|  | # Normal Ballast Condition: |  | 15.44 M |  |  | 7.66 M |  |  |  |  | 50,249 MT |  |  | 75,249 MT |
| # 5.3 | # Does vessel have multiple SDWT? If yes, please provide all assigned loadlines: |  |  |  |  |  |  |  |  |  | YES, SDWT 158902 / 149,988.7 |  |  |  |
| Cargo Tank Capacities |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.4 | Number of cargo tanks and total cubic capacity (98%): |  |  |  |  |  |  |  |  |  |  | 12 |  | 166308.1, Cu. Meters |
| 5.5 | Capacity (98%) of each natural segregation with double valve (specify tanks): |  |  |  |  |  |  |  |  |  |  | 1: NO1 COT (P&S) NO4 COT (P&S) SLOP (S) CAPACITY (98%): 53,058.9 CUM 2: NO2 COT (P&S) NO5 COT (P&S) CAPACITY (98%): 58,477.0 CUM 3: NO3 COT (P&S) NO6 COT (P&S) SLOP (P) CAPACITY (98%): 58,206.3 CUM |  |  |
| 5.6 | Number of slop tanks and total cubic capacity (98%): |  |  |  |  |  |  |  |  |  |  | 2 |  | 3,434 Cu. Meters |
| 5.7 | Specify segregations which slops tanks belong to and their capacity with double valve: |  |  |  |  |  |  |  |  |  |  | Segregation No.1 (Slop S) and Segregation No.3 (Slop P), Single Valve |  |  |
| 5.8 | Residual/Retention oil tank(s) capacity (98%), if applicable: |  |  |  |  |  |  |  |  |  |  | N/A |  |  |
| 5.9 | Does vessel have Segregated Ballast Tanks (SBT) or Clean Ballast Tanks (CBT): |  |  |  |  |  |  |  |  |  |  | SBT |  |  |
| SBT Vessels |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.10 | What is total SBT capacity and percentage of SDWT vessel can maintain? |  |  |  |  |  |  |  |  |  |  | 52,856.7 Cu. Meters |  | 35.2 % |
| 5.11 | Does vessel meet the requirements of MARPOL Annex I Reg 18.2: |  |  |  |  |  |  |  |  |  |  | Yes |  |  |
| Cargo Handling and Pumping Systems |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.12 | How many grades/products can vessel load/discharge with double valve segregation: |  |  |  |  |  |  |  |  |  |  | 3 |  |  |
| 5.13 | Are there any cargo tank filling restrictions? If yes, specify number of slack tanks, max s.g., ullage restrictions etc.: |  |  |  |  |  |  |  |  |  |  | No |  |  |
| 5.14 | Pumps |  |  |  | No. | Type |  |  |  |  |  | Capacity |  | At What Head (sg=1.0) |
|  | Cargo Pumps: |  |  |  | 3 | Single stage steam turbine driven, centrifugal |  |  |  |  |  | 4000 M3/HR |  | 135 MetersTH |
|  | Cargo Eductors: |  |  |  | 1 | Driving liquid from cargo oil pump |  |  |  |  |  | 400 Cu. Metres/Hour |  | 25 MetresTH |
|  | Stripping: |  |  |  | 1 | Steam driven reciprocating |  |  |  |  |  | 300 Cu. Metres/Hour |  | 135 MetresTH |
|  | Ballast Pumps: |  |  |  | 2 | Vertical centrifugal single stage, driven by single speed electric motor in engine room |  |  |  |  |  | 2,000 Cu. Metres/Hour (Each) |  | 35 MetresTH (Each) |
|  | Ballast Eductors: |  |  |  | 2 | Driving water from water ballast pumps |  |  |  |  |  | 400 Cu.Meters/Hour (Each) |  | 22 MetresTH |
| 5.15 | Max loading rate for homogenous cargo per manifold connection: |  |  |  |  |  |  |  |  |  |  | 5,666 Cu. Metres/Hour |  |  |
| 5.16 | Max loading rate for homogenous cargo loaded simultaneously through all manifolds: |  |  |  |  |  |  |  |  |  |  | 17,000 Cu. Metres/Hour |  |  |
| 5.17 | How many cargo pumps can be run simultaneously at full capacity: |  |  |  |  |  |  |  |  |  |  | 3 |  |  |
| Cargo Control Room |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.18 | Is ship fitted with a Cargo Control Room (CCR)? |  |  |  |  |  |  |  |  |  |  | Yes |  |  |
| 5.19 | Can tank innage / ullage be read from the CCR? |  |  |  |  |  |  |  |  |  |  | Yes |  |  |
| Gauging and Sampling |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.20 | Can cargo be transferred under closed loading conditions in accordance with ISGOTT 11.1.6.6? |  |  |  |  |  |  |  |  |  |  | Yes |  |  |
| 5.21 | What type of fixed closed tank gauging system is fitted: |  |  |  |  |  |  |  |  |  |  | RADAR BEAM |  |  |
| 5.22 | Number of portable gauging units (example- MMC) on board: |  |  |  |  |  |  |  |  |  |  | 4 |  |  |
| 5.23 | Are overfill (high) alarms fitted? If Yes, indicate whether to all tanks or partial: |  |  |  |  |  |  |  |  |  |  | Yes, All |  |  |
| 5.24 | Are cargo tanks fitted with multipoint gauging? If yes, specify type and locations: |  |  |  |  |  |  |  |  |  |  | Yes, Vapor Locks, Fore/Middle/Aft |  |  |
| 5.25 | Is gauging system certified and calibrated? If no, specify which ones are not calibrated: |  |  |  |  |  |  |  |  |  |  | Yes |  |  |
| Vapor Emission Control System (VECS) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.26 | Is a Vapour Emission Control System (VECS) fitted? |  |  |  |  |  |  |  |  |  |  | Yes |  |  |
| 5.27 | Number/size of VECS manifolds (per side): |  |  |  |  |  |  |  |  |  |  | 2 |  | 500 Millimeters |
| 5.28 | Number / size / type of VECS reducers: |  |  |  |  |  |  |  |  |  |  | I.G SYSTEM: VECS: 2 / 20” X 12” ANSI EXT. SUPPLY: 1 / 20”X10” ANSI |  |  |
| Venting |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.29 | State what type of venting system is fitted: |  |  |  |  |  |  |  |  |  |  | FULL CAPACITY HIGH VELOCITY P/V VALVES |  |  |
| Cargo Manifolds and Reducers |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.30 | Does vessel comply with the latest edition of the OCIMF 'Recommendations for Oil Tanker Manifolds and Associated Equipment’? |  |  |  |  |  |  |  |  |  |  | Yes |  |  |
| 5.31 | Total number / size of cargo manifold connections on each side: |  |  |  |  |  |  |  |  |  |  | 3 / 600 Millimeters |  |  |
| 5.32 | What type of valves are fitted at manifold: |  |  |  |  |  |  |  |  |  |  | MANUAL BUTTERFLY |  |  |
| 5.33 | What is the material/rating of the manifold: |  |  |  |  |  |  |  |  |  |  | STEEL |  |  |
| 5.34 | Does the vessel have a Common Line Manifold connection? If yes, describe: |  |  |  |  |  |  |  |  |  |  | N/A |  |  |
| 5.35 | Distance between cargo manifold centers: |  |  |  |  |  |  |  |  |  |  | 2,500 MM |  |  |
| 5.36 | Distance ships rail to manifold: |  |  |  |  |  |  |  |  |  |  | 4,300 MM |  |  |
| 5.37 | Distance manifold to ships side: |  |  |  |  |  |  |  |  |  |  | 4,600 MM |  |  |
| 5.38 | Top of rail to center of manifold: |  |  |  |  |  |  |  |  |  |  | 500 MM |  |  |
| 5.39 | Distance main deck to center of manifold: |  |  |  |  |  |  |  |  |  |  | 2,100 MM |  |  |
| 5.40 | Spill tank grating to center of manifold: |  |  |  |  |  |  |  |  |  |  | 900 MM |  |  |
| 5.41 | Manifold height above the waterline in normal ballast / at SDWT condition: |  |  |  |  |  |  |  |  |  |  | 17.22 M |  | 8.04 M |
| 5.42 | Number / size / type of reducers: |  |  |  |  |  |  |  |  |  |  | C.O. SYSTEM: 3 / 24”X12” ANSI 3 / 24”X10” ANSI 3 / 24”X8” ANSI F.O& D.O SYSTEM: 1 / 12”X8” ANSI 1 / 10”X8” ANSI 1 / 8”X6” ANSI |  |  |
| 5.43 | Is vessel fitted with a stern manifold? If yes, state size: |  |  |  |  |  |  |  |  |  |  | No |  |  |
| Heating |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.44 | Cargo / slop tanks fitted with a cargo heating system? |  |  |  |  |  |  | Type |  |  |  | Coiled |  | Material |
|  | Cargo Tanks: |  |  |  |  |  |  | HEATING COILS |  |  |  | YES |  | STEEL |
|  | Slop Tanks: |  |  |  |  |  |  | HEATING COILS |  |  |  | YES |  | STEEL |
| 5.45 | Maximum temperature cargo can be loaded / maintained: |  |  |  |  |  |  |  |  |  |  | 70° |  | 66° |
| 5.46 | Minimum temperature cargo can be loaded / maintained: |  |  |  |  |  |  |  |  |  |  | 5° |  | 5° |
| Coating / Anodes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.47 | Tank Coating |  |  | Coated |  | Type |  |  |  |  |  | To What Extent |  | Anodes |
|  | Cargo tanks: |  |  | YES |  | MODIFIED EPOXY |  |  |  |  |  | SLOP TANKS 100%, COTs 1-6: UNDERDECK UP TO 2 M FROM DECK, BOTTOM UP TO 1.5 M HEIGHT |  | NO |
|  | Ballast tanks: |  |  | YES |  | MODIFIED EPOXY |  |  |  |  |  | FULL |  | YES |
|  | Slop tanks: |  |  | YES |  | MODIFIED EPOXY |  |  |  |  |  | WHOLE TANK |  | YES |
| 6. | INERT GAS AND CRUDE OIL WASHING |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6.1 | Is a Crude Oil Washing (COW) installation fitted / operational? |  |  |  |  |  |  |  |  |  |  | Yes / Yes |  |  |
| 6.2 | Is an Inert Gas System (IGS) fitted / operational? |  |  |  |  |  |  |  |  |  |  | Yes / Yes |  |  |
| 6.3 | Is IGS supplied by flue gas, inert gas (IG) generator and/or nitrogen: |  |  |  |  |  |  |  |  |  |  | Flue Gas |  |  |
| 7. | MOORING |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7.1 | Wires (on drums) | No. |  | Diameter |  | Material |  |  |  |  |  | Length |  | Breaking Strength |
|  | Forecastle: | 4 |  | 35 Millimetres |  | STEEL |  |  |  |  |  | 275 Metres |  | 84 Metric Tonnes |
|  | Main deck fwd: | 4 |  | 35 Millimetres |  | STEEL |  |  |  |  |  | 275 Metres |  | 84 Metric Tonnes |
|  | Main deck aft: | 2 |  | 35 Millimetres |  | STEEL |  |  |  |  |  | 275 Metres |  | 84 Metric Tonnes |
|  | Poop deck: | 6 |  | 35 Millimetres |  | STEEL |  |  |  |  |  | 275 Metres |  | 84 Metric Tonnes |
| 7.2 | Wire tails | No. |  | Diameter |  | Material |  |  |  |  |  | Length |  | Breaking Strength |
|  | Forecastle: | 4 |  | 80 Millimetres |  | NYLON |  |  |  |  |  | 11 Meters |  | 115 Metric Tonnes |
|  | Main deck fwd: | 4 |  | 80 Millimetres |  | NYLON |  |  |  |  |  | 11 Meters |  | 115 Metric Tonnes |
|  | Main deck aft: | 2 |  | 80 Millimetres |  | NYLON |  |  |  |  |  | 11 Meters |  | 115 Metric Tonnes |
|  | Poop deck: | 6 |  | 80 Millimetres |  | NYLON |  |  |  |  |  | 11 Meters |  | 115 Metric Tonnes |
| 7.3 | Ropes (on drums) | No. |  | Diameter |  | Material |  |  |  |  |  | Length |  | Breaking Strength |
|  | Forecastle: |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Main deck fwd: |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Main deck aft: |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Poop deck: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7.4 | Other lines | No. |  | Diameter |  | Material |  |  |  |  |  | Length |  | Breaking Strength |
|  | Forecastle: |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Main deck fwd: |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Main deck aft: |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Poop deck: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7.5 | Winches | No. |  | No. Drums |  | Motive Power |  |  |  |  |  | Brake Capacity |  | Type of Brake |
|  | Forecastle: | 2 |  | Double Drums |  | Electro-Hydraulic |  |  |  |  |  | 50.4 Metric Tonnes |  | Manually Spring Applied Fail Safe |
|  | Main deck fwd: | 2 |  | Double Drums |  | Electro-Hydraulic |  |  |  |  |  | 50.4 Metric Tonnes |  | Manually Spring Applied Fail Safe |
|  | Main deck mid: | 1 |  | Double Drums |  | Electro-Hydraulic |  |  |  |  |  | 50.4 Metric Tonnes |  | Manually Spring Applied Fail Safe |
|  | Poop deck: | 3 |  | Double Drums |  | Electro-Hydraulic |  |  |  |  |  | 50.4 Metric Tonnes |  | Manually Spring Applied Fail Safe |
| 7.6 | Bitts, closed chocks/fairleads |  |  | No. Bitts |  | SWL Bitts |  |  |  |  |  | No. Closed Chocks |  | SWL Closed Chocks |
|  | Forecastle: |  |  | 7 |  | 84 MT |  |  |  |  |  | 7 |  | 84 MT |
|  | Main deck fwd: |  |  | 4 |  | 84 MT |  |  |  |  |  | 9 |  | 84 MT |
|  | Main deck aft: |  |  | 2 |  | 92 MT |  |  |  |  |  | 4 |  | 84/92 MT |
|  | Poop deck: |  |  | 7 |  | 84/90 MT |  |  |  |  |  | 13 |  | 84/90 MT |
| Anchors/Emergency Towing System |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7.7 | Number of shackles on port / starboard cable: |  |  |  |  |  |  |  |  |  |  | 13/14 |  |  |
| 7.8 | Type / SWL of Emergency Towing system forward: |  |  |  |  |  |  |  |  |  |  | CHAIN STOPPER |  | 204 |
| 7.9 | Type / SWL of Emergency Towing system aft: |  |  |  |  |  |  |  |  |  |  | STRONG POINT |  | 200 Metric Tonnes |
| Escort Tug |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7.10 | What is size / SWL of closed chock and/or fairleads of enclosed type on stern: |  |  |  |  |  |  |  |  |  |  | 600 X 450 |  | 200 Metric Tonnes |
| 7.11 | What is SWL of bollard on poop deck suitable for escort tug: |  |  |  |  |  |  |  |  |  |  | 200 Metric Tonnes |  |  |
| Bow/Stern Thruster |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7.12 | What is brake horse power of bow thruster (if fitted): |  |  |  |  |  |  |  |  |  |  | N/A |  |  |
| 7.13 | What is brake horse power of stern thruster (if fitted): |  |  |  |  |  |  |  |  |  |  | N/A |  |  |
| Single Point Mooring (SPM) Equipment |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7.14 | Does the vessel meet the recommendations in the latest edition of OCIMF 'Recommendations for Equipment Employed in the Bow Mooring of Conventional Tankers at Single Point Moorings (SPM)’? |  |  |  |  |  |  |  |  |  |  | Yes |  |  |
| 7.15 | If fitted, how many chain stoppers: |  |  |  |  |  |  |  |  |  |  | 2 |  |  |
| 7.16 | State type / SWL of chain stopper(s): |  |  |  |  |  |  |  |  |  |  | TONGUE TYPE |  | 350 Metric Tonnes |
| 7.17 | What is the maximum size chain diameter the bow stopper(s) can handle: |  |  |  |  |  |  |  |  |  |  | 76 Millimeters |  |  |
| 7.18 | Distance between the bow fairlead and chain stopper/bracket: |  |  |  |  |  |  |  |  |  |  | 2.75 M |  |  |
| 7.19 | Is bow chock and/or fairlead of enclosed type of OCIMF recommended size (600mm x 450mm)? If not, give details of size: |  |  |  |  |  |  |  |  |  |  | Yes |  |  |
| Lifting Equipment |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7.20 | Derrick / Crane description (Number, SWL and location): |  |  |  |  |  |  |  |  |  |  | HHC: 2 X 20 TONS- MIDSHIPS PROV. CRANE: 1 X 8 TONS (PORT) PROV. CRANE: 1 X 2 TONS (STD) |  |  |
| 7.21 | What is maximum outreach of cranes / derricks outboard of the ship’s side: |  |  |  |  |  |  |  |  |  |  | HHC: 7.5 M PROV. CRANES (P&S) : 4 M |  |  |
| Ship To Ship Transfer (STS) / Helicopter Operations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7.22 | Does vessel comply with recommendations contained in OCIMF/ICS Ship To Ship Transfer Guide (Petroleum, Chemicals or Liquified Gas, as applicable)? |  |  |  |  |  |  |  |  |  |  | Yes |  |  |
| 7.23 | Can the ship comply with the ICS Helicopter Guidelines? If Yes, state whether winching or landing area provided and diameter of the circle provided: |  |  |  |  |  |  |  |  |  |  | Yes, Landing 16.00 Meters |  |  |
| 8. | MISCELLANEOUS |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Engine |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.1 | Speed |  |  |  |  |  |  |  |  |  |  | Maximum |  | Economic |
|  | Ballast speed: |  |  |  |  |  |  |  |  |  |  | As per C/P |  | As per C/P |
|  | Laden speed: |  |  |  |  |  |  |  |  |  |  | As per C/P |  | As per C/P |
| 8.2 | What type of fuel is used for main propulsion / generating plant: |  |  |  |  |  |  |  |  |  |  | IFO 380 cst / LSMGO |  | IFO 380 cst / LSMGO |
| 8.3 | Type / Capacity of bunker tanks: |  |  |  |  |  |  |  |  |  |  | Fuel Oil: ABT 3,400 Cu. Meters Gas Oil: ABT 600 Cu. Meters |  |  |
| 8.4 | Is vessel fitted with fixed or controllable pitch propeller(s): |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.5 | Engines |  |  |  |  |  |  |  |  | No |  | Capacity |  | Make/Type |
|  | Main engine: |  |  |  |  |  |  |  |  | 1 |  | MCR: 16,590 Kw at 77.1 RPM |  | HYUNDAI-MAN B&W 6G70ME-C9.5 |
|  | Aux engine: (Diesel Engine) |  |  |  |  |  |  |  |  | 3 |  | ABT 1,320 kW @ 900 rpm |  | Hyundai-Himsen 6H21/32 |
|  | (Generator) |  |  |  |  |  |  |  |  | 3 |  | ABT 1,240 kW @ 900 rpm |  | Marine Design IP 23 Enclosure, brushless |
|  | Power packs: |  |  |  |  |  |  |  |  | N/A |  |  |  |  |
|  | Boilers: (Aux. Boiler) |  |  |  |  |  |  |  |  | 2 1 |  | 35,000 kg/hr evaporation, 16/6 kg/cm2, saturated Composite boiler (1.6 T/hr) |  | Automatic, forced draft, heavy fuel oil burning, marine boiler |
| Emissions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.6 | Main engine IMO NOx emission standard: |  |  |  |  |  |  |  |  |  |  | TIER II |  |  |
| 8.7 | Energy Efficiency Design Index (EEDI) rating number: |  |  |  |  |  |  |  |  |  |  | N/A |  |  |
| Insurance |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.8 | P & I Club - Full Style: |  |  |  |  |  |  |  | UK P+I CLUB c/o Thomas Miller P&I Ltd 90 Fenchurch Str., London, EC3M 4ST, UK Tel. +44207204219 Tel: 00306944965411 (EMERGENT) Email: TONY.FIELDER@THOMASMILLER.COM |  |  |  |  |  |
| 8.9 | P & I Club pollution liability coverage / expiration date: |  |  |  |  |  |  |  |  |  |  | 1,000,000,000 US$ |  | 20 FEB 2017 |
| 8.10 | Hull & Machinery insured by - Full Style: |  |  |  |  |  |  |  | Hull & Machinery ins. is placed at Lloyds/ Companies, in USA and Italy |  |  |  |  |  |
| 8.11 | Hull & Machinery insured value / expiration date: |  |  |  |  |  |  |  |  |  |  | 80,000,000 USD |  | 31 DEC 2017 |
| Recent Operational History |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.12 | Date and place of last Port State Control inspection: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.13 | Any outstanding deficiencies as reported by any Port State Control? If yes, provide details: |  |  |  |  |  |  |  |  |  |  | No |  |  |
| 8.14 | Has vessel been involved in a pollution, grounding, serious casualty or collision incident during the past 12 months? If yes, full description: |  |  |  |  |  |  |  |  |  |  | Pollution: No Grounding: No Casualty: No Collision: No |  |  |
| 8.15 | Last three cargoes / charterers / voyages (Last / 2nd Last / 3rd Last): |  |  |  |  |  |  |  |  |  |  | CURRENT: CRUDE OIL / PETROBRAS / ANGRA DOS REIS / ROTTERDAM 2ND LAST: GASOIL / TRAFIGURA / YOSU & ULSAN / LUANDA & TEMA |  |  |
| 8.16 | Date/place of last STS operation: |  |  |  |  |  |  |  |  |  |  | 14 SEP 2016/TEMA-GHANA |  |  |
| Vetting |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.17 | Date of last SIRE inspection: |  |  |  |  |  |  |  |  |  |  | 22 July 2016 |  |  |
| 8.18 | Date of last CDI inspection: |  |  |  |  |  |  |  |  |  |  | - |  |  |
| 8.19 | Recent Oil company inspections/screenings (To the best of owners knowledge and without guarantee of acceptance for future business)*: * "Approvals" are not given by Oil Majors and ships are accepted for the voyage on a case by case basis. |  |  |  |  |  |  |  |  |  |  | Motoroil, Trafigura, GSCaltex, SK. |  |  |
| Additional Information |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.20 | Additional information relating to features of the ship or operational characteristics: |  |  |  |  |  |  |  |  |  |  | N/A |  |  |

Rev 2015 (INTERTANKO / Q88.com)

Form completed on http://www.q88.com/integration.aspx  Please email support@q88.com an updated copy if this is not the latest version.