- INTERTANKO TANKER CHARTERING QUESTIONNAIRE 88 	Version 4

| 1. | VESSEL DESCRIPTION |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1.1 | Date updated: |  |  |  |  |  |  |  |  |  |  | 22.MAy.2018 |  |  |
| 1.2 | Vessel’s name (IMO number): |  |  |  |  |  |  |  |  |  |  | Sidra Ras Laffan (9339648) |  |  |
| 1.3 | Vessel’s previous name(s) and date(s) of change: |  |  |  |  |  |  |  |  |  |  | Alam Cergas (Dec 21, 2007) |  |  |
| 1.4 | Date delivered / Builder (where built): |  |  |  |  |  |  |  |  |  |  | Jun 20, 2007 / Dalian Shipbuilding Industry Co. Ltd |  |  |
| 1.5 | Flag / Port of Registry: |  |  |  |  |  |  |  |  |  |  | Liberia / MONROVIA |  |  |
| 1.6 | Call sign / MMSI: |  |  |  |  |  |  |  |  |  |  | A8OT6 / 636 013 705 |  |  |
| 1.7 | Vessel’s contact details (satcom/fax/email etc.): |  |  |  |  |  |  |  |  |  |  | Tel: +870 773 140 235 Fax: Email: Sidra.raslaffan@woqodmarine.com.qa |  |  |
| 1.8 | Type of vessel (as described in Form A or Form B Q1.11 of the IOPPC): |  |  |  |  |  |  |  |  |  |  | Oil Tanker |  |  |
| 1.9 | Type of hull: |  |  |  |  |  |  |  |  |  |  | Double Hull |  |  |
| Classification |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1.10 | Classification society: |  |  |  |  |  |  |  |  |  |  | Lloyds Register |  |  |
| 1.11 | Class notation: |  |  |  |  |  |  |  |  |  |  | LRS +100A1 Double Hull Oil & Chem, Tanker type 3 Veg. oil only, ESP, Shipright (FDA, SDA, CM) *IES, SPM, LI, +LMC, IGS, UMS, Shipright (SM, SERS, BWMP (F,S+F)), COW, SBT, ETA |  |  |
| 1.12 | Is the vessel subject to any conditions of class, class extensions, outstanding memorandums or class recommendations? If yes, give details: |  |  |  |  |  |  |  |  |  |  | No |  |  |
| 1.13 | If classification society changed, name of previous and date of change: |  |  |  |  |  |  |  |  |  |  | New Construction, |  |  |
| 1.14 | IMO type, if applicable: |  |  |  |  |  |  |  |  |  |  | 3 |  |  |
| 1.15 | Does the vessel have ice class? If yes, state what level: |  |  |  |  |  |  |  |  |  |  | No, |  |  |
| 1.16 | Date / place of last dry-dock: |  |  |  |  |  |  |  |  |  |  | Oct 16, 2017 / Colombo |  |  |
| 1.17 | Date next dry dock due / next annual survey due: |  |  |  |  |  |  |  |  |  |  | Jul 19, 2020 |  | Jul 19, 2018 |
| 1.18 | Date of last special survey / next special survey due: |  |  |  |  |  |  |  |  |  |  | Oct 25, 2017 |  | Jul 19, 2022 |
| 1.19 | If ship has Condition Assessment Program (CAP), what is the latest overall rating: |  |  |  |  |  |  |  |  |  |  | No, |  |  |
| 1.20 | Does the vessel have a statement of compliance issued under the provisions of the Condition Assessment Scheme (CAS): If yes, what is the expiry date? |  |  |  |  |  |  |  |  |  |  | N/A |  |  |
| Dimensions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1.21 | Length overall (LOA): |  |  |  |  |  |  |  |  |  |  | 171.20 Metres |  |  |
| 1.22 | Length between perpendiculars (LBP): |  |  |  |  |  |  |  |  |  |  | 163.40 Metres |  |  |
| 1.23 | Extreme breadth (Beam): |  |  |  |  |  |  |  |  |  |  | 27.42 Metres |  |  |
| 1.24 | Moulded depth: |  |  |  |  |  |  |  |  |  |  | 17.31 Metres |  |  |
| 1.25 | Keel to masthead (KTM)/ Keel to masthead (KTM) in collapsed condition, if applicable: |  |  |  |  |  |  |  |  |  |  | 44.70 Metres |  |  |
| 1.26 | Bow to center manifold (BCM) / Stern to center manifold (SCM): |  |  |  |  |  |  |  |  |  |  | 83.30 Metres |  | 87.90 Metres |
| 1.27 | Distance bridge front to center of manifold: |  |  |  |  |  |  |  |  |  |  | 51.70 Metres |  |  |
| 1.28 | Parallel body distances |  |  |  |  | Lightship |  |  |  |  |  | Normal Ballast |  | Summer Dwt |
|  | # Forward to mid-point manifold: |  |  |  |  | 35.21 Metres |  |  |  |  |  | 39.18 Metres |  | 49.28 Metres |
|  | # Aft to mid-point manifold: |  |  |  |  | 17.16 Metres |  |  |  |  |  | 34.57 Metres |  | 58.04 Metres |
|  | # Parallel body length: |  |  |  |  | 52.37 Metres |  |  |  |  |  | 70.13 Metres |  | 107.32 Metres |
| # 1.29 | # FWA/TPC at summer draft: |  |  |  |  |  |  |  |  |  |  | 264.00 Millimetres |  | 41.50 Metric Tonnes |
| 1.30 | Constant (excluding fresh water): |  |  |  |  |  |  |  |  |  |  | 100 Metric Tonnes |  |  |
| 1.31 | What is the company guidelines for Under Keel Clearance (UKC) for this vessel? |  |  |  |  |  |  |  |  |  |  | Ocean passages - 20% of the deepest draft Fairways - 15% of the deepest draft Inside ports/Canals- 10% of the deepest draft Whilst Moored (Berth/SBM/CBM)- 1.5% of the ship’s extreme breadth or 0.3m |  |  |
| 1.32 | What is the max height of mast above waterline (air draft) |  |  |  |  |  |  |  |  |  |  | Full Mast |  | Collapsed Mast |
|  | Lightship: |  |  |  |  |  |  |  |  |  |  | 41.73 Metres |  | 0 Metres |
|  | Normal ballast: |  |  |  |  |  |  |  |  |  |  | 37.65 Metres |  | 0 Metres |
|  | At loaded summer deadweight: |  |  |  |  |  |  |  |  |  |  | 32.81 Metres |  | 0 Metres |
| Tonnages |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1.33 | Net Tonnage: |  |  |  |  |  |  |  |  |  |  | 9,431.00 |  |  |
| 1.34 | Gross Tonnage / Reduced Gross Tonnage (if applicable): |  |  |  |  |  |  |  |  |  |  | 22,184.00 |  |  |
| 1.35 | Suez Canal Tonnage - Gross (SCGT) / Net (SCNT): |  |  |  |  |  |  |  |  |  |  | 23,621.64 |  | 20,439.95 |
| # 1.36 | Panama Canal Net Tonnage (PCNT): |  |  |  |  |  |  |  |  |  |  | 18,492.00 |  |  |
| Ownership and Operation |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1.37 | Registered owner - Full style: |  |  |  |  |  | Sidra Ras Laffan Shipping Company 80 Broad Street Monrovia Liberia Company IMO#: 5357881 |  |  |  |  |  |  |  |
| 1.38 | Technical operator - Full style: |  |  |  |  |  | WOQOD MARINE SERVICES |  |  |  |  |  |  |  |
| 1.39 | Commercial operator - Full style: |  |  |  |  |  | WSC SHIPPING PTE LTD 20 Collyer Quay, #09-02, S 049319 Ph: +65 62270115, Fax: +65 62245704 |  |  |  |  |  |  |  |
| 1.40 | Disponent owner - Full style: |  |  |  |  |  | WSC SHIPPING PTE LTD 20 Collyer Quay, #09-02, S 049319 Ph: +65 62270115, Fax: +65 62245704 |  |  |  |  |  |  |  |
| 2. | CERTIFICATION |  |  |  |  | Issued |  |  |  |  |  | Last Annual |  | Expires |
| 2.1 | Safety Equipment Certificate (SEC): |  |  |  |  | Nov 12, 2017 |  |  |  |  |  |  |  | June 19, 2022 |
| 2.2 | Safety Radio Certificate (SRC): |  |  |  |  | Nov 12, 2017 |  |  |  |  |  |  |  | June 19, 2022 |
| 2.3 | Safety Construction Certificate (SCC): |  |  |  |  | Nov 12, 2017 |  |  |  |  |  |  |  | June 19, 2022 |
| 2.4 | International Loadline Certificate (ILC): |  |  |  |  | Nov 12, 2017 |  |  |  |  |  |  |  | June 19, 2022 |
| 2.5 | International Oil Pollution Prevention Certificate (IOPPC): |  |  |  |  | Feb 14, 2018 |  |  |  |  |  |  |  | Aug 07, 2018 |
| 2.6 | ISM Safety Management Certificate (SMC): |  |  |  |  | Nov 22, 2015 |  |  |  |  |  | None |  | Nov 21, 2020 |
| # 2.7 | # Document of Compliance (DOC): |  |  |  |  | # Aug 11, 2015 |  |  |  |  |  | Aug 21, 2017 |  | Aug 10, 2020 |
| 2.8 | USCG Certificate of Compliance (COC): |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2.9 | Civil Liability Convention (CLC) 1992 Certificate: |  |  |  |  | Feb 20, 2018 |  |  |  |  |  | Not Applicable |  | Feb 20, 2019 |
| 2.10 | Civil Liability for Bunker Oil Pollution Damage Convention (CLBC) Certificate: |  |  |  |  | Feb 20, 2018 |  |  |  |  |  | Not Applicable |  | Feb 20, 2019 |
| 2.11 | Ship Sanitation Control (SSCC)/Ship Sanitation Control Exemption (SSCE) Certificate: |  |  |  |  | april, 2018 |  |  |  |  |  | Not Applicable |  | OCT, 2018 |
| 2.12 | U.S. Certificate of Financial Responsibility (COFR): |  |  |  |  |  |  |  |  |  |  | Not Applicable |  |  |
| 2.13 | Certificate of Class (COC): |  |  |  |  | Nov 12, 2017 |  |  |  |  |  |  |  | June 19, 2022 |
| # 2.14 | # International Sewage Pollution Prevention Certificate (ISPPC): |  |  |  |  | # Jun 17, 2017 |  |  |  |  |  | # Not Applicable |  | June 19, 2022 |
| # 2.15 | # Certificate of Fitness (COF): |  |  |  |  | # Nov 12, 2017 |  |  |  |  |  |  |  | Jul 19, 2022 |
| # 2.16 | # International Energy Efficiency Certificate (IEEC): |  |  |  |  | # Jul 12, 2015 |  |  |  |  |  | Not Applicable |  | Not Applicable |
| # 2.17 | # International Ship Security Certificate (ISSC): |  |  |  |  | # Nov 22, 2015 |  |  |  |  |  | # None |  | Nov 21, 2020 |
| # 2.18 | # International Air Pollution Prevention Certificate (IAPPC): |  |  |  |  | # Nov 10, 2017 |  |  |  |  |  |  |  | June 19, 2022 |
| # 2.19 | # Maritime Labour Certificate (MLC): |  |  |  |  | # Aug 26, 2013 |  |  |  |  |  | Not Applicable |  | Aug 25, 2018 |
| Documentation |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2.20 | Owner warrant that vessel is member of ITOPF and will remain so for the entire duration of this voyage/contract: |  |  |  |  |  |  |  |  |  |  | Yes |  |  |
| 2.21 | Does vessel have in place a Drug and Alcohol Policy complying with OCIMF guidelines for Control of Drugs and Alcohol Onboard Ship? |  |  |  |  |  |  |  |  |  |  | Yes |  |  |
| 2.22 | Is the ITF Special Agreement on board (if applicable)? |  |  |  |  |  |  |  |  |  |  | N/A |  |  |
| 2.23 | ITF Blue Card expiry date: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3. | CREW |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3.1 | Nationality of Master: |  |  |  |  |  |  |  |  |  |  | Myanmar |  |  |
| 3.2 | Number and Nationality of Officers: |  |  |  |  |  |  |  |  |  |  | 10 Myanmar, Pakistani, Jordanian, Egyptian, Eritrean, Indian, Sri Lankan |  |  |
| 3.3 | Number and Nationality of Crew: |  |  |  |  |  |  |  |  |  |  | 15 Indian, Myanmar, Pakistan, Sudanese, Ethiopian, Nepali , Ghanaian |  |  |
| 3.4 | What is the common working language onboard: |  |  |  |  |  |  |  |  |  |  | ENGLISH |  |  |
| 3.5 | Do officers speak and understand English? |  |  |  |  |  |  |  |  |  |  | Yes |  |  |
| 3.6 | If Officers/Crew employed by a Manning Agency - Full style: |  |  |  |  |  | Officers: Hausbau Marine Co. P. O. Box: 1298, Post Code. 11821 Amman, Jordan Tel: +962 6 5820 805 Fax: +962 6 5820 767 Email: mpd@hausbaumarine.com Crew: Hausbau Marine Co. P. O. Box: 1298, Post Code. 11821 Amman, Jordan Tel: +962 6 5820805 Fax: +962 6 5820767 Email: mpd@hausbaumarine.com |  |  |  |  |  |  |  |
| 4. | FOR USA CALLS |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4.1 | Has the vessel Operator submitted a Vessel Spill Response Plan to the US Coast Guard which has been approved by official USCG letter? |  |  |  |  |  |  |  |  |  |  |  | N/A |  |
| 4.2 | Qualified individual (QI) - Full style: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4.3 | Oil Spill Response Organization (OSRO) - Full style: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5. | CARGO AND BALLAST HANDLING |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Double Hull Vessels |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.1 | Is vessel fitted with centerline bulkhead in all cargo tanks? If Yes, solid or perforated: |  |  |  |  |  |  |  |  |  |  | Yes, Solid |  |  |
| # Loadline Information |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| # 5.2 | # Loadline |  | # Freeboard |  |  | # Draft |  |  |  |  | # Deadweight |  |  | # Displacement |
|  | # Summer: |  | 5.42 Metres |  |  | 11.89 Metres |  |  |  |  | # 34,671.00 Metric Tonnes |  |  | 43,730.00 Metric Tonnes |
|  | # Winter: |  | 5.67 Metres |  |  | 11.64 Metres |  |  |  |  | 33,602.00 Metric Tonnes |  |  | 42,661.00 Metric Tonnes |
|  | # Tropical: |  | 5.18 Metres |  |  | 12.13 Metres |  |  |  |  | 35,368.00 Metric Tonnes |  |  | 44,697.00 Metric Tonnes |
|  | # Lightship: |  | 14.34 Metres |  |  | 2.97 Metres |  |  |  |  | Not Applicable |  |  | 0.00 Metric Tonnes |
|  | # Normal Ballast Condition: |  | 10.14 Metres |  |  | 7.17 Metres |  |  |  |  | 15,921.00 Metric Tonnes |  |  | 24,980.00 Metric Tonnes |
| # 5.3 | # Does vessel have multiple SDWT? If yes, please provide all assigned loadlines: |  |  |  |  |  |  |  |  |  | No |  |  |  |
| Cargo Tank Capacities |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.4 | Number of cargo tanks and total cubic capacity (98%): |  |  |  |  |  |  |  |  |  |  | 12 |  | 36,736.4 m3 |
| 5.5 | Capacity (98%) of each natural segregation with double valve (specify tanks): |  |  |  |  |  |  |  |  |  |  | 1 P&S: 4,404.8 m3 2 P&S: 6,275.9 m3 3 P&S: 7,548.9 m3 4 P&S: 8,516.6 m3 5 P&S: 7,089.2 m3 6 P&S: 2,901.0 m3 |  |  |
| 5.6 | Number of slop tanks and total cubic capacity (98%): |  |  |  |  |  |  |  |  |  |  | 2 |  | 1,315.6 m3 |
| 5.7 | Specify segregations which slops tanks belong to and their capacity with double valve: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.8 | Residual/Retention oil tank(s) capacity (98%), if applicable: |  |  |  |  |  |  |  |  |  |  | 142.9 m3 |  |  |
| 5.9 | Does vessel have Segregated Ballast Tanks (SBT) or Clean Ballast Tanks (CBT): |  |  |  |  |  |  |  |  |  |  | SBT |  |  |
| SBT Vessels |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.10 | What is total SBT capacity and percentage of SDWT vessel can maintain? |  |  |  |  |  |  |  |  |  |  | 15,949.70 Cu. Metres |  | 47.00 % |
| 5.11 | Does vessel meet the requirements of MARPOL Annex I Reg 18.2: |  |  |  |  |  |  |  |  |  |  | Yes |  |  |
| Cargo Handling and Pumping Systems |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.12 | How many grades/products can vessel load/discharge with double valve segregation: |  |  |  |  |  |  |  |  |  |  | 6 |  |  |
| 5.13 | Are there any cargo tank filling restrictions? If yes, specify number of slack tanks, max s.g., ullage restrictions etc.: |  |  |  |  |  |  |  |  |  |  | No Nil |  |  |
| 5.14 | Pumps |  |  |  | No. | Type |  |  |  |  |  | Capacity |  | At What Head (sg=1.0) |
|  | Cargo Pumps: |  |  |  | 10 | FRAMO P/P |  |  |  |  |  | 500 CU M/HR |  |  |
|  | Cargo Eductors: |  |  |  | 1 | FRAMO P/P |  |  |  |  |  | 70 CU M /HR |  |  |
|  | Stripping: |  |  |  | 4 | FRAMO P/P |  |  |  |  |  | 300 CU M /HR |  |  |
|  | Ballast Pumps: |  |  |  | 2 | Framo 300-3 |  |  |  |  |  | 1,000 Cu. Metres/Hour |  | 25 Metres |
|  | Ballast Eductors: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.15 | Max loading rate for homogenous cargo per manifold connection: |  |  |  |  |  |  |  |  |  |  | 768 CU M /HR |  |  |
| 5.16 | Max loading rate for homogenous cargo loaded simultaneously through all manifolds: |  |  |  |  |  |  |  |  |  |  | 4,608.00 Cu. Metres/Hour |  |  |
| 5.17 | How many cargo pumps can be run simultaneously at full capacity: |  |  |  |  |  |  |  |  |  |  | 6 |  |  |
| Cargo Control Room |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.18 | Is ship fitted with a Cargo Control Room (CCR)? |  |  |  |  |  |  |  |  |  |  | Yes |  |  |
| 5.19 | Can tank innage / ullage be read from the CCR? |  |  |  |  |  |  |  |  |  |  | Yes |  |  |
| Gauging and Sampling |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.20 | Can cargo be transferred under closed loading conditions in accordance with ISGOTT 11.1.6.6? |  |  |  |  |  |  |  |  |  |  | Yes |  |  |
| 5.21 | What type of fixed closed tank gauging system is fitted: |  |  |  |  |  |  |  |  |  |  | Radar |  |  |
| 5.22 | Number of portable gauging units (example- MMC) on board: |  |  |  |  |  |  |  |  |  |  | 2 |  |  |
| 5.23 | Are overfill (high) alarms fitted? If Yes, indicate whether to all tanks or partial: |  |  |  |  |  |  |  |  |  |  | Yes, All |  |  |
| 5.24 | Are cargo tanks fitted with multipoint gauging? If yes, specify type and locations: |  |  |  |  |  |  |  |  |  |  | , |  |  |
| 5.25 | Is gauging system certified and calibrated? If no, specify which ones are not calibrated: |  |  |  |  |  |  |  |  |  |  | Yes, |  |  |
| Vapor Emission Control System (VECS) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.26 | Is a Vapour Emission Control System (VECS) fitted? |  |  |  |  |  |  |  |  |  |  | Yes |  |  |
| 5.27 | Number/size of VECS manifolds (per side): |  |  |  |  |  |  |  |  |  |  | 2 |  | 400 Millimetres |
| 5.28 | Number / size / type of VECS reducers: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Venting |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.29 | State what type of venting system is fitted: |  |  |  |  |  |  |  |  |  |  | High Velocity P/V |  |  |
| Cargo Manifolds and Reducers |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.30 | Does vessel comply with the latest edition of the OCIMF 'Recommendations for Oil Tanker Manifolds and Associated Equipment’? |  |  |  |  |  |  |  |  |  |  | Yes |  |  |
| 5.31 | Total number / size of cargo manifold connections on each side: |  |  |  |  |  |  |  |  |  |  | 6 / 400 Millimetres |  |  |
| 5.32 | What type of valves are fitted at manifold: |  |  |  |  |  |  |  |  |  |  | Butterfly |  |  |
| 5.33 | What is the material/rating of the manifold: |  |  |  |  |  |  |  |  |  |  | MS / |  |  |
| 5.34 | Does the vessel have a Common Line Manifold connection? If yes, describe: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.35 | Distance between cargo manifold centers: |  |  |  |  |  |  |  |  |  |  | 2,000.00 Millimetres |  |  |
| 5.36 | Distance ships rail to manifold: |  |  |  |  |  |  |  |  |  |  | 4,000.00 Millimetres |  |  |
| 5.37 | Distance manifold to ships side: |  |  |  |  |  |  |  |  |  |  | 4,600.00 Millimetres |  |  |
| 5.38 | Top of rail to center of manifold: |  |  |  |  |  |  |  |  |  |  | 600.00 Millimetres |  |  |
| 5.39 | Distance main deck to center of manifold: |  |  |  |  |  |  |  |  |  |  | 1,950.00 Millimetres |  |  |
| 5.40 | Spill tank grating to center of manifold: |  |  |  |  |  |  |  |  |  |  | 900.00 Millimetres |  |  |
| 5.41 | Manifold height above the waterline in normal ballast / at SDWT condition: |  |  |  |  |  |  |  |  |  |  | 12.09 Metres |  | 7.37 Metres |
| 5.42 | Number / size / type of reducers: |  |  |  |  |  |  |  |  |  |  | None ANSI |  |  |
| 5.43 | Is vessel fitted with a stern manifold? If yes, state size: |  |  |  |  |  |  |  |  |  |  | No, |  |  |
| Heating |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.44 | Cargo / slop tanks fitted with a cargo heating system? |  |  |  |  |  |  | Type |  |  |  | Coiled |  | Material |
|  | Cargo Tanks: |  |  |  |  |  |  | Steam |  |  |  | Yes |  | Mildsteel |
|  | Slop Tanks: |  |  |  |  |  |  | Steam |  |  |  | Yes |  | MS |
| 5.45 | Maximum temperature cargo can be loaded / maintained: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.46 | Minimum temperature cargo can be loaded / maintained: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Coating / Anodes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.47 | Tank Coating |  |  | Coated |  | Type |  |  |  |  |  | To What Extent |  | Anodes |
|  | Cargo tanks: |  |  | Yes |  | Epoxy, Sidmagard |  |  |  |  |  | Whole Tank |  | No |
|  | Ballast tanks: |  |  | Yes |  | Epoxy |  |  |  |  |  | Whole |  | Yes |
|  | Slop tanks: |  |  | Yes |  | Epoxy |  |  |  |  |  | Whole Tank |  | No |
| 6. | INERT GAS AND CRUDE OIL WASHING |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6.1 | Is a Crude Oil Washing (COW) installation fitted / operational? |  |  |  |  |  |  |  |  |  |  | Yes / Yes |  |  |
| 6.2 | Is an Inert Gas System (IGS) fitted / operational? |  |  |  |  |  |  |  |  |  |  | Yes / Yes |  |  |
| 6.3 | Is IGS supplied by flue gas, inert gas (IG) generator and/or nitrogen: |  |  |  |  |  |  |  |  |  |  | IG Generator |  |  |
| 7. | MOORING |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7.1 | Wires (on drums) | No. |  | Diameter |  | Material |  |  |  |  |  | Length |  | Breaking Strength |
|  | Forecastle: |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Main deck fwd: |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Main deck aft: |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Poop deck: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7.2 | Wire tails | No. |  | Diameter |  | Material |  |  |  |  |  | Length |  | Breaking Strength |
|  | Forecastle: |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Main deck fwd: |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Main deck aft: |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Poop deck: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7.3 | Ropes (on drums) | No. |  | Diameter |  | Material |  |  |  |  |  | Length |  | Breaking Strength |
|  | Forecastle: | 6 |  | 56.00 Millimetres |  | Polymix Polyester 50/50 |  |  |  |  |  | 220.00 Metres |  | 80.30 Metric Tonnes |
|  | Main deck fwd: | 2 |  | 56.00 Millimetres |  | Polymix Polyester 50/50 |  |  |  |  |  | 220.00 Metres |  | 80.30 Metric Tonnes |
|  | Main deck aft: | 2 |  | 56.00 Millimetres |  | Polymix Polyester 50/50 |  |  |  |  |  | 220.00 Metres |  | 63.00 Metric Tonnes |
|  | Poop deck: | 5 |  | 56.00 Millimetres |  | Polymix Polyester 50/50 |  |  |  |  |  | 220.00 Metres |  | 63.00 Metric Tonnes |
| 7.4 | Other lines | No. |  | Diameter |  | Material |  |  |  |  |  | Length |  | Breaking Strength |
|  | Forecastle: | 1 |  | 56.00 Millimetres |  | Polymix Polyester 50/50 |  |  |  |  |  | 220.00 Metres |  | 80.30 Metric Tonnes |
|  | Main deck fwd: |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Main deck aft: |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Poop deck: | 1 |  | 56.00 Millimetres |  | Polymix Polyester 50/50 |  |  |  |  |  | 220.00 Metres |  | 80.30 Metric Tonnes |
| 7.5 | Winches | No. |  | No. Drums |  | Motive Power |  |  |  |  |  | Brake Capacity |  | Type of Brake |
|  | Forecastle: | 3 |  | Double Drums |  | Hydraulic |  |  |  |  |  | 50.00 Metric Tonnes |  |  |
|  | Main deck fwd: | 1 |  | Double Drums |  | Hydraulic |  |  |  |  |  | 50.00 Metric Tonnes |  |  |
|  | Main deck aft: | 1 |  | Double Drums |  | Hydraulic |  |  |  |  |  | 50.00 Metric Tonnes |  |  |
|  | Poop deck: | 3 |  | Double Drums |  | Hydraulic |  |  |  |  |  | 50.00 Metric Tonnes |  |  |
| 7.6 | Bitts, closed chocks/fairleads |  |  | No. Bitts |  | SWL Bitts |  |  |  |  |  | No. Closed Chocks |  | SWL Closed Chocks |
|  | Forecastle: |  |  | 6 |  |  |  |  |  |  |  |  |  |  |
|  | Main deck fwd: |  |  | 1 |  |  |  |  |  |  |  |  |  |  |
|  | Main deck aft: |  |  | 2 |  |  |  |  |  |  |  |  |  |  |
|  | Poop deck: |  |  | 8 |  |  |  |  |  |  |  |  |  |  |
| Anchors/Emergency Towing System |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7.7 | Number of shackles on port / starboard cable: |  |  |  |  |  |  |  |  |  |  | 12 / 11 |  |  |
| 7.8 | Type / SWL of Emergency Towing system forward: |  |  |  |  |  |  |  |  |  |  | Chafing Chain |  | 200 Metric Tonnes |
| 7.9 | Type / SWL of Emergency Towing system aft: |  |  |  |  |  |  |  |  |  |  | Tubular Strong Point |  | 200 Metric Tonnes |
| Escort Tug |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7.10 | What is size / SWL of closed chock and/or fairleads of enclosed type on stern: |  |  |  |  |  |  |  |  |  |  | 600 |  | 116.00 Metric Tonnes |
| 7.11 | What is SWL of bollard on poop deck suitable for escort tug: |  |  |  |  |  |  |  |  |  |  | 64.00 Metric Tonnes |  |  |
| Bow/Stern Thruster |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7.12 | What is brake horse power of bow thruster (if fitted): |  |  |  |  |  |  |  |  |  |  | No, |  |  |
| 7.13 | What is brake horse power of stern thruster (if fitted): |  |  |  |  |  |  |  |  |  |  | No, |  |  |
| Single Point Mooring (SPM) Equipment |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7.14 | Does the vessel meet the recommendations in the latest edition of OCIMF 'Recommendations for Equipment Employed in the Bow Mooring of Conventional Tankers at Single Point Moorings (SPM)’? |  |  |  |  |  |  |  |  |  |  | Yes |  |  |
| 7.15 | If fitted, how many chain stoppers: |  |  |  |  |  |  |  |  |  |  | 1 |  |  |
| 7.16 | State type / SWL of chain stopper(s): |  |  |  |  |  |  |  |  |  |  | Tongue Type |  | 200.00 Metric Tonnes |
| 7.17 | What is the maximum size chain diameter the bow stopper(s) can handle: |  |  |  |  |  |  |  |  |  |  | 76.00 Millimetres |  |  |
| 7.18 | Distance between the bow fairlead and chain stopper/bracket: |  |  |  |  |  |  |  |  |  |  | 3450.00 Milimetres |  |  |
| 7.19 | Is bow chock and/or fairlead of enclosed type of OCIMF recommended size (600mm x 450mm)? If not, give details of size: |  |  |  |  |  |  |  |  |  |  | Yes 600 |  |  |
| Lifting Equipment |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7.20 | Derrick / Crane description (Number, SWL and location): |  |  |  |  |  |  |  |  |  |  | Cranes: 1 x 10.00 Tonnes center |  |  |
| 7.21 | What is maximum outreach of cranes / derricks outboard of the ship’s side: |  |  |  |  |  |  |  |  |  |  | 5.00 Metres |  |  |
| Ship To Ship Transfer (STS) / Helicopter Operations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7.22 | Does vessel comply with recommendations contained in OCIMF/ICS Ship To Ship Transfer Guide (Petroleum, Chemicals or Liquified Gas, as applicable)? |  |  |  |  |  |  |  |  |  |  | Yes |  |  |
| 7.23 | Can the ship comply with the ICS Helicopter Guidelines? If Yes, state whether winching or landing area provided and diameter of the circle provided: |  |  |  |  |  |  |  |  |  |  | Yes, Winching 5.00 Metres |  |  |
| 8. | MISCELLANEOUS |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Engine |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.1 | Speed |  |  |  |  |  |  |  |  |  |  | Maximum |  | Economic |
|  | Ballast speed: |  |  |  |  |  |  |  |  |  |  | 12 Knots (WSNP) |  | 11 Knots (WSNP) |
|  | Laden speed: |  |  |  |  |  |  |  |  |  |  | 11.50 Knots (WSNP) |  | 10 Knots (WSNP) |
| 8.2 | What type of fuel is used for main propulsion / generating plant: |  |  |  |  |  |  |  |  |  |  | HFO 380 CST |  | HFO 380 CST |
| 8.3 | Type / Capacity of bunker tanks: |  |  |  |  |  |  |  |  |  |  | Fuel Oil: 1,479 Cu. Metres Diesel Oil: 176 Cu. Metres Gas Oil: |  |  |
| 8.4 | Is vessel fitted with fixed or controllable pitch propeller(s): |  |  |  |  |  |  |  |  |  |  | Fixed |  |  |
| 8.5 | Engines |  |  |  |  |  |  |  |  | No |  | Capacity |  | Make/Type |
|  | Main engine: |  |  |  |  |  |  |  |  | 1 |  | 7,150 Kilowatt |  | MAN B&W 5S50MC |
|  | Aux engine: |  |  |  |  |  |  |  |  | 3 |  |  |  |  |
|  | Power packs: |  |  |  |  |  |  |  |  | 3 |  |  |  | FRAMO |
|  | Boilers: |  |  |  |  |  |  |  |  | 1 |  | 18.00 Metric Tonnes/Hour |  |  |
| Emissions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.6 | Main engine IMO NOx emission standard: |  |  |  |  |  |  |  |  |  |  | Tier II |  |  |
| 8.7 | Energy Efficiency Design Index (EEDI) rating number: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Insurance |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.8 | P & I Club - Full Style: |  |  |  |  |  |  |  | SKULD P.O.Box 1376 Vika No-0114 Oslo, Norway Tel: +47 22002200 Fax: +47 85028302 Email: oslo2@skuld.com Web: www.skuld.com |  |  |  |  |  |
| 8.9 | P & I Club pollution liability coverage / expiration date: |  |  |  |  |  |  |  |  |  |  | 1,000,000,000 US$ |  | Feb 20, 2019 |
| 8.10 | Hull & Machinery insured by - Full Style: |  |  |  |  |  |  |  | QATAR INSURANCE COMPANY P.O. Box 666- Tamin St West Bay - Doha Qatar Tel: +974 4496 2222 Fax: +974 4483 1569 |  |  |  |  |  |
| 8.11 | Hull & Machinery insured value / expiration date: |  |  |  |  |  |  |  |  |  |  | 20,000,000 US$ |  | Dec 31, 2018 |
| Recent Operational History |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.12 | Date and place of last Port State Control inspection: |  |  |  |  |  |  |  |  |  |  | Sep 18, 2017 |  |  |
| 8.13 | Any outstanding deficiencies as reported by any Port State Control? If yes, provide details: |  |  |  |  |  |  |  |  |  |  | No |  |  |
| 8.14 | Has vessel been involved in a pollution, grounding, serious casualty or collision incident during the past 12 months? If yes, full description: |  |  |  |  |  |  |  |  |  |  | Pollution: No, Grounding: No, Casualty: No, Collision: No, |  |  |
| 8.15 | Last three cargoes / charterers / voyages (Last / 2nd Last / 3rd Last): |  |  |  |  |  |  |  |  |  |  | Fuel oil /HFO / HFO |  |  |
| 8.16 | Date/place of last STS operation: |  |  |  |  |  |  |  |  |  |  | 18.DEC. 2017 TANJUNG PELAPAS, MALAYSIA |  |  |
| Vetting |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.17 | Date of last SIRE inspection: |  |  |  |  |  |  |  |  |  |  | 20TH May // shell |  |  |
| 8.18 | Date of last CDI inspection: |  |  |  |  |  |  |  |  |  |  | N/A |  |  |
| 8.19 | Recent Oil company inspections/screenings (To the best of owners knowledge and without guarantee of acceptance for future business)*: * "Approvals" are not given by Oil Majors and ships are accepted for the voyage on a case by case basis. |  |  |  |  |  |  |  |  |  |  | SHELL, TASWEEQ, VIVA |  |  |
| Additional Information |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.20 | Additional information relating to features of the ship or operational characteristics: |  |  |  |  |  |  |  |  |  |  |  |  |  |

Rev 2015 (INTERTANKO / Q88.com)

Form completed on http://www.q88.com/integration.aspx  Please email support@q88.com an updated copy if this is not the latest version.

To the best of owner's knowledge all information is true and given without any guarantee.

Each person consumes almost 6 trees worth of paper per year. Please consider the environment before printing this e-mail. 
 
This e-mail message may contain confidential, proprietary or legally privileged information. It should not be used by anyone who is not the original intended recipient. If you have erroneously received this message, please delete it immediately and notify the sender. The recipien