- INTERTANKO CHARTERING QUESTIONNAIRE 88 - OIL/CHEMICAL	Version 5

| 1. | GENERAL INFORMATION |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1.1 | Date updated: |  |  |  |  |  |  | Jul 12, 2019 |  |
| 1.2 | Vessel’s name (IMO number): |  |  |  |  |  |  | Flores Palm (9385051) |  |
| 1.3 | Vessel’s previous name(s) and date(s) of change: |  |  |  |  |  |  | Not Applicable |  |
| 1.4 | Date delivered/Builder (where built): |  |  |  |  |  |  | Jul 22, 2008/Fujian Ningde Baimashun Shipbuilding Co. Ltd, 4807 Factory, Xiabaishi Town , Fuan County, Fujian Province, China |  |
| 1.5 | Flag/Port of Registry: |  |  |  |  |  |  | Singapore/SINGAPORE |  |
| 1.6 | Call sign/MMSI: |  |  |  |  |  |  | 9VBF6/565684000 |  |
| 1.7 | Vessel’s contact details (satcom/fax/email etc.): |  |  |  |  |  |  | Tel: +870 773302632 Fax: +870 783209510 Email: florespalm@paccfleet.com |  |
| 1.8 | Type of vessel (as described in Form A or Form B Q1.11 of the IOPPC): |  |  |  |  |  |  | Chemical |  |
| 1.9 | Type of hull: |  |  |  |  |  |  | Double Hull |  |
| Ownership and Operation |  |  |  |  |  |  |  |  |  |
| 1.10 | Registered owner - Full style: |  |  |  |  | Petroil Shipping Pte Ltd (IMO No. 4112896) No.1 Kim Seng Promenade, Great World City, #07-02, Singapore 237994 Singapore Tel: 65-6733 3500 Fax: 65-6839 6083 Email: ptmmarine@pacctanker.com.sg; ptmtech@pacctanker.com.sg Web: www.pclsg.com |  |  |  |
| 1.11 | Technical operator - Full style: |  |  |  |  | PACC TANKER MANAGEMENT PTE LTD No.1 Kim Seng Promenade, #07-02 Great World City, Singapore 237994 Singapore Tel: 65-6733 3500 Fax: 65-6737 3966 Email: ptmmarine@pacctanker.com.sg; ptmtech@pacctanker.com.sg Web: www.pclsg.com Company IMO#: 1627986 |  |  |  |
| 1.12 | Commercial operator - Full style: |  |  |  |  | PCL TANKERS PTE LTD No.1 Kim Seng Promenade #07-02, Great World City, Singapore 237994 Singapore Tel: +65-6733 3500 Fax: +65-6839 6007 Email: ops.tanker@pacificcarriers.com.sg Web: www.pclsg.com |  |  |  |
| 1.13 | Disponent owner - Full style: |  |  |  |  |  |  |  |  |
| Insurance |  |  |  |  |  |  |  |  |  |
| 1.14 | P & I Club - Full Style: |  |  |  |  | SKULD P.O. Box 1376 Vika N-0114 Oslo Norway |  |  |  |
| 1.15 | P & I Club pollution liability coverage/expiration date: |  |  |  |  |  |  | 1,000,000,000 US$ | Feb 20, 2020 |
| 1.16 | Hull & Machinery insured by - Full Style: (Specify broker or leading underwriter) |  |  |  |  | Norwegian Hull Club Bergen Norway Value: USD 13.6m Exp: 31st May 2018 |  |  |  |
| 1.17 | Hull & Machinery insured value/expiration date: |  |  |  |  |  |  | 12,480,000 US$ | Sep 14, 2020 |
| Classification |  |  |  |  |  |  |  |  |  |
| 1.18 | Classification society: |  |  |  |  |  |  | American Bureau of Shipping |  |
| 1.19 | Class notation: |  |  |  |  |  |  | +A1, Chemical Carrier, Oil Carrier, (E), +AMS, +ACCU |  |
| 1.20 | Is the vessel subject to any conditions of class, class extensions, outstanding memorandums or class recommendations? If yes, give details: |  |  |  |  |  |  | No |  |
| 1.21 | If classification society changed, name of previous and date of change: |  |  |  |  |  |  | , Not Applicable |  |
| 1.22 | Does the vessel have ice class? If yes, state what level: |  |  |  |  |  |  | No, |  |
| 1.23 | Date/place of last dry-dock: |  |  |  |  |  |  | Aug 02, 2018/SINGAPORE |  |
| 1.24 | Date next dry dock due/next annual survey due: |  |  |  |  |  |  | Jul 21, 2021 | Aug 01, 2019 |
| 1.25 | Date of last special survey/next special survey due: |  |  |  |  |  |  | Aug 02, 2018 | Aug 01, 2023 |
| 1.26 | If ship has Condition Assessment Program (CAP), what is the latest overall rating: |  |  |  |  |  |  | No, |  |
| Dimensions |  |  |  |  |  |  |  |  |  |
| 1.27 | Length overall (LOA): |  |  |  |  |  |  | 144.47 Metres |  |
| 1.28 | Length between perpendiculars (LBP): |  |  |  |  |  |  | 135.60 Metres |  |
| 1.29 | Extreme breadth (Beam): |  |  |  |  |  |  | 23.00 Metres |  |
| 1.30 | Moulded depth: |  |  |  |  |  |  | 12.50 Metres |  |
| 1.31 | Keel to masthead (KTM)/ Keel to masthead (KTM) in collapsed condition, if applicable: |  |  |  |  |  |  | 36 Metres |  |
| 1.32 | Distance bridge front to center of manifold: |  |  |  |  |  |  | 43.60 Metres |  |
| 1.33 | Bow to center manifold (BCM)/Stern to center manifold (SCM): |  |  |  |  |  |  | 73.00 Metres | 70.80 Metres |
| 1.34 | Parallel body distances |  |  |  |  | Lightship |  | Normal Ballast | Summer Dwt |
|  | # Forward to mid-point manifold: |  |  |  |  | 26.80 Metres |  | 31.20 Metres | 34.20 Metres |
|  | # Aft to mid-point manifold: |  |  |  |  | 25.20 Metres |  | 33.60 Metres | 37.90 Metres |
|  | # Parallel body length: |  |  |  |  | 52 Metres |  | 64.80 Metres | 72.10 Metres |
| Tonnages |  |  |  |  |  |  |  |  |  |
| 1.35 | Net Tonnage: |  |  |  |  |  |  | 5,040 |  |
| 1.36 | Gross Tonnage/Reduced Gross Tonnage (if applicable): |  |  |  |  |  |  | 11,248 |  |
| 1.37 | Suez Canal Tonnage - Gross (SCGT)/Net (SCNT): |  |  |  |  |  |  | 11,998.72 | 10,151.20 |
| # 1.38 | Panama Canal Net Tonnage (PCNT): |  |  |  |  |  |  | 9,468 |  |
| # Loadline Information |  |  |  |  |  |  |  |  |  |
| # 1.39 | # Loadline |  |  | # Freeboard |  | # Draft |  | # Deadweight | # Displacement |
|  | # Summer: |  |  | 3.72 Metres |  | 8.80 Metres |  | # 17,056 Metric Tonnes | 22,078 Metric Tonnes |
|  | # Winter: |  |  | 3.90 Metres |  | 8.617 Metres |  | 16,286 Metric Tonnes | 21,861 Metric Tonnes |
|  | # Tropical: |  |  | 3.53 Metres |  | 8.983 Metres |  | 17,314 Metric Tonnes | 22,889 Metric Tonnes |
|  | # Lightship: |  |  | 9.94 Metres |  | 2.58 Metres |  | - | 5,022 Metric Tonnes |
|  | # Normal Ballast Condition: |  |  | 6.58 Metres |  | 5.94 Metres |  | 8,648 Metric Tonnes | 14,223 Metric Tonnes |
|  | # Segregated Ballast Condition: |  |  |  |  |  |  |  |  |
| # 1.40 | # FWA/TPC at summer draft: |  |  |  |  |  |  | 197.00 Millimetres | 28.09 Metric Tonnes |
| # 1.41 | # Does vessel have multiple SDWT? If yes, please provide all assigned loadlines: |  |  |  |  |  |  | No |  |
| 1.42 | Constant (excluding fresh water): |  |  |  |  |  |  | 98.00 Metric Tonnes |  |
| 1.43 | What is the company guidelines for Under Keel Clearance (UKC) for this vessel? |  |  |  |  |  |  | At berth: 0.4m; SBM,CBM, Offshore Installation, STS Area or Cargo Work Anchorage: 2.0 M, Pilotage water/River passage: 10% max draft; Shallow water: 15% max draft; Coastal water: 20% max draft; Ocean passage: 2 times ship's static draft. |  |
| 1.44 | What is the max height of mast above waterline (air draft) |  |  |  |  |  |  | Full Mast | Collapsed Mast |
|  | Summer deadweight: |  |  |  |  |  |  | 27.20 Metres | 0 Metres |
|  | Normal ballast: |  |  |  |  |  |  | 30 Metres | 0 Metres |
|  | Lightship: |  |  |  |  |  |  | 33.42 Metres | 0 Metres |
| 2. | CERTIFICATES |  |  | Issued |  | Last Annual |  | Last Intermediate | Expires |
| 2.1 | Safety Equipment Certificate (SEC): |  |  | Aug 02, 2018 |  | Jul 10, 2019 |  |  | Jul 21, 2023 |
| 2.2 | Safety Radio Certificate (SRC): |  |  | Jul 03, 2018 |  | Jul 10, 2019 |  |  | Jul 21, 2023 |
| 2.3 | Safety Construction Certificate (SCC): |  |  | Aug 02, 2018 |  | Jul 10, 2019 |  |  | Jul 21, 2023 |
| 2.4 | International Loadline Certificate (ILC): |  |  | Aug 02, 2018 |  | Jul 10, 2019 |  |  | Jul 21, 2023 |
| 2.5 | International Oil Pollution Prevention Certificate (IOPPC): |  |  | Aug 02, 2018 |  | Jul 10, 2019 |  |  | Jul 21, 2023 |
| # 2.6 | # International Ship Security Certificate (ISSC): |  |  | # Jan 16, 2018 |  | # Not Applicable |  |  | Jan 15, 2023 |
| # 2.7 | # Maritime Labour Certificate (MLC): |  |  | # Jan 16, 2018 |  | # N/A |  |  | Jan 15, 2023 |
| 2.8 | ISM Safety Management Certificate (SMC): |  |  | Jan 16, 2018 |  | # Not Applicable |  | # Not Applicable | Jan 15, 2023 |
| # 2.9 | # Document of Compliance (DOC): |  |  | # Dec 15, 2017 |  |  |  |  | Dec 14, 2022 |
| 2.10 | USCG Certificate of Compliance(USCGCOC): |  |  |  |  | Not Applicable |  |  |  |
| 2.11 | Civil Liability Convention (CLC) 1992 Certificate: |  |  | Jan 23, 2019 |  | N/A |  | N/A | Feb 20, 2020 |
| 2.12 | Civil Liability for Bunker Oil Pollution Damage Convention (CLBC) Certificate: |  |  | Jan 22, 2019 |  | N/A |  | N/A | Feb 20, 2020 |
| # 2.13 | # Liability for the Removal of Wrecks Certificate (WRC): |  |  | # Jan 22, 2019 |  | N/A |  | N/A | Feb 20, 2020 |
| 2.14 | U.S. Certificate of Financial Responsibility (COFR): |  |  | Not Applicable |  | N/A |  | N/A | Not Applicable |
| 2.15 | Certificate of Class (COC): |  |  | Aug 02, 2018 |  |  |  |  | Jul 21, 2023 |
| # 2.16 | # International Sewage Pollution Prevention Certificate (ISPPC): |  |  | # Jul 03, 2018 |  | # N/A |  | # N/A | Jul 21, 2023 |
| # 2.17 | # Certificate of Fitness (COF): |  |  | # Aug 02, 2018 |  |  |  |  | Jul 21, 2023 |
| # 2.18 | # International Energy Efficiency Certificate (IEEC): |  |  | # Jun 15, 2016 |  | N/A |  | N/A | N/A |
| # 2.19 | # International Air Pollution Prevention Certificate (IAPPC): |  |  | # Jul 03, 2018 |  |  |  |  | Jul 21, 2023 |
| Documentation |  |  |  |  |  |  |  |  |  |
| 2.20 | Owner warrant that vessel is member of ITOPF and will remain so for the entire duration of this voyage/contract: |  |  |  |  |  |  | Yes |  |
| 2.21 | Does vessel have in place a Drug and Alcohol Policy complying with OCIMF guidelines for Control of Drugs and Alcohol Onboard Ship? |  |  |  |  |  |  | Yes |  |
| 2.22 | Is the ITF Special Agreement on board (if applicable)? |  |  |  |  |  |  | Yes |  |
| 2.23 | ITF Blue Card expiry date (if applicable): |  |  |  |  |  |  | Dec 31, 2019 |  |
| 3. | CREW |  |  |  |  |  |  |  |  |
| 3.1 | Nationality of Master: |  |  |  |  |  |  | Filipino |  |
| 3.2 | Number and nationality of Officers: |  |  |  |  | 9 |  | Filipino, Indian, Myanmar, Chinese |  |
| 3.3 | Number and nationality of Crew: |  |  |  |  | 14 |  | Filipino, Myanmar , Indian , Malaysian and Chinese |  |
| 3.4 | What is the common working language onboard: |  |  |  |  |  |  | English |  |
| 3.5 | Do officers speak and understand English? |  |  |  |  |  |  | Yes |  |
| 3.6 | If Officers/ratings employed by a manning agency - Full style: |  |  | Officers: POSH FLEET SERVICES PTE LTD 1 Kim Seng Promenade#15-05/06,Great World City,East Tower,Singapore 237994 Tel: +65-6839 7064 Email: mpd_psm@paccoffshore.com.sg |  |  |  | Ratings: |  |
| 4. | FOR USA CALLS |  |  |  |  |  |  |  |  |
| 4.1 | Has the vessel Operator submitted a Vessel Spill Response Plan to the US Coast Guard which has been approved by official USCG letter? |  |  |  |  |  |  | N/A |  |
| 4.2 | Qualified individual (QI) - Full style: |  |  |  |  |  |  |  |  |
| 4.3 | Oil Spill Response Organization (OSRO) - Full style: |  |  |  |  |  |  |  |  |
| 4.4 | Salvage and Marine Firefighting Services (SMFF) - Full Style: |  |  |  |  |  |  |  |  |
| 5. | SAFETY/HELICOPTER |  |  |  |  |  |  |  |  |
| 5.1 | Is the vessel operated under a Quality Management System? If Yes, what type of system? (ISO9001 or IMO Resolution A.741(18) as amended): |  |  |  |  |  |  | Yes IMO Resolution A.741(18) |  |
| 5.2 | Can the ship comply with the ICS Helicopter Guidelines? |  |  |  |  |  |  | No |  |
| 5.2.1 | If Yes, state whether winching or landing area provided: |  |  |  |  |  |  |  |  |
| 5.2.2 | If Yes, what is the diameter of the circle provided: |  |  |  |  |  |  |  |  |
| 6. | COATING/ANODES |  |  |  |  |  |  |  |  |
| 6.1 | Tank Coating |  |  | Coated |  | Type |  | To What Extent | Anodes |
|  | Cargo tanks: |  |  | Yes |  | Phenolic Epoxy |  | Whole tanks | No |
|  | Ballast tanks: |  |  | Yes |  | Epoxy |  | Whole tanks | Yes |
|  | Slop tanks: |  |  | Yes |  | Epoxy |  | Whole Tank | No |
| 7. | BALLAST |  |  |  |  |  |  |  |  |
| 7.1 | Pumps |  |  |  | No. | Type |  | Capacity | At What Head (sg=1.0) |
|  | Ballast Pumps: |  |  |  | 2 | Centrifugal |  |  | 25 Metres |
|  | Ballast Eductors: |  |  |  | 1 | water drive |  |  |  |
| 8. | CARGO |  |  |  |  |  |  |  |  |
| Double Hull Vessels |  |  |  |  |  |  |  |  |  |
| 8.1 | Is vessel fitted with centerline bulkhead in all cargo tanks? If Yes, solid or perforated: |  |  |  |  |  |  | Yes, Solid |  |
| Cargo Tank Capacities |  |  |  |  |  |  |  |  |  |
| 8.2 | Number of cargo tanks and total cubic capacity (98%): |  |  |  |  |  |  | 12 | 19,077.712 Cu. Metres |
| 8.2.1 | Capacity (98%) of each natural segregation with double valve (specify tanks): |  |  |  |  |  |  | Seg#1: 5999.55 m3 (1P/S & 4P/S) Seg#2: 7077.347 m3 (2P/S & 5P/S) Seg#3: 2366.91 m3 (6P/S) Seg#4: 4013.91 m3 (3P/S & Slop P/S) |  |
| 8.2.2 | IMO class (Oil/Chemical Ship Type 1, 2 or 3): |  |  |  |  |  |  | 2 |  |
| 8.3 | Number of slop tanks and total cubic capacity (98%): |  |  |  |  |  |  | 2 | 380 Cu. Metres |
| 8.3.1 | Specify segregations which slops tanks belong to and their capacity with double valve: |  |  |  |  |  |  |  |  |
| 8.3.2 | Residual/retention oil tank(s) capacity (98%), if applicable: |  |  |  |  |  |  | 2.94 Cu. Metres |  |
| SBT Vessels |  |  |  |  |  |  |  |  |  |
| 8.3.3 | What is total SBT capacity and percentage of SDWT vessel can maintain? |  |  |  |  |  |  | 7,437.20 Cu. Metres | 43.60 % |
| 8.3.4 | Does vessel meet the requirements of MARPOL Annex I Reg 18.2: |  |  |  |  |  |  | Yes |  |
| Cargo Handling and Pumping Systems |  |  |  |  |  |  |  |  |  |
| 8.4 | How many grades/products can vessel load/discharge with double valve segregation: |  |  |  |  |  |  | 4 |  |
| 8.4.1 | State type of cargo containment (integral, independent, gravity or pressure tanks): |  |  |  |  |  |  |  |  |
| 8.5 | Are there any cargo tank filling restrictions? If yes, specify number of slack tanks, max s.g., ullage restrictions etc.: |  |  |  |  |  |  | N/A No restrictions below SG1.025 t/m3but above 1.025 Vsl can load heavy cargo up to 1.3t/m3 only up to 78.8 of full of tank |  |
| 8.6 | Max loading rate for homogenous cargo |  |  |  |  |  |  | With VECS | Without VECS |
|  | Loaded per manifold connection: |  |  |  |  |  |  |  | 1,050 Cu. Metres/Hour |
|  | Loaded simultaneously through all manifolds: |  |  |  |  |  |  |  | 2,100.00 Cu. Metres/Hour |
| Cargo Control Room |  |  |  |  |  |  |  |  |  |
| 8.7 | Is ship fitted with a Cargo Control Room (CCR)? |  |  |  |  |  |  | Yes |  |
| 8.8 | Can tank innage/ullage be read from the CCR? |  |  |  |  |  |  | Yes |  |
| Gauging and Sampling |  |  |  |  |  |  |  |  |  |
| 8.9 | Is gauging system certified and calibrated? If no, specify which ones are not calibrated: |  |  |  |  |  |  | Yes, |  |
|  | What type of gauging system as per IBC 13.1 is fitted (Open/Restricted/Closed )? |  |  |  |  |  |  |  |  |
|  | What type of fixed closed tank gauging system is fitted: |  |  |  |  |  |  | Radar |  |
|  | Is a tank overflow control system fitted? If yes, then state if system includes automatic closing of valves? |  |  |  |  |  |  | Yes, No |  |
|  | Are high level alarms fitted to the cargo tanks? If Yes, indicate whether to all tanks or partial: |  |  |  |  |  |  | Yes, All |  |
| 8.9.1 | Can cargo be transferred under closed loading conditions in accordance with ISGOTT 11.1.6.6? |  |  |  |  |  |  | Yes |  |
| 8.9.2 | Are cargo tanks fitted with multipoint gauging? If yes, specify type and locations: |  |  |  |  |  |  | No, |  |
| 8.10 | Number of portable gauging units (example- MMC) on board: |  |  |  |  |  |  | 3 |  |
| Vapor Emission Control System (VECS) |  |  |  |  |  |  |  |  |  |
| 8.11 | Is a vapour return system (VRS) fitted? |  |  |  |  |  |  | Yes |  |
| 8.12 | Number/size of VECS manifolds (per side): |  |  |  |  |  |  | 1 | 304.80 Millimetres |
| 8.13 | Number/size/type of VECS reducers: |  |  |  |  |  |  |  |  |
| Venting |  |  |  |  |  |  |  |  |  |
| 8.14 | State what type of venting system is fitted: |  |  |  |  |  |  | High Velocity P/V Valve |  |
| Cargo Manifolds and Reducers |  |  |  |  |  |  |  |  |  |
| 8.15 | Total number/size of cargo manifold connections on each side: |  |  |  |  |  |  | 4/304.80 Millimetres |  |
| 8.15.1 | Does the vessel have a Common Line Manifold connection? If yes, describe: |  |  |  |  |  |  | One common line, size 300 millimetres |  |
| 8.16 | What type of valves are fitted at manifold: |  |  |  |  |  |  | Butterfly |  |
| 8.17 | What is the material/rating of the manifold: |  |  |  |  |  |  | Stainless Steel/ANSI |  |
| 8.17.1 | Does vessel comply with the latest edition of the OCIMF 'Recommendations for Oil Tanker Manifolds and Associated Equipment’? |  |  |  |  |  |  | Yes |  |
| 8.18 | Distance between cargo manifold centers: |  |  |  |  |  |  | 1,500 Millimetres |  |
| 8.19 | Distance ships rail to manifold: |  |  |  |  |  |  | 4,550 Millimetres |  |
| 8.20 | Distance manifold to ships side: |  |  |  |  |  |  | 4,600 Millimetres |  |
| 8.21 | Top of rail to center of manifold: |  |  |  |  |  |  | 800 Millimetres |  |
| 8.22 | Distance main deck to center of manifold: |  |  |  |  |  |  | 1,950 Millimetres |  |
| 8.23 | Spill tank grating to center of manifold: |  |  |  |  |  |  | 780 Millimetres |  |
| 8.24 | Manifold height above the waterline in normal ballast/at SDWT condition: |  |  |  |  |  |  | 8.46 Metres | 5.65 Metres |
| 8.25 | Number/size/type of reducers: |  |  |  |  |  |  | 4 x 304.8/254mm (12/10") 4 x 304.8/203.2mm (12/8") ANSI |  |
| 8.26 | Is vessel fitted with a stern manifold? If yes, state size: |  |  |  |  |  |  | No, |  |
| Heating |  |  |  |  |  |  |  |  |  |
| 8.27 | Cargo/slop tanks fitted with a cargo heating system? |  |  |  |  | Type |  | Coiled | Material |
|  | Cargo Tanks: |  |  |  |  | Steam heating coils |  | Yes | SS |
|  | Slop Tanks: |  |  |  |  | heating coil |  | Yes | SS |
| 8.27.1 | Is a Thermal Oil Heating system fitted? If yes, identify tanks? |  |  |  |  |  |  | No, |  |
| 8.28 | Maximum temperature cargo can be loaded/maintained: |  |  |  |  |  |  | 65.0 °C / 149.0 °F | 65 °C / 149 °F |
| 8.28.1 | Minimum temperature cargo can be loaded/maintained: |  |  |  |  |  |  |  |  |
| Inert Gas and Crude Oil Washing |  |  |  |  |  |  |  |  |  |
| 8.29 | Is an Inert Gas System (IGS) fitted/operational? |  |  |  |  |  |  | N/A/N/A |  |
| 8.29.1 | Is a Crude Oil Washing (COW) installation fitted/operational? |  |  |  |  |  |  | N/A/N/A |  |
| 8.30 | Is IGS supplied by flue gas, inert gas (IG) generator and/or nitrogen: |  |  |  |  |  |  |  |  |
| 8.30.1 | If nitrogen generator, specify the applicable flow rate for each of the designed purity modes: |  |  |  |  |  |  |  |  |
| Cargo Pumps |  |  |  |  |  |  |  |  |  |
| 8.31 | How many cargo pumps can be run simultaneously at full capacity: |  |  |  |  |  |  | 4 |  |
| 8.32 | Pumps |  |  | No. |  | Type |  | Capacity | At What Head (sg=1.0) |
|  | Cargo Pumps: |  |  | 4 |  | Screw |  | 500 M3/HR | 100 Meters 100 Meters 100 Meters 100 Meters |
|  | Cargo Eductors: |  |  |  |  | N/A |  |  |  |
|  | Stripping: |  |  | 1 |  | Screw |  | 25 Cu. Metres/Hour |  |
| 8.33 | Is at least one emergency portable cargo pump provided? |  |  |  |  |  |  | N/A |  |
| Tank Cleaning Systems |  |  |  |  |  |  |  |  |  |
| 8.34 | Is tank cleaning equipment fixed in cargo tanks? |  |  |  |  |  |  | Yes |  |
| 8.35 | Is portable tank cleaning equipment provided? |  |  |  |  |  |  | Yes |  |
| 8.36 | Tank washing pump capacity: |  |  |  |  |  |  | 150 Cu. Metres/Hour |  |
| 8.37 | Is a washing water heater fitted? If yes is it operational and state max washing water temperature: |  |  |  |  |  |  | Yes, Yes 70 Degrees Celsius |  |
| 8.38 | What is the maximum number of machines that can be operated at their designed max pressure? |  |  |  |  |  |  | 4 |  |
| Other Deck Equipment |  |  |  |  |  |  |  |  |  |
| 8.39 | Is vessel fitted with a remote cargo tank temperature monitoring system. If yes, is it operational? |  |  |  |  |  |  | Yes, Yes |  |
| 8.40 | Is vessel fitted with a remote cargo tank pressure monitoring system. If yes, is it operational? |  |  |  |  |  |  | Yes, Yes |  |
| 8.41 | Is vessel fitted with a cargo tank drier. If yes is it operational and state capacity: |  |  |  |  |  |  | No, N/A |  |
| 8.42 | Is vessel fitted with a cargo cooling system. If yes is it operational and state tanks applicable: |  |  |  |  |  |  | No, N/A |  |
| 8.43 | Is steam available on deck? |  |  |  |  |  |  | Yes |  |
| 9. | MOORING |  |  |  |  |  |  |  |  |
| 9.1 | Wires (on drums) |  | No. | Diameter |  | Material |  | Length | Breaking Strength |
|  | Forecastle: |  |  |  |  |  |  |  |  |
|  | Main deck fwd: |  |  |  |  |  |  |  |  |
|  | Main deck aft: |  |  |  |  |  |  |  |  |
|  | Poop deck: |  |  |  |  |  |  |  |  |
| 9.2 | Wire tails |  | No. | Diameter |  | Material |  | Length | Breaking Strength |
|  | Forecastle: |  |  |  |  |  |  |  |  |
|  | Main deck fwd: |  |  |  |  |  |  |  |  |
|  | Main deck aft: |  |  |  |  |  |  |  |  |
|  | Poop deck: |  |  |  |  |  |  |  |  |
| 9.3 | Ropes (on drums) |  | No. | Diameter |  | Material |  | Length | Breaking Strength |
|  | Forecastle: |  | 4 | 48 Millimetres |  | Polyester-Polysteel 40/60 |  | 220 Metres | 48.90 Metric Tonnes |
|  | Main deck fwd: |  |  |  |  |  |  |  |  |
|  | Main deck aft: |  |  |  |  |  |  |  |  |
|  | Poop deck: |  | 4 | 48 Millimetres |  | Polyester-Polysteel 40/60 |  | 220 Metres | 48.90 Metric Tonnes |
| 9.4 | Other lines |  | No. | Diameter |  | Material |  | Length | Breaking Strength |
|  | Forecastle: |  | 4 | 48 Millimetres |  | Polyester-Polysteel 40/60 |  | 220 Metres | 48.90 Metric Tonnes |
|  | Main deck fwd: |  |  |  |  |  |  |  |  |
|  | Main deck aft: |  |  |  |  |  |  |  |  |
|  | Poop deck: |  | 4 | 48 Millimetres |  | Polyester-Polysteel 40/60 |  | 220 Metres | 48.90 Metric Tonnes |
| 9.5 | Winches |  | No. | No. Drums |  | Motive Power |  | Brake Capacity | Type of Brake |
|  | Forecastle: |  | 2 | Dbl drum |  | Hydraulic |  | 23.00 Metric Tonnes | Manual Band/Friction |
|  | Main deck fwd: |  |  |  |  |  |  |  |  |
|  | Main deck aft: |  |  |  |  |  |  |  |  |
|  | Poop deck: |  | 2 | Dbl drum |  | Hydraulic |  | 23.00 Metric Tonnes | Manual Band/Friction |
| 9.6 | Bitts, closed chocks/fairleads |  |  | No. Bitts |  | SWL Bitts |  | No. Closed Chocks | SWL Closed Chocks |
|  | Forecastle: |  |  | 8 |  | 35 Metric Tonnes |  | 12 | 62 Metric Tonnes |
|  | Main deck fwd: |  |  | 2 |  | 35 Metric Tonnes |  | 2 | 62 Metric Tonnes |
|  | Main deck aft: |  |  | 2 |  | 35 Metric Tonnes |  | 2 | 62 Metric Tonnes |
|  | Poop deck: |  |  | 7 |  | 35 Metric Tonnes |  | 10 | 62 Metric Tonnes |
| Anchors/Emergency Towing System |  |  |  |  |  |  |  |  |  |
| 9.7 | Number of shackles on port/starboard cable: |  |  |  |  |  |  | 11/10 |  |
| 9.8 | Type/SWL of Emergency Towing system forward: |  |  |  |  |  |  |  |  |
| 9.9 | Type/SWL of Emergency Towing system aft: |  |  |  |  |  |  |  |  |
| 9.10.1 | What is size of closed chock and/or fairleads of enclosed type on stern |  |  |  |  |  |  |  |  |
| Escort Tug |  |  |  |  |  |  |  |  |  |
| 9.10.2 | What is SWL of closed chock and/or fairleads of enclosed type on stern: |  |  |  |  |  |  | 84 Metric Tonnes |  |
| 9.11 | What is SWL of bollard on poop deck suitable for escort tug: |  |  |  |  |  |  | 35 Metric Tonnes |  |
| Lifting Equipment/Gangway |  |  |  |  |  |  |  |  |  |
| 9.12 | Derrick/Crane description (Number, SWL and location): |  |  |  |  |  |  | Cranes: 1 x 10.00 Tonnes center |  |
| 9.13 | Accommodation ladder direction: |  |  |  |  |  |  | Aft |  |
|  | Does vessel have a portable gangway? If yes, state length: |  |  |  |  |  |  | Yes, |  |
| Single Point Mooring (SPM) Equipment |  |  |  |  |  |  |  |  |  |
| 9.14 | Does the vessel meet the recommendations in the latest edition of OCIMF 'Recommendations for Equipment Employed in the Bow Mooring of Conventional Tankers at Single Point Moorings (SPM)’:? |  |  |  |  |  |  | No |  |
| 9.15 | If fitted, how many chain stoppers: |  |  |  |  |  |  |  |  |
| 9.16 | State type/SWL of chain stopper(s): |  |  |  |  |  |  |  |  |
| 9.17 | What is the maximum size chain diameter the bow stopper(s) can handle: |  |  |  |  |  |  |  |  |
| 9.18 | Distance between the bow fairlead and chain stopper/bracket: |  |  |  |  |  |  |  |  |
| 9.19 | Is bow chock and/or fairlead of enclosed type of OCIMF recommended size (600mm x 450mm)? If not, give details of size: |  |  |  |  |  |  | Yes |  |
| 10. | PROPULSION |  |  |  |  |  |  |  |  |
| 10.1 | Speed |  |  |  |  |  |  | Maximum | Economical |
|  | Ballast speed: |  |  |  |  |  |  | 13 Knots (WSNP) | 11 Knots (WSNP) |
|  | Laden speed: |  |  |  |  |  |  | 12 Knots (WSNP) | 11 Knots (WSNP) |
| 10.2 | What type of fuel is used for main propulsion/generating plant: |  |  |  |  |  |  | IFO 380 CST | IFO 380CST |
| 10.3 | Type/Capacity of bunker tanks: |  |  |  |  |  |  | Fuel Oil: 649.40 Cu. Metres Diesel Oil: 78.60 Cu. Metres Gas Oil: 0 Cu. Metres |  |
| 10.4 | Is vessel fitted with fixed or controllable pitch propeller(s): |  |  |  |  |  |  | Fixed |  |
| 10.5 | Engines |  |  |  |  |  | No | Capacity | Make/Type |
|  | Main engine: |  |  |  |  |  | 1 | 4,440 Kilowatt | MAN B&W 6S35MC MK7 |
|  | Aux engine: |  |  |  |  |  | 3 | 660 Kilowatt | Anqing Marine Diesel / 5DK-20 |
|  | Power packs: |  |  |  |  |  |  |  |  |
|  | Boilers: |  |  |  |  |  | 2 | 3.50 Metric Tonnes/Hour |  |
| Bow/Stern Thruster |  |  |  |  |  |  |  |  |  |
| 10.6 | What is brake horse power of bow thruster (if fitted): |  |  |  |  |  |  | No, |  |
| 10.7 | What is brake horse power of stern thruster (if fitted): |  |  |  |  |  |  | No, |  |
| Emissions |  |  |  |  |  |  |  |  |  |
| 10.8 | Main engine IMO NOx emission standard: |  |  |  |  |  |  |  |  |
| 10.9 | Energy Efficiency Design Index (EEDI) rating number: |  |  |  |  |  |  |  |  |
| 11. | SHIP TO SHIP TRANSFER |  |  |  |  |  |  |  |  |
| 11.1 | Does vessel comply with recommendations contained in OCIMF/ICS Ship To Ship Transfer Guide (Petroleum, Chemicals or Liquified Gas, as applicable)? |  |  |  |  |  |  | Yes |  |
| 11.2 | What is maximum outreach of cranes/derricks outboard of the ship’s side: |  |  |  |  |  |  | 3.00 Metres |  |
| 11.3 | Date/place of last STS operation: |  |  |  |  |  |  |  |  |
| 12. | RECENT OPERATIONAL HISTORY |  |  |  |  |  |  |  |  |
| 12.1 | Last three cargoes/charterers/voyages (Last/2nd Last/3rd Last): |  |  |  |  |  |  | 1ST Last Cargo: Jet A1 2ND Last Cargo: Gasoline 3RD Last Cargo: Naphtha & Jet A1 |  |
| 12.2 | Has vessel been involved in a pollution, grounding, serious casualty, unscheduled repair or collision incident during the past 12 months? If yes, provide details: |  |  |  |  |  |  | Pollution: No, Grounding: No, Casualty: No, Repair: No, Collision: No, |  |
| 12.3 | Date and place of last Port State Control inspection: |  |  |  |  |  |  | Mar 27, 2019 / Subic, Philippines |  |
| 12.4 | Any outstanding deficiencies as reported by any Port State Control? If yes, provide details: |  |  |  |  |  |  | No |  |
| 12.5 | Recent Oil company inspections/screenings (To the best of owners knowledge and without guarantee of acceptance for future business)*: * "Approvals" are not given by Oil Majors and ships are accepted for the voyage on a case by case basis. |  |  |  |  |  |  | Vopak Terminal, PSC-Tokyo MOU, ENOC, PHILLIPS66, Viva Energy, Petron |  |
| 12.6 | Date/Place of last SIRE inspection: |  |  |  |  |  |  | Feb 13, 2019 / Sandakan |  |
| 12.6.1 | Date/Place of last CDI inspection: |  |  |  |  |  |  |  |  |
| 12.7 | Additional information relating to features of the ship or operational characteristics: |  |  |  |  |  |  |  |  |

Revised 2018 (INTERTANKO/Q88.com)

Form completed on http://www.q88.com/integration.aspx  Please email support@q88.com an updated copy if this is not the latest version.

To the best of owners knowledge all information is true and given without any guarantee.