- INTERTANKO CHARTERING QUESTIONNAIRE 88 - OIL 	Version 5

| 1. | GENERAL INFORMATION |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1.1 | Date updated: |  |  |  |  |  |  | Sep 01, 2020 |  |
| 1.2 | Vessel’s name (IMO number): |  |  |  |  |  |  | Maersk Navigator (9724594) |  |
| 1.3 | Vessel’s previous name(s) and date(s) of change: |  |  |  |  |  |  | British Navigator (06 Aug 2020) |  |
| 1.4 | Date delivered/Builder (where built): |  |  |  |  |  |  | Apr 26, 2016/Hyundai Mipo Dockyard Ltd |  |
| 1.5 | Flag/Port of Registry: |  |  |  |  |  |  | Danish Flag / Svendborg |  |
| 1.6 | Call sign/MMSI: |  |  |  |  |  |  | OYQK2 / 219322000 |  |
| 1.7 | Vessel’s contact details (satcom/fax/email etc.): |  |  |  |  |  |  | Tel: +4589876685 Fax: +870783260241 Email: master@navigator.maersktankers.com |  |
| 1.8 | Type of vessel (as described in Form A or Form B Q1.11 of the IOPPC): |  |  |  |  |  |  | Oil Tanker |  |
| 1.9 | Type of hull: |  |  |  |  |  |  | Double Hull |  |
| Ownership and Operation |  |  |  |  |  |  |  |  |  |
| 1.10 | Registered owner - Full style: |  |  |  |  | Hai Kuo Shipping 1507 Limited Trust Company Complex Ajeltalky Road Ajeltalke Island Majuro Marshall Islands MH96960 Marshall Islands |  |  |  |
| 1.11 | Technical operator - Full style: |  |  |  |  | Maersk Tankers AS 133 Holmbladsgade Copenhagen S 2300 Copenhagen Denmark Tel: +4531563499 Email: marine@maersktankers.com Web: www.maersktankers.com Company IMO#: 5676362 |  |  |  |
| 1.12 | Commercial operator - Full style: |  |  |  |  | BP Shipping Limited Chertsey Road Sunbury on Thames Middlesex TW16 7BP United Kingdom Tel: 01932 76200 Telex: 290851 BPSHIP G Email: Operations@bpshipping.com |  |  |  |
| 1.13 | Disponent owner - Full style: |  |  |  |  | BP Singapore Pte Limited 7 Straits View #26-01 Marina One East Tower Singapore 018936 Tel: +65 6751 3005 Email: operations@bpshipping.com |  |  |  |
| Insurance |  |  |  |  |  |  |  |  |  |
| 1.14 | P & I Club - Full Style: |  |  |  |  | SKULD P.O. Box, 1376 Vika, N-0114 Oslo, Norway, Tel: +4795292200 |  |  |  |
| 1.15 | P & I Club pollution liability coverage/expiration date: |  |  |  |  |  |  | 1,000,000,000 US$ | Feb 20, 2021 |
| 1.16 | Hull & Machinery insured by - Full Style: (Specify broker or leading underwriter) |  |  |  |  | Gard AS PO Box 31 NO 4809 Arendal Norway Tel +47 37 01 91 00 Fax +44 37 02 48 10 |  |  |  |
| 1.17 | Hull & Machinery insured value/expiration date: |  |  |  |  |  |  | 43,355,000 US$ | Dec 31, 2020 |
| Classification |  |  |  |  |  |  |  |  |  |
| 1.18 | Classification society: |  |  |  |  |  |  | Lloyds Register |  |
| 1.19 | Class notation: |  |  |  |  |  |  | +100A1, Double Hull Oil and Chemical Tanker, ESP, Ship Type 3, CSR, +LMC, UMS, ShipRight (ACS(B), FDA Plus (40,WW), CM), *IWS, LI, DSPM4, ECO (BWT, EEDI-1, IHM, P), IGS, CCS with descriptive notes COW(LR), ETA, Part Higher Tensile Steel, ShipRight (MCM, MPMS, SCM, SERS) |  |
| 1.20 | Is the vessel subject to any conditions of class, class extensions, outstanding memorandums or class recommendations? If yes, give details: |  |  |  |  |  |  | No |  |
| 1.21 | If classification society changed, name of previous and date of change: |  |  |  |  |  |  | Lloyds Register, Not Applicable |  |
| 1.22 | Does the vessel have ice class? If yes, state what level: |  |  |  |  |  |  | No, None |  |
| 1.23 | Date/place of last dry-dock: |  |  |  |  |  |  | Apr 26, 2016/HMD Shipyard - Delivery |  |
| 1.24 | Date next dry dock due/next annual survey due: |  |  |  |  |  |  | Apr 24, 2021 | Feb 26, 2020 |
| 1.25 | Date of last special survey/next special survey due: |  |  |  |  |  |  | Apr 26, 2016 | Apr 25, 2021 |
| 1.26 | If ship has Condition Assessment Program (CAP), what is the latest overall rating: |  |  |  |  |  |  | No, |  |
| Dimensions |  |  |  |  |  |  |  |  |  |
| 1.27 | Length overall (LOA): |  |  |  |  |  |  | 183.06 Metres |  |
| 1.28 | Length between perpendiculars (LBP): |  |  |  |  |  |  | 174.00 Metres |  |
| 1.29 | Extreme breadth (Beam): |  |  |  |  |  |  | 32.20 Metres |  |
| 1.30 | Moulded depth: |  |  |  |  |  |  | 19.10 Metres |  |
| 1.31 | Keel to masthead (KTM)/ Keel to masthead (KTM) in collapsed condition, if applicable: |  |  |  |  |  |  | 48.95 Metres | 46.41 Metres |
| 1.32 | Distance bridge front to center of manifold: |  |  |  |  |  |  | 57.83 Metres |  |
| 1.33 | Bow to center manifold (BCM)/Stern to center manifold (SCM): |  |  |  |  |  |  | 90.20 Metres | 92.87 Metres |
| 1.34 | Parallel body distances |  |  |  |  | Lightship |  | Normal Ballast | Summer Dwt |
|  | # Forward to mid-point manifold: |  |  |  |  | 26.28 Metres |  | 42.59 Metres | 46.63 Metres |
|  | # Aft to mid-point manifold: |  |  |  |  | 25.89 Metres |  | 43.15 Metres | 57.60 Metres |
|  | # Parallel body length: |  |  |  |  | 52.17 Metres |  | 85.74 Metres | 104.23 Metres |
| Tonnages |  |  |  |  |  |  |  |  |  |
| 1.35 | Net Tonnage: |  |  |  |  |  |  | 11,988.00 |  |
| 1.36 | Gross Tonnage/Reduced Gross Tonnage (if applicable): |  |  |  |  |  |  | 30,948.00 | 24,117 |
| 1.37 | Suez Canal Tonnage - Gross (SCGT)/Net (SCNT): |  |  |  |  |  |  | 32,764.34 | 28,034.39 |
| # 1.38 | Panama Canal Net Tonnage (PCNT): |  |  |  |  |  |  | 25,729.00 |  |
| # Loadline Information |  |  |  |  |  |  |  |  |  |
| # 1.39 | # Loadline |  |  | # Freeboard |  | # Draft |  | # Deadweight | # Displacement |
|  | # Summer: |  |  | 6.805 Metres |  | 12.323 Metres |  | # 45,999 Metric Tonnes | 57,318 Metric Tonnes |
|  | # Winter: |  |  | 7.061 Metres |  | 12.067 Metres |  | 44,617 Metric Tonnes | 55,936 Metric Tonnes |
|  | # Tropical: |  |  | 6.549 Metres |  | 12.579 Metres |  | 47,378 Metric Tonnes | 58,697 Metric Tonnes |
|  | # Lightship: |  |  | 15.95 Metres |  | 3.18 Metres |  | - | 11,319.00 Metric Tonnes |
|  | # Normal Ballast Condition: |  |  | 11.96 Metres |  | 7.17 Metres |  | 18,881.00 Metric Tonnes | 30,200.00 Metric Tonnes |
|  | # Segregated Ballast Condition: |  |  | 11.67 Metres |  | 7.46 Metres |  | 20,356.00 Metric Tonnes | 31,675.00 Metric Tonnes |
| # 1.40 | # FWA/TPC at summer draft: |  |  |  |  |  |  | 266 Millimetres | 54.14 Metric Tonnes |
| # 1.41 | # Does vessel have multiple SDWT? If yes, please provide all assigned loadlines: |  |  |  |  |  |  | Yes 45,999 44,999 39,999 34,999 |  |
| 1.42 | Constant (excluding fresh water): |  |  |  |  |  |  | 150 Metric Tonnes |  |
| 1.43 | What is the company guidelines for Under Keel Clearance (UKC) for this vessel? |  |  |  |  |  |  | 1) When alongside, at anchor or at SBM/CBM within a protected harbour area. Min. UKC to apply is 0.30 m or allowance for one degree list, whichever is greater. As soon as possible after berthing, hand lead soundings should be taken around the vessel to confirm the depth alongside, unless recent reliable soundings are available that exclude any possibility that the vessel may stand on the bottom. 2. Approaches, Anchorages and SBM/CBM outside harbour areas, Confined waters, and buoyed channels. Min. UKC should be 0.90 m or allowance for three degrees list; whichever is greater. The Master may proceed to the destination with less than the above minimum underkeel clearance only after a further assessment and on the advice of the applicable pilot, together with the concurrence of the appropriate nautical department. 3. Deep Sea, while underway during open sea navigation. When laying off courses for large vessels consideration shall be given to ensure the greatest possible depth of water for the vessels track and, whenever possible, such vessels shall follow recommended routes. Min. UKC should be 50% of the vessels draught or a minimum of 3.5 m., whichever is the greater. Emphasis is placed on the importance of considering all the variable factors listed above. |  |
| 1.441) When alongside, at anchor or at SBM/CBM within a protected harbour area. Min. UKC to apply is 0.30 m or allowance for one degree list, whichever is greater. As soon as possible after berthing, hand lead soundings should be taken around the vessel to confirm the depth alongside, unless recent reliable soundings are available that exclude any possibility that the vessel may stand on the bottom. 2. Approaches, Anchorages and SBM/CBM outside harbour areas, Confined waters, and buoyed channels. Min. UKC should be 0.90 m or allowance for three degrees list; whichever is greater. The Master may proceed to the destination with less than the above minimum underkeel clearance only after a further assessment and on the advice of the applicable pilot, together with the concurrence of the appropriate nautical department. 3. Deep Sea, while underway during open sea navigation. When laying off courses for large vessels consideration shall be given to ensure the greatest possible depth of water for the vessels track and, whenever possible, such vessels shall follow recommended routes. Min. UKC should be 50% of the vessels draught or a minimum of 3.5 m., whichever is the greater. Emphasis is placed on the importance of considering all the variable factors listed above. | What is the max height of mast above waterline (air draft) |  |  |  |  |  |  | Full Mast | Collapsed Mast |
|  | Summer deadweight: |  |  |  |  |  |  | 12.323 Metres | 9.783 Metres |
|  | Normal ballast: |  |  |  |  |  |  | 41.66 Metres | 39.12 Metres |
|  | Lightship: |  |  |  |  |  |  | 3.07 Metres | 0.53 Metres |
| 2. | CERTIFICATES |  |  | Issued |  | Last Annual |  | Last Intermediate | Expires |
| 2.1 | Safety Equipment Certificate (SEC): |  |  | Aug 07, 2020 |  | May 17, 2020 |  | Feb 26, 2019 | Apr 25, 2021 |
| 2.2 | Safety Radio Certificate (SRC): |  |  | Aug 07, 2020 |  | May 17, 2020 |  | Feb 26, 2019 | Apr 25, 2021 |
| 2.3 | Safety Construction Certificate (SCC): |  |  | Aug 07, 2020 |  | May 17, 2020 |  | Feb 26, 2019 | Apr 25, 2021 |
| 2.4 | International Loadline Certificate (ILC): |  |  | Aug 07, 2020 |  | Feb 23, 2018 |  |  | Apr 25, 2021 |
| 2.5 | International Oil Pollution Prevention Certificate (IOPPC): |  |  | Aug 07, 2020 |  | May 17, 2020 |  | Feb 22, 2019 | Apr 25, 2021 |
| # 2.6 | # International Ship Security Certificate (ISSC): |  |  | # Aug 07, 2020 |  | # Not Applicable |  |  | Feb 09, 2021 |
| # 2.7 | # Maritime Labour Certificate (MLC): |  |  | # Aug 07, 2020 |  | # Not Applicable |  |  | Feb 09, 2021 |
| 2.8 | ISM Safety Management Certificate (SMC): |  |  | Aug 07, 2020 |  | # Not Applicable |  |  | Feb 09, 2021 |
| # 2.9 | # Document of Compliance (DOC): |  |  | # Nov 06, 2019 |  | Not Applicable |  |  | Nov 17, 2024 |
| 2.10 | USCG Certificate of Compliance (USCGCOC): |  |  | Not Applicable |  | Not Applicable |  | Not Applicable | Not Applicable |
| 2.11 | Civil Liability Convention (CLC) 1992 Certificate: |  |  | Jul 28, 2020 |  | N/A |  | N/A | Feb 20, 2021 |
| 2.12 | Civil Liability for Bunker Oil Pollution Damage Convention (CLBC) Certificate: |  |  | Jul 28, 2020 |  | N/A |  | N/A | Feb 20, 2021 |
| # 2.13 | # Liability for the Removal of Wrecks Certificate (WRC): |  |  | # Jul 28, 2020 |  | N/A |  | N/A | Feb 20, 2021 |
| 2.14 | U.S. Certificate of Financial Responsibility (COFR): |  |  | N/A |  | N/A |  | N/A | N/A |
| 2.15 | Certificate of Class (COC): |  |  | Apr 26, 2016 |  | Not Applicable |  | Feb 26, 2019 | Apr 25, 2021 |
| # 2.16 | # International Sewage Pollution Prevention Certificate (ISPPC): |  |  | # Aug 07, 2020 |  | # N/A |  | # N/A | Apr 25, 2021 |
| # 2.17 | # Certificate of Fitness (COF): |  |  | # Aug 07, 2020 |  |  |  |  | Apr 25, 2021 |
| # 2.18 | # International Energy Efficiency Certificate (IEEC): |  |  | # Aug 07, 2020 |  | N/A |  | N/A | N/A |
| # 2.19 | # International Air Pollution Prevention Certificate (IAPPC): |  |  | # Aug 07, 2020 |  | May 17, 2020 |  | Feb 26, 2019 | Apr 25, 2021 |
| Documentation |  |  |  |  |  |  |  |  |  |
| 2.20 | Owner warrant that vessel is member of ITOPF and will remain so for the entire duration of this voyage/contract: |  |  |  |  |  |  | Yes |  |
| 2.21 | Does vessel have in place a Drug and Alcohol Policy complying with OCIMF guidelines for Control of Drugs and Alcohol Onboard Ship? |  |  |  |  |  |  | Yes |  |
| 2.22 | Is the ITF Special Agreement on board (if applicable)? |  |  |  |  |  |  | N/A |  |
| 2.23 | ITF Blue Card expiry date (if applicable): |  |  |  |  |  |  | Not Applicable |  |
| 3. | CREW |  |  |  |  |  |  |  |  |
| 3.1 | Nationality of Master: |  |  |  |  |  |  | FILIPINO |  |
| 3.2 | Number and nationality of Officers: |  |  |  |  | 10 |  | INDIAN, ROMANIAN, RUSSIAN, FILIPINO |  |
| 3.3 | Number and nationality of Crew: |  |  |  |  | 8 |  | FILIPINO |  |
| 3.4 | What is the common working language onboard: |  |  |  |  |  |  | English |  |
| 3.5 | Do officers speak and understand English? |  |  |  |  |  |  | Yes |  |
| 3.6 | If Officers/ratings employed by a manning agency - Full style: |  |  | Officers: Maersk Tankers India Private Limited, 702 Tower A, 7th Floor, 247 Park, LBS Road, Vikhroli West Mumbai 400083 India |  |  |  | Ratings: Jebsen Maritime Inc. Pilgrim Building, 111 Aguirre Street, Legaspi Village, Makati City, Philippines Tel: +639178318244 Fax: +632 804-028 |  |
| 4. | FOR USA CALLS |  |  |  |  |  |  |  |  |
| 4.1 | Has the vessel Operator submitted a Vessel Spill Response Plan to the US Coast Guard which has been approved by official USCG letter? |  |  |  |  |  |  | Yes |  |
| 4.2 | Qualified individual (QI) - Full style: |  |  |  |  | Gallagher Marine Systems (GMS) 305 Harper Drive Moorestown NJ 08054 USA Tel: +1 703 683 4700 Fax: +1 856 642 3945 Email: info@chgms.com Web: http://www.gallaghermarine.com |  |  |  |
| 4.3 | Oil Spill Response Organization (OSRO) - Full style: |  |  |  |  | National Response Corporation 3500 Sunrise Highway, Building 200, Suite 200 Great River, New York 11739 USA Tel: +1 631 224 9141 Fax: +1 631 224 9082 Email: Email: iocdo@nrcc.com |  |  |  |
| 4.4 | Salvage and Marine Firefighting Services (SMFF) - Full Style: |  |  |  |  | Donjon-SMIT, LLC 15402 Vantage Pkwy E. Suite 316 Houston, Texas 77032 24/7 OPA-90 Emergency or Drill, please dial: +1 206 332 8200 Tel: +1 703 299 0081 Email: Opa90@ardent-americas.com Web: www.ardent-americas.com |  |  |  |
| 5. | SAFETY/HELICOPTER |  |  |  |  |  |  |  |  |
| 5.1 | Is the vessel operated under a Quality Management System? If Yes, what type of system? (ISO9001 or IMO Resolution A.741(18) as amended): |  |  |  |  |  |  | Yes IMO Resolution A.741(18) as amended |  |
| 5.2 | Can the ship comply with the ICS Helicopter Guidelines? |  |  |  |  |  |  | Yes |  |
| 5.2.1 | If Yes, state whether winching or landing area provided: |  |  |  |  |  |  | Winching |  |
| 5.2.2 | If Yes, what is the diameter of the circle provided: |  |  |  |  |  |  | 5.60 Metres |  |
| 6. | COATING/ANODES |  |  |  |  |  |  |  |  |
| 6.1 | Tank Coating |  |  | Coated |  | Type |  | To What Extent | Anodes |
|  | Cargo tanks: |  |  | Yes |  | Phenolic Epoxy |  | Full | No |
|  | Ballast tanks: |  |  | Yes |  | Epoxy |  | Full | Yes |
|  | Slop tanks: |  |  | Yes |  | Phenolic Epoxy |  | Whole Tank | No |
| 7. | BALLAST |  |  |  |  |  |  |  |  |
| 7.1 | Pumps |  |  |  | No. | Type |  | Capacity | At What Head (sg=1.0) |
|  | Ballast Pumps: |  |  |  | 2 | Framo |  | 750 Cu. Metres/Hour | 30 Metres |
|  | Ballast Eductors: |  |  |  | 2 | Other |  | 100 Cu. Metres/Hour |  |
| 8. | CARGO |  |  |  |  |  |  |  |  |
| Double Hull Vessels |  |  |  |  |  |  |  |  |  |
| 8.1 | Is vessel fitted with centerline bulkhead in all cargo tanks? If Yes, solid or perforated: |  |  |  |  |  |  | No, |  |
| Cargo Tank Capacities |  |  |  |  |  |  |  |  |  |
| 8.2 | Number of cargo tanks and total cubic capacity (98%): |  |  |  |  |  |  | 12 | 52,692.20 Cu. Metres |
| 8.2.1 | Capacity (98%) of each natural segregation with double valve (specify tanks): |  |  |  |  |  |  | EACH SET OF TANKS HAVE NATURAL DOUBLE VALVE SEGREGATION. VESSEL FITTED WITH 6 GROUP. SEG 1- COT 1P & 1S 7061.9 CUB M 98% SEG 2- COT 2P & 2S 9226.3 CUB M 98% SEG 3- COT 3P & 3S 9383.1 CUB M 98% SEG 4- COT 4P & 4S 9382.3 CUB M 98% SEG 5- COT 5P & 5S 9360.47 CUB M 98% SEG 6- COT 6P & 6S 8278.15 CUB M 98% |  |
| 8.2.2 | IMO class (Oil/Chemical Ship Type 1, 2 or 3): |  |  |  |  |  |  | 3 |  |
| 8.3 | Number of slop tanks and total cubic capacity (98%): |  |  |  |  |  |  | 2 | 1,145.40 Cu. Metres |
| 8.3.1 | Specify segregations which slops tanks belong to and their capacity with double valve: |  |  |  |  |  |  | Segregated from all tanks by 2 valves. |  |
| 8.3.2 | Residual/retention oil tank(s) capacity (98%), if applicable: |  |  |  |  |  |  | 152.40 Cu. Metres |  |
| SBT Vessels |  |  |  |  |  |  |  |  |  |
| 8.3.3 | What is total SBT capacity and percentage of SDWT vessel can maintain? |  |  |  |  |  |  | 22,752.10 Cu. Metres | 44 % |
| 8.3.4 | Does vessel meet the requirements of MARPOL Annex I Reg 18.2: |  |  |  |  |  |  | Yes |  |
| Cargo Handling and Pumping Systems |  |  |  |  |  |  |  |  |  |
| 8.4 | How many grades/products can vessel load/discharge with double valve segregation: |  |  |  |  |  |  | 7 |  |
| 8.5 | Are there any cargo tank filling restrictions? If yes, specify number of slack tanks, max s.g., ullage restrictions etc.: |  |  |  |  |  |  | No |  |
| 8.6 | Max loading rate for homogenous cargo |  |  |  |  |  |  | With VECS | Without VECS |
|  | Loaded per manifold connection: |  |  |  |  |  |  | 3,054 Cu. Metres/Hour | 3,054 Cu. Metres/Hour |
|  | Loaded simultaneously through all manifolds: |  |  |  |  |  |  | 4,900 Cu. Metres/Hour | 4,900.00 Cu. Metres/Hour |
| Cargo Control Room |  |  |  |  |  |  |  |  |  |
| 8.7 | Is ship fitted with a Cargo Control Room (CCR)? |  |  |  |  |  |  | Yes |  |
| 8.8 | Can tank innage/ullage be read from the CCR? |  |  |  |  |  |  | Yes |  |
| Gauging and Sampling |  |  |  |  |  |  |  |  |  |
| 8.9 | Is gauging system certified and calibrated? If no, specify which ones are not calibrated: |  |  |  |  |  |  | No, Tank Centre only |  |
|  | What type of fixed closed tank gauging system is fitted: |  |  |  |  |  |  | Radar |  |
|  | Are high level alarms fitted to the cargo tanks? If Yes, indicate whether to all tanks or partial: |  |  |  |  |  |  | Yes, All |  |
| 8.9.1 | Can cargo be transferred under closed loading conditions in accordance with ISGOTT 11.1.6.6? |  |  |  |  |  |  | Yes |  |
| 8.9.2 | Are cargo tanks fitted with multipoint gauging? If yes, specify type and locations: |  |  |  |  |  |  | Yes, Fwd, Mid , Aft MMC vapour Lock type C-50, 2" for tank gauging F-100 4" diameter for sampling in tank centre. D-25 ,1" diameter for and aft for dipping |  |
| 8.10 | Number of portable gauging units (example- MMC) on board: |  |  |  |  |  |  | 2 |  |
| Vapor Emission Control System (VECS) |  |  |  |  |  |  |  |  |  |
| 8.11 | Is a vapour return system (VRS) fitted? |  |  |  |  |  |  | Yes |  |
| 8.12 | Number/size of VECS manifolds (per side): |  |  |  |  |  |  | 2 | 300 Millimetres |
| 8.13 | Number/size/type of VECS reducers: |  |  |  |  |  |  | 4 x 300/300 2 x 300/400 1 x 300/250 |  |
| Venting |  |  |  |  |  |  |  |  |  |
| 8.14 | State what type of venting system is fitted: |  |  |  |  |  |  | Mastriser and Individual Tank PV Valves |  |
| Cargo Manifolds and Reducers |  |  |  |  |  |  |  |  |  |
| 8.15 | Total number/size of cargo manifold connections on each side: |  |  |  |  |  |  | 7 (6 for cargo and 1 for slop tanks)/300.00 Millimetres (300 millimetres for cargo, 200 millimetres for slop) |  |
| 8.16 | What type of valves are fitted at manifold: |  |  |  |  |  |  | Manual |  |
| 8.17 | What is the material/rating of the manifold: |  |  |  |  |  |  | 316 Stainless Steel/ANSI 150 |  |
| 8.17.1 | Does vessel comply with the latest edition of the OCIMF 'Recommendations for Oil Tanker Manifolds and Associated Equipment’? |  |  |  |  |  |  | Yes |  |
| 8.18 | Distance between cargo manifold centers: |  |  |  |  |  |  | 2,000.00 Millimetres |  |
| 8.19 | Distance ships rail to manifold: |  |  |  |  |  |  | 4,420.00 Millimetres |  |
| 8.20 | Distance manifold to ships side: |  |  |  |  |  |  | 4,600.00 Millimetres |  |
| 8.21 | Top of rail to center of manifold: |  |  |  |  |  |  | 840.00 Millimetres |  |
| 8.22 | Distance main deck to center of manifold: |  |  |  |  |  |  | 2,100.00 Millimetres |  |
| 8.23 | Spill tank grating to center of manifold: |  |  |  |  |  |  | 900.00 Millimetres |  |
| 8.24 | Manifold height above the waterline in normal ballast/at SDWT condition: |  |  |  |  |  |  | 13.82 Metres | 8.905 Metres |
| 8.25 | Number/size/type of reducers: |  |  |  |  |  |  | 12 x 300/400mm (12/16") 2 x 200/400mm (8/16") 6 x 300/200mm (12/8") 6 x 300/250mm (12/10") 6 x 300/300mm (12/12") ANSI |  |
| 8.26 | Is vessel fitted with a stern manifold? If yes, state size: |  |  |  |  |  |  | No, |  |
| Heating |  |  |  |  |  |  |  |  |  |
| 8.27 | Cargo/slop tanks fitted with a cargo heating system? |  |  |  |  | Type |  | Coiled | Material |
|  | Cargo Tanks: |  |  |  |  | Steam |  | No | SS |
|  | Slop Tanks: |  |  |  |  | SS Coils |  | Yes | 316 SUS |
| 8.28 | Maximum temperature cargo can be loaded/maintained: |  |  |  |  |  |  | 80.0 °C / 176.0 °F | 70 °C / 158 °F |
| 8.28.1 | Minimum temperature cargo can be loaded/maintained: |  |  |  |  |  |  | -10.0 °C / 14.0 °F | 0.0 °C / 32.0 °F |
| Inert Gas and Crude Oil Washing |  |  |  |  |  |  |  |  |  |
| 8.29 | Is an Inert Gas System (IGS) fitted/operational? |  |  |  |  |  |  | Yes/Yes |  |
| 8.29.1 | Is a Crude Oil Washing (COW) installation fitted/operational? |  |  |  |  |  |  | Yes/Yes |  |
| 8.30 | Is IGS supplied by flue gas, inert gas (IG) generator and/or nitrogen: |  |  |  |  |  |  | IG Generator |  |
| Cargo Pumps |  |  |  |  |  |  |  |  |  |
| 8.31 | How many cargo pumps can be run simultaneously at full capacity: |  |  |  |  |  |  | 6 |  |
| 8.32 | Pumps |  |  | No. |  | Type |  | Capacity | At What Head (sg=1.0) |
|  | Cargo Pumps: |  |  | 12 2 1 |  | Centrifugal Centrifugal Centrifugal |  | 600 M3/HR 300 M3/HR 75 M3/HR | 125 Meters 125 Meters 125 Meters |
|  | Cargo Eductors: |  |  |  |  |  |  |  |  |
|  | Stripping: |  |  |  |  |  |  |  |  |
| 8.33 | Is at least one emergency portable cargo pump provided? |  |  |  |  |  |  | Yes |  |
| 9. | MOORING |  |  |  |  |  |  |  |  |
| 9.1 | Wires (on drums) |  | No. | Diameter |  | Material |  | Length | Breaking Strength |
|  | Forecastle: |  |  |  |  |  |  |  |  |
|  | Main deck fwd: |  |  |  |  |  |  |  |  |
|  | Main deck aft: |  |  |  |  |  |  |  |  |
|  | Poop deck: |  |  |  |  |  |  |  |  |
| 9.2 | Wire tails |  | No. | Diameter |  | Material |  | Length | Breaking Strength |
|  | Forecastle: |  | 4 | 48.00 Millimetres |  | Dyneema |  | 11.00 Metres | 75.80 Metric Tonnes |
|  | Main deck fwd: |  | 2 | 48.00 Millimetres |  | Dyneema |  | 11.00 Metres | 75.80 Metric Tonnes |
|  | Main deck aft: |  | 2 | 48.00 Millimetres |  | Dyneema |  | 11.00 Metres | 75.80 Metric Tonnes |
|  | Poop deck: |  | 4 | 48.00 Millimetres |  | Dyneema |  | 11.00 Metres | 75.80 Metric Tonnes |
| 9.3 | Ropes (on drums) |  | No. | Diameter |  | Material |  | Length | Breaking Strength |
|  | Forecastle: |  | 4 | 26.00 Millimetres |  | Amsteel Blue AS-78 |  | 220.00 Metres | 54.70 Metric Tonnes |
|  | Main deck fwd: |  | 2 | 26.00 Millimetres |  | Amsteel Blue AS-78 |  | 220.00 Metres | 54.70 Metric Tonnes |
|  | Main deck aft: |  | 2 | 26.00 Millimetres |  | Amsteel Blue AS-78 |  | 220.00 Metres | 54.70 Metric Tonnes |
|  | Poop deck: |  | 4 | 26 Millimetres |  | Amsteel Blue AS-78 |  | 220.00 Metres | 54.70 Metric Tonnes |
| 9.4 | Other lines |  | No. | Diameter |  | Material |  | Length | Breaking Strength |
|  | Forecastle: |  | 3 | 58.00 Millimetres |  | D Flex Rope |  | 220.00 Metres | 65.00 Metric Tonnes |
|  | Main deck fwd: |  |  |  |  |  |  |  |  |
|  | Main deck aft: |  |  |  |  |  |  |  |  |
|  | Poop deck: |  | 3 | 58.00 Millimetres |  | D Flex Rope |  | 220.00 Metres | 65.00 Metric Tonnes |
| 9.5 | Winches |  | No. | No. Drums |  | Motive Power |  | Brake Capacity | Type of Brake |
|  | Forecastle: |  | 2 | Dbl |  | Hydraulic |  | 50.40 Metric Tonnes | Friction Band |
|  | Main deck fwd: |  | 1 | Dbl |  | Hydraulic |  | 50.40 Metric Tonnes | Friction Band |
|  | Main deck aft: |  | 1 | Dbl |  | Hydraulic |  | 50.40 Metric Tonnes | Friction Band |
|  | Poop deck: |  | 2 | Dbl |  | Hydraulic |  | 50.40 Metric Tonnes | Friction Band |
| 9.6 | Bitts, closed chocks/fairleads |  |  | No. Bitts |  | SWL Bitts |  | No. Closed Chocks | SWL Closed Chocks |
|  | Forecastle: |  |  | 6 |  | 64 Metric Tonnes (2 x 63, 2 x 64, 2 x 90) |  | 10 | 63 Metric Tonnes (4 x 90 , 4 x 64, 2 x 204) |
|  | Main deck fwd: |  |  | 12 |  | 63 Metric Tonnes (2 x 55 , 8 x 63, 2 x 25) |  | 18 | 63 Metric Tonnes (2 x 55 , 12 x 63, 4 x 25) |
|  | Main deck aft: |  |  | 6 |  | 63 Metric Tonnes (2 x 25, 4 x 63) |  | 10 | 63 Metric Tonnes (4 x 25, 6 x 63) |
|  | Poop deck: |  |  | 8 |  | 64 Metric Tonnes (2 x 92, 6 x 64) |  | 17 | 64 Metric Tonnes (4 x 90, 6 x 64, 6 x 63, 1 x 204) |
| Anchors/Emergency Towing System |  |  |  |  |  |  |  |  |  |
| 9.7 | Number of shackles on port/starboard cable: |  |  |  |  |  |  | 12/12 |  |
| 9.8 | Type/SWL of Emergency Towing system forward: |  |  |  |  |  |  | Tongue Type | 204 Metric Tonnes |
| 9.9 | Type/SWL of Emergency Towing system aft: |  |  |  |  |  |  | Escort and Pull Back | 204 Metric Tonnes |
| 9.10.1 | What is size of closed chock and/or fairleads of enclosed type on stern |  |  |  |  |  |  | 450mm x 600mm |  |
| Escort Tug |  |  |  |  |  |  |  |  |  |
| 9.10.2 | What is SWL of closed chock and/or fairleads of enclosed type on stern: |  |  |  |  |  |  | 204.00 Metric Tonnes |  |
| 9.11 | What is SWL of bollard on poop deck suitable for escort tug: |  |  |  |  |  |  | 204.00 Metric Tonnes |  |
| Lifting Equipment/Gangway |  |  |  |  |  |  |  |  |  |
| 9.12 | Derrick/Crane description (Number, SWL and location): |  |  |  |  |  |  | Cranes: 1 x 10.00 Tonnes Centre |  |
| 9.13 | Accommodation ladder direction: |  |  |  |  |  |  | Aft |  |
|  | Does vessel have a portable gangway? If yes, state length: |  |  |  |  |  |  | Yes, 15 Metres |  |
| Single Point Mooring (SPM) Equipment |  |  |  |  |  |  |  |  |  |
| 9.14 | Does the vessel meet the recommendations in the latest edition of OCIMF ‘Recommendations for Equipment Employed in the Bow Mooring of Conventional Tankers at Single Point Moorings (SPM)’? |  |  |  |  |  |  | Yes |  |
| 9.15 | If fitted, how many chain stoppers: |  |  |  |  |  |  | 2 |  |
| 9.16 | State type/SWL of chain stopper(s): |  |  |  |  |  |  | Tongue | 204.00 Metric Tonnes |
| 9.17 | What is the maximum size chain diameter the bow stopper(s) can handle: |  |  |  |  |  |  | 76.00 Millimetres |  |
| 9.18 | Distance between the bow fairlead and chain stopper/bracket: |  |  |  |  |  |  | 3.37 Metres |  |
| 9.19 | Is bow chock and/or fairlead of enclosed type of OCIMF recommended size (600mm x 450mm)? If not, give details of size: |  |  |  |  |  |  | Yes |  |
| 10. | PROPULSION |  |  |  |  |  |  |  |  |
| 10.1 | Speed |  |  |  |  |  |  | Maximum | Economical |
|  | Ballast speed: |  |  |  |  |  |  | 16.00 Knots (WSNP) | 12.00 Knots (WSNP) |
|  | Laden speed: |  |  |  |  |  |  | 15.26 Knots (WSNP) | 11.40 Knots (WSNP) |
| 10.2 | What type of fuel is used for main propulsion/generating plant: |  |  |  |  |  |  | VLSHFO & MGO | VLSHFO & MGO |
| 10.3 | Type/Capacity of bunker tanks: |  |  |  |  |  |  | Fuel Oil: 1,957.40 Cu. Metres Diesel Oil: 185.40 Cu. Metres Gas Oil: 66.30 Cu. Metres |  |
| 10.4 | Is vessel fitted with fixed or controllable pitch propeller(s): |  |  |  |  |  |  |  |  |
| 10.5 | Engines |  |  |  |  |  | No | Capacity | Make/Type |
|  | Main engine: |  |  |  |  |  | 1 | 9,160 Kilowatt | Hyundai MAN B&W 6G50ME-B9.3 |
|  | Aux engine: |  |  |  |  |  | 3 | 960 Kilowatt | Himsen 6H21/32 |
|  | Power packs: |  |  |  |  |  | 4 | 0.08 Cu. Metres | Electric and Cummins diesel |
|  | Boilers: |  |  |  |  |  | 2 | 18.00 Metric Tonnes/Hour | Alfa Laval / KBM 150 |
| Bow/Stern Thruster |  |  |  |  |  |  |  |  |  |
| 10.6 | What is brake horse power of bow thruster (if fitted): |  |  |  |  |  |  | No, |  |
| 10.7 | What is brake horse power of stern thruster (if fitted): |  |  |  |  |  |  | No, |  |
| Emissions |  |  |  |  |  |  |  |  |  |
| 10.8 | Main engine IMO NOx emission standard: |  |  |  |  |  |  | Tier II |  |
| 10.9 | Energy Efficiency Design Index (EEDI) rating number: |  |  |  |  |  |  | 4.97 |  |
| 11. | SHIP TO SHIP TRANSFER |  |  |  |  |  |  |  |  |
| 11.1 | Does vessel comply with recommendations contained in OCIMF/ICS Ship To Ship Transfer Guide (Petroleum, Chemicals or Liquified Gas, as applicable)? |  |  |  |  |  |  | Yes |  |
| 11.2 | What is maximum outreach of cranes/derricks outboard of the ship’s side: |  |  |  |  |  |  | 8.90 Metres |  |
| 11.3 | Date/place of last STS operation: |  |  |  |  |  |  |  |  |
| 12. | RECENT OPERATIONAL HISTORY |  |  |  |  |  |  |  |  |
| 12.1 | Last three cargoes/charterers/voyages (Last/2nd Last/3rd Last): |  |  |  |  |  |  | Last Cargo: Naphtha / Sahara Energy / Vadinar to Pengerang 2nd Last Cargo: ULSD / BP / Cherry Point - Botany Bay & Newcastle 3rd Last Cargo; Gas Oil, ULP 91, ULP 95, ULP 98 Ron & Gas Oil / BP / Kwinana - Newcastle & Botany Bay |  |
| 12.2 | Has vessel been involved in a pollution, grounding, serious casualty, unscheduled repair or collision incident during the past 12 months? If yes, provide details: |  |  |  |  |  |  | Pollution: No, Grounding: No, Casualty: No, Repair: No, Collision: No, |  |
| 12.3 | Date and place of last Port State Control inspection: |  |  |  |  |  |  | Apr 22, 2020 / Hobart, Australia |  |
| 12.4 | Any outstanding deficiencies as reported by any Port State Control? If yes, provide details: |  |  |  |  |  |  | No No Outstanding Deficiencies |  |
| 12.5 | Recent Oil company inspections/screenings (To the best of owners knowledge and without guarantee of acceptance for future business)*: * "Approvals" are not given by Oil Majors and ships are accepted for the voyage on a case by case basis. |  |  |  |  |  |  |  |  |
| 12.6 | Date/Place of last SIRE inspection: |  |  |  |  |  |  | 12 Aug 2020 / BP / Singapore |  |
| 12.7 | Additional information relating to features of the ship or operational characteristics: |  |  |  |  |  |  |  |  |

Revised 2018 (INTERTANKO/Q88.com)

Form completed on http://www.q88.com/integration.aspx  Please email support@q88.com an updated copy if this is not the latest version.