- INTERTANKO CHARTERING QUESTIONNAIRE 88 - OIL 	Version 5

| 1. | GENERAL INFORMATION |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1.1 | Date updated: |  |  |  |  |  |  | Sep 19, 2020 |  |
| 1.2 | Vessel’s name (IMO number): |  |  |  |  |  |  | Sauger (9273442) |  |
| 1.3 | Vessel’s previous name(s) and date(s) of change: |  |  |  |  |  |  | Kaspar Schulte (Jun 24, 2015) |  |
| 1.4 | Date delivered/Builder (where built): |  |  |  |  |  |  | Feb 10, 2004/Samsung Heavy Industries Co., Ltd Koje Shipyard |  |
| 1.5 | Flag/Port of Registry: |  |  |  |  |  |  | Marshall Islands/Majuro |  |
| 1.6 | Call sign/MMSI: |  |  |  |  |  |  | V7NI8/538006461 |  |
| 1.7 | Vessel’s contact details (satcom/fax/email etc.): |  |  |  |  |  |  | Tel: +870 7739 10492 // +870 771 301 762 Fax: +870 783928402 Email: sauger@gtships.com |  |
| 1.8 | Type of vessel (as described in Form A or Form B Q1.11 of the IOPPC): |  |  |  |  |  |  | Oil Tanker |  |
| 1.9 | Type of hull: |  |  |  |  |  |  | Double Hull |  |
| Ownership and Operation |  |  |  |  |  |  |  |  |  |
| 1.10 | Registered owner - Full style: |  |  |  |  | Sauger Solutions Inc. C/O Prime Tanker Management Inc |  |  |  |
| 1.11 | Technical operator - Full style: |  |  |  |  | Prime Tanker Management Inc 4, Possidonos ave, 176-74, Kallithea, Athens, Greece Tel: +30-210-9464800 Email: operations@prime-marine.net Web: www. prime-marine.net |  |  |  |
| 1.12 | Commercial operator - Full style: |  |  |  |  | Prime Tanker Management Inc 4, Possidonos ave, 176-74, Kallithea, Athens, Greece Tel: +30-210-9464800 Email: operations@prime-marine.net Web: www. prime-marine.net |  |  |  |
| 1.13 | Disponent owner - Full style: |  |  |  |  | NORTHWIND TRANSENERGY AS. SANDVIGGVEIEN 195, 4816 KOLBJORNSVIK 0906 ARENDAL, NORWAY |  |  |  |
| Insurance |  |  |  |  |  |  |  |  |  |
| 1.14 | P & I Club - Full Style: |  |  |  |  | The United Kingdom Mutual Steam Ship Assurance Association (Europe) Limited. Registered in England No 22215. Registered office: 90 Fenchurch Street, London, EC3M 4ST. Tel: +44 20 7283 4646 Email: Hellas1.ukclub@thomasmiller.com |  |  |  |
| 1.15 | P & I Club pollution liability coverage/expiration date: |  |  |  |  |  |  | 1,000,000,000 US$ | Feb 20, 2021 |
| 1.16 | Hull & Machinery insured by - Full Style: (Specify broker or leading underwriter) |  |  |  |  | Norwegian Hull Club Radhusgaten 25, NO-0158 Oslo, Norway Emergency Nr +47-22428844 Tel: +47 55 55 95 00 nhcmail@norclub.no |  |  |  |
| 1.17 | Hull & Machinery insured value/expiration date: |  |  |  |  |  |  | 19,000,000 US$ | Jan 08, 2021 |
| Classification |  |  |  |  |  |  |  |  |  |
| 1.18 | Classification society: |  |  |  |  |  |  | American Bureau of Shipping |  |
| 1.19 | Class notation: |  |  |  |  |  |  | +A1, Oil Carrier, E, +AMS, +ACCU, VEC, SH, SHCM |  |
| 1.20 | Is the vessel subject to any conditions of class, class extensions, outstanding memorandums or class recommendations? If yes, give details: |  |  |  |  |  |  | No n/a |  |
| 1.21 | If classification society changed, name of previous and date of change: |  |  |  |  |  |  | N/S |  |
| 1.22 | Does the vessel have ice class? If yes, state what level: |  |  |  |  |  |  | No, |  |
| 1.23 | Date/place of last dry-dock: |  |  |  |  |  |  | Dec 30, 2018/Qinhuangdao |  |
| 1.24 | Date next dry dock due/next annual survey due: |  |  |  |  |  |  | Feb 09, 2024 | Feb 09, 2021 |
| 1.25 | Date of last special survey/next special survey due: |  |  |  |  |  |  | Dec 30, 2018 | Feb 09, 2024 |
| 1.26 | If ship has Condition Assessment Program (CAP), what is the latest overall rating: |  |  |  |  |  |  | Yes, 1 |  |
| Dimensions |  |  |  |  |  |  |  |  |  |
| 1.27 | Length overall (LOA): |  |  |  |  |  |  | 227.83 Metres |  |
| 1.28 | Length between perpendiculars (LBP): |  |  |  |  |  |  | 219.00 Metres |  |
| 1.29 | Extreme breadth (Beam): |  |  |  |  |  |  | 32.28 Metres |  |
| 1.30 | Moulded depth: |  |  |  |  |  |  | 20.60 Metres |  |
| 1.31 | Keel to masthead (KTM)/ Keel to masthead (KTM) in collapsed condition, if applicable: |  |  |  |  |  |  | 47.65 Metres |  |
| 1.32 | Distance bridge front to center of manifold: |  |  |  |  |  |  | 76.65 Metres |  |
| 1.33 | Bow to center manifold (BCM)/Stern to center manifold (SCM): |  |  |  |  |  |  | 113.11 Metres | 114.72 Metres |
| 1.34 | Parallel body distances |  |  |  |  | Lightship |  | Normal Ballast | Summer Dwt |
|  | # Forward to mid-point manifold: |  |  |  |  | 51.39 Metres |  | 68.05 Metres | 68.57 Metres |
|  | # Aft to mid-point manifold: |  |  |  |  | 47.90 Metres |  | 60.27 Metres | 72.28 Metres |
|  | # Parallel body length: |  |  |  |  | 99.29 Metres |  | 128.32 Metres | 140.336 Metres |
| Tonnages |  |  |  |  |  |  |  |  |  |
| 1.35 | Net Tonnage: |  |  |  |  |  |  | 20,972.00 |  |
| 1.36 | Gross Tonnage/Reduced Gross Tonnage (if applicable): |  |  |  |  |  |  | 41,503.00 | 32,428 |
| 1.37 | Suez Canal Tonnage - Gross (SCGT)/Net (SCNT): |  |  |  |  |  |  | 43,153.84 | 38,517.12 |
| # 1.38 | Panama Canal Net Tonnage (PCNT): |  |  |  |  |  |  | 34,292.00 |  |
| # Loadline Information |  |  |  |  |  |  |  |  |  |
| # 1.39 | # Loadline |  |  | # Freeboard |  | # Draft |  | # Deadweight | # Displacement |
|  | # Summer: |  |  | 6.619 Metres |  | 14.022 Metres |  | # 72,652 Metric Tonnes | 86,139 Metric Tonnes |
|  | # Winter: |  |  | 6.911 Metres |  | 13.73 Metres |  | 70.769 Metric Tonnes | 84,190 Metric Tonnes |
|  | # Tropical: |  |  | 6.327 Metres |  | 14.314 Metres |  | 74,605 Metric Tonnes | 88,092 Metric Tonnes |
|  | # Lightship: |  |  | 18.04 Metres |  | 2.60 Metres |  | - | 13,487.00 Metric Tonnes |
|  | # Normal Ballast Condition: |  |  | 13.44 Metres |  | 7.11 Metres |  | 27,822.00 Metric Tonnes | 41,243.00 Metric Tonnes |
|  | # Segregated Ballast Condition: |  |  |  |  |  |  |  |  |
| # 1.40 | # FWA/TPC at summer draft: |  |  |  |  |  |  | 322.00 Millimetres | 66.80 Metric Tonnes |
| # 1.41 | # Does vessel have multiple SDWT? If yes, please provide all assigned loadlines: |  |  |  |  |  |  | Yes Dwt 64.999.0 Draft 12.872 m Dwt 59.997.4 Draft 12.115 m |  |
| 1.42 | Constant (excluding fresh water): |  |  |  |  |  |  | 260 Metric Tonnes |  |
| 1.43 | What is the company guidelines for Under Keel Clearance (UKC) for this vessel? |  |  |  |  |  |  | Channels & Fairways: Min 0.50m dynamic (at all times) Whilst alongside a berth, the minimum required UKC is 1.5% of the moulded breadth of the vessel, but not less than 0.3m |  |
| 1.44 | What is the max height of mast above waterline (air draft) |  |  |  |  |  |  | Full Mast | Collapsed Mast |
|  | Summer deadweight: |  |  |  |  |  |  | 33.628 Metres | 0 Metres |
|  | Normal ballast: |  |  |  |  |  |  | 40.45 Metres | 0 Metres |
|  | Lightship: |  |  |  |  |  |  | 45.05 Metres | 0 Metres |
| 2. | CERTIFICATES |  |  | Issued |  | Last Annual |  | Last Intermediate | Expires |
| 2.1 | Safety Equipment Certificate (SEC): |  |  | Dec 30, 2018 |  | Jul 05, 2020 |  |  | Feb 09, 2024 |
| 2.2 | Safety Radio Certificate (SRC): |  |  | Dec 30, 2018 |  | Jul 05, 2020 |  |  | Feb 09, 2024 |
| 2.3 | Safety Construction Certificate (SCC): |  |  | Dec 30, 2018 |  | Jul 05, 2020 |  |  | Feb 09, 2024 |
| 2.4 | International Loadline Certificate (ILC): |  |  | Dec 30, 2018 |  | Jul 05, 2020 |  |  | Feb 09, 2024 |
| 2.5 | International Oil Pollution Prevention Certificate (IOPPC): |  |  | Dec 30, 2018 |  | Jul 05, 2020 |  |  | Feb 09, 2024 |
| # 2.6 | # International Ship Security Certificate (ISSC): |  |  | # Nov 01, 2015 |  |  |  | # Sep 10, 2018 | Oct 31, 2020 |
| # 2.7 | # Maritime Labour Certificate (MLC): |  |  | # Sep 12, 2018 |  | # N/A |  | # Sep 12, 2018 | Oct 31, 2020 |
| 2.8 | ISM Safety Management Certificate (SMC): |  |  | Nov 01, 2015 |  |  |  | # Sep 10, 2018 | Oct 31, 2020 |
| # 2.9 | # Document of Compliance (DOC): |  |  | # Apr 19, 2019 |  | Jul 15, 2020 |  |  | Apr 21, 2024 |
| 2.10 | USCG Certificate of Compliance (USCGCOC): |  |  | Jan 29, 2019 |  |  |  |  | Jan 29, 2021 |
| 2.11 | Civil Liability Convention (CLC) 1992 Certificate: |  |  | Feb 20, 2020 |  | N/A |  | N/A | Feb 20, 2021 |
| 2.12 | Civil Liability for Bunker Oil Pollution Damage Convention (CLBC) Certificate: |  |  | Feb 20, 2020 |  | N/A |  | N/A | Feb 20, 2021 |
| # 2.13 | # Liability for the Removal of Wrecks Certificate (WRC): |  |  | # Feb 20, 2020 |  | N/A |  | N/A | Feb 20, 2021 |
| 2.14 | U.S. Certificate of Financial Responsibility (COFR): |  |  | Aug 07, 2019 |  | N/A |  | N/A | Aug 07, 2022 |
| 2.15 | Certificate of Class (COC): |  |  | Dec 30, 2018 |  | Jul 05, 2020 |  |  | Feb 09, 2024 |
| # 2.16 | # International Sewage Pollution Prevention Certificate (ISPPC): |  |  | # Dec 30, 2018 |  | # N/A |  | # N/A | Feb 09, 2024 |
| # 2.17 | # Certificate of Fitness (COF): |  |  |  |  |  |  |  |  |
| # 2.18 | # International Energy Efficiency Certificate (IEEC): |  |  | # Jun 25, 2015 |  | N/A |  | N/A | N/A |
| # 2.19 | # International Air Pollution Prevention Certificate (IAPPC): |  |  | # Dec 30, 2018 |  | Jul 05, 2020 |  |  | Feb 09, 2024 |
| Documentation |  |  |  |  |  |  |  |  |  |
| 2.20 | Owner warrant that vessel is member of ITOPF and will remain so for the entire duration of this voyage/contract: |  |  |  |  |  |  | Yes |  |
| 2.21 | Does vessel have in place a Drug and Alcohol Policy complying with OCIMF guidelines for Control of Drugs and Alcohol Onboard Ship? |  |  |  |  |  |  | Yes |  |
| 2.22 | Is the ITF Special Agreement on board (if applicable)? |  |  |  |  |  |  | Yes |  |
| 2.23 | ITF Blue Card expiry date (if applicable): |  |  |  |  |  |  | Jun 23, 2021 |  |
| 3. | CREW |  |  |  |  |  |  |  |  |
| 3.1 | Nationality of Master: |  |  |  |  |  |  | Russian |  |
| 3.2 | Number and nationality of Officers: |  |  |  |  | 8 |  | Ukrainian, Russian, Georgian |  |
| 3.3 | Number and nationality of Crew: |  |  |  |  | 13 |  | Ukranian, Georgian |  |
| 3.4 | What is the common working language onboard: |  |  |  |  |  |  | English |  |
| 3.5 | Do officers speak and understand English? |  |  |  |  |  |  | Yes |  |
| 3.6 | If Officers/ratings employed by a manning agency - Full style: |  |  | Officers: Uniprime Ltd 18 / B, Armeyskaya str Odessa 65009, Ukraine Tel: +380 482 372264 Email: office@uniprime.od.ua |  |  |  | Ratings: Uniprime Ltd 18 / B, Armeyskaya str Odessa 65009m Ukraine Tel: +380 482 372264 Email: office@uniprime.od.ua |  |
| 4. | FOR USA CALLS |  |  |  |  |  |  |  |  |
| 4.1 | Has the vessel Operator submitted a Vessel Spill Response Plan to the US Coast Guard which has been approved by official USCG letter? |  |  |  |  |  |  | Yes |  |
| 4.2 | Qualified individual (QI) - Full style: |  |  |  |  | Gallagher Marine Systems Inc 305 Harper Drive, Moorestown, New Jersey, USA 08057 Tel: +1 703 683 4700 Email: info@chgms.com |  |  |  |
| 4.3 | Oil Spill Response Organization (OSRO) - Full style: |  |  |  |  | National Response Corporation 3500 Sunrise Highway, Suite – 103, Great River, NY 11739 Tel: +1 631 224 9141 Email: iocdo@nrcc.com |  |  |  |
| 4.4 | Salvage and Marine Firefighting Services (SMFF) - Full Style: |  |  |  |  | Resolve Marine Group 1850 SE 17th Street, Suite #204, FL 33316, USA Tel: +1 (954) 764 8700 Email: info@resolvemarine.com |  |  |  |
| 5. | SAFETY/HELICOPTER |  |  |  |  |  |  |  |  |
| 5.1 | Is the vessel operated under a Quality Management System? If Yes, what type of system? (ISO9001 or IMO Resolution A.741(18) as amended): |  |  |  |  |  |  | Yes IMO A 741(18) ISM Code |  |
| 5.2 | Can the ship comply with the ICS Helicopter Guidelines? |  |  |  |  |  |  | Yes |  |
| 5.2.1 | If Yes, state whether winching or landing area provided: |  |  |  |  |  |  | Winching |  |
| 5.2.2 | If Yes, what is the diameter of the circle provided: |  |  |  |  |  |  | 5.00 Metres |  |
| 6. | COATING/ANODES |  |  |  |  |  |  |  |  |
| 6.1 | Tank Coating |  |  | Coated |  | Type |  | To What Extent | Anodes |
|  | Cargo tanks: |  |  | Yes |  | Pure epoxy |  | Whole Tank | No |
|  | Ballast tanks: |  |  | Yes |  | Pure Epoxy |  | Whole Tank | Yes |
|  | Slop tanks: |  |  | Yes |  | Pure Epoxy |  | Whole Tank | No |
| 7. | BALLAST |  |  |  |  |  |  |  |  |
| 7.1 | Pumps |  |  |  | No. | Type |  | Capacity | At What Head (sg=1.0) |
|  | Ballast Pumps: |  |  |  | 2 | Centrifugal |  | 1,000 Cu. Metres/Hour | 25 Metres |
|  | Ballast Eductors: |  |  |  | 1 | Eductor |  | 200 Cu. Metres/Hour | 15 Metres |
| 8. | CARGO |  |  |  |  |  |  |  |  |
| Double Hull Vessels |  |  |  |  |  |  |  |  |  |
| 8.1 | Is vessel fitted with centerline bulkhead in all cargo tanks? If Yes, solid or perforated: |  |  |  |  |  |  | Yes, Solid |  |
| Cargo Tank Capacities |  |  |  |  |  |  |  |  |  |
| 8.2 | Number of cargo tanks and total cubic capacity (max% per company policy: 98%, 97%, 96% or 95%) excluding slops tanks: |  |  |  |  |  |  | 12 | 78,812.80 Cu. Metres |
| 8.2.1 | Capacity (max% per company policy: 98%, 97%, 96% or 95%) of each natural segregation with double valve (specify tanks): |  |  |  |  |  |  | Seg#1: 26570.9 m3 (1w, 4w, Slop S) Seg#2: 29281.1 m3 (2w,5w, Slop P) Seg#3: 27216.4 m3 (3w, 6w) |  |
| 8.2.2 | IMO class (Oil/Chemical Ship Type 1, 2 or 3): |  |  |  |  |  |  | N/A |  |
| 8.3 | Number of slop tanks and total cubic capacity (max% per company policy: 98%, 97%, 96% or 95%): |  |  |  |  |  |  | 2 | 4,255.80 Cu. Metres |
| 8.3.1 | Specify segregations which slops tanks belong to and their capacity with double valve: |  |  |  |  |  |  | As per 8.2.1 |  |
| 8.3.2 | Residual/retention oil tank(s) capacity (98%), if applicable: |  |  |  |  |  |  | 275 Cu. Metres |  |
| SBT Vessels |  |  |  |  |  |  |  |  |  |
| 8.3.3 | What is total SBT capacity and percentage of SDWT vessel can maintain? |  |  |  |  |  |  | 29,696.50 Cu. Metres | 49.40 % |
| 8.3.4 | Does vessel meet the requirements of MARPOL Annex I Reg 18.2: |  |  |  |  |  |  | Yes |  |
| Cargo Handling and Pumping Systems |  |  |  |  |  |  |  |  |  |
| 8.4 | How many grades/products can vessel load/discharge with double valve segregation: |  |  |  |  |  |  | 3 |  |
| 8.5 | Are there any cargo tank filling restrictions? If yes, specify number of slack tanks, max s.g., ullage restrictions etc.: |  |  |  |  |  |  | No |  |
| 8.6 | Max loading rate for homogenous cargo |  |  |  |  |  |  | With VECS | Without VECS |
|  | Loaded per manifold connection: |  |  |  |  |  |  |  | 4,000 Cu. Metres/Hour |
|  | Loaded simultaneously through all manifolds: |  |  |  |  |  |  |  | 6,000 Cu. Metres/Hour |
| Cargo Control Room |  |  |  |  |  |  |  |  |  |
| 8.7 | Is ship fitted with a Cargo Control Room (CCR)? |  |  |  |  |  |  | Yes |  |
| 8.8 | Can tank innage/ullage be read from the CCR? |  |  |  |  |  |  | Yes |  |
| Gauging and Sampling |  |  |  |  |  |  |  |  |  |
| 8.9 | Is gauging system certified and calibrated? If no, specify which ones are not calibrated: |  |  |  |  |  |  | Yes, |  |
|  | What type of fixed closed tank gauging system is fitted: |  |  |  |  |  |  | Radar |  |
|  | Are high level alarms fitted to the cargo tanks? If Yes, indicate whether to all tanks or partial: |  |  |  |  |  |  | Yes, All |  |
| 8.9.1 | Can cargo be transferred under closed loading conditions in accordance with ISGOTT 11.1.6.6? |  |  |  |  |  |  | Yes |  |
| 8.9.2 | Are cargo tanks fitted with multipoint gauging? If yes, specify type and locations: |  |  |  |  |  |  | No, |  |
| 8.10 | Number of portable gauging units (example- MMC) on board: |  |  |  |  |  |  | 3 |  |
| Vapor Emission Control System (VECS) |  |  |  |  |  |  |  |  |  |
| 8.11 | Is a vapour return system (VRS) fitted? |  |  |  |  |  |  | Yes |  |
| 8.12 | Number/size of VECS manifolds (per side): |  |  |  |  |  |  | 2 | 400 Millimetres |
| 8.13 | Number/size/type of VECS reducers: |  |  |  |  |  |  | 400 x 300 2 pcs 400 x 200 1 pc |  |
| Venting |  |  |  |  |  |  |  |  |  |
| 8.14 | State what type of venting system is fitted: |  |  |  |  |  |  | Common line, P/Vs and pressure gauges |  |
| Cargo Manifolds and Reducers |  |  |  |  |  |  |  |  |  |
| 8.15 | Total number/size of cargo manifold connections on each side: |  |  |  |  |  |  | 3/400 Millimetres |  |
| 8.16 | What type of valves are fitted at manifold: |  |  |  |  |  |  | Butterfly |  |
| 8.17 | What is the material/rating of the manifold: |  |  |  |  |  |  | steel/ANSI B16.5 150LB |  |
| 8.17.1 | Does vessel comply with the latest edition of the OCIMF 'Recommendations for Oil Tanker Manifolds and Associated Equipment’? |  |  |  |  |  |  | Yes |  |
| 8.18 | Distance between cargo manifold centers: |  |  |  |  |  |  | 2,500.00 Millimetres |  |
| 8.19 | Distance ships rail to manifold: |  |  |  |  |  |  | 4,600.00 Millimetres |  |
| 8.20 | Distance manifold to ships side: |  |  |  |  |  |  | 4,600.00 Millimetres |  |
| 8.21 | Top of rail to center of manifold: |  |  |  |  |  |  | 750.00 Millimetres |  |
| 8.22 | Distance main deck to center of manifold: |  |  |  |  |  |  | 2,100.00 Millimetres |  |
| 8.23 | Spill tank grating to center of manifold: |  |  |  |  |  |  | 900.00 Millimetres |  |
| 8.24 | Manifold height above the waterline in normal ballast/at SDWT condition: |  |  |  |  |  |  | 14.97 Metres | 8.72 Metres |
| 8.25 | Number/size/type of reducers: |  |  |  |  |  |  | 3 x 400/300mm (16/12") 3 x 400/250mm (16/10") 3 x 400/200mm (16/8") 1 x 300/200mm (12/8") 2 x 400/300mm (16/12") // ANSI |  |
| 8.26 | Is vessel fitted with a stern manifold? If yes, state size: |  |  |  |  |  |  | No, |  |
| Heating |  |  |  |  |  |  |  |  |  |
| 8.27 | Cargo/slop tanks fitted with a cargo heating system? |  |  |  |  | Type |  | Coiled | Material |
|  | Cargo Tanks: |  |  |  |  | Steam heating coils |  | Yes | SS |
|  | Slop Tanks: |  |  |  |  | Steam heating coils |  | Yes | Stainless steel |
| 8.28 | Maximum temperature cargo can be loaded/maintained: |  |  |  |  |  |  | 73.9 °C / 165.0 °F | 57.2 °C / 134.96 °F |
| 8.28.1 | Minimum temperature cargo can be loaded/maintained: |  |  |  |  |  |  |  |  |
| Inert Gas and Crude Oil Washing |  |  |  |  |  |  |  |  |  |
| 8.29 | Is an Inert Gas System (IGS) fitted/operational? |  |  |  |  |  |  | Yes/Yes |  |
| 8.29.1 | Is a Crude Oil Washing (COW) installation fitted/operational? |  |  |  |  |  |  | Yes/Yes |  |
| 8.30 | Is IGS supplied by flue gas, inert gas (IG) generator and/or nitrogen: |  |  |  |  |  |  | IG Generator |  |
| Cargo Pumps |  |  |  |  |  |  |  |  |  |
| 8.31 | How many cargo pumps can be run simultaneously at full capacity: |  |  |  |  |  |  | 3 |  |
| 8.32 | Pumps |  |  | No. |  | Type |  | Capacity | At What Head (sg=1.0) |
|  | Cargo Pumps: |  |  | 3 |  | Centrifugal |  | 2000 Cu. Metres/Hour | 130 Meters |
|  | Cargo Eductors: |  |  | 1 |  | Eductor |  | 400 Cu. Metres/Hour | 25 Metres |
|  | Stripping: |  |  | 1 |  | Reciprocating |  | 160 Cu. Metres/Hour | 130 Metres |
| 8.33 | Is at least one emergency portable cargo pump provided? |  |  |  |  |  |  |  |  |
| 9. | MOORING |  |  |  |  |  |  |  |  |
| 9.1 | Wires (on drums) |  | No. | Diameter |  | Material |  | Length | Breaking Strength |
|  | Forecastle: |  | 6 | 28.00 Millimetres |  | Galvanized steel |  | 200.00 Metres | 53.00 Metric Tonnes |
|  | Main deck fwd: |  | 2 | 28.00 Millimetres |  | Galvanized steel |  | 200.00 Metres | 53.00 Metric Tonnes |
|  | Main deck aft: |  | 2 | 28.00 Millimetres |  | Galvanized steel |  | 200.00 Metres | 53.00 Metric Tonnes |
|  | Poop deck: |  | 6 | 28.00 Millimetres |  | Galvaized steel |  | 200.00 Metres | 53.00 Metric Tonnes |
| 9.2 | Wire tails |  | No. | Diameter |  | Material |  | Length | Breaking Strength |
|  | Forecastle: |  | 6 | 64.00 Millimetres |  | Polyester |  | 11.00 Metres | 80 Metric Tonnes |
|  | Main deck fwd: |  | 2 | 64 Millimetres |  | Polyester |  | 11.00 Metres | 80 Metric Tonnes |
|  | Main deck aft: |  | 2 | 64 Millimetres |  | Polyester |  | 11.00 Metres | 80 Metric Tonnes |
|  | Poop deck: |  | 6 | 64.00 Millimetres |  | Polyester |  | 11.00 Metres | 80 Metric Tonnes |
| 9.3 | Ropes (on drums) |  | No. | Diameter |  | Material |  | Length | Breaking Strength |
|  | Forecastle: |  |  |  |  |  |  |  |  |
|  | Main deck fwd: |  |  |  |  |  |  |  |  |
|  | Main deck aft: |  |  |  |  |  |  |  |  |
|  | Poop deck: |  |  |  |  |  |  |  |  |
| 9.4 | Other lines |  | No. | Diameter |  | Material |  | Length | Breaking Strength |
|  | Forecastle: |  | 6 | 64.00 Millimetres |  | Polydacron |  | 220.00 Metres | 79.00 Metric Tonnes |
|  | Main deck fwd: |  | 2 | 72.00 Millimetres |  | Polypropylene |  | 220.00 Metres | 90.60 Metric Tonnes |
|  | Main deck aft: |  | 2 | 64.00 Millimetres |  | Polyproylen |  | 220.00 Metres | 83.60 Metric Tonnes |
|  | Poop deck: |  | 5 | 64.00 Millimetres |  | Polydacron |  | 220.00 Metres | 79.00 Metric Tonnes |
| 9.5 | Winches |  | No. | No. Drums |  | Motive Power |  | Brake Capacity | Type of Brake |
|  | Forecastle: |  | 2 | Triple Drums |  | Hydraulic |  | 46.40 Metric Tonnes | Band Brake |
|  | Main deck fwd: |  | 1 | Double Drums |  | Hydraulic |  | 46.40 Metric Tonnes | Band Brake |
|  | Main deck aft: |  | 1 | Double Drums |  | Hydraulic |  | 46.40 Metric Tonnes | Band Brake |
|  | Poop deck: |  | 2 | Triple Drums |  | Hydraulic |  | 46.40 Metric Tonnes | Band Brake |
| 9.6 | Bitts, closed chocks/fairleads |  |  | No. Bitts |  | SWL Bitts |  | No. Closed Chocks | SWL Closed Chocks |
|  | Forecastle: |  |  | 6 |  | 43 Metric Tonnes |  | 12 | 43 Metric Tonnes (PLUS 2 closed chocks fwd with SWL 200 tons for SPM PLUS 2 Panama chocks fwd with SWL 64 tons for Tugs) |
|  | Main deck fwd: |  |  | 4 |  | 43 Metric Tonnes |  | 6 | 43 Metric Tonnes |
|  | Main deck aft: |  |  | 6 |  | 43 Metric Tonnes |  | 6 | 40 Metric Tonnes |
|  | Poop deck: |  |  | 8 |  | 43 Metric Tonnes (4 of the 8 SETS OF BITTS HAVE SWL 64 tons for Tugs) |  | 12 | 43 Metric Tonnes (PLUS 3 Panama chocks aft with SWL 64 tons for Tugs PLUS 2 Panama chocks aft with SWL 46 tons) |
| Anchors/Emergency Towing System |  |  |  |  |  |  |  |  |  |
| 9.7 | Number of shackles on port/starboard cable: |  |  |  |  |  |  | 12/12 |  |
| 9.8 | Type/SWL of Emergency Towing system forward: |  |  |  |  |  |  | Tanktech KETA-45F | 200 Metric Tonnes |
| 9.9 | Type/SWL of Emergency Towing system aft: |  |  |  |  |  |  | Tanktech KETA-40A | 200 Metric Tonnes |
| 9.10.1 | What is size of closed chock and/or fairleads of enclosed type on stern |  |  |  |  |  |  | 400 x 250 |  |
| Escort Tug |  |  |  |  |  |  |  |  |  |
| 9.10.2 | What is SWL of closed chock and/or fairleads of enclosed type on stern: |  |  |  |  |  |  | 64 Metric Tonnes |  |
| 9.11 | What is SWL of bollard on poop deck suitable for escort tug: |  |  |  |  |  |  | 64 Metric Tonnes |  |
| Lifting Equipment/Gangway |  |  |  |  |  |  |  |  |  |
| 9.12 | Derrick/Crane description (Number, SWL and location): |  |  |  |  |  |  | Cranes: 1 x 15.00 Tonnes Center, serving both manifolds |  |
| 9.13 | Accommodation ladder direction: |  |  |  |  |  |  | Aft |  |
|  | Does vessel have a portable gangway? If yes, state length: |  |  |  |  |  |  | Yes, 15.24 Metres |  |
| Single Point Mooring (SPM) Equipment |  |  |  |  |  |  |  |  |  |
| 9.14 | Does the vessel meet the recommendations in the latest edition of OCIMF ‘Recommendations for Equipment Employed in the Bow Mooring of Conventional Tankers at Single Point Moorings (SPM)’? |  |  |  |  |  |  | Yes |  |
| 9.15 | If fitted, how many chain stoppers: |  |  |  |  |  |  | 2 |  |
| 9.16 | State type/SWL of chain stopper(s): |  |  |  |  |  |  | Tongue | 200.00 Metric Tonnes |
| 9.17 | What is the maximum size chain diameter the bow stopper(s) can handle: |  |  |  |  |  |  | 76.00 Millimetres |  |
| 9.18 | Distance between the bow fairlead and chain stopper/bracket: |  |  |  |  |  |  | 3,500.00 Metres |  |
| 9.19 | Is bow chock and/or fairlead of enclosed type of OCIMF recommended size (600mm x 450mm)? If not, give details of size: |  |  |  |  |  |  | Yes |  |
| 10. | PROPULSION |  |  |  |  |  |  |  |  |
| 10.1 | Speed |  |  |  |  |  |  | Maximum | Economical |
|  | Ballast speed: |  |  |  |  |  |  | 13.50 Knots (WSNP) | 12.50 Knots (WSNP) |
|  | Laden speed: |  |  |  |  |  |  | 13.50 Knots (WSNP) | 12.50 Knots (WSNP) |
| 10.2 | What type of fuel is used for main propulsion/generating plant: |  |  |  |  |  |  | VLSFO or LSMGO | VLSFO or LSMGO |
| 10.3 | Type/Capacity of bunker tanks: |  |  |  |  |  |  | Fuel Oil: 2,245 Cu. Metres Diesel Oil: Gas Oil: 599 Cu. Metres |  |
| 10.4 | Is vessel fitted with fixed or controllable pitch propeller(s): |  |  |  |  |  |  | Fixed |  |
| 10.5 | Engines |  |  |  |  |  | No | Capacity | Make/Type |
|  | Main engine: |  |  |  |  |  | 1 | 12,240 Kilowatt | B&W 6S60MC |
|  | Aux engine: |  |  |  |  |  | 3 | 670 Kilowatt | MAN B&W 6L23/30 |
|  | Power packs: |  |  |  |  |  | n/a |  | n/a |
|  | Boilers: |  |  |  |  |  | 2 | 20 Metric Tonnes/Hour | AALBORG MISSION OL OIL FIRED |
| Bow/Stern Thruster |  |  |  |  |  |  |  |  |  |
| 10.6 | What is brake horse power of bow thruster (if fitted): |  |  |  |  |  |  | No, |  |
| 10.7 | What is brake horse power of stern thruster (if fitted): |  |  |  |  |  |  | No, |  |
| Emissions |  |  |  |  |  |  |  |  |  |
| 10.8 | Main engine IMO NOx emission standard: |  |  |  |  |  |  | Tier I |  |
| 10.9 | Energy Efficiency Design Index (EEDI) rating number: |  |  |  |  |  |  | Not Applicable. vessel built prior 2013 |  |
| 11. | SHIP TO SHIP TRANSFER |  |  |  |  |  |  |  |  |
| 11.1 | Does vessel comply with recommendations contained in OCIMF/ICS Ship To Ship Transfer Guide (Petroleum, Chemicals or Liquified Gas, as applicable)? |  |  |  |  |  |  | Yes |  |
| 11.2 | What is maximum outreach of cranes/derricks outboard of the ship’s side: |  |  |  |  |  |  | 4.38 Metres |  |
| 11.3 | Date/place of last STS operation: |  |  |  |  |  |  | June 10 2020, Off Lome |  |
| 12. | RECENT OPERATIONAL HISTORY |  |  |  |  |  |  |  |  |
| 12.1 | Last three cargoes/charterers/voyages (Last/2nd Last/3rd Last): |  |  |  |  |  |  | Last: Naphtha / Vitol / Tuapse to Yosu. 2nd: Jet + Gasoil + UMS / CSSA / Sikka to Lome + Dakar. 3rd: Gasoline / Vitol / Sikka + Fujairah to Port Qasim |  |
| 12.2 | Has vessel been involved in a pollution, grounding, serious casualty, unscheduled repair or collision incident during the past 12 months? If yes, provide details: |  |  |  |  |  |  | Pollution: No, Grounding: No, Casualty: No, Repair: No, Collision: No, |  |
| 12.3 | Date and place of last Port State Control inspection: |  |  |  |  |  |  | Oct 03, 2019 / Yokohama, Japan |  |
| 12.4 | Any outstanding deficiencies as reported by any Port State Control? If yes, provide details: |  |  |  |  |  |  | No n/a |  |
| 12.5 | Recent Oil company inspections/screenings (To the best of owners knowledge and without guarantee of acceptance for future business)*: * "Approvals" are not given by Oil Majors and ships are accepted for the voyage on a case by case basis. |  |  |  |  |  |  | P66 / SHELL / ENOC / GAZPROM / IDEMITSU / OTHERS VIA SIRE |  |
| 12.6 | Date/Place of last SIRE inspection: |  |  |  |  |  |  | Jun 04, 2020 / Lome |  |
| 12.7 | Additional information relating to features of the ship or operational characteristics: |  |  |  |  |  |  | N/A |  |

Revised 2018 (INTERTANKO/Q88.com)