- INTERTANKO CHARTERING QUESTIONNAIRE 88 - OIL/CHEMICAL	Version 5

| 1. | GENERAL INFORMATION |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1.1 | Date updated: |  |  |  |  |  |  | Dec 28, 2020 |  |
| 1.2 | Vessel’s name (IMO number): |  |  |  |  |  |  | Ardmore Sealion (9708203) |  |
| 1.3 | Vessel’s previous name(s) and date(s) of change: |  |  |  |  |  |  | Not Applicable |  |
| 1.4 | Date delivered/Builder (where built): |  |  |  |  |  |  | May 26, 2015/SPP Ship Building Co. Ltd |  |
| 1.5 | Flag/Port of Registry: |  |  |  |  |  |  | Marshall Islands/Majuro |  |
| 1.6 | Call sign/MMSI: |  |  |  |  |  |  | V7JZ3/538006129 |  |
| 1.7 | Vessel’s contact details (satcom/fax/email etc.): |  |  |  |  |  |  | Tel: +1 929 425 3025 (VSAT),+ 88 1677752676 (IOP ) Fax: Email: asealion@skyfile.com |  |
| 1.8 | Type of vessel (as described in Form A or Form B Q1.11 of the IOPPC): |  |  |  |  |  |  | Oil Tanker |  |
| 1.9 | Type of hull: |  |  |  |  |  |  | Double Hull |  |
| Ownership and Operation |  |  |  |  |  |  |  |  |  |
| 1.10 | Registered owner - Full style: |  |  |  |  | Fair Isle Shipco LLC Trust Company Complex, Ajeltake Road, Ajeltake Island Majuro MH96960 Marshall Islands Tel: +353212409500 Fax: +353 212409501 Telex: Not Applicable Email: marine@ardmoreshipping.com Web: www.ardmoreshipping.com |  |  |  |
| 1.11 | Technical operator - Full style: |  |  |  |  | Anglo Ardmore Ship Management Limited 17/F Kingston International Centre 19 Wang Chiu Road, Kowloon Bay, Kowloon, Hong Kong Hong Kong Tel: +852 3940 7000 Fax: +852 2861 2419 Telex: Not Applicable Email: vetting@angloardmore.com Web: NA Company IMO#: 5993395 |  |  |  |
| 1.12 | Commercial operator - Full style: |  |  |  |  | Ardmore MR Pool LLC Trust Company Complex Ajeltake Road Ajeltake Island Majuro Marshall Islands MH 969690 Tel: +65 63299400 Fax: N/A Telex: Not Applicable Email: commercialoperations@ardmoreshipping.com Web: www.ardmoreshipping.com |  |  |  |
| 1.13 | Disponent owner - Full style: |  |  |  |  | Ardmore MR Pool LLC Trust Company Complex Ajeltake Road Ajeltake Island Majuro Marshall Islands MH 969690 Tel: +65 63299400 Fax: N/A Telex: Not Applicable Email: commercialoperations@ardmoreshipping.com Web: www.ardmoreshipping.com |  |  |  |
| Insurance |  |  |  |  |  |  |  |  |  |
| 1.14 | P & I Club - Full Style: |  |  |  |  | WEST OF ENGLAND Tower Bridge Court 226 Tower Bridge Road London SE1 2UP Tel: +44 20 7716 6000 Fax: +44 20 7716 6100 Telex: NA Email: mail@westpandi.com Web: www.westpandi.com |  |  |  |
| 1.15 | P & I Club pollution liability coverage/expiration date: |  |  |  |  |  |  | 1,000,000,000 US$ | Feb 20, 2021 |
| 1.16 | Hull & Machinery insured by - Full Style: (Specify broker or leading underwriter) |  |  |  |  | Dashwood Brewer & Philip Limited, London DBP House, 63 Mark lane, London EC3R 7NQ www.dashwood.co.uk Tel: +44 20 7626 3711 Fax: +44 20 7283 4175 |  |  |  |
| 1.17 | Hull & Machinery insured value/expiration date: |  |  |  |  |  |  | Contact owners for details | Jul 01, 2021 |
| Classification |  |  |  |  |  |  |  |  |  |
| 1.18 | Classification society: |  |  |  |  |  |  | Lloyds Register |  |
| 1.19 | Class notation: |  |  |  |  |  |  | +100A1, Double Hull Oil and Chemical Tanker, Ship Type 2, CSR, ESP, ShipRight (CM, ACS(B)), *IWS, LI, SPM4, +LMC, IGS, UMS, with descriptive notes COW(LR), ETA, Part Higher Tensile Steel, ShipRight (BWMP(S, T), IHM, SERS, SCM). |  |
| 1.20 | Is the vessel subject to any conditions of class, class extensions, outstanding memorandums or class recommendations? If yes, give details: |  |  |  |  |  |  | No Nil |  |
| 1.21 | If classification society changed, name of previous and date of change: |  |  |  |  |  |  | , Not Applicable |  |
| 1.22 | Does the vessel have ice class? If yes, state what level: |  |  |  |  |  |  | No, Not Applicable |  |
| 1.23 | Date/place of last dry-dock: |  |  |  |  |  |  | Nov 06, 2020/Singapore |  |
| 1.24 | Date next dry dock due/next annual survey due: |  |  |  |  |  |  | May 25, 2025 | May 25, 2021 |
| 1.25 | Date of last special survey/next special survey due: |  |  |  |  |  |  | Nov 06, 2020 | May 25, 2025 |
| 1.26 | If ship has Condition Assessment Program (CAP), what is the latest overall rating: |  |  |  |  |  |  | No, |  |
| Dimensions |  |  |  |  |  |  |  |  |  |
| 1.27 | Length overall (LOA): |  |  |  |  |  |  | 183.00 Metres |  |
| 1.28 | Length between perpendiculars (LBP): |  |  |  |  |  |  | 174.00 Metres |  |
| 1.29 | Extreme breadth (Beam): |  |  |  |  |  |  | 32.23 Metres |  |
| 1.30 | Moulded depth: |  |  |  |  |  |  | 19.127 Metres |  |
| 1.31 | Keel to masthead (KTM)/ Keel to masthead (KTM) in collapsed condition, if applicable: |  |  |  |  |  |  | 48.45 Metres |  |
| 1.32 | Distance bridge front to center of manifold: |  |  |  |  |  |  | 58.40 Metres |  |
| 1.33 | Bow to center manifold (BCM)/Stern to center manifold (SCM): |  |  |  |  |  |  | 89.90 Metres | 93.10 Metres |
| 1.34 | Parallel body distances |  |  |  |  | Lightship |  | Normal Ballast | Summer Dwt |
|  | # Forward to mid-point manifold: |  |  |  |  | 26.08 Metres |  | 33.74 Metres | 33.88 Metres |
|  | # Aft to mid-point manifold: |  |  |  |  | 23.58 Metres |  | 42.08 Metres | 56.57 Metres |
|  | # Parallel body length: |  |  |  |  | 48.66 Metres |  | 75.82 Metres | 90.45 Metres |
| Tonnages |  |  |  |  |  |  |  |  |  |
| 1.35 | Net Tonnage: |  |  |  |  |  |  | 13,763 |  |
| 1.36 | Gross Tonnage/Reduced Gross Tonnage (if applicable): |  |  |  |  |  |  | 29,737 | 23,009 |
| 1.37 | Suez Canal Tonnage - Gross (SCGT)/Net (SCNT): |  |  |  |  |  |  | 31,223 | 26,631 |
| # 1.38 | Panama Canal Net Tonnage (PCNT): |  |  |  |  |  |  | 24,764 |  |
| # Loadline Information |  |  |  |  |  |  |  |  |  |
| # 1.39 | # Loadline |  |  | # Freeboard |  | # Draft |  | # Deadweight | # Displacement |
|  | # Summer: |  |  | 5.841 Metres |  | 13.286 Metres |  | # 49,999 Metric Tonnes | 60,809.70 Metric Tonnes |
|  | # Winter: |  |  | 6.117 Metres |  | 13.01 Metres |  | 48,550.90 Metric Tonnes | 59,361.60 Metric Tonnes |
|  | # Tropical: |  |  | 5.565 Metres |  | 13.562 Metres |  | 51,444.40 Metric Tonnes | 62,255.10 Metric Tonnes |
|  | # Lightship: |  |  | 16.096 Metres |  | 3.031 Metres |  | - | 10,811 Metric Tonnes |
|  | # Normal Ballast Condition: |  |  | 11.448 Metres |  | 7.679 Metres |  | 21,572 Metric Tonnes | 32,383 Metric Tonnes |
|  | # Segregated Ballast Condition: |  |  | 12.43 Metres |  | 6.67 Metres |  | 17,375 Metric Tonnes | 27,600 Metric Tonnes |
| # 1.40 | # FWA/TPC at summer draft: |  |  |  |  |  |  | 290.00 Millimetres | 52.40 Metric Tonnes |
| # 1.41 | # Does vessel have multiple SDWT? If yes, please provide all assigned loadlines: |  |  |  |  |  |  | Yes 49,999/44,999/39,999/34,999/29,999 |  |
| 1.42 | Constant (excluding fresh water): |  |  |  |  |  |  | 211 Metric Tonnes |  |
| 1.43 | What is the company guidelines for Under Keel Clearance (UKC) for this vessel? |  |  |  |  |  |  | After making allowance for factors like squat, swell, change of water density, list, heel and prevailing weather conditions including other variables such as allowance due to, siltation and reduced depths over pipelines and Ice accretion.: A. While Underway: 1.Pilotage waters, Channels, Fairways, Rivers.: 10% of static draft. 2.Coastal Waters when transiting at distance less than 25NM from coastline : 20% of Max Static Draft. 3.Open Waters When transiting more than or equal to 25 NM from coastline: 50% of static draft. B. While In Port 1. Alongside / Engaged in mooring or un-mooring.: 1.5% of Vessel's Extreme Breadth or 0.3m, whichever is greater. 2. CBM, SBM : not less than 20% of vessels static draft. 3a.At Anchor Unprotected Waters : 20 % of vessels static draft. 3b.At Anchor Protected/ Sheltered Waters : 10 % of vessels static draft. *** If Local requirements (charterers, port, canal authorities) have rules that are stricter than the above criteria then adhere to such higher UKC allowances. |  |
| 1.44 | What is the max height of mast above waterline (air draft) |  |  |  |  |  |  | Full Mast | Collapsed Mast |
|  | Summer deadweight: |  |  |  |  |  |  | 37.09 Metres | 0 Metres |
|  | Normal ballast: |  |  |  |  |  |  | 40.77 Metres | 0 Metres |
|  | Lightship: |  |  |  |  |  |  | 45.42 Metres | 0 Metres |
| 2. | CERTIFICATES |  |  | Issued |  | Last Annual |  | Last Intermediate | Expires |
| 2.1 | Safety Equipment Certificate (SEC): |  |  | Nov 06, 2020 |  |  |  |  | May 25, 2025 |
| 2.2 | Safety Radio Certificate (SRC): |  |  | May 23, 2020 |  |  |  |  | May 25, 2025 |
| 2.3 | Safety Construction Certificate (SCC): |  |  | Nov 06, 2020 |  |  |  |  | May 25, 2025 |
| 2.4 | International Loadline Certificate (ILC): |  |  | Sep 25, 2020 |  |  |  |  | May 25, 2025 |
| 2.5 | International Oil Pollution Prevention Certificate (IOPPC): |  |  | Sep 25, 2020 |  |  |  |  | May 25, 2025 |
| # 2.6 | # International Ship Security Certificate (ISSC): |  |  | # Aug 09, 2020 |  | # Not Applicable |  |  | Oct 12, 2025 |
| # 2.7 | # Maritime Labour Certificate (MLC): |  |  | # Aug 09, 2020 |  | # N/A |  |  | Oct 12, 2025 |
| 2.8 | ISM Safety Management Certificate (SMC): |  |  | Aug 09, 2020 |  | # Not Applicable |  |  | Oct 12, 2025 |
| # 2.9 | # Document of Compliance (DOC): |  |  | # Apr 21, 2020 |  | Sep 09, 2020 |  |  | Oct 23, 2021 |
| 2.10 | USCG Certificate of Compliance(USCGCOC): |  |  | Jan 29, 2019 |  | Not Applicable |  | Not Applicable | Jan 29, 2021 |
| 2.11 | Civil Liability Convention (CLC) 1992 Certificate: |  |  | Feb 14, 2020 |  | N/A |  | N/A | Feb 20, 2021 |
| 2.12 | Civil Liability for Bunker Oil Pollution Damage Convention (CLBC) Certificate: |  |  | Feb 14, 2020 |  | N/A |  | N/A | Feb 20, 2021 |
| # 2.13 | # Liability for the Removal of Wrecks Certificate (WRC): |  |  | # Feb 14, 2020 |  | N/A |  | N/A | Feb 20, 2021 |
| 2.14 | U.S. Certificate of Financial Responsibility (COFR): |  |  | May 28, 2018 |  | N/A |  | N/A | May 28, 2021 |
| 2.15 | Certificate of Class (COC): |  |  | Nov 06, 2020 |  |  |  |  | May 25, 2025 |
| # 2.16 | # International Sewage Pollution Prevention Certificate (ISPPC): |  |  | # May 23, 2020 |  | # N/A |  | # N/A | May 25, 2025 |
| # 2.17 | # Certificate of Fitness (COF): |  |  | # Nov 06, 2020 |  |  |  |  | May 25, 2025 |
| # 2.18 | # International Energy Efficiency Certificate (IEEC): |  |  | # May 26, 2015 |  | N/A |  | N/A | N/A |
| # 2.19 | # International Air Pollution Prevention Certificate (IAPPC): |  |  | # Sep 25, 2020 |  |  |  |  | May 25, 2025 |
| Documentation |  |  |  |  |  |  |  |  |  |
| 2.20 | Owner warrant that vessel is member of ITOPF and will remain so for the entire duration of this voyage/contract: |  |  |  |  |  |  | Yes |  |
| 2.21 | Does vessel have in place a Drug and Alcohol Policy complying with OCIMF guidelines for Control of Drugs and Alcohol Onboard Ship? |  |  |  |  |  |  | Yes |  |
| 2.22 | Is the ITF Special Agreement on board (if applicable)? |  |  |  |  |  |  | Yes |  |
| 2.23 | ITF Blue Card expiry date (if applicable): |  |  |  |  |  |  | May 25, 2021 |  |
| 3. | CREW |  |  |  |  |  |  |  |  |
| 3.1 | Nationality of Master: |  |  |  |  |  |  | Indian |  |
| 3.2 | Number and nationality of Officers: |  |  |  |  | 10 |  | Indian |  |
| 3.3 | Number and nationality of Crew: |  |  |  |  | 9 |  | Indian |  |
| 3.4 | What is the common working language onboard: |  |  |  |  |  |  | English |  |
| 3.5 | Do officers speak and understand English? |  |  |  |  |  |  | Yes |  |
| 3.6 | If Officers/ratings employed by a manning agency - Full style: |  |  | Officers: Anglo Ardmore Ship Management Limited 17/F Kingston International Centre 19 Wang Chiu Road, Kowloon Bay, Kowloon, Hong Kong Tel: +852 3940 7000 Fax: +852 2861 2419 Telex: NA Email: operations@angloardmore.com Web: NA |  |  |  | Ratings: Anglo Ardmore Ship Management Limited 17/F Kingston International Centre 19 Wang Chiu Road, Kowloon Bay, Kowloon, Hong Kong Tel: +852 3940 7000 Fax: +852 2861 2419 Telex: NA Email: operations@angloardmore.com Web: NA |  |
| 4. | FOR USA CALLS |  |  |  |  |  |  |  |  |
| 4.1 | Has the vessel Operator submitted a Vessel Spill Response Plan to the US Coast Guard which has been approved by official USCG letter? |  |  |  |  |  |  | Yes |  |
| 4.2 | Qualified individual (QI) - Full style: |  |  |  |  | O'Brien's Oil Pollution Service O'Brien's Response Management 818 Town and Country Blvd., Suite 200 Houston, TX 77024, USA Tel: +1 (281) 606-4854 Fax: +1 985 781 0580 Telex: 24Hr (alternate): +1 Email: commandcenter@wittobriens.com Web: Not Applicable |  |  |  |
| 4.3 | Oil Spill Response Organization (OSRO) - Full style: |  |  |  |  | National Response Corporation 3500 Sunrise Highway Suite T103 Great River NY 11739 USA Tel: +1-631-224-9141 Fax: +1-631-224-9086 Telex: Not Applicable Email: iocdo@nrcc.com Web: Not Applicable |  |  |  |
| 4.4 | Salvage and Marine Firefighting Services (SMFF) - Full Style: |  |  |  |  | RESOLVE MARINE GROUP INC. 1510 SE 17th Street, Suite 400 Fort Lauderdale, FL 33316 Tel: +1-954-764-8700 Fax: +1-954-764-8724 Email: opa90@resolvemarine.com Web: www.resolvemarine.com |  |  |  |
| 5. | SAFETY/HELICOPTER |  |  |  |  |  |  |  |  |
| 5.1 | Is the vessel operated under a Quality Management System? If Yes, what type of system? (ISO9001 or IMO Resolution A.741(18) as amended): |  |  |  |  |  |  | Yes ISO 9001:2008 |  |
| 5.2 | Can the ship comply with the ICS Helicopter Guidelines? |  |  |  |  |  |  | Yes |  |
| 5.2.1 | If Yes, state whether winching or landing area provided: |  |  |  |  |  |  | Winching |  |
| 5.2.2 | If Yes, what is the diameter of the circle provided: |  |  |  |  |  |  | 5.00 Metres |  |
| 6. | COATING/ANODES |  |  |  |  |  |  |  |  |
| 6.1 | Tank Coating |  |  | Coated |  | Type |  | To What Extent | Anodes |
|  | Cargo tanks: |  |  | Yes |  | Phenolic Epoxy |  | 100% | No |
|  | Ballast tanks: |  |  | Yes |  | Epoxy |  | 100% | Yes |
|  | Slop tanks: |  |  | Yes |  | Phenolic Epoxy |  | Whole Tank | No |
| 7. | BALLAST |  |  |  |  |  |  |  |  |
| 7.1 | Pumps |  |  |  | No. | Type |  | Capacity | At What Head (sg=1.0) |
|  | Ballast Pumps: |  |  |  | 2 | FRAMO SB 300 |  | 750 Cu. Metres/Hour | 30 Metres |
|  | Ballast Eductors: |  |  |  | 1 | Ballast Sea Water Driven |  | 100 Cu. Metres/Hour | 30 Metres |
| 8. | CARGO |  |  |  |  |  |  |  |  |
| Double Hull Vessels |  |  |  |  |  |  |  |  |  |
| 8.1 | Is vessel fitted with centerline bulkhead in all cargo tanks? If Yes, solid or perforated: |  |  |  |  |  |  | Yes, Solid |  |
| Cargo Tank Capacities |  |  |  |  |  |  |  |  |  |
| 8.2 | Number of cargo tanks and total cubic capacity (max% per company policy: 98%, 97%, 96% or 95%) excluding slops tanks: |  |  |  |  |  |  | 12 | 51,734.50 Cu. Metres |
| 8.2.1 | Capacity (max% per company policy: 98%, 97%, 96% or 95%) of each natural segregation with double valve (specify tanks): |  |  |  |  |  |  | As per Company Policy 98% Seg No.1 W - 6143.0 cbm Seg No.2 W - 9057.9 cbm Seg No.3 W - 9446.4 cbm Seg No.4 W - 9441.1 cbm Seg No.5 W - 9391.20 cbm Seg No.6 W - 8254.90 cbm |  |
| 8.2.2 | IMO class (Oil/Chemical Ship Type 1, 2 or 3): |  |  |  |  |  |  | 2,3 |  |
| 8.3 | Number of slop tanks and total cubic capacity (max% per company policy: 98%, 97%, 96% or 95%): |  |  |  |  |  |  | 2 | 1,194.10 Cu. Metres |
| 8.3.1 | Specify segregations which slops tanks belong to and their capacity with double valve: |  |  |  |  |  |  | Seg No.7, Slop wings , 1194.10 cbm |  |
| 8.3.2 | Residual/retention oil tank(s) capacity (98%), if applicable: |  |  |  |  |  |  | 157.90 Cu. Metres |  |
| SBT Vessels |  |  |  |  |  |  |  |  |  |
| 8.3.3 | What is total SBT capacity and percentage of SDWT vessel can maintain? |  |  |  |  |  |  | 22,420 Cu. Metres | 45.96 % |
| 8.3.4 | Does vessel meet the requirements of MARPOL Annex I Reg 18.2: |  |  |  |  |  |  | Yes |  |
| Cargo Handling and Pumping Systems |  |  |  |  |  |  |  |  |  |
| 8.4 | How many grades/products can vessel load/discharge with double valve segregation: |  |  |  |  |  |  | 7 |  |
| 8.4.1 | State type of cargo containment (integral, independent, gravity or pressure tanks): |  |  |  |  |  |  |  |  |
| 8.5 | Are there any cargo tank filling restrictions? If yes, specify number of slack tanks, max s.g., ullage restrictions etc.: |  |  |  |  |  |  | Yes 1) All cargo tanks are designed for 100% filling at 1.025 S.G and there are no sloshing restrictions 2) All cargo tanks are designed for max SG 1.53 under slack loading condition up to 67% of the tank height. 3)The filling restrictions for intermediate SGs shall be in accordance with the following formula => %(percentage of filling height)=(1.025x100)/ X ( where x = intermediate S.G). |  |
| 8.6 | Max loading rate for homogenous cargo |  |  |  |  |  |  | With VECS | Without VECS |
|  | Loaded per manifold connection: |  |  |  |  |  |  | 4,050 Cu. Metres/Hour | 4,050 Cu. Metres/Hour |
|  | Loaded simultaneously through all manifolds: |  |  |  |  |  |  | 4,560 Cu. Metres/Hour | 4,560 Cu. Metres/Hour |
| Cargo Control Room |  |  |  |  |  |  |  |  |  |
| 8.7 | Is ship fitted with a Cargo Control Room (CCR)? |  |  |  |  |  |  | Yes |  |
| 8.8 | Can tank innage/ullage be read from the CCR? |  |  |  |  |  |  | Yes |  |
| Gauging and Sampling |  |  |  |  |  |  |  |  |  |
| 8.9 | Is gauging system certified and calibrated? If no, specify which ones are not calibrated: |  |  |  |  |  |  | Yes, N/A |  |
|  | What type of gauging system as per IBC 13.1 is fitted (Open/Restricted/Closed )? |  |  |  |  |  |  | Closed |  |
|  | What type of fixed closed tank gauging system is fitted: |  |  |  |  |  |  | Radar |  |
|  | Is a tank overflow control system fitted? If yes, then state if system includes automatic closing of valves? |  |  |  |  |  |  | No, N/A |  |
|  | Are high level alarms fitted to the cargo tanks? If Yes, indicate whether to all tanks or partial: |  |  |  |  |  |  | Yes, All |  |
| 8.9.1 | Can cargo be transferred under closed loading conditions in accordance with ISGOTT 11.1.6.6? |  |  |  |  |  |  | Yes |  |
| 8.9.2 | Are cargo tanks fitted with multipoint gauging? If yes, specify type and locations: |  |  |  |  |  |  | Yes, Tanktech - Fwd, Centre & Aft of each Cargo tank |  |
| 8.10 | Number of portable gauging units (example- MMC) on board: |  |  |  |  |  |  | 4 |  |
| Vapor Emission Control System (VECS) |  |  |  |  |  |  |  |  |  |
| 8.11 | Is a vapour return system (VRS) fitted? |  |  |  |  |  |  | Yes |  |
| 8.12 | Number/size of VECS manifolds (per side): |  |  |  |  |  |  | 2 | 300 Millimetres |
| 8.13 | Number/size/type of VECS reducers: |  |  |  |  |  |  | 2 Nos. 300 mm X 400 mm 1 Nos. 300 mm X 200 mm 1 Nos. 300 mm x 150 mm |  |
| Venting |  |  |  |  |  |  |  |  |  |
| 8.14 | State what type of venting system is fitted: |  |  |  |  |  |  | Common Mast Riser, Individual tank high Velocity vents and Individual tank pressure sensors. |  |
| Cargo Manifolds and Reducers |  |  |  |  |  |  |  |  |  |
| 8.15 | Total number/size of cargo manifold connections on each side: |  |  |  |  |  |  | 7/350 Millimetres |  |
| 8.15.1 | Does the vessel have a Common Line Manifold connection? If yes, describe: |  |  |  |  |  |  | All 7 manifold cargo lines fitted with manifold cross overs. |  |
| 8.16 | What type of valves are fitted at manifold: |  |  |  |  |  |  | BUTTERFLY VALVES |  |
| 8.17 | What is the material/rating of the manifold: |  |  |  |  |  |  | STAINLESS STEEL/Not Applicable |  |
| 8.17.1 | Does vessel comply with the latest edition of the OCIMF 'Recommendations for Oil Tanker Manifolds and Associated Equipment’? |  |  |  |  |  |  | Yes |  |
| 8.18 | Distance between cargo manifold centers: |  |  |  |  |  |  | 2,000 Millimetres |  |
| 8.19 | Distance ships rail to manifold: |  |  |  |  |  |  | 4,600 Millimetres |  |
| 8.20 | Distance manifold to ships side: |  |  |  |  |  |  | 4,600 Millimetres |  |
| 8.21 | Top of rail to center of manifold: |  |  |  |  |  |  | 600 Millimetres |  |
| 8.22 | Distance main deck to center of manifold: |  |  |  |  |  |  | 2,100 Millimetres |  |
| 8.23 | Spill tank grating to center of manifold: |  |  |  |  |  |  | 900 Millimetres |  |
| 8.24 | Manifold height above the waterline in normal ballast/at SDWT condition: |  |  |  |  |  |  | 13.548 Metres | 7.941 Metres |
| 8.25 | Number/size/type of reducers: |  |  |  |  |  |  | 6 x 200/350mm (8/14") 6 x 250/350mm (10/14") 6 x 300/350mm (12/14") 12 x 400/350mm (16/14") 1 x 200/300mm (8/12") ANSI |  |
| 8.26 | Is vessel fitted with a stern manifold? If yes, state size: |  |  |  |  |  |  | No, 0 Millimetres |  |
| Heating |  |  |  |  |  |  |  |  |  |
| 8.27 | Cargo/slop tanks fitted with a cargo heating system? |  |  |  |  | Type |  | Coiled | Material |
|  | Cargo Tanks: |  |  |  |  | Heating Coils |  | Yes | SS |
|  | Slop Tanks: |  |  |  |  | Heating Coils |  | Yes | Stainless Steel |
| 8.27.1 | Is a Thermal Oil Heating system fitted? If yes, identify tanks? |  |  |  |  |  |  | , |  |
| 8.28 | Maximum temperature cargo can be loaded/maintained: |  |  |  |  |  |  | 80.0 °C / 176.0 °F | 70 °C / 158 °F |
| 8.28.1 | Minimum temperature cargo can be loaded/maintained: |  |  |  |  |  |  | 15.0 °C / 59.0 °F | 15.0 °C / 59.0 °F |
| Inert Gas and Crude Oil Washing |  |  |  |  |  |  |  |  |  |
| 8.29 | Is an Inert Gas System (IGS) fitted/operational? |  |  |  |  |  |  | Yes/Yes |  |
| 8.29.1 | Is a Crude Oil Washing (COW) installation fitted/operational? |  |  |  |  |  |  | Yes/Yes |  |
| 8.30 | Is IGS supplied by flue gas, inert gas (IG) generator and/or nitrogen: |  |  |  |  |  |  | IG Generator |  |
| 8.30.1 | If nitrogen generator, specify the applicable flow rate for each of the designed purity modes: |  |  |  |  |  |  |  |  |
| Cargo Pumps |  |  |  |  |  |  |  |  |  |
| 8.31 | How many cargo pumps can be run simultaneously at full capacity: |  |  |  |  |  |  | 6 |  |
| 8.32 | Pumps |  |  | No. |  | Type |  | Capacity | At What Head (sg=1.0) |
|  | Cargo Pumps: |  |  | 12 2 1 |  | Deepwell Submerged Deepwell Submerged Deepwell Submerged |  | 600 M3/HR 300 M3/HR 100 M3/HR | 125 Meters 125 Meters 125 Meters |
|  | Cargo Eductors: |  |  | 0 |  | 0 |  | 0 Cu. Metres/Hour | 0 Metres |
|  | Stripping: |  |  |  |  | 0 |  | 0 Cu. Metres/Hour | 0 Metres |
| 8.33 | Is at least one emergency portable cargo pump provided? |  |  |  |  |  |  | Yes |  |
| Tank Cleaning Systems |  |  |  |  |  |  |  |  |  |
| 8.34 | Is tank cleaning equipment fixed in cargo tanks? |  |  |  |  |  |  | Yes |  |
| 8.35 | Is portable tank cleaning equipment provided? |  |  |  |  |  |  | Yes |  |
| 8.36 | Tank washing pump capacity: |  |  |  |  |  |  | 120 Cu. Metres/Hour |  |
| 8.37 | Is a washing water heater fitted? If yes is it operational and state max washing water temperature: |  |  |  |  |  |  | Yes, 75 Degrees Celsius |  |
| 8.38 | What is the maximum number of machines that can be operated at their designed max pressure? |  |  |  |  |  |  | 4 |  |
| Other Deck Equipment |  |  |  |  |  |  |  |  |  |
| 8.39 | Is vessel fitted with a remote cargo tank temperature monitoring system. If yes, is it operational? |  |  |  |  |  |  | Yes, Yes |  |
| 8.40 | Is vessel fitted with a remote cargo tank pressure monitoring system. If yes, is it operational? |  |  |  |  |  |  | Yes, Yes |  |
| 8.41 | Is vessel fitted with a cargo tank drier. If yes is it operational and state capacity: |  |  |  |  |  |  | N/A, N/A |  |
| 8.42 | Is vessel fitted with a cargo cooling system. If yes is it operational and state tanks applicable: |  |  |  |  |  |  | N/A, N/A |  |
| 8.43 | Is steam available on deck? |  |  |  |  |  |  | Yes |  |
| 9. | MOORING |  |  |  |  |  |  |  |  |
| 9.1 | Wires (on drums) |  | No. | Diameter |  | Material |  | Length | Breaking Strength |
|  | Forecastle: |  | 0 | 0 Millimetres |  | 0 |  | 0 Metres | 0 Metric Tonnes |
|  | Main deck fwd: |  | 0 | 0 Millimetres |  | 0 |  | 0 Metres | 0 Metric Tonnes |
|  | Main deck aft: |  | 0 | 0 Millimetres |  | 0 |  | 0 Metres | 0 Metric Tonnes |
|  | Poop deck: |  | 0 | 0 Millimetres |  | 0 |  | 0 Metres | 0 Metric Tonnes |
| 9.2 | Wire tails |  | No. | Diameter |  | Material |  | Length | Breaking Strength |
|  | Forecastle: |  | 0 | 0 Millimetres |  | 0 |  | 0 Metres | 0 Metric Tonnes |
|  | Main deck fwd: |  | 0 | 0 Millimetres |  | 0 |  | 0 Metres | 0 Metric Tonnes |
|  | Main deck aft: |  | 0 | 0 Millimetres |  | 0 |  | 0 Metres | 0 Metric Tonnes |
|  | Poop deck: |  | 0 | 0 Millimetres |  | 0 |  | 0 Metres | 0 Metric Tonnes |
| 9.3 | Ropes (on drums) |  | No. | Diameter |  | Material |  | Length | Breaking Strength |
|  | Forecastle: |  | 4 | 56 Millimetres |  | HBRFLEX (polypropylene/Polyester) |  | 220 Metres | 67.70 Metric Tonnes |
|  | Main deck fwd: |  | 4 | 62 Millimetres |  | WSS SBA (Mixed Polyolefins) |  | 220 Metres | 67 Metric Tonnes |
|  | Main deck aft: |  | 2 | 62 Millimetres |  | WSS SBA (Mixed Polyolefins) |  | 220 Metres | 67 Metric Tonnes |
|  | Poop deck: |  | 6 | 56 Millimetres |  | HBR flex + Superflex (polypropylene/Polyester) |  | 220 Metres | 67.70 Metric Tonnes |
| 9.4 | Other lines |  | No. | Diameter |  | Material |  | Length | Breaking Strength |
|  | Forecastle: |  | 1 | 62 Millimetres |  | WSS SBA (Mixed Polyolefins) |  | 220 Metres | 67 Metric Tonnes |
|  | Main deck fwd: |  | 1 | 56 Millimetres |  | Dongyang Magnaro (Polyester + polyolefin) |  | 220 Metres | 67 Metric Tonnes |
|  | Main deck aft: |  | 0 | 0 Millimetres |  | 0 |  | 0 Metres | 0 Metric Tonnes |
|  | Poop deck: |  | 2 | 56 Millimetres |  | Dongyang Magnaro (Polyester + polyolefin) |  | 220 Metres | 67.70 Metric Tonnes |
| 9.5 | Winches |  | No. | No. Drums |  | Motive Power |  | Brake Capacity | Type of Brake |
|  | Forecastle: |  | 2 | DOUBLE DRUM |  | Hydraulic |  | 53.60 Metric Tonnes (Brake Rendering Capacity adjusted to- 40.2 T) | Manual Spring Loaded |
|  | Main deck fwd: |  | 2 | DOUBLE DRUM |  | Hydraulic |  | 53.60 Metric Tonnes (Brake Rendering Capacity adjusted to- 40.2 T) | Manual Spring Loaded |
|  | Main deck aft: |  | 1 | DOUBLE DRUM |  | Hydraulic |  | 53.60 Metric Tonnes (Brake Rendering Capacity adjusted to- 40.2 T) | Manual Spring Loaded |
|  | Poop deck: |  | 2 | TRIPLE DRUM |  | Hydraulic |  | 53.60 Metric Tonnes (Brake Rendering Capacity adjusted to- 40.2 T) | Manual Spring Loaded |
| 9.6 | Bitts, closed chocks/fairleads |  |  | No. Bitts |  | SWL Bitts |  | No. Closed Chocks | SWL Closed Chocks |
|  | Forecastle: |  |  | 6 |  | 92 Metric Tonnes |  | 8 | 67 Metric Tonnes (6 X 67 Tons &amp; 2 X 92 Tons) |
|  | Main deck fwd: |  |  | 4 |  | 64 Metric Tonnes |  | 16 | 64 Metric Tonnes |
|  | Main deck aft: |  |  | 4 |  | 92 Metric Tonnes (2 X 92 Tons &amp; 2 X 67 Tons) |  | 8 | 67 Metric Tonnes (6 X 67 Tons &amp; 2 X 92 Tons) |
|  | Poop deck: |  |  | 6 |  | 64 Metric Tonnes (6 X 64 Tons &amp; 2 X 67 Tons) |  | 18 | 67 Metric Tonnes (6 X 64 Tons &amp; 12 X 67 Tons) |
| Anchors/Emergency Towing System |  |  |  |  |  |  |  |  |  |
| 9.7 | Number of shackles on port/starboard cable: |  |  |  |  |  |  | 11/12 |  |
| 9.8 | Type/SWL of Emergency Towing system forward: |  |  |  |  |  |  | Tongue Type O-SM200F | 204 Metric Tonnes |
| 9.9 | Type/SWL of Emergency Towing system aft: |  |  |  |  |  |  | Reel Type C-SM200A | 204 Metric Tonnes |
| 9.10.1 | What is size of closed chock and/or fairleads of enclosed type on stern |  |  |  |  |  |  | 1100 mm X 500 mm X 1100 mm |  |
| Escort Tug |  |  |  |  |  |  |  |  |  |
| 9.10.2 | What is SWL of closed chock and/or fairleads of enclosed type on stern: |  |  |  |  |  |  | 204 Metric Tonnes |  |
| 9.11 | What is SWL of bollard on poop deck suitable for escort tug: |  |  |  |  |  |  | 204 Metric Tonnes |  |
| Lifting Equipment/Gangway |  |  |  |  |  |  |  |  |  |
| 9.12 | Derrick/Crane description (Number, SWL and location): |  |  |  |  |  |  | Cranes: 1 x 10.00 Tonnes Centre |  |
| 9.13 | Accommodation ladder direction: |  |  |  |  |  |  | Aft |  |
|  | Does vessel have a portable gangway? If yes, state length: |  |  |  |  |  |  | Yes, 15.00 Metres |  |
| Single Point Mooring (SPM) Equipment |  |  |  |  |  |  |  |  |  |
| 9.14 | Does the vessel meet the recommendations in the latest edition of OCIMF 'Recommendations for Equipment Employed in the Bow Mooring of Conventional Tankers at Single Point Moorings (SPM)’:? |  |  |  |  |  |  | Yes |  |
| 9.15 | If fitted, how many chain stoppers: |  |  |  |  |  |  | 1 |  |
| 9.16 | State type/SWL of chain stopper(s): |  |  |  |  |  |  | Tongue type- O- SM 200F | 204 Metric Tonnes |
| 9.17 | What is the maximum size chain diameter the bow stopper(s) can handle: |  |  |  |  |  |  | 76 Millimetres |  |
| 9.18 | Distance between the bow fairlead and chain stopper/bracket: |  |  |  |  |  |  | 3 Metres |  |
| 9.19 | Is bow chock and/or fairlead of enclosed type of OCIMF recommended size (600mm x 450mm)? If not, give details of size: |  |  |  |  |  |  | Yes NA |  |
| 10. | PROPULSION |  |  |  |  |  |  |  |  |
| 10.1 | Speed |  |  |  |  |  |  | Maximum | Economical |
|  | Ballast speed: |  |  |  |  |  |  | Contact Owners for Details | Contact Owners for Details |
|  | Laden speed: |  |  |  |  |  |  | Contact Owners for Details | Contact Owners for Details |
| 10.2 | What type of fuel is used for main propulsion/generating plant: |  |  |  |  |  |  | VLSFO and LS FO (MGO) | VLSFO and LS FO (MGO) |
| 10.3 | Type/Capacity of bunker tanks: |  |  |  |  |  |  | Fuel Oil: 903.30 Cu. Metres Diesel Oil: 0 Cu. Metres Gas Oil: 390.80 Cu. Metres |  |
| 10.4 | Is vessel fitted with fixed or controllable pitch propeller(s): |  |  |  |  |  |  | Fixed |  |
| 10.5 | Engines |  |  |  |  |  | No | Capacity | Make/Type |
|  | Main engine: |  |  |  |  |  | 1 | 7,240 Kilowatt | STX = MAN B&W 6S50ME-B9-2 |
|  | Aux engine: |  |  |  |  |  | 3 | 900 Kilowatt | YANMAR = 6EY22ALW |
|  | Power packs: |  |  |  |  |  | 4 Sets (2 x Engine Driven & 2 x Electric Driven) | 3.224 Cu. Metres/Hour | FRAMO Cummins Diesel Engines |
|  | Boilers: |  |  |  |  |  | 2 | 19.20 Metric Tonnes/Hour | SPP Shipbuilding MW-18T & SPP Shipbuilding MC-1.2T |
| Bow/Stern Thruster |  |  |  |  |  |  |  |  |  |
| 10.6 | What is brake horse power of bow thruster (if fitted): |  |  |  |  |  |  | No, 0 bhp |  |
| 10.7 | What is brake horse power of stern thruster (if fitted): |  |  |  |  |  |  | No, 0 bhp |  |
| Emissions |  |  |  |  |  |  |  |  |  |
| 10.8 | Main engine IMO NOx emission standard: |  |  |  |  |  |  | Tier II |  |
| 10.9 | Energy Efficiency Design Index (EEDI) rating number: |  |  |  |  |  |  | Attained EEDI = 4.2189 |  |
| 11. | SHIP TO SHIP TRANSFER |  |  |  |  |  |  |  |  |
| 11.1 | Does vessel comply with recommendations contained in OCIMF/ICS Ship To Ship Transfer Guide (Petroleum, Chemicals or Liquified Gas, as applicable)? |  |  |  |  |  |  | Yes |  |
| 11.2 | What is maximum outreach of cranes/derricks outboard of the ship’s side: |  |  |  |  |  |  | 9 Metres |  |
| 11.3 | Date/place of last STS operation: |  |  |  |  |  |  | 09 July 2020, Tanjung Pelepas STS,Malaysia |  |
| 12. | RECENT OPERATIONAL HISTORY |  |  |  |  |  |  |  |  |
| 12.1 | Last three cargoes/charterers/voyages (Last/2nd Last/3rd Last): |  |  |  |  |  |  | 1st Last : JET A1 / Petrochina / Mailiao, Taiwan / Hong Kong 2nd Last : Naphtha / Petco / Kerteh, Malaysia /Pengerang, Singapore, Hong Kong 3rd Last : Gasoil 10 ppm/ BP / Muara, Brunei /Port Hedland - Australia |  |
| 12.2 | Has vessel been involved in a pollution, grounding, serious casualty, unscheduled repair or collision incident during the past 12 months? If yes, provide details: |  |  |  |  |  |  | Pollution: No, Nil Grounding: No, Nil Casualty: No, Nil Repair: No, Collision: No, Nil |  |
| 12.3 | Date and place of last Port State Control inspection: |  |  |  |  |  |  | Nov 27, 2020 / Port Hedland, Australia |  |
| 12.4 | Any outstanding deficiencies as reported by any Port State Control? If yes, provide details: |  |  |  |  |  |  | No Nil |  |
| 12.5 | Recent Oil company inspections/screenings (To the best of owners knowledge and without guarantee of acceptance for future business)*: * "Approvals" are not given by Oil Majors and ships are accepted for the voyage on a case by case basis. |  |  |  |  |  |  | MAXCOM Petroli Spa |  |
| 12.6 | Date/Place of last SIRE inspection: |  |  |  |  |  |  | Sep 30, 2020 / New Zealand Wellington [NZWLG] |  |
| 12.6.1 | Date/Place of last CDI inspection: |  |  |  |  |  |  | N/A |  |
| 12.7 | Additional information relating to features of the ship or operational characteristics: |  |  |  |  |  |  | Nil |  |

Revised 2018 (INTERTANKO/Q88.com)

Form completed on http://www.q88.com/integration.aspx  Please email support@q88.com an updated copy if this is not the latest version.