- INTERTANKO CHARTERING QUESTIONNAIRE 88 - OIL 	Version 5

| 1. | GENERAL INFORMATION |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1.1 | Date updated: |  |  |  |  |  |  | Apr 28, 2021 |  |
| 1.2 | Vessel’s name (IMO number): |  |  |  |  |  |  | Maersk Kara (9374428) |  |
| 1.3 | Vessel’s previous name(s) and date(s) of change: |  |  |  |  |  |  | Saffo (Nov 23, 2015) |  |
| 1.4 | Date delivered/Builder (where built): |  |  |  |  |  |  | Jan 03, 2008/Guangzhou Shipyard International, China |  |
| 1.5 | Flag/Port of Registry: |  |  |  |  |  |  | Singapore/Singapore |  |
| 1.6 | Call sign/MMSI: |  |  |  |  |  |  | 9V3814/565186000 |  |
| 1.7 | Vessel’s contact details (satcom/fax/email etc.): |  |  |  |  |  |  | Tel: +870 773935369 Fax: +870 783929929 Email: master@kara.maersktankers.com |  |
| 1.8 | Type of vessel (as described in Form A or Form B Q1.11 of the IOPPC): |  |  |  |  |  |  | Oil Tanker |  |
| 1.9 | Type of hull: |  |  |  |  |  |  | Double Hull |  |
| Ownership and Operation |  |  |  |  |  |  |  |  |  |
| 1.10 | Registered owner - Full style: |  |  |  |  | Maersk Tankers Singapore Pte Ltd 39 Robinson Road, #15-01, Robinson Point, Singapore 068911 Singapore Tel: +45 23296858 Fax: Not Applicable Telex: Not Applicable Email: marine@maersktankers.com Web: www.maersktankers.com |  |  |  |
| 1.11 | Technical operator - Full style: |  |  |  |  | Maersk Tankers A/S Holmbladsgade 133, 2300 Copenhagen S Denmark Tel: +45 23296858 Fax: Not Applicable Telex: Not Applicable Email: marine@maersktankers.com Web: www.maersktankers.com Company IMO#: 5638479 |  |  |  |
| 1.12 | Commercial operator - Full style: |  |  |  |  | Handytankers K/S as agents to (disponent) Owners Holmbladsgade 133, 2300 Copenhagen S Denmark Tel: +91 2262 600 222 Fax: Not Applicable Telex: Not Applicable Email: operations@maersktankers.com Web: www.maersktankers.com |  |  |  |
| 1.13 | Disponent owner - Full style: |  |  |  |  | NA NA Tel: NA Fax: N/A Telex: NA Email: N/A Web: NA |  |  |  |
| Insurance |  |  |  |  |  |  |  |  |  |
| 1.14 | P & I Club - Full Style: |  |  |  |  | GARD P&I (Bermuda Ltd. Norwegian Branch Kittelsbuktveien 31 4836 Arendal Norway Tel: +47 90 52 41 00 Fax: +47 37 02 48 10 Email: companymail@gard.no Web: http://www.gard.no |  |  |  |
| 1.15 | P & I Club pollution liability coverage/expiration date: |  |  |  |  |  |  | 1,000,000,000 US$ | Feb 20, 2022 |
| 1.16 | Hull & Machinery insured by - Full Style: (Specify broker or leading underwriter) |  |  |  |  | Gard AS PO BOX 31 NO 4809 Arendal Norway Tel: +47 37 01 91 00 Fax: +47 37 02 48 10 |  |  |  |
| 1.17 | Hull & Machinery insured value/expiration date: |  |  |  |  |  |  | 12,250,000 US$ | Dec 31, 2021 |
| Classification |  |  |  |  |  |  |  |  |  |
| 1.18 | Classification society: |  |  |  |  |  |  | Lloyds Register |  |
| 1.19 | Class notation: |  |  |  |  |  |  | I HULL MACH Oil tanker ESP; Chemical tanker ESP Unrestricted Navigation VeriSTAR-HULL AUT-UMS , AUT-PORT, MON-SHAFT, INWATERSURVEY, SPM, VCS-TRANSFER, IHM-EU |  |
| 1.20 | Is the vessel subject to any conditions of class, class extensions, outstanding memorandums or class recommendations? If yes, give details: |  |  |  |  |  |  | No NA |  |
| 1.21 | If classification society changed, name of previous and date of change: |  |  |  |  |  |  | Bureau Veritas, Apr 25, 2017 |  |
| 1.22 | Does the vessel have ice class? If yes, state what level: |  |  |  |  |  |  | No, N/A |  |
| 1.23 | Date/place of last dry-dock: |  |  |  |  |  |  | Dec 23, 2017/Singapore |  |
| 1.24 | Date next dry dock due/next annual survey due: |  |  |  |  |  |  | Jan 02, 2023 | Jan 02, 2022 |
| 1.25 | Date of last special survey/next special survey due: |  |  |  |  |  |  | Dec 22, 2017 | Jan 02, 2023 |
| 1.26 | If ship has Condition Assessment Program (CAP), what is the latest overall rating: |  |  |  |  |  |  | No, No |  |
| Dimensions |  |  |  |  |  |  |  |  |  |
| 1.27 | Length overall (LOA): |  |  |  |  |  |  | 182.86 Metres |  |
| 1.28 | Length between perpendiculars (LBP): |  |  |  |  |  |  | 174.50 Metres |  |
| 1.29 | Extreme breadth (Beam): |  |  |  |  |  |  | 27.40 Metres |  |
| 1.30 | Moulded depth: |  |  |  |  |  |  | 16.80 Metres |  |
| 1.31 | Keel to masthead (KTM)/ Keel to masthead (KTM) in collapsed condition, if applicable: |  |  |  |  |  |  | 44.00 Metres |  |
| 1.32 | Distance bridge front to center of manifold: |  |  |  |  |  |  | 59.00 Metres |  |
| 1.33 | Bow to center manifold (BCM)/Stern to center manifold (SCM): |  |  |  |  |  |  | 89.80 Metres | 93.20 Metres |
| 1.34 | Parallel body distances |  |  |  |  | Lightship |  | Normal Ballast | Summer Dwt |
|  | # Forward to mid-point manifold: |  |  |  |  | 33.80 Metres |  | 47.20 Metres | 52.00 Metres |
|  | # Aft to mid-point manifold: |  |  |  |  | 37.40 Metres |  | 47.60 Metres | 62.60 Metres |
|  | # Parallel body length: |  |  |  |  | 71.20 Metres |  | 94.80 Metres | 114.60 Metres |
| Tonnages |  |  |  |  |  |  |  |  |  |
| 1.35 | Net Tonnage: |  |  |  |  |  |  | 11,118.00 |  |
| 1.36 | Gross Tonnage/Reduced Gross Tonnage (if applicable): |  |  |  |  |  |  | 24,112.00 | 18,898 |
| 1.37 | Suez Canal Tonnage - Gross (SCGT)/Net (SCNT): |  |  |  |  |  |  | 25,628.64 | 21,646.27 |
| # 1.38 | Panama Canal Net Tonnage (PCNT): |  |  |  |  |  |  | 20,200.00 |  |
| # Loadline Information |  |  |  |  |  |  |  |  |  |
| # 1.39 | # Loadline |  |  | # Freeboard |  | # Draft |  | # Deadweight | # Displacement |
|  | # Summer: |  |  | 5.21 Metres |  | 11.62 Metres |  | # 38,395.86 Metric Tonnes | 47,849.40 Metric Tonnes |
|  | # Winter: |  |  | 5.455 Metres |  | 11.375 Metres |  | 37,274.66 Metric Tonnes | 46,728.20 Metric Tonnes |
|  | # Tropical: |  |  | 4.971 Metres |  | 11.859 Metres |  | 39,519.96 Metric Tonnes | 48,973.50 Metric Tonnes |
|  | # Lightship: |  |  | 14.05 Metres |  | 2.76 Metres |  | - | 9,453.54 Metric Tonnes |
|  | # Normal Ballast Condition: |  |  | 10.09 Metres |  | 6.75 Metres |  | 16,705.00 Metric Tonnes | 26,158.00 Metric Tonnes |
|  | # Segregated Ballast Condition: |  |  | 9.84 Metres |  | 6.98 Metres |  | 17,749.00 Metric Tonnes | 27,203.00 Metric Tonnes |
| # 1.40 | # FWA/TPC at summer draft: |  |  |  |  |  |  | 258.00 Millimetres | 46.51 Metric Tonnes |
| # 1.41 | # Does vessel have multiple SDWT? If yes, please provide all assigned loadlines: |  |  |  |  |  |  | Yes FB S=7.041m,DWT=29,999,Dis=39,452.5/MT FB S=5.948m,DWT=34,999,Dis=44,452,5/MT FB S=5.213m,DWT=38,395,Dis=47,849.4/MT |  |
| 1.42 | Constant (excluding fresh water): |  |  |  |  |  |  | 250 Metric Tonnes |  |
| 1.43 | What is the company guidelines for Under Keel Clearance (UKC) for this vessel? |  |  |  |  |  |  | 1. When alongside, at anchor or at SBM/CBM within a protected harbour area. Min. UKC to apply is 0.30 m or allowance for one degree list, whichever is greater. 2. Approaches, Anchorages and SBM/CBM outside harbour areas, Confined waters, and buoyed channels. Min. UKC should be 0.90 m or allowance for three degrees list; whichever is greater. 3. Deep Sea, while underway during open sea navigation , Min. UKC should be 50% of the vessels draught or a minimum of 3.5 m., whichever is the greater. |  |
| 1.44 | What is the max height of mast above waterline (air draft) |  |  |  |  |  |  | Full Mast | Collapsed Mast |
|  | Summer deadweight: |  |  |  |  |  |  | 32.38 Metres | 0 Metres |
|  | Normal ballast: |  |  |  |  |  |  | 37.00 Metres | 0 Metres |
|  | Lightship: |  |  |  |  |  |  | 41.24 Metres | 0 Metres |
| 2. | CERTIFICATES |  |  | Issued |  | Last Annual |  | Last Intermediate | Expires |
| 2.1 | Safety Equipment Certificate (SEC): |  |  | Dec 24, 2019 |  | Jan 31, 2021 |  | Jan 31, 2021 | Jan 02, 2023 |
| 2.2 | Safety Radio Certificate (SRC): |  |  | Dec 10, 2020 |  | Dec 10, 2020 |  | Dec 10, 2020 | Jan 02, 2023 |
| 2.3 | Safety Construction Certificate (SCC): |  |  | Jan 06, 2018 |  | Dec 24, 2019 |  | Feb 03, 2021 | Jan 02, 2023 |
| 2.4 | International Loadline Certificate (ILC): |  |  | Dec 24, 2019 |  | Jan 31, 2021 |  | Jan 31, 2021 | Jan 02, 2023 |
| 2.5 | International Oil Pollution Prevention Certificate (IOPPC): |  |  | Mar 03, 2020 |  | Dec 24, 2019 |  | Jan 31, 2021 | Jan 02, 2023 |
| # 2.6 | # International Ship Security Certificate (ISSC): |  |  | # Feb 09, 2021 |  | # Not Applicable |  |  | Mar 21, 2026 |
| # 2.7 | # Maritime Labour Certificate (MLC): |  |  | # Feb 09, 2021 |  | # N/A |  |  | Mar 21, 2026 |
| 2.8 | ISM Safety Management Certificate (SMC): |  |  | Feb 09, 2021 |  | # Not Applicable |  |  | Mar 21, 2026 |
| # 2.9 | # Document of Compliance (DOC): |  |  | # Nov 06, 2019 |  | Oct 28, 2020 |  |  | Nov 17, 2024 |
| 2.10 | USCG Certificate of Compliance (USCGCOC): |  |  | Jul 07, 2015 |  | Not Applicable |  | Not Applicable | Jul 07, 2017 |
| 2.11 | Civil Liability Convention (CLC) 1992 Certificate: |  |  | Feb 20, 2021 |  | N/A |  | N/A | Feb 20, 2022 |
| 2.12 | Civil Liability for Bunker Oil Pollution Damage Convention (CLBC) Certificate: |  |  | Feb 20, 2021 |  | N/A |  | N/A | Feb 20, 2022 |
| # 2.13 | # Liability for the Removal of Wrecks Certificate (WRC): |  |  | # Feb 20, 2021 |  | N/A |  | N/A | Feb 20, 2022 |
| 2.14 | U.S. Certificate of Financial Responsibility (COFR): |  |  | Sep 28, 2020 |  | N/A |  | N/A | Sep 28, 2023 |
| 2.15 | Certificate of Class (COC): |  |  | Dec 24, 2019 |  | Dec 24, 2019 |  | Jan 31, 2021 | Jan 02, 2023 |
| # 2.16 | # International Sewage Pollution Prevention Certificate (ISPPC): |  |  | # Dec 22, 2017 |  | # N/A |  | # N/A | Jan 02, 2023 |
| # 2.17 | # Certificate of Fitness (COF): |  |  | # Jan 01, 2021 |  | Dec 24, 2019 |  | Jan 31, 2021 | Jan 02, 2023 |
| # 2.18 | # International Energy Efficiency Certificate (IEEC): |  |  | # Dec 24, 2019 |  | N/A |  | N/A | N/A |
| # 2.19 | # International Air Pollution Prevention Certificate (IAPPC): |  |  | # Sep 03, 2020 |  | Dec 24, 2019 |  | Jan 31, 2021 | Jan 02, 2023 |
| Documentation |  |  |  |  |  |  |  |  |  |
| 2.20 | Owner warrant that vessel is member of ITOPF and will remain so for the entire duration of this voyage/contract: |  |  |  |  |  |  | Yes |  |
| 2.21 | Does vessel have in place a Drug and Alcohol Policy complying with OCIMF guidelines for Control of Drugs and Alcohol Onboard Ship? |  |  |  |  |  |  | Yes |  |
| 2.22 | Is the ITF Special Agreement on board (if applicable)? |  |  |  |  |  |  | Yes |  |
| 2.23 | ITF Blue Card expiry date (if applicable): |  |  |  |  |  |  | Jun 30, 2021 |  |
| 3. | CREW |  |  |  |  |  |  |  |  |
| 3.1 | Nationality of Master: |  |  |  |  |  |  | Ukrainian |  |
| 3.2 | Number and nationality of Officers: |  |  |  |  | 9 |  | Indian, Romanian, Ukrainian, Indonesian |  |
| 3.3 | Number and nationality of Crew: |  |  |  |  | 10 |  | Filipino |  |
| 3.4 | What is the common working language onboard: |  |  |  |  |  |  | English |  |
| 3.5 | Do officers speak and understand English? |  |  |  |  |  |  | Yes |  |
| 3.6 | If Officers/ratings employed by a manning agency - Full style: |  |  | Officers: In-house manning Maersk Tankers India Pvt. Ltd. (For Maersk Tankers A/S, As Agents to Owners) 702, 07th Floor ¦ Tower A, 247 Park ¦ LBS Marg ¦ Vikhroli (West) ¦ Mumbai 400 083 ¦ Maharashtra ¦ INDIA Tel: +65 6318 3256 Fax: +65 6223 7191 Email: crew1@maersktankers.com Web: http://www.MaerskTankers.com |  |  |  | Ratings: In-house manning Jebsen Maritime Incorporated Pilgrim Building, 111 Aguirre Street ,Legaspi Village, Makati City Philippines 1229 Tel: +63 9 5279980 Email: marinehrmnl@abojeb.com.ph Web: http://www.maersktankers.com |  |
| 4. | FOR USA CALLS |  |  |  |  |  |  |  |  |
| 4.1 | Has the vessel Operator submitted a Vessel Spill Response Plan to the US Coast Guard which has been approved by official USCG letter? |  |  |  |  |  |  | Yes |  |
| 4.2 | Qualified individual (QI) - Full style: |  |  |  |  | Gallagher Marine Systems Inc 100 Century Pkwy, Mt Laurel, NJ 08054, U.S.A. Tel: +12037616030 Fax: +1 856 642 3945 Email: info@chgms.com Web: http://www.gallaghermarine.com/ |  |  |  |
| 4.3 | Oil Spill Response Organization (OSRO) - Full style: |  |  |  |  | National Response Corporation 3500 Sunrise Hwy South Service Rd, Great River, NY 11739, U.S.A. Tel: +18008994672 Fax: +1-631-224-9082 Email: iocdo@nrcc.com Web: http://nrcc.com/ |  |  |  |
| 4.4 | Salvage and Marine Firefighting Services (SMFF) - Full Style: |  |  |  |  | Donjon-SMIT, LLC 15402 Vantage Pkwy E. Suite 316 Houston, Texas 77032 Tel: +1 703 299 0081 24/7 OPA-90 Emergency or Drill, please dial: +1 206 332 8200 Tel: +1 703 299 0081 Email: Opa90@ardent-americas.com Web: https://www.ardent-americas.com/ |  |  |  |
| 5. | SAFETY/HELICOPTER |  |  |  |  |  |  |  |  |
| 5.1 | Is the vessel operated under a Quality Management System? If Yes, what type of system? (ISO9001 or IMO Resolution A.741(18) as amended): |  |  |  |  |  |  | Yes ISO14001 |  |
| 5.2 | Can the ship comply with the ICS Helicopter Guidelines? |  |  |  |  |  |  | Yes |  |
| 5.2.1 | If Yes, state whether winching or landing area provided: |  |  |  |  |  |  | Winching |  |
| 5.2.2 | If Yes, what is the diameter of the circle provided: |  |  |  |  |  |  | 5.00 Metres |  |
| 6. | COATING/ANODES |  |  |  |  |  |  |  |  |
| 6.1 | Tank Coating |  |  | Coated |  | Type |  | To What Extent | Anodes |
|  | Cargo tanks: |  |  | Yes |  | Pure Epoxy |  | Whole Tank | No |
|  | Ballast tanks: |  |  | Yes |  | Epoxy |  | Whole Tank | Yes |
|  | Slop tanks: |  |  | Yes |  | Epoxy |  | Whole Tank | No |
| 7. | BALLAST |  |  |  |  |  |  |  |  |
| 7.1 | Pumps |  |  |  | No. | Type |  | Capacity | At What Head (sg=1.0) |
|  | Ballast Pumps: |  |  |  | 2 | Centrifugal |  | 1,000 Cu. Metres/Hour | 25 Metres |
|  | Ballast Eductors: |  |  |  | 1 | Other |  | 80 Cu. Metres/Hour |  |
| 8. | CARGO |  |  |  |  |  |  |  |  |
| Double Hull Vessels |  |  |  |  |  |  |  |  |  |
| 8.1 | Is vessel fitted with centerline bulkhead in all cargo tanks? If Yes, solid or perforated: |  |  |  |  |  |  | Yes, Solid |  |
| Cargo Tank Capacities |  |  |  |  |  |  |  |  |  |
| 8.2 | Number of cargo tanks and total cubic capacity (max% per company policy: 98%, 97%, 96% or 95%) excluding slops tanks: |  |  |  |  |  |  | 10 | 43,444.10 Cu. Metres |
| 8.2.1 | Capacity (max% per company policy: 98%, 97%, 96% or 95%) of each natural segregation with double valve (specify tanks): |  |  |  |  |  |  | 1Port & 1 Stbd: 6396cbm(98%) 2Port & 2 stbd: 9663cbm(98%) 3Port & 3 stbd: 7306cbm(98%) 4Port & 4 stbd: 7306cbm(98%) 5Port & 5 stbd: 7291cbm(98%) ((Residue tank, Slop Port & slop stbd: 5781cbm(98%) are also connected to 2W group)) |  |
| 8.2.2 | IMO class (Oil/Chemical Ship Type 1, 2 or 3): |  |  |  |  |  |  | 3 |  |
| 8.3 | Number of slop tanks and total cubic capacity (max% per company policy: 98%, 97%, 96% or 95%): |  |  |  |  |  |  | 2 | 0 Cu. Metres |
| 8.3.1 | Specify segregations which slops tanks belong to and their capacity with double valve: |  |  |  |  |  |  | ROT 306.52 M3 |  |
| 8.3.2 | Residual/retention oil tank(s) capacity (98%), if applicable: |  |  |  |  |  |  | 300.39 Cu. Metres |  |
| SBT Vessels |  |  |  |  |  |  |  |  |  |
| 8.3.3 | What is total SBT capacity and percentage of SDWT vessel can maintain? |  |  |  |  |  |  | 17,733.30 Cu. Metres | 47.00 % |
| 8.3.4 | Does vessel meet the requirements of MARPOL Annex I Reg 18.2: |  |  |  |  |  |  | Yes |  |
| Cargo Handling and Pumping Systems |  |  |  |  |  |  |  |  |  |
| 8.4 | How many grades/products can vessel load/discharge with double valve segregation: |  |  |  |  |  |  | 5 |  |
| 8.5 | Are there any cargo tank filling restrictions? If yes, specify number of slack tanks, max s.g., ullage restrictions etc.: |  |  |  |  |  |  | No Not Applicable |  |
| 8.6 | Max loading rate for homogenous cargo |  |  |  |  |  |  | With VECS | Without VECS |
|  | Loaded per manifold connection: |  |  |  |  |  |  | 2,500 Cu. Metres/Hour | 2,500 Cu. Metres/Hour |
|  | Loaded simultaneously through all manifolds: |  |  |  |  |  |  | 4,500 Cu. Metres/Hour | 4,500.00 Cu. Metres/Hour |
| Cargo Control Room |  |  |  |  |  |  |  |  |  |
| 8.7 | Is ship fitted with a Cargo Control Room (CCR)? |  |  |  |  |  |  | Yes |  |
| 8.8 | Can tank innage/ullage be read from the CCR? |  |  |  |  |  |  | Yes |  |
| Gauging and Sampling |  |  |  |  |  |  |  |  |  |
| 8.9 | Is gauging system certified and calibrated? If no, specify which ones are not calibrated: |  |  |  |  |  |  | Yes, |  |
|  | What type of fixed closed tank gauging system is fitted: |  |  |  |  |  |  | Radar |  |
|  | Are high level alarms fitted to the cargo tanks? If Yes, indicate whether to all tanks or partial: |  |  |  |  |  |  | Yes, All |  |
| 8.9.1 | Can cargo be transferred under closed loading conditions in accordance with ISGOTT 11.1.6.6? |  |  |  |  |  |  | Yes |  |
| 8.9.2 | Are cargo tanks fitted with multipoint gauging? If yes, specify type and locations: |  |  |  |  |  |  | Yes, |  |
| 8.10 | Number of portable gauging units (example- MMC) on board: |  |  |  |  |  |  | 4 |  |
| Vapor Emission Control System (VECS) |  |  |  |  |  |  |  |  |  |
| 8.11 | Is a vapour return system (VRS) fitted? |  |  |  |  |  |  | Yes |  |
| 8.12 | Number/size of VECS manifolds (per side): |  |  |  |  |  |  | 2 | 350 Millimetres |
| 8.13 | Number/size/type of VECS reducers: |  |  |  |  |  |  | 4 - 12" 2 -16" |  |
| Venting |  |  |  |  |  |  |  |  |  |
| 8.14 | State what type of venting system is fitted: |  |  |  |  |  |  | High Velocity PV Valves |  |
| Cargo Manifolds and Reducers |  |  |  |  |  |  |  |  |  |
| 8.15 | Total number/size of cargo manifold connections on each side: |  |  |  |  |  |  | 5/350.00 Millimetres |  |
| 8.16 | What type of valves are fitted at manifold: |  |  |  |  |  |  | Butterfly |  |
| 8.17 | What is the material/rating of the manifold: |  |  |  |  |  |  | ANSI SS (Stainless steel)/ANSI B 16.5 CLASS 150 |  |
| 8.17.1 | Does vessel comply with the latest edition of the OCIMF 'Recommendations for Oil Tanker Manifolds and Associated Equipment’? |  |  |  |  |  |  | Yes |  |
| 8.18 | Distance between cargo manifold centers: |  |  |  |  |  |  | 2,010.00 Millimetres |  |
| 8.19 | Distance ships rail to manifold: |  |  |  |  |  |  | 4,300.00 Millimetres |  |
| 8.20 | Distance manifold to ships side: |  |  |  |  |  |  | 4,600.00 Millimetres |  |
| 8.21 | Top of rail to center of manifold: |  |  |  |  |  |  | 600.00 Millimetres |  |
| 8.22 | Distance main deck to center of manifold: |  |  |  |  |  |  | 1,950.00 Millimetres |  |
| 8.23 | Spill tank grating to center of manifold: |  |  |  |  |  |  | 900.00 Millimetres |  |
| 8.24 | Manifold height above the waterline in normal ballast/at SDWT condition: |  |  |  |  |  |  | 12.20 Metres | 7.30 Metres |
| 8.25 | Number/size/type of reducers: |  |  |  |  |  |  | 10 x 350/400mm (14/16") 4 x 350/300mm (14/12") 4 x 350/250mm (14/10") 4 x 350/200mm (14/8") ANSI |  |
| 8.26 | Is vessel fitted with a stern manifold? If yes, state size: |  |  |  |  |  |  | No, 0.00 Millimetres |  |
| Heating |  |  |  |  |  |  |  |  |  |
| 8.27 | Cargo/slop tanks fitted with a cargo heating system? |  |  |  |  | Type |  | Coiled | Material |
|  | Cargo Tanks: |  |  |  |  | Coils |  | Yes | SS |
|  | Slop Tanks: |  |  |  |  | Coils |  | Yes | SS |
| 8.28 | Maximum temperature cargo can be loaded/maintained: |  |  |  |  |  |  | 75.0 °C / 167.0 °F | 66 °C / 150.8 °F |
| 8.28.1 | Minimum temperature cargo can be loaded/maintained: |  |  |  |  |  |  |  |  |
| Inert Gas and Crude Oil Washing |  |  |  |  |  |  |  |  |  |
| 8.29 | Is an Inert Gas System (IGS) fitted/operational? |  |  |  |  |  |  | Yes/Yes |  |
| 8.29.1 | Is a Crude Oil Washing (COW) installation fitted/operational? |  |  |  |  |  |  | Yes/Yes |  |
| 8.30 | Is IGS supplied by flue gas, inert gas (IG) generator and/or nitrogen: |  |  |  |  |  |  | IG Generator |  |
| Cargo Pumps |  |  |  |  |  |  |  |  |  |
| 8.31 | How many cargo pumps can be run simultaneously at full capacity: |  |  |  |  |  |  | 6 |  |
| 8.32 | Pumps |  |  | No. |  | Type |  | Capacity | At What Head (sg=1.0) |
|  | Cargo Pumps: |  |  | 10 2 1 |  | Marflex Marflex Marflex |  | 540 M3/HR 300 M3/HR 100 M3/HR | 130 Meters 130 Meters 60 Meters |
|  | Cargo Eductors: |  |  | 0 |  |  |  |  |  |
|  | Stripping: |  |  | 0 |  |  |  |  |  |
| 8.33 | Is at least one emergency portable cargo pump provided? |  |  |  |  |  |  | Yes |  |
| 9. | MOORING |  |  |  |  |  |  |  |  |
| 9.1 | Wires (on drums) |  | No. | Diameter |  | Material |  | Length | Breaking Strength |
|  | Forecastle: |  | 0 | 0.00 Millimetres |  |  |  | 0.00 Metres | 0.00 Metric Tonnes |
|  | Main deck fwd: |  | 0 | 0.00 Millimetres |  |  |  | 0.00 Metres | 0.00 Metric Tonnes |
|  | Main deck aft: |  | 0 | 0.00 Millimetres |  |  |  | 0.00 Metres | 0.00 Metric Tonnes |
|  | Poop deck: |  | 0 | 0.00 Millimetres |  |  |  | 0.00 Metres | 0.00 Metric Tonnes |
| 9.2 | Wire tails |  | No. | Diameter |  | Material |  | Length | Breaking Strength |
|  | Forecastle: |  | 0 | 0.00 Millimetres |  |  |  | 0.00 Metres | 0.00 Metric Tonnes |
|  | Main deck fwd: |  | 0 | 0.00 Millimetres |  |  |  | 0.00 Metres | 0.00 Metric Tonnes |
|  | Main deck aft: |  | 0 | 0.00 Millimetres |  |  |  | 0.00 Metres | 0.00 Metric Tonnes |
|  | Poop deck: |  | 0 | 0.00 Millimetres |  |  |  | 0.00 Metres | 0.00 Metric Tonnes |
| 9.3 | Ropes (on drums) |  | No. | Diameter |  | Material |  | Length | Breaking Strength |
|  | Forecastle: |  | 4 | 52.00 Millimetres |  | SignalB5YARN |  | 220.00 Metres | 53.50 Metric Tonnes |
|  | Main deck fwd: |  | 2 | 52.00 Millimetres |  | MAXIFLEX |  | 220.00 Metres | 57.00 Metric Tonnes |
|  | Main deck aft: |  | 2 | 52.00 Millimetres |  | MAXIFLEX |  | 220.00 Metres | 57.00 Metric Tonnes |
|  | Poop deck: |  | 4 | 52.00 Millimetres |  | SignalB5YARN |  | 220.00 Metres | 53.50 Metric Tonnes |
| 9.4 | Other lines |  | No. | Diameter |  | Material |  | Length | Breaking Strength |
|  | Forecastle: |  | 3 | 52.00 Millimetres |  | SignalB5YARN |  | 220.00 Metres | 53.50 Metric Tonnes |
|  | Main deck fwd: |  | 0 |  |  |  |  | 0.00 Metres |  |
|  | Main deck aft: |  | 0 | 0.00 Millimetres |  |  |  | 0.00 Metres | 0.00 Metric Tonnes |
|  | Poop deck: |  | 2 | 52.00 Millimetres |  | SignalB5YARN&KAPAFLOAT |  | 220.00 Metres | 57.00 Metric Tonnes |
| 9.5 | Winches |  | No. | No. Drums |  | Motive Power |  | Brake Capacity | Type of Brake |
|  | Forecastle: |  | 2 | Double Drums |  | Hydraulic |  | 36.80 Metric Tonnes | Friction bands |
|  | Main deck fwd: |  | 1 | Double Drums |  | Hydraulic |  | 36.80 Metric Tonnes | Friction bands |
|  | Main deck aft: |  | 1 | Double Drums |  | Hydraulic |  | 36.80 Metric Tonnes | Friction bands |
|  | Poop deck: |  | 2 | Double Drums |  | Hydraulic |  | 36.80 Metric Tonnes | Friction bands |
| 9.6 | Bitts, closed chocks/fairleads |  |  | No. Bitts |  | SWL Bitts |  | No. Closed Chocks | SWL Closed Chocks |
|  | Forecastle: |  |  | 4 |  | 49 Metric Tonnes |  | 14 | 48 Metric Tonnes |
|  | Main deck fwd: |  |  | 4 |  | 49 Metric Tonnes |  | 8 | 49 Metric Tonnes |
|  | Main deck aft: |  |  | 2 |  | 49 Metric Tonnes |  | 6 | 49 Metric Tonnes |
|  | Poop deck: |  |  | 6 |  | 49 Metric Tonnes |  | 13 | 49 Metric Tonnes |
| Anchors/Emergency Towing System |  |  |  |  |  |  |  |  |  |
| 9.7 | Number of shackles on port/starboard cable: |  |  |  |  |  |  | 11/12 |  |
| 9.8 | Type/SWL of Emergency Towing system forward: |  |  |  |  |  |  | Chain Stopper | 200 Metric Tonnes |
| 9.9 | Type/SWL of Emergency Towing system aft: |  |  |  |  |  |  | ETS | 100 Metric Tonnes |
| 9.10.1 | What is size of closed chock and/or fairleads of enclosed type on stern |  |  |  |  |  |  | Millimetres |  |
| Escort Tug |  |  |  |  |  |  |  |  |  |
| 9.10.2 | What is SWL of closed chock and/or fairleads of enclosed type on stern: |  |  |  |  |  |  | 64.00 Metric Tonnes |  |
| 9.11 | What is SWL of bollard on poop deck suitable for escort tug: |  |  |  |  |  |  | 64.00 Metric Tonnes |  |
| Lifting Equipment/Gangway |  |  |  |  |  |  |  |  |  |
| 9.12 | Derrick/Crane description (Number, SWL and location): |  |  |  |  |  |  | Cranes: 1 x 10.00 Tonnes center |  |
| 9.13 | Accommodation ladder direction: |  |  |  |  |  |  | Parrelel to shipside |  |
|  | Does vessel have a portable gangway? If yes, state length: |  |  |  |  |  |  | Yes, 12.00 Metres |  |
| Single Point Mooring (SPM) Equipment |  |  |  |  |  |  |  |  |  |
| 9.14 | Does the vessel meet the recommendations in the latest edition of OCIMF ‘Recommendations for Equipment Employed in the Bow Mooring of Conventional Tankers at Single Point Moorings (SPM)’? |  |  |  |  |  |  | Yes |  |
| 9.15 | If fitted, how many chain stoppers: |  |  |  |  |  |  | 1 |  |
| 9.16 | State type/SWL of chain stopper(s): |  |  |  |  |  |  | Tongue Type chain stopper | 200.00 Metric Tonnes |
| 9.17 | What is the maximum size chain diameter the bow stopper(s) can handle: |  |  |  |  |  |  | 76.00 Millimetres |  |
| 9.18 | Distance between the bow fairlead and chain stopper/bracket: |  |  |  |  |  |  | 3,200.00 Metres |  |
| 9.19 | Is bow chock and/or fairlead of enclosed type of OCIMF recommended size (600mm x 450mm)? If not, give details of size: |  |  |  |  |  |  | Yes |  |
| 10. | PROPULSION |  |  |  |  |  |  |  |  |
| 10.1 | Speed |  |  |  |  |  |  | Maximum | Economical |
|  | Ballast speed: |  |  |  |  |  |  | 16 Knots (WSNP) | 13 Knots (WSNP) |
|  | Laden speed: |  |  |  |  |  |  | 15.40 Knots (WSNP) | 12.50 Knots (WSNP) |
| 10.2 | What type of fuel is used for main propulsion/generating plant: |  |  |  |  |  |  | IFO | IFO |
| 10.3 | Type/Capacity of bunker tanks: |  |  |  |  |  |  | Fuel Oil: 1,275.42 Cu. Metres Diesel Oil: 143.90 Cu. Metres Gas Oil: 0 Cu. Metres |  |
| 10.4 | Is vessel fitted with fixed or controllable pitch propeller(s): |  |  |  |  |  |  | Fixed |  |
| 10.5 | Engines |  |  |  |  |  | No | Capacity | Make/Type |
|  | Main engine: |  |  |  |  |  | 1 | 9,480 Kilowatt | MAN B&W 6S50MC - C |
|  | Aux engine: |  |  |  |  |  | 3 | 910 Kilowatt | ZJMD-MAN- B&W L23/30H |
|  | Power packs: |  |  |  |  |  | N/A |  | N/A |
|  | Boilers: |  |  |  |  |  | 1 | 18.00 Metric Tonnes/Hour | AALBORG |
| Bow/Stern Thruster |  |  |  |  |  |  |  |  |  |
| 10.6 | What is brake horse power of bow thruster (if fitted): |  |  |  |  |  |  | Yes, 1,020.00 bhp |  |
| 10.7 | What is brake horse power of stern thruster (if fitted): |  |  |  |  |  |  | No, |  |
| Emissions |  |  |  |  |  |  |  |  |  |
| 10.8 | Main engine IMO NOx emission standard: |  |  |  |  |  |  | Not Applicable |  |
| 10.9 | Energy Efficiency Design Index (EEDI) rating number: |  |  |  |  |  |  | N/A |  |
| 11. | SHIP TO SHIP TRANSFER |  |  |  |  |  |  |  |  |
| 11.1 | Does vessel comply with recommendations contained in OCIMF/ICS Ship To Ship Transfer Guide (Petroleum, Chemicals or Liquified Gas, as applicable)? |  |  |  |  |  |  | Yes |  |
| 11.2 | What is maximum outreach of cranes/derricks outboard of the ship’s side: |  |  |  |  |  |  | 4.80 Metres |  |
| 11.3 | Date/place of last STS operation: |  |  |  |  |  |  | 04th June, 2020 / Kavkaz OPL |  |
| 12. | RECENT OPERATIONAL HISTORY |  |  |  |  |  |  |  |  |
| 12.1 | Last three cargoes/charterers/voyages (Last/2nd Last/3rd Last): |  |  |  |  |  |  | Pls see commercial operator for details. L3C FO FO FO |  |
| 12.2 | Has vessel been involved in a pollution, grounding, serious casualty, unscheduled repair or collision incident during the past 12 months? If yes, provide details: |  |  |  |  |  |  | Pollution: No, NA Grounding: No, NA Casualty: No, Repair: No, Not Applicable Collision: No, NA |  |
| 12.3 | Date and place of last Port State Control inspection: |  |  |  |  |  |  | Mar 20, 2020 / Taman |  |
| 12.4 | Any outstanding deficiencies as reported by any Port State Control? If yes, provide details: |  |  |  |  |  |  | No nil |  |
| 12.5 | Recent Oil company inspections/screenings (To the best of owners knowledge and without guarantee of acceptance for future business)*: * "Approvals" are not given by Oil Majors and ships are accepted for the voyage on a case by case basis. |  |  |  |  |  |  | Koch, Vivo Energy, IMT |  |
| 12.6 | Date/Place of last SIRE inspection: |  |  |  |  |  |  | Mar 02, 2021 / Mombasa |  |
| 12.7 | Additional information relating to features of the ship or operational characteristics: |  |  |  |  |  |  | Not Applicable |  |

Revised 2018 (INTERTANKO/Q88.com)

Form completed on http://www.q88.com/integration.aspx  Please email support@q88.com an updated copy if this is not the latest version.