- INTERTANKO CHARTERING QUESTIONNAIRE 88 - OIL/CHEMICAL	Version 5

| 1. | GENERAL INFORMATION |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1.1 | Date updated: |  |  |  |  | 30-06- 2021 |  |
| 1.2 | Vessel’s name (IMO number): |  |  |  |  | STARGAZER (9843675) |  |
| 1.3 | Vessel’s previous name(s) and date(s) of change: |  |  |  |  | NIL |  |
| 1.4 | Date delivered/Builder (where built): |  |  |  |  | 20th JAN-2021/ FUJIAN MAWEI SHIPBUILDING LTD, CHINA |  |
| 1.5 | Flag/Port of Registry: |  |  |  |  | SINGAPORE / SINGAPORE |  |
| 1.6 | Call sign/MMSI: |  |  |  |  | 9V5425 / 563120100 |  |
| 1.7 | Vessel’s contact details (1arian/fax/email etc.): |  |  |  |  | +870 7739 45202(FBB) +65 31650132(V-SAT SGP) +4723410578 (V-SAT NORWAY) Fax: +870 7739 45202(FBB) Email: stargazer@honglam.com.sg |  |
| 1.8 | Type of vessel (as described in Form A or Form B Q1.11 of the IOPPC): |  |  |  |  | Oil/Chemical Tanker IMO II |  |
| 1.9 | Type of hull: |  |  |  |  | DOUBLE HULL |  |
| Ownership and Operation |  |  |  |  |  |  |  |
| 1.10 | Registered owner – Full style: |  |  |  | HONG LAM MARINE PTE LTD 6 Shenton Way, #16-08 OUE Down town 2 Singapore 068809 OFF.TEL: + 65 – 63336577 OFF. FAX NO. + 65 – 63336077 OFF.E-MAIL: honglam@hanglam.com.sg maungtun.khin@honglam.com.sg |  |  |
| 1.11 | Technical operator – Full style: |  |  |  | HONG LAM MARINE PTE LTD 6 Shenton Way, #16-08 OUE Down town 2 Singapore 068809 OFF.TEL: + 65 – 63336577 OFF. FAX NO. + 65 – 63336077 OFF.E-MAIL: honglam@hanglam.com.sg maungtun.khin@honglam.com.sg |  |  |
| 1.12 | Commercial operator – Full style: |  |  |  | HONG LAM MARINE PTE LTD 6 Shenton Way, #16-08 OUE Down town 2 Singapore 068809 OFF.TEL: + 65 – 63336577 OFF. FAX NO. + 65 – 63336077 OFF.E-MAIL: honglam@hanglam.com.sg maungtun.khin@honglam.com.sg |  |  |
| 1.13 | Disponent owner – Full style: |  |  |  | N/A |  |  |
| Insurance |  |  |  |  |  |  |  |
| 1.14 | P & I Club – Full Style: |  |  |  | Steamship Mutual Underwriting Association Limited Aquatical House, 39 Bell Lane, London E1 7LU Tel: +44 20 7247 5490 Website: www.steamshipmutual.com Registered No: OC376859 Represented in Singapore by: Steamship P&I Management (Singapore) PTE. LTD 80 Anson Road #25-05A Fuji Xerox Towers Singapore (079907)Tel: +6567309339 |  |  |
| 1.15 | P & I Club pollution liability coverage/expiration date: |  |  |  |  | 1 BILLION US$ | Feb 20, 2022 |
| 1.16 | Hull & Machinery insured by – Full Style: (Specify broker or leading underwriter) |  |  |  | As brokers:- 1.Howden Specialty Asia Pacific Pte Ltd 61 Robinson Road #20-01 Robinson Centre Singapore 068893 Ph. +65 6302 9699; Fax +65 6557 0564 2.Cambiaso Risso Asia Pte Ltd 6 Battery Road #34-01 Singapore 049909 Ph. +6565860602 ; Fax +6564916591 |  |  |
| 1.17 | Hull & Machinery insured value/expiration date: |  |  |  |  | 11 200 000US$ | 30-Sep-2021 |
| 1.17 | Hull & Machinery insured value/expiration date: |  |  |  |  | ( Cambiaso Risso Asia Pte. Ltd. ) 50% USD 11,200,000.00 | 30-Sep-2021 |
| Classification |  |  |  |  |  |  |  |
| 1.18 | Classification society: |  |  |  |  | ABS (AMERICAN BUREAU OF SHIPPING) |  |
| 1.19 | Class notation: |  |  |  |  | +A1 Oil and Chemical Carrier, (E) +AMS, +ACCU, ESP, RRDA, VEC-L, CPS, TCM, PMA, UWILD, IHM, ENVIRO, CRC, POT, MLC-ACCOM,PMP,BWT |  |
| 1.20 | Is the vessel subject to any conditions of class, class extensions, outstanding memorandums or class recommendations? If yes, give details: |  |  |  |  | N/A |  |
| 1.21 | If classification society changed, name of previous and date of change: |  |  |  |  | N/A, Not Applicable |  |
| 1.22 | Does the vessel have ice class? If yes, state what level: |  |  |  |  | No, n/a |  |
| 1.23 | Date/place of last dry-dock: |  |  |  |  | NA |  |
| 1.24 | Date next dry dock due/next annual survey due: |  |  |  |  | 19/Jan/2024 | 19/Jan/2022 |
| 1.25 | Date of last special survey/next special survey due: |  |  |  |  | NA | 20/Jan/2026 |
| 1.26 | If ship has Condition Assessment Program (CAP), what is the latest overall rating: |  |  |  |  | No, |  |
| Dimensions |  |  |  |  |  |  |  |
| 1.27 | Length overall (LOA): |  |  |  |  | 139.98 Meters |  |
| 1.28 | Length between perpendiculars (LBP): |  |  |  |  | 134.37 Meters |  |
| 1.29 | Extreme breadth (Beam): |  |  |  |  | 20.20 Meters |  |
| 1.30 | Moulded depth: |  |  |  |  | 11.20 Meters |  |
| 1.31 | Keel to masthead (KTM)/ Keel to masthead (KTM) in collapsed condition, if applicable: |  |  |  |  | 34.4 Meters | N/A |
| 1.32 | Distance bridge front to center of manifold: |  |  |  |  | 54.27 Meters |  |
| 1.33 | Bow to center manifold (BCM)/Stern to center manifold (SCM): |  |  |  |  | 60.14 Meters | 79.84 Meters |
| 1.34 | Parallel body distances |  |  |  | Lightship | Normal Ballast | Summer Dwt |
|  | # Forward to mid-point manifold: |  |  |  | 13.4 M | 18.41 M | 21.78 M |
|  | # Aft to mid-point manifold: |  |  |  | 23.45 M | 40.29 M | 55.09 M |
|  | # Parallel body length: |  |  |  | 36.85 M | 58.70 M | 76.87 M |
| Tonnages |  |  |  |  |  |  |  |
| 1.35 | Net Tonnage: |  |  |  |  | 4433T |  |
| 1.36 | Gross Tonnage/Reduced Gross Tonnage (if applicable): |  |  |  |  | 8916T | 8916 T |
| 1.37 | Suez Canal Tonnage - Gross (SCGT)/Net (SCNT): |  |  |  |  | 9552T | 8116T |
| # 1.38 | Panama Canal Net Tonnage (PCNT): |  |  |  |  | 7531T |  |
| # Loadline Information |  |  |  |  |  |  |  |
| # 1.39 | # Loadline |  |  | # Freeboard | # Draft | # Deadweight | # Displacement |
|  | # Summer: |  |  | 3.017 M | 8.196 M | 13799 MT | 18222.8MT |
|  | # Winter: |  |  | 3.188 M | 8.025 M | 13362.1MT | 17785.9MT |
|  | # Tropical: |  |  | 2.846 M | 8.367 M | 14233.8MT | 18657.6MT |
|  | # Lightship: |  |  | 8.760 M | 2.453 M |  | 4423.8MT |
|  | # Normal Ballast Condition: |  |  | 6.203 M | 5.010 M | 6010.4MT | 10434.2MT |
|  | # Segregated Ballast Condition: |  |  | 6.203 M | 5.010 M | 6010.4MT | 10434.2MT |
| # 1.40 | # FWA/TPC at summer draft: |  |  |  |  | 179 MM | 25.508 T |
| # 1.41 | # Does vessel have multiple SDWT? If yes, please provide all assigned loadlines: |  |  |  |  | 13799/11281.2/9997.9MT |  |
| 1.42 | Constant (excluding fresh water): |  |  |  |  | 40 MT |  |
| 1.43 | What is the company guidelines for Under Keel Clearance (UKC) for this vessel? |  |  |  |  | -Open sea a minimum of 50% of the ship’s maximum static draft. -In port or at shallow water at least 1.0 meter. -At terminal (alongside, during loading or discharging) ship to maintain minimum UKC of 0.3 meter |  |
| 1.44 | What is the max height of mast above waterline (air draft) |  |  |  |  | Full Mast | Collapsed Mast |
|  | Summer deadweight: |  |  |  |  | 26.22M | N/A |
|  | Normal ballast: |  |  |  |  | 28.56 M | N/A |
|  | Lightship: |  |  |  |  | 29.30M | N/A |
| 2. | CERTIFICATES |  |  | Issued | Last Annual | Last Intermediate | Expires |
| 2.1 | Safety Equipment Certificate (SEC): |  |  | 20th Jan 2021 | N/A | N/A | 19th Jan 2026 |
| 2.2 | Safety Radio Certificate (SRC): |  |  | 20th Jan 2021 | N/A | N/A | 19th Jan 2026 |
| 2.3 | Safety Construction Certificate (SCC): |  |  | 20th Jan 2021 | N/A | N/A | 19th Jan 2026 |
| 2.4 | International Load line Certificate (ILC): |  |  | 20th Jan 2021 | N/A | N/A | 19th Jan 2026 |
| 2.5 | International Oil Pollution Prevention Certificate (IOPPC): |  |  | 20th Jan 2021 | N/A | N/A | 19th Jan 2026 |
| # 2.6 | # International Ship Security Certificate (ISSC): |  |  | 10th Jun 2021 | N/A | # N/A | 09th Jun 2026 |
| # 2.7 | # Maritime Labour Certificate (MLC): |  |  | 10th Jun 2021 | N/A |  | 09th Jun 2026 |
| 2.8 | ISM Safety Management Certificate (SMC): |  |  | 10th Jun 2021 | N/A | # N/A | 09th Jun 2026 |
| # 2.9 | # Document of Compliance (DOC): |  |  | 02nd Nov 2017 | 10th Feb 2021 | N/A | 19th Nov 2022 |
| 2.10 | USCG Certificate of Compliance(USCGCOC): |  |  | NA | N/A | N/A | Not Applicable |
| 2.11 | Civil Liability Convention (CLC) 1992 Certificate: |  |  | 05-Feb-2021 | N/A | N/A | 20-FEB-2022 |
| 2.12 | Civil Liability for Bunker Oil Pollution Damage Convention (CLBC) Certificate: |  |  | 05-Feb-2021 | N/A | N/A | 20-FEB-2022 |
| # 2.13 | # Liability for the Removal of Wrecks Certificate (WRC): |  |  | 06-Feb-2021 | N/A | N/A | 20-FEB-2022 |
| 2.14 | U.S. Certificate of Financial Responsibility (COFR): |  |  | NA | N/A | N/A | Not Applicable |
| 2.15 | Certificate of Class (COC): |  |  | 25th Feb 2021 | N/A | N/A | 19th Jun 2026 |
| # 2.16 | # International Sewage Pollution Prevention Certificate (ISPPC): |  |  | 20th Jan 2021 | N/A | N/A | 19th Jan 2026 |
| # 2.17 | # Certificate of Fitness (COF): |  |  | 20th Jan 2021 | N/A | N/A | 19th Jan 2026 |
| # 2.18 | # International Energy Efficiency Certificate (IEEC): |  |  | 20th Jan 2021 | N/A | N/A | N/A |
| # 2.19 | # International Air Pollution Prevention Certificate (IAPPC): |  |  | 20th Jan 2021 | N/A | N/A | 19th Jan 2026 |
| Documentation |  |  |  |  |  |  |  |
| 2.20 | Owner warrant that vessel is member of ITOPF and will remain so for the entire duration of this voyage/contract: |  |  |  |  | Yes |  |
| 2.21 | Does vessel have in place a Drug and Alcohol Policy complying with OCIMF guidelines for Control of Drugs and Alcohol Onboard Ship? |  |  |  |  | Yes |  |
| 2.22 | Is the ITF Special Agreement on board (if applicable)? |  |  |  |  | N/A |  |
| 2.23 | ITF Blue Card expiry date (if applicable): |  |  |  |  | N/A |  |
| 3. | CREW |  |  |  |  |  |  |
| 3.1 | Nationality of Master: |  |  |  |  | CHINESE |  |
| 3.2 | Number and nationality of Officers: |  |  |  | 8 | CHINESE |  |
| 3.3 | Number and nationality of Crew: |  |  |  | 8 | CHINESE |  |
| 3.4 | What is the common working language onboard: |  |  |  |  | English |  |
| 3.5 | Do officers speak and understand English? |  |  |  |  | Yes |  |
| 3.6 | If Officers/ratings employed by a manning agency - Full style: |  |  | SINGHAI MARINE SERVICES 380 Jalan Besar, #05-06 ARC 380, Singapore 209000. Building 4, No 600 Min Sheng Road, Shanghai, China 200135 Mobile: +65 81810283; +86 13681962512 Email: zhupeipei@singhai.com |  | SINGHAI MARINE SERVICES 380 Jalan Besar, #05-06 ARC 380, Singapore 209000. Building 4, No 600 Min Sheng Road, Shanghai, China 200135 Mobile: +65 81810283; +86 13681962512 Email: zhupeipei@singhai.com |  |
| 4. | FOR USA CALLS |  |  |  |  |  |  |
| 4.1 | Has the vessel Operator submitted a Vessel Spill Response Plan to the US Coast Guard which has been approved by official USCG letter? |  |  |  |  | No |  |
| 4.2 | Qualified individual (QI) - Full style: |  |  |  | N/A |  |  |
| 4.3 | Oil Spill Response Organization (OSRO) - Full style: |  |  |  | N/A |  |  |
| 4.4 | Salvage and Marine Firefighting Services (SMFF) - Full Style: |  |  |  | N/A |  |  |
| 5. | SAFETY/HELICOPTER |  |  |  |  |  |  |
| 5.1 | Is the vessel operated under a Quality Management System? If Yes, what type of system? (ISO9001 or IMO Resolution A.741(18) as amended): |  |  |  |  | Yes ISO9001:2015, ISO14001:2015, OHSAS 18001:2007 |  |
| 5.2 | Can the ship comply with the ICS Helicopter Guidelines? |  |  |  |  | No |  |
| 5.2.1 | If Yes, state whether winching or landing area provided: |  |  |  |  | N/A |  |
| 5.2.2 | If Yes, what is the diameter of the circle provided: |  |  |  |  | N/A |  |
| 6. | COATING/ANODES |  |  |  |  |  |  |
| 6.1 | Tank Coating |  |  | Coated | Type | To What Extent | Anodes |
|  | Cargo tanks: |  |  | Yes | Phenolic epoxy | Whole tank | No |
|  | Ballast tanks: |  |  | Yes | EPOXY | Whole tank | Yes |
|  | Slop tanks: |  |  | Yes | Phenolic epoxy | Whole tank | No |
| 7. | BALLAST |  |  |  |  |  |  |
| 7.1 | Pumps |  |  | No. | Type | Capacity | At What Head (sg=1.0) |
|  | Ballast Pumps: |  |  | 2 | Horizantal Centrifugal Pump | Each 250 M3/HR | N/A |
|  | Ballast Eductors: |  |  | N/A | N/A | NA | N/A |
| 8. | CARGO |  |  |  |  |  |  |
| Double Hull Vessels |  |  |  |  |  |  |  |
| 8.1 | Is vessel fitted with centerline bulkhead in all cargo tanks? If Yes, solid or perforated: |  |  |  |  | Yes, Solid |  |
| Cargo Tank Capacities |  |  |  |  |  |  |  |
| 8.2 | Number of cargo tanks and total cubic capacity (98%): (excluding two slop tanks) |  |  |  |  | 12 | 16231.424 M3 |
| 8.2.1 | Capacity (98%) of each natural segregation with double valve (specify tanks): |  |  |  |  | Port Stbd NO.1 981.046 981.711 NO.2 1377.360 1378.667 NO.3 1447.119 1449.832 NO.4 1448.682 1451.064 NO.5 1430.517 1429.535 NO.6 1240.427 1235.824 Slop tank 186.864 192.776 |  |
| 8.2.2 | IMO class (Oil/Chemical Ship Type 1, 2 or 3): |  |  |  |  | 2 |  |
| 8.3 | Number of slop tanks and total cubic capacity (98%): |  |  |  |  | 2 | 379.640 M3 |
| 8.3.1 | Specify segregations which slops tanks belong to and their capacity with double valve: |  |  |  |  | YES |  |
| 8.3.2 | Residual/retention oil tank(s) capacity (98%), if applicable: |  |  |  |  | N/A |  |
| SBT Vessels |  |  |  |  |  |  |  |
| 8.3.3 | What is total SBT capacity and percentage of SDWT vessel can maintain? |  |  |  |  | 5715.94M3 | 36 % |
| 8.3.4 | Does vessel meet the requirements of MARPOL Annex I Reg 18.2: |  |  |  |  | YES |  |
| Cargo Handling and Pumping Systems |  |  |  |  |  |  |  |
| 8.4 | How many grades/products can vessel load/discharge with double valve segregation: |  |  |  |  | 3 (Including slops) |  |
| 8.4.1 | State type of cargo containment (integral, independent, gravity or pressure tanks): |  |  |  |  | Gravity |  |
| 8.5 | Are there any cargo tank filling restrictions? If yes, specify number of slack tanks, max s.g., ullage restrictions etc.: |  |  |  |  | Yes 98% volume at Max s.g 1.025 |  |
| 8.6 | Max loading rate for homogenous cargo |  |  |  |  | With VECS | Without VECS |
|  | Loaded per manifold connection: |  |  |  |  | 1600 M3/Hr | 1600 M3/HR |
|  | Loaded simultaneously through all manifolds: |  |  |  |  | 2500 M3/Hr | 2500 M3/HR |
| Cargo Control Room |  |  |  |  |  |  |  |
| 8.7 | Is ship fitted with a Cargo Control Room (CCR)? |  |  |  |  | Yes |  |
| 8.8 | Can tank innage/ullage be read from the CCR? |  |  |  |  | Yes |  |
| Gauging and Sampling |  |  |  |  |  |  |  |
| 8.9 | Is gauging system certified and calibrated? If no, specify which ones are not calibrated: |  |  |  |  | Yes, |  |
|  | What type of gauging system as per IBC 13.1 is fitted (Open/Restricted/Closed )? |  |  |  |  | Closed |  |
|  | What type of fixed closed tank gauging system is fitted: |  |  |  |  | TANK RADAR |  |
|  | Is a tank overflow control system fitted? If yes, then state if system includes automatic closing of valves? |  |  |  |  | N/A |  |
|  | Are high level alarms fitted to the cargo tanks? If Yes, indicate whether to all tanks or partial: |  |  |  |  | Yes, All |  |
| 8.9.1 | Can cargo be transferred under closed loading conditions in accordance with ISGOTT 11.1.6.6? |  |  |  |  | Yes |  |
| 8.9.2 | Are cargo tanks fitted with multipoint gauging? If yes, specify type and locations: |  |  |  |  | N/A, |  |
| 8.10 | Number of portable gauging units (example- MMC) on board: |  |  |  |  | 2 |  |
| Vapor Emission Control System (VECS) |  |  |  |  |  |  |  |
| 8.11 | Is a Vapour Emission Control System (VECS) fitted? |  |  |  |  | Yes |  |
| 8.12 | Number/size of VECS manifolds (per side): |  |  |  |  | 2 | 250 MM |
| 8.13 | Number/size/type of VECS reducers: |  |  |  |  | (10 x 6) & (10 x 4) / ANSI |  |
| Venting |  |  |  |  |  |  |  |
| 8.14 | State what type of venting system is fitted: |  |  |  |  | HIGH VELOCITY PV VALVES |  |
| Cargo Manifolds and Reducers |  |  |  |  |  |  |  |
| 8.15 | Total number/size of cargo manifold connections on each side: |  |  |  |  | 3 x 300mm |  |
| 8.15.1 | Does the vessel have a Common Line Manifold connection? If yes, describe: |  |  |  |  | NO |  |
| 8.16 | What type of valves are fitted at manifold: |  |  |  |  | Manual gate type valves |  |
| 8.17 | What is the material/rating of the manifold: |  |  |  |  | SUS 316L |  |
| 8.17.1 | Does vessel comply with the latest edition of the OCIMF 'Recommendations for Oil Tanker Manifolds and Associated Equipment’? |  |  |  |  | Yes |  |
| 8.18 | Distance between cargo manifold centers: |  |  |  |  | 1500mm |  |
| 8.19 | Distance ships rail to manifold: |  |  |  |  | 4920mm |  |
| 8.20 | Distance manifold to ships side: |  |  |  |  | 5150mm |  |
| 8.21 | Top of rail to center of manifold: |  |  |  |  | 1470mm |  |
| 8.22 | Distance main deck to center of manifold: |  |  |  |  | 2300mm |  |
| 8.23 | Spill tank grating to center of manifold: |  |  |  |  | 900mm |  |
| 8.24 | Manifold height above the waterline in normal ballast/at SDWT condition: |  |  |  |  | 9.00 M | 5.82 M |
| 8.25 | Number/size/type of reducers: |  |  |  |  | Reducers ANSI to ANSI 6Pc =12"ANSI TO 10"ANSI 6Pc=12"ANSI TO 8"ANSI 2Pc=12"ANSI TO 6"ANSI 4Pc=14"ANSI TO 12"ANSI 1Pc=16"ANSI TO 12"ANSI 2Pc=10"ANSI TO 8"ANSI 2Pc=10"ANSI TO 4"ANSI 2Pc=10"ANSI TO 5"ANSI 2Pc=12"ANSI TO 2*12"ANSI Y-PIECE Reducers ANSI to JIS 2Pc=12"ANSI TO 10"JIS 2Pc=12" ANSI TO 8"JIS 2Pc=12"ANSI TO 6"JIS 2Pc=12"ANSI TO 5"JIS 2Pc=10"ANSI TO 8"JIS 2Pc=10"ANSI TO 4"JIS |  |
| 8.26 | Is vessel fitted with a stern manifold? If yes, state size: |  |  |  |  | No, |  |
| Heating |  |  |  |  |  |  |  |
| 8.27 | Cargo/slop tanks fitted with a cargo heating system? |  |  |  | Type | Coiled | Material |
|  | Cargo Tanks: |  |  |  | STEAM | HEATING COILS | STAINLESS STEEL |
|  | Slop Tanks: |  |  |  | STEAM | HEATING COILS | STAINLESS STEEL |
| 8.27.1 | Is a Thermal Oil Heating system fitted? If yes, identify tanks? |  |  |  |  | N/A |  |
| 8.28 | Maximum temperature cargo can be loaded/maintained: |  |  |  |  | 65° C | 60° C |
| 8.28.1 | Minimum temperature cargo can be loaded/maintained: |  |  |  |  | AMBIENT | AMBIENT |
| Inert Gas and Crude Oil Washing |  |  |  |  |  |  |  |
| 8.29 | Is an Inert Gas System (IGS) fitted/operational? |  |  |  |  | YES |  |
| 8.29.1 | Is a Crude Oil Washing (COW) installation fitted/operational? |  |  |  |  | N/A |  |
| 8.30 | Is IGS supplied by flue gas, inert gas (IG) generator and/or nitrogen: |  |  |  |  | YES |  |
| 8.30.1 | If nitrogen generator, specify the applicable flow rate for each of the designed purity modes: |  |  |  |  | N/A |  |
| Cargo Pumps |  |  |  |  |  |  |  |
| 8.31 | How many cargo pumps can be run simultaneously at full capacity: |  |  |  |  | 3 |  |
| 8.32 | Pumps |  |  | No. | Type | Capacity | At What Head (sg=1.0) |
|  | Cargo Pumps: |  |  | 3 | Twin-screw Pump(motor driven) | EACH 750 M3/HR | 1.0MPa, S.G. 1.025 t/m3, |
|  | Cargo Eductors: |  |  | N/A | N/A | M3/HR | N/A |
|  | Stripping: |  |  | 1 | Twin-screw Pump(motor driven) | 50 M3/HR | 1.0MPa, S.G. 1.025 t/m3 |
| 8.33 | Is at least one emergency portable cargo pump provided? |  |  |  |  | YES |  |
| Tank Cleaning Systems |  |  |  |  |  |  |  |
| 8.34 | Is tank cleaning equipment fixed in cargo tanks? |  |  |  |  | Yes |  |
| 8.35 | Is portable tank cleaning equipment provided? |  |  |  |  | Yes |  |
| 8.36 | Tank washing pump capacity: |  |  |  |  | 66 M3/Hour |  |
| 8.37 | Is a washing water heater fitted? If yes is it operational and state max washing water temperature: |  |  |  |  | Yes , 70’C |  |
| 8.38 | What is the maximum number of machines that can be operated at their designed max pressure? |  |  |  |  | 6 |  |
| Other Deck Equipment |  |  |  |  |  |  |  |
| 8.39 | Is vessel fitted with a remote cargo tank temperature monitoring system? If yes, is it operational? |  |  |  |  | Yes, Yes |  |
| 8.40 | Is vessel fitted with a remote cargo tank pressure monitoring system? If yes, is it operational? |  |  |  |  | Yes, Yes |  |
| 8.41 | Is vessel fitted with a cargo tank drier? If yes is it operational and state capacity: |  |  |  |  | No (Gas free fan only) |  |
| 8.42 | Is vessel fitted with a cargo cooling system? If yes is it operational and state tanks applicable: |  |  |  |  | N/A |  |
| 8.43 | Is steam available on deck? |  |  |  |  | Yes |  |
| 9. | MOORING |  |  |  |  |  |  |
| 9.1 | Wires (on drums) |  | No. | Diameter | Material | Length | Breaking Strength |
|  | Forecastle: |  |  |  |  |  |  |
|  | Main deck fwd: |  |  |  |  |  |  |
|  | Main deck aft: |  |  |  |  |  |  |
|  | Poop deck: |  |  |  |  |  |  |
| 9.2 | Wire tails |  | No. | Diameter | Material | Length | Breaking Strength |
|  | Forecastle: |  |  |  |  |  |  |
|  | Main deck fwd: |  |  |  |  |  |  |
|  | Main deck aft: |  |  |  |  |  |  |
|  | Poop deck: |  |  |  |  |  |  |
| 9.3 | Ropes (on drums) |  | No. | Diameter | Material | Length | Breaking Strength |
|  | Forecastle: |  | 4 | 48 MM | Polyamide | 220 M | 340 KN |
|  | Main deck fwd: |  | N/A | N/A | N/A | N/A | N/A |
|  | Main deck aft: |  | N/A | N/A | N/A | N/A | N/A |
|  | Poop deck: |  | 4 | 48 MM | Polyamide | 220 M | 340 KN |
| 9.4 | Other lines |  | No. | Diameter | Material | Length | Breaking Strength |
|  | Forecastle: |  | 4 | 48 MM | PP/PE | 220 M | 364 KN |
|  | Main deck fwd: |  | N/A | MM | N/A | M | MT |
|  | Main deck aft: |  | N/A | MM | N/A | M | MT |
|  | Poop deck: |  | 4 | 48 MM | PP/PE | 220 M | 364 KN |
| 9.5 | Winches |  | No. | No. Drums | Motive Power | Brake Capacity | Type of Brake |
|  | Forecastle: |  | 4 | SINGLE | HYDRAULIC | 26.45 MT | Friction Band |
|  | Main deck fwd: |  | N/A | N/A | N/A | N/A | N/A |
|  | Main deck aft: |  | N/A | N/A | N/A | N/A | N/A |
|  | Poop deck: |  | 4 | SINGLE | HYDRAULIC | 26.45 MT | Friction Band |
| 9.6 | Bitts, closed chocks/fairleads |  |  | No. Bitts | SWL Bitts | No. Closed Chocks | SWL Closed Chocks |
|  | Forecastle: |  |  | 6 | 35 MT | 13 | 35 MT |
|  | Main deck fwd: |  |  | 2 | 35 MT | 2 | 35 MT |
|  | Main deck aft: |  |  | 2 | 35 MT | 2 | 35 MT |
|  | Poop deck: |  |  | 6 | 35 MT | 13 | 35 MT |
| Anchors/Emergency Towing System |  |  |  |  |  |  |  |
| 9.7 | Number of shackles on port/starboard cable: |  |  |  |  | 9/9 |  |
| 9.8 | Type/SWL of Emergency Towing system forward: |  |  |  |  | N/A |  |
| 9.9 | Type/SWL of Emergency Towing system aft: |  |  |  |  | N/A |  |
| Escort Tug |  |  |  |  |  |  |  |
| 9.10 | What is size/SWL of closed chock and/or fairleads of enclosed type on stern: |  |  |  |  | 360 MM X 250 MM | 35.00 MT |
| 9.11 | What is SWL of bollard on poop deck suitable for escort tug: |  |  |  |  | 35.00 MT |  |
| Lifting Equipment/Gangway |  |  |  |  |  |  |  |
| 9.12 | Derrick/Crane description (Number, SWL and location): |  |  |  |  | CRANE: 1 X 10.0 TONNES Center |  |
| 9.13 | Accommodation ladder direction: |  |  |  |  | Facing Aft |  |
|  | Does vessel have a portable gangway? If yes, state length: |  |  |  |  | Yes, 8 M |  |
| Single Point Mooring (SPM) Equipment |  |  |  |  |  |  |  |
| 9.14 | Does the vessel meet the recommendations in the latest edition of OCIMF 'Recommendations for Equipment Employed in the Bow Mooring of Conventional Tankers at Single Point Moorings (SPM)’? |  |  |  |  | No |  |
| 9.15 | If fitted, how many chain stoppers: |  |  |  |  | N/A |  |
| 9.16 | State type/SWL of chain stopper(s): |  |  |  |  | N/A |  |
| 9.17 | What is the maximum size chain diameter the bow stopper(s) can handle: |  |  |  |  | N/A |  |
| 9.18 | Distance between the bow fairlead and chain stopper/bracket: |  |  |  |  | N/A |  |
| 9.19 | Is bow chock and/or fairlead of enclosed type of OCIMF recommended size (600mm x 450mm)? If not, give details of size: |  |  |  |  | N/A |  |
| 10. | PROPULSION |  |  |  |  |  |  |
| 10.1 | Speed |  |  |  |  | Maximum | Economical |
|  | Ballast speed: |  |  |  |  | 13.0 KTS | 12.0 KTS |
|  | Laden speed: |  |  |  |  | 12.0 KTS | 11.5 KTS |
| 10.2 | What type of fuel is used for main propulsion/generating plant: |  |  |  |  | VLSFO | VLSFO |
| 10.3 | Type/Capacity of bunker tanks: |  |  |  |  | Fuel Oil: 364.20 Cu. Meters Diesel Oil: 81.67 M3 Gas Oil: 0 M3 |  |
| 10.4 | Is vessel fitted with fixed or controllable pitch propeller(s): |  |  |  |  | Fixed |  |
| 10.5 | Engines |  |  |  | No | Capacity | Make/Type |
|  | Main engine: |  |  |  | 1 | 3575kw @142rpm | YCMP-WINGD / W5X-35-B |
|  | Aux engine: |  |  |  | 3 | 615 KW @720rpm | CMP MAN, CSSC Marine Power CO.,LTD/5L23/30H |
|  | Power packs: |  |  |  | NIL | NIL | NIL |
|  | Boilers: |  |  |  | 1 | 6,000 kg/h | LSK8-0-0.7/vertical oil fired |
| Bow/Stern Thruster |  |  |  |  |  |  |  |
| 10.6 | What is brake horse power of bow thruster (if fitted): |  |  |  |  | 422 BHP / 315 KW |  |
| 10.7 | What is brake horse power of stern thruster (if fitted): |  |  |  |  | NA |  |
| Emissions |  |  |  |  |  |  |  |
| 10.8 | Main engine IMO NOx emission standard: |  |  |  |  | IMO Tier II compliant |  |
| 10.9 | Energy Efficiency Design Index (EEDI) rating number: |  |  |  |  | Phase 3 |  |
| 11. | SHIP TO SHIP TRANSFER |  |  |  |  |  |  |
| 11.1 | Does vessel comply with recommendations contained in OCIMF/ICS Ship To Ship Transfer Guide (Petroleum, Chemicals or Liquified Gas, as applicable)? |  |  |  |  | YES |  |
| 11.2 | What is maximum outreach of cranes/derricks outboard of the ship’s side: |  |  |  |  | 5.5 M |  |
| 11.3 | Date/place of last STS operation: |  |  |  |  | Nil |  |
| 12. | RECENT OPERATIONAL HISTORY |  |  |  |  |  |  |
| 12.1 | Last three cargoes/charterers/voyages (Last/2nd Last/3rd Last): |  |  |  |  | 1. AGO 10PPM/SHELL TIMUR SDN BHD/ V0921 2. LCO/BP SINGAPORE PTE LTD/V1021 3.MTBE/INTEGRA PETROCHEMICALS PTE LTD/V1121 |  |
| 12.2 | Has vessel been involved in a pollution, grounding, serious casualty, unscheduled repair or collision incident during the past 12 months? If yes, provide details: |  |  |  |  | Pollution: No, Grounding: No, Casualty: No, Repair: No, Collision: No, |  |
| 12.3 | Date and place of last Port State Control inspection: |  |  |  |  | 2021-04-15/JAKARTA |  |
| 12.4 | Any outstanding deficiencies as reported by any Port State Control? If yes, provide details: |  |  |  |  | No |  |
| 12.5 | Recent Oil company inspections/screenings (To the best of owners knowledge and without guarantee of acceptance for future business)*: * "Approvals" are not given by Oil Majors and ships are accepted for the voyage on a case by case basis. |  |  |  |  | Contact owner for details |  |
| 12.6 | Date/Place of last SIRE inspection: |  |  |  |  | 22-MAR.-2021/HALDIA,INDIA |  |
| 12.6.1 | Date/Place of last CDI inspection: |  |  |  |  | Nil |  |
| 12.7 | Additional information relating to features of the ship or operational characteristics: |  |  |  |  | None |  |

Revised 2018 (INTERTANKO/Q88.com)

Form completed on http://www.q88.com/integration.aspx  Please email support@q88.com an updated copy if this is not the latest version.