- INTERTANKO CHARTERING QUESTIONNAIRE 88 - OIL/CHEMICAL	Version 5

| 1. | GENERAL INFORMATION |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1.1 | Date updated: |  |  |  |  |  |  | Sep 06, 2023 |  |
| 1.2 | Vessel’s name (IMO number): |  |  |  |  |  |  | STI Black Hawk (9704453) |  |
| 1.3 | Vessel’s previous name(s) and date(s) of change: |  |  |  |  |  |  | Not Applicable |  |
| 1.4 | Date delivered/Builder (where built): |  |  |  |  |  |  | Sep 16, 2015/HYUNDAI MIPO DOCKYARD,SOUTH KOREA |  |
| 1.5 | Flag/Port of Registry: |  |  |  |  |  |  | Marshall Islands/Majuro |  |
| 1.6 | Call sign/MMSI: |  |  |  |  |  |  | V7HK6/538005836 |  |
| 1.7 | Vessel’s contact details (satcom/fax/email etc.): |  |  |  |  |  |  | Tel: VSAT : +1 203 346 2959 / FBB : +870 773 923 691 Fax: +870 783 917 628 Email: master@blackhawk.scorpiotankers.com |  |
| 1.8 | Type of vessel (as described in Form A or Form B Q1.11 of the IOPPC): |  |  |  |  |  |  | Oil Tanker (Crude Oil/Product Carrier) |  |
| 1.9 | Type of hull: |  |  |  |  |  |  | Double Hull |  |
| Ownership and Operation |  |  |  |  |  |  |  |  |  |
| 1.10 | Registered owner - Full style: |  |  |  |  | TAIPING & SINOPEC TJ14 SHIPPING LEASING CO., LTD. Room 202, office area of inspection warehouse No.6262 Aozhou Road, Dongjiang Free Trade Port Zone, Tianjin Pilot Free Trade Zone, Tianjin, China IMO no. 6215621 China |  |  |  |
| 1.11 | Technical operator - Full style: |  |  |  |  | Scorpio Marine Management India Pvt Ltd Scorpio House,Central Avenue,Hiranandani Business Park,Powai,Mumbai - 400 076 India Tel: +9122-6106-0000 Fax: +91-22-6150-4350 Email: ssm.vetting@scorpiogroup.net Company IMO#: 5562457 |  |  |  |
| 1.12 | Commercial operator - Full style: |  |  |  |  | Scorpio Commercial Management S.A.M. “L'Exotique” 99 Bd. du Jardin Exotique MC 98000 Monaco Monaco Email: opsmr@scorpiogroup.net |  |  |  |
| 1.13 | Disponent owner - Full style: |  |  |  |  | SCORPIO MR POOL Ltd. C/O Scorpio Commercial Management Sam “L'Exotique” 99 Bd. du Jardin Exotique MC 98000 Monaco Email: opsmr@scorpiogroup.net |  |  |  |
| Insurance |  |  |  |  |  |  |  |  |  |
| 1.14 | P & I Club - Full Style: |  |  |  |  | The Britannia Steam Ship Insurance Association Ltd Regis House, 45 King William Street, London. EC4R 9AN Tel: +44 0 20 7407 3588 Fax: +44 0 20 7403 3942 Email: info@britanniapandi.com Web: britanniapandi.com |  |  |  |
| 1.15 | P & I Club pollution liability coverage/expiration date: |  |  |  |  |  |  | 1,000,000,000 US$ | Feb 20, 2024 |
| 1.16 | Hull & Machinery insured by - Full Style: (Specify broker or leading underwriter) |  |  |  |  | Ital Brokers S.p.A. Via Albaro, 3 16145 Genoa Tel: +39 010 3195315 Fax: +390103623862 |  |  |  |
| 1.17 | Hull & Machinery insured value/expiration date: |  |  |  |  |  |  | 35,500,000 US$ | Nov 28, 2024 |
| Classification |  |  |  |  |  |  |  |  |  |
| 1.18 | Classification society: |  |  |  |  |  |  | American Bureau of Shipping |  |
| 1.19 | Class notation: |  |  |  |  |  |  | A1 ,Chemical Carrier ,Oil Carrier ,ESP ,AMS ,ACCU ,CPS ,CSR, AB-CM, BWE, BWT, CRC, ENVIRO, GP, POT, RRDA, RW, SPMA, TCM, UWILD, VEC |  |
| 1.20 | Is the vessel subject to any conditions of class, class extensions, outstanding memorandums or class recommendations? If yes, give details: |  |  |  |  |  |  | No NO |  |
| 1.21 | If classification society changed, name of previous and date of change: |  |  |  |  |  |  | American Bureau of Shipping, Not Applicable |  |
| 1.22 | Does the vessel have ice class? If yes, state what level: |  |  |  |  |  |  | N/A, NA |  |
| 1.23 | Date/place of last dry-dock: |  |  |  |  |  |  | Aug 10, 2020/Shanghai,China |  |
| 1.24 | Date next dry dock due/next annual survey due: |  |  |  |  |  |  | Sep 16, 2025 | Sep 15, 2023 |
| 1.25 | Date of last special survey/next special survey due: |  |  |  |  |  |  | Aug 07, 2020 | Sep 16, 2025 |
| 1.26 | If ship has Condition Assessment Program (CAP), what is the latest overall rating: |  |  |  |  |  |  | No, |  |
| Dimensions |  |  |  |  |  |  |  |  |  |
| 1.27 | Length overall (LOA): |  |  |  |  |  |  | 183.12 Metres |  |
| 1.28 | Length between perpendiculars (LBP): |  |  |  |  |  |  | 174.00 Metres |  |
| 1.29 | Extreme breadth (Beam): |  |  |  |  |  |  | 32.23 Metres |  |
| 1.30 | Moulded depth: |  |  |  |  |  |  | 19.40 Metres |  |
| 1.31 | Keel to masthead (KTM)/ Keel to masthead (KTM) in collapsed condition, if applicable: |  |  |  |  |  |  | 49.10 Metres | NA |
| 1.32 | Distance bridge front to center of manifold: |  |  |  |  |  |  | 57.11 Metres |  |
| 1.33 | Bow to center manifold (BCM)/Stern to center manifold (SCM): |  |  |  |  |  |  | 90.96 Metres | 92.16 Metres |
| 1.34 | Parallel body distances |  |  |  |  | Lightship |  | Normal Ballast | Summer Dwt |
|  | # Forward to mid-point manifold: |  |  |  |  | 32.10 Metres |  | 34.30 Metres | 33.14 Metres |
|  | # Aft to mid-point manifold: |  |  |  |  | 22.04 Metres |  | 40.90 Metres | 55.90 Metres |
|  | # Parallel body length: |  |  |  |  | 50.35 Metres |  | 75.20 Metres | 89.04 Metres |
| Tonnages |  |  |  |  |  |  |  |  |  |
| 1.35 | Net Tonnage: |  |  |  |  |  |  | 13,224.00 |  |
| 1.36 | Gross Tonnage/Reduced Gross Tonnage (if applicable): |  |  |  |  |  |  | 29,899 | 23,167 |
| 1.37 | Suez Canal Tonnage - Gross (SCGT)/Net (SCNT): |  |  |  |  |  |  | 31,750.37 | 27,642.05 |
| # 1.38 | Panama Canal Net Tonnage (PCNT): |  |  |  |  |  |  | 24,817 |  |
| # Loadline Information |  |  |  |  |  |  |  |  |  |
| # 1.39 | # Loadline |  |  | # Freeboard |  | # Draft |  | # Deadweight | # Displacement |
|  | # Summer: |  |  | 7.051 Metres |  | 12.377 Metres |  | # 44,999 Metric Tonnes | 55,622 Metric Tonnes |
|  | # Winter: |  |  |  |  |  |  |  |  |
|  | # Tropical: |  |  |  |  |  |  |  |  |
|  | # Lightship: |  |  | 16.368 Metres |  | 3.06 Metres |  | - | 10,633.00 Metric Tonnes |
|  | # Normal Ballast Condition: |  |  | 12.178 Metres |  | 7.25 Metres |  | 19,204.00 Metric Tonnes | 29,827.50 Metric Tonnes |
|  | # Segregated Ballast Condition: |  |  | 11.76 Metres |  | 7.64 Metres |  | 20,998.50 Metric Tonnes | 31,621.50 Metric Tonnes |
| # 1.40 | # FWA/TPC at summer draft: |  |  |  |  |  |  | 267 Millimetres | 52.09 Metric Tonnes |
| # 1.41 | # Does vessel have multiple SDWT? If yes, please provide all assigned loadlines: |  |  |  |  |  |  | Yes 39999MT/44999MT/49990MT |  |
| 1.42 | Constant (excluding fresh water): |  |  |  |  |  |  | 200 Metric Tonnes |  |
| 1.43 | What is the company guidelines for Under Keel Clearance (UKC) for this vessel? |  |  |  |  |  |  | Ocean Passage/ Deep Sea: Minimum net Under Keel Clearance (UKC) of 50% of deepest static draught is to be maintained at all times. Confined Waters (narrow channels / rivers / coastal areas / SBM / CBM): Minimum Net Under Keel Clearance (UKC) of 10% of deepest static draught or 1 meter, whichever is greater, is to be maintained at all times. Within Port Limits (If not marked on chart then fairway buoy or pilot station whichever comes first / moored alongside / certain river approaches not clearly marked by a fairway buoy) and while alongside: Minimum Net Under Keel Clearance (UKC) of 10% of deepest static draught. Can be reduced to 1.5% of ship’s beam after office approval. |  |
| 1.44 | What is the max height of mast above waterline (air draft) |  |  |  |  |  |  | Full Mast | Collapsed Mast |
|  | Summer deadweight: |  |  |  |  |  |  | 37.686 Metres | 0 Metres |
|  | Normal ballast: |  |  |  |  |  |  | 41.85 Metres | 0 Metres |
|  | Lightship: |  |  |  |  |  |  | 46.22 Metres | 0 Metres |
| 2. | CERTIFICATES |  |  | Issued |  | Last Annual |  | Last Intermediate | Expires |
| 2.1 | Safety Equipment Certificate (SEC): |  |  | Aug 07, 2020 |  | Oct 13, 2022 |  |  | Sep 15, 2025 |
| 2.2 | Safety Radio Certificate (SRC): |  |  | Aug 07, 2020 |  | Oct 13, 2022 |  |  | Sep 15, 2025 |
| 2.3 | Safety Construction Certificate (SCC): |  |  | Aug 07, 2020 |  | Oct 13, 2022 |  |  | Sep 15, 2025 |
| 2.4 | International Loadline Certificate (ILC): |  |  | Aug 07, 2020 |  | Oct 13, 2022 |  |  | Sep 15, 2025 |
| 2.5 | International Oil Pollution Prevention Certificate (IOPPC): |  |  | Aug 07, 2020 |  | Oct 13, 2022 |  |  | Sep 15, 2025 |
| # 2.6 | # International Ship Security Certificate (ISSC): |  |  | # Nov 02, 2020 |  |  |  |  | Jan 11, 2026 |
| # 2.7 | # Maritime Labour Certificate (MLC): |  |  | # Nov 02, 2020 |  | # N/A |  |  | Jan 11, 2026 |
| 2.8 | ISM Safety Management Certificate (SMC): |  |  | Nov 02, 2020 |  |  |  |  | Jan 11, 2026 |
| # 2.9 | # Document of Compliance (DOC): |  |  | # Nov 25, 2022 |  |  |  |  | Jan 28, 2028 |
| 2.10 | USCG Certificate of Compliance(USCGCOC): |  |  | Apr 06, 2022 |  |  |  |  | Apr 06, 2024 |
| 2.11 | Civil Liability Convention (CLC) 1992 Certificate: |  |  | Feb 20, 2023 |  | N/A |  | N/A | Feb 20, 2024 |
| 2.12 | Civil Liability for Bunker Oil Pollution Damage Convention (CLBC) Certificate: |  |  | Feb 20, 2023 |  | N/A |  | N/A | Feb 20, 2024 |
| # 2.13 | # Liability for the Removal of Wrecks Certificate (WRC): |  |  | # Feb 20, 2023 |  | N/A |  | N/A | Feb 20, 2024 |
| 2.14 | U.S. Certificate of Financial Responsibility (COFR): |  |  | Apr 28, 2021 |  | N/A |  | N/A | Apr 28, 2024 |
| 2.15 | Certificate of Class (COC): |  |  | Aug 07, 2020 |  | Oct 13, 2022 |  |  | Sep 15, 2025 |
| # 2.16 | # International Sewage Pollution Prevention Certificate (ISPPC): |  |  | # Aug 07, 2020 |  | # N/A |  | # N/A | Sep 15, 2025 |
| # 2.17 | # Certificate of Fitness (COF): |  |  | # Dec 30, 2020 |  | Oct 13, 2022 |  |  | Sep 15, 2025 |
| # 2.18 | # International Energy Efficiency Certificate (IEEC): |  |  | # Aug 07, 2020 |  | N/A |  | N/A | N/A |
| # 2.19 | # International Air Pollution Prevention Certificate (IAPPC): |  |  | # Apr 26, 2023 |  |  |  |  | Sep 15, 2025 |
| Documentation |  |  |  |  |  |  |  |  |  |
| 2.20 | Owner warrant that vessel is member of ITOPF and will remain so for the entire duration of this voyage/contract: |  |  |  |  |  |  | Yes |  |
| 2.21 | Does vessel have in place a Drug and Alcohol Policy complying with OCIMF guidelines for Control of Drugs and Alcohol Onboard Ship? |  |  |  |  |  |  | Yes |  |
| 2.22 | Is the ITF Special Agreement on board (if applicable)? |  |  |  |  |  |  | Yes |  |
| 2.23 | ITF Blue Card expiry date (if applicable): |  |  |  |  |  |  | Jan 31, 2024 |  |
| 3. | CREW |  |  |  |  |  |  |  |  |
| 3.1 | Nationality of Master: |  |  |  |  |  |  | Indian |  |
| 3.2 | Number and nationality of Officers: |  |  |  |  | 11 |  | Indian, Montenegrin, Croatian |  |
| 3.3 | Number and nationality of Crew: |  |  |  |  | 12 |  | Indian |  |
| 3.4 | What is the common working language onboard: |  |  |  |  |  |  | English |  |
| 3.5 | Do officers speak and understand English? |  |  |  |  |  |  | Yes |  |
| 3.6 | If Officers/ratings employed by a manning agency - Full style: |  |  | Officers: SMM (I) Pvt Ltd/Astor Ship Management Scorpio Marine Management (I) Pvt Ltd ,Scorpio House, Central Avenue, Hiranandani Business Park, Powai, Mumbai : 400076. India Astor Ship Management ,15, Ieriku Street, 1st Building LV-1084 Riga, Latvia Tel: +91 22 61504300/+371 66 06 59 70 Fax: +91 22 61504350/+371 66 06 53 73 Email: crew.operations@scorpiogroup.net/info@astorship.co |  |  |  | Ratings: Scorpio Marine Management (India) Pvt. Ltd. Scorpio House, Central Avenue, Hiranandani Business Park, Powai, Mumbai : 400076. India Tel: +91 22 61504300 Fax: +91 22 61504350 Email: crew.operations@scorpiogroup.net |  |
| 4. | FOR USA CALLS |  |  |  |  |  |  |  |  |
| 4.1 | Has the vessel Operator submitted a Vessel Spill Response Plan to the US Coast Guard which has been approved by official USCG letter? |  |  |  |  |  |  | Yes |  |
| 4.2 | Qualified individual (QI) - Full style: |  |  |  |  | ECM Maritime Services LLC 43 Danbury Road, Suite A Wilton, CT 06897, USA Tel: +1 203 857 0444/+1 281 464 3328 Fax: +1 203 857 0428 Email: qi@ecmmaritime.com/ ecm@ecmmaritime.com |  |  |  |
| 4.3 | Oil Spill Response Organization (OSRO) - Full style: |  |  |  |  | National Response Corporation 3500 Sunrise Highway Suite T103 Great River, NY 11739 Tel: +1 800 899 4672 Fax: +1 631 224 9141 Email: iocdo@oopsusa.com |  |  |  |
| 4.4 | Salvage and Marine Firefighting Services (SMFF) - Full Style: |  |  |  |  | Resolve Salvage & Fire (Americans), Inc. 1510 SE 17th Street, Suite 400 Fort Lauderdale, Fl 33316 Tel: +1.954.764.8700 Fax: +1.954.764.8724 Email: EMX@resolvemarine.com |  |  |  |
| 5. | SAFETY/HELICOPTER |  |  |  |  |  |  |  |  |
| 5.1 | Is the vessel operated under a Quality Management System? If Yes, what type of system? (ISO9001 or IMO Resolution A.741(18) as amended): |  |  |  |  |  |  | Yes IMO Resolution A.741(18) |  |
| 5.2 | Can the ship comply with the ICS Helicopter Guidelines? |  |  |  |  |  |  | Yes |  |
| 5.2.1 | If Yes, state whether winching or landing area provided: |  |  |  |  |  |  | Winching |  |
| 5.2.2 | If Yes, what is the diameter of the circle provided: |  |  |  |  |  |  | 5.00 Metres |  |
| 6. | COATING/ANODES |  |  |  |  |  |  |  |  |
| 6.1 | Tank Coating |  |  | Coated |  | Type |  | To What Extent | Anodes |
|  | Cargo tanks: |  |  | Yes |  | Phenolic Epoxy |  | Whole Tank | No |
|  | Ballast tanks: |  |  | Yes |  | Epoxy |  |  | No |
|  | Slop tanks: |  |  | Yes |  | Epicon T-800 Grey |  | Whole Tank | No |
| 7. | BALLAST |  |  |  |  |  |  |  |  |
| 7.1 | Pumps |  |  |  | No. | Type |  | Capacity | At What Head (sg=1.0) |
|  | Ballast Pumps: |  |  |  | 2 | Centrifugal & Submerged |  | 750 Cu. Metres/Hour | 30 Metres |
|  | Ballast Eductors: |  |  |  | 0 |  |  |  |  |
| 8. | CARGO |  |  |  |  |  |  |  |  |
| Double Hull Vessels |  |  |  |  |  |  |  |  |  |
| 8.1 | Is vessel fitted with centerline bulkhead in all cargo tanks? If Yes, solid or perforated: |  |  |  |  |  |  | Yes, Solid |  |
| Cargo Tank Capacities |  |  |  |  |  |  |  |  |  |
| 8.2 | Number of cargo tanks and total cubic capacity (max% per company policy: 98%, 97%, 96% or 95%) excluding slops tanks: |  |  |  |  |  |  | 12 | 52,434.31 Cu. Metres |
| 8.2.1 | Capacity (max% per company policy: 98%, 97%, 96% or 95%) of each natural segregation with double valve (specify tanks): |  |  |  |  |  |  | Seg # 1 : 1W – 5999.188 m3 ( 1P/1S ) Seg # 2 : 2W – 9125.408 m3 ( 2P/2S ) Seg # 3 : 3W – 9618.474 m3 ( 3P/3S ) Seg # 4 : 4W – 9619.964 m3 ( 4P/4S ) Seg # 5 : 5W – 9617.524 m3 ( 5P/5S ) Seg # 6 : 6W – 8453.754 m3 ( 6P/6S ) Seg # 7 : Slop W – 1119.337 m3 ( SLP P/ SLP S ) |  |
| 8.2.2 | IMO class (Oil/Chemical Ship Type 1, 2 or 3): |  |  |  |  |  |  |  |  |
| 8.3 | Number of slop tanks and total cubic capacity (max% per company policy: 98%, 97%, 96% or 95%): |  |  |  |  |  |  | 2 | 1,119.337 Cu. Metres |
| 8.3.1 | Specify segregations which slops tanks belong to and their capacity with double valve: |  |  |  |  |  |  | Double Valve / P – 653.621 m3 Double Valve / S – 465.716 m3 |  |
| 8.3.2 | Residual/retention oil tank(s) capacity (98%), if applicable: |  |  |  |  |  |  | 166.286 Cu. Metres |  |
| SBT Vessels |  |  |  |  |  |  |  |  |  |
| 8.3.3 | What is total SBT capacity and percentage of SDWT vessel can maintain? |  |  |  |  |  |  | 22,443.50 Cu. Metres | 46.67 % |
| 8.3.4 | Does vessel meet the requirements of MARPOL Annex I Reg 18.2: |  |  |  |  |  |  | Yes |  |
| Cargo Handling and Pumping Systems |  |  |  |  |  |  |  |  |  |
| 8.4 | How many grades/products can vessel load/discharge with double valve segregation: |  |  |  |  |  |  | 7 |  |
| 8.4.1 | State type of cargo containment (integral, independent, gravity or pressure tanks): |  |  |  |  |  |  | 2P (Integral Pressure) |  |
| 8.5 | Are there any cargo tank filling restrictions? If yes, specify number of slack tanks, max s.g., ullage restrictions etc.: |  |  |  |  |  |  | Yes Max S.g of cargo =1.55, (filling height =66%of tank height) |  |
| 8.6 | Max loading rate for homogenous cargo |  |  |  |  |  |  | With VECS | Without VECS |
|  | Loaded per manifold connection: |  |  |  |  |  |  | 3,054 Cu. Metres/Hour | 3,054 Cu. Metres/Hour (Simultaneous loading in at least 6 Cargo tanks) |
|  | Loaded simultaneously through all manifolds: |  |  |  |  |  |  | 3,600 Cu. Metres/Hour | 3,600.00 Cu. Metres/Hour |
| Cargo Control Room |  |  |  |  |  |  |  |  |  |
| 8.7 | Is ship fitted with a Cargo Control Room (CCR)? |  |  |  |  |  |  | Yes |  |
| 8.8 | Can tank innage/ullage be read from the CCR? |  |  |  |  |  |  | Yes |  |
| Gauging and Sampling |  |  |  |  |  |  |  |  |  |
| 8.9 | Is gauging system certified and calibrated? If no, specify which ones are not calibrated: |  |  |  |  |  |  | Yes, NA |  |
|  | What type of gauging system as per IBC 13.1 is fitted (Open/Restricted/Closed )? |  |  |  |  |  |  | Closed |  |
|  | What type of fixed closed tank gauging system is fitted: |  |  |  |  |  |  | Radar |  |
|  | Is a tank overflow control system fitted? If yes, then state if system includes automatic closing of valves? |  |  |  |  |  |  | Yes, Yes |  |
|  | Are high level alarms fitted to the cargo tanks? If Yes, indicate whether to all tanks or partial: |  |  |  |  |  |  | Yes, All |  |
| 8.9.1 | Can cargo be transferred under closed loading conditions in accordance with ISGOTT 11.1.6.6? |  |  |  |  |  |  | Yes |  |
| 8.9.2 | Are cargo tanks fitted with multipoint gauging? If yes, specify type and locations: |  |  |  |  |  |  | Yes, MMC / FWD, MID,AFT |  |
| 8.10 | Number of portable gauging units (example- MMC) on board: |  |  |  |  |  |  | 3 |  |
| Vapor Emission Control System (VECS) |  |  |  |  |  |  |  |  |  |
| 8.11 | Is a vapour return system (VRS) fitted? |  |  |  |  |  |  | Yes |  |
| 8.12 | Number/size of VECS manifolds (per side): |  |  |  |  |  |  | 2 | 300 Millimetres |
| 8.13 | Number/size/type of VECS reducers: |  |  |  |  |  |  | 4 X 12” & 2 X 16” |  |
| Venting |  |  |  |  |  |  |  |  |  |
| 8.14 | State what type of venting system is fitted: |  |  |  |  |  |  | High Velocity (PV Valves) |  |
| Cargo Manifolds and Reducers |  |  |  |  |  |  |  |  |  |
| 8.15 | Total number/size of cargo manifold connections on each side: |  |  |  |  |  |  | 12/305 Millimetres |  |
| 8.15.1 | Does the vessel have a Common Line Manifold connection? If yes, describe: |  |  |  |  |  |  | 7 on each side /400 Millimetres |  |
| 8.16 | What type of valves are fitted at manifold: |  |  |  |  |  |  | Manual Butterfly |  |
| 8.17 | What is the material/rating of the manifold: |  |  |  |  |  |  | Stainless Steel/24 |  |
| 8.17.1 | Does vessel comply with the latest edition of the OCIMF 'Recommendations for Oil Tanker Manifolds and Associated Equipment’? |  |  |  |  |  |  | Yes |  |
| 8.18 | Distance between cargo manifold centers: |  |  |  |  |  |  | 2,000.00 Millimetres |  |
| 8.19 | Distance ships rail to manifold: |  |  |  |  |  |  | 4,200.00 Millimetres |  |
| 8.20 | Distance manifold to ships side: |  |  |  |  |  |  | 4,600.00 Millimetres |  |
| 8.21 | Top of rail to center of manifold: |  |  |  |  |  |  | 700.00 Millimetres |  |
| 8.22 | Distance main deck to center of manifold: |  |  |  |  |  |  | 2,100.00 Millimetres |  |
| 8.23 | Spill tank grating to center of manifold: |  |  |  |  |  |  | 900.00 Millimetres |  |
| 8.24 | Manifold height above the waterline in normal ballast/at SDWT condition: |  |  |  |  |  |  | 14.28 Metres | 9.20 Metres |
| 8.25 | Number/size/type of reducers: |  |  |  |  |  |  | 12 x 300/400mm (12/16") 2 x 200/400mm (8/16") 6 x 300/300mm (12/12") 6 x 300/250mm (12/10") 6 x 300/200mm (12/8") (1x200/300(8" x 12") 1x200/250(8" x 10") 1x200/200(8" x 8")) ANSI |  |
| 8.26 | Is vessel fitted with a stern manifold? If yes, state size: |  |  |  |  |  |  | No, 0 Millimetres |  |
| Heating |  |  |  |  |  |  |  |  |  |
| 8.27 | Cargo/slop tanks fitted with a cargo heating system? |  |  |  |  | Type |  | Coiled | Material |
|  | Cargo Tanks: |  |  |  |  | Deck Mounted Heat Exchanger |  | No | SS |
|  | Slop Tanks: |  |  |  |  | HEATING COILS |  | Yes | SS |
| 8.27.1 | Is a Thermal Oil Heating system fitted? If yes, identify tanks? |  |  |  |  |  |  | No, NA |  |
| 8.28 | Maximum temperature cargo can be loaded/maintained: |  |  |  |  |  |  | 75.0 °C / 167.0 °F | 60 °C / 140 °F |
| 8.28.1 | Minimum temperature cargo can be loaded/maintained: |  |  |  |  |  |  | NA |  |
| Inert Gas and Crude Oil Washing |  |  |  |  |  |  |  |  |  |
| 8.29 | Is an Inert Gas System (IGS) fitted/operational? |  |  |  |  |  |  | Yes/Yes |  |
| 8.29.1 | Is a Crude Oil Washing (COW) installation fitted/operational? |  |  |  |  |  |  | Yes/Yes |  |
| 8.30 | Is IGS supplied by flue gas, inert gas (IG) generator and/or nitrogen: |  |  |  |  |  |  | IG Generator |  |
| 8.30.1 | If nitrogen generator, specify the applicable flow rate for each of the designed purity modes: |  |  |  |  |  |  | No |  |
| Cargo Pumps |  |  |  |  |  |  |  |  |  |
| 8.31 | How many cargo pumps can be run simultaneously at full capacity: |  |  |  |  |  |  | 6 |  |
| 8.32 | Pumps |  |  | No. |  | Type |  | Capacity | At What Head (sg=1.0) |
|  | Cargo Pumps: |  |  | 12 2 |  | Submerdged Submerdged |  | 600 M3/HR 300 M3/HR | 125 Meters |
|  | Cargo Eductors: |  |  | 0 |  |  |  |  |  |
|  | Stripping: |  |  | 0 |  |  |  |  |  |
| 8.33 | Is at least one emergency portable cargo pump provided? |  |  |  |  |  |  | Yes |  |
| Tank Cleaning Systems |  |  |  |  |  |  |  |  |  |
| 8.34 | Is tank cleaning equipment fixed in cargo tanks? |  |  |  |  |  |  | Yes |  |
| 8.35 | Is portable tank cleaning equipment provided? |  |  |  |  |  |  | Yes |  |
| 8.36 | Tank washing pump capacity: |  |  |  |  |  |  | 120.00 Cu. Metres/Hour |  |
| 8.37 | Is a washing water heater fitted? If yes is it operational and state max washing water temperature: |  |  |  |  |  |  | Yes, Yes 75.00 Degrees Celsius |  |
| 8.38 | What is the maximum number of machines that can be operated at their designed max pressure? |  |  |  |  |  |  | 4 |  |
| Other Deck Equipment |  |  |  |  |  |  |  |  |  |
| 8.39 | Is vessel fitted with a remote cargo tank temperature monitoring system. If yes, is it operational? |  |  |  |  |  |  | Yes, Yes |  |
| 8.40 | Is vessel fitted with a remote cargo tank pressure monitoring system. If yes, is it operational? |  |  |  |  |  |  | Yes, Yes |  |
| 8.41 | Is vessel fitted with a cargo tank drier. If yes is it operational and state capacity: |  |  |  |  |  |  | No, No |  |
| 8.42 | Is vessel fitted with a cargo cooling system. If yes is it operational and state tanks applicable: |  |  |  |  |  |  | No, No NA |  |
| 8.43 | Is steam available on deck? |  |  |  |  |  |  | Yes |  |
| 9. | MOORING |  |  |  |  |  |  |  |  |
| 9.1 | Wires (on drums) |  | No. | Diameter |  | Material |  | Length | Breaking Strength |
|  | Forecastle: |  | 0 |  |  |  |  |  |  |
|  | Main deck fwd: |  | 0 |  |  |  |  |  |  |
|  | Main deck aft: |  | 0 |  |  |  |  |  |  |
|  | Poop deck: |  | 0 |  |  |  |  |  |  |
| 9.2 | Wire tails |  | No. | Diameter |  | Material |  | Length | Breaking Strength |
|  | Forecastle: |  | 4 | 56.00 Millimetres |  | 50% H.T Polyester - 50% H.T.Olefin |  | 11.00 Metres | 68 Metric Tonnes |
|  | Main deck fwd: |  | 4 | 56.00 Millimetres |  | 50% H.T Polyester - 50% H.T.Olefin |  | 11.00 Metres | 68 Metric Tonnes |
|  | Main deck aft: |  | 2 | 56.00 Millimetres |  | 50% H.T Polyester - 50% H.T.Olefin |  | 11.00 Metres | 68 Metric Tonnes |
|  | Poop deck: |  | 6 | 56.00 Millimetres |  | 50% H.T Polyester - 50% H.T.Olefin |  | 11.00 Metres | 68 Metric Tonnes |
| 9.3 | Ropes (on drums) |  | No. | Diameter |  | Material |  | Length | Breaking Strength |
|  | Forecastle: |  | 4 | 26.00 Millimetres |  | Kapaneema (Ultra High Modulus PolyethyleneFiberRope 75 Dyneema Core) |  | 220.00 Metres | 53 Metric Tonnes |
|  | Main deck fwd: |  | 4 | 26.00 Millimetres |  | Kapaneema (Ultra High Modulus PolyethyleneFiberRope 75 Dyneema Core) |  | 220.00 Metres | 53 Metric Tonnes |
|  | Main deck aft: |  | 2 | 26.00 Millimetres |  | Kapaneema (Ultra High Modulus PolyethyleneFiberRope 75 Dyneema Core) |  | 220.00 Metres | 53 Metric Tonnes |
|  | Poop deck: |  | 6 | 26.00 Millimetres |  | Kapaneema (Ultra High Modulus PolyethyleneFiberRope 75 Dyneema Core) |  | 220.00 Metres | 53 Metric Tonnes |
| 9.4 | Other lines |  | No. | Diameter |  | Material |  | Length | Breaking Strength |
|  | Forecastle: |  | 4 | 52 Millimetres |  | 25% Polyester - 75% Polysteel (Polyester (40%) / Nikasteel (60%)) |  | 220 Metres | 53.5 &amp; 53.0 |
|  | Main deck fwd: |  | 3 | 48 Millimetres |  | Polyester & Polyolefin |  | 220.00 Metres | 50.50 Metric Tonnes |
|  | Main deck aft: |  | 2 | 48 Millimetres |  | Polyester & Polyolefin |  | 220.00 Metres | 50.50 Metric Tonnes |
|  | Poop deck: |  | 4 | 52 Millimetres (2x48) |  | 25% Polyester - 75% Polysteel (Polyester (40%) / Nikasteel (60%)) |  | 220 Metres | 53.5 &amp; 53.0 |
| 9.5 | Winches |  | No. | No. Drums |  | Motive Power |  | Brake Capacity | Type of Brake |
|  | Forecastle: |  | 2 | Double Drums |  | Hydraulic |  | 42 Metric Tonnes | MANUAL |
|  | Main deck fwd: |  | 2 | Double Drums |  | Hydraulic |  | 42 Metric Tonnes | MANUAL |
|  | Main deck aft: |  | 1 | Double Drums |  | Hydraulic |  | 42 Metric Tonnes | MANUAL |
|  | Poop deck: |  | 3 | Double Drums |  | Hydraulic |  | 42 Metric Tonnes | MANUAL |
| 9.6 | Bitts, closed chocks/fairleads |  |  | No. Bitts |  | SWL Bitts |  | No. Closed Chocks | SWL Closed Chocks |
|  | Forecastle: |  |  | 4 |  | 53 Metric Tonnes (64) |  | 6 | 64 Metric Tonnes (53) |
|  | Main deck fwd: |  |  | 7 |  | 53 Metric Tonnes |  | 13 | 53 Metric Tonnes |
|  | Main deck aft: |  |  | 5 |  | 53 Metric Tonnes |  | 7 | 53 Metric Tonnes |
|  | Poop deck: |  |  | 10 |  | 64/55/53 |  | 17 | 64/53 |
| Anchors/Emergency Towing System |  |  |  |  |  |  |  |  |  |
| 9.7 | Number of shackles on port/starboard cable: |  |  |  |  |  |  | 11/12 |  |
| 9.8 | Type/SWL of Emergency Towing system forward: |  |  |  |  |  |  | Tongue stopper + Chain / ETS-4000FSR-SJ | 204 Metric Tonnes |
| 9.9 | Type/SWL of Emergency Towing system aft: |  |  |  |  |  |  | Storage Drum / KBETS-4000AS-SJ | 204 Metric Tonnes |
| 9.10.1 | What is size of closed chock and/or fairleads of enclosed type on stern |  |  |  |  |  |  | 600 Millimeters x 450 Millimeters |  |
| Escort Tug |  |  |  |  |  |  |  |  |  |
| 9.10.2 | What is SWL of closed chock and/or fairleads of enclosed type on stern: |  |  |  |  |  |  | 204.00 Metric Tonnes |  |
| 9.11 | What is SWL of bollard on poop deck suitable for escort tug: |  |  |  |  |  |  | 204.00 Metric Tonnes |  |
| Lifting Equipment/Gangway |  |  |  |  |  |  |  |  |  |
| 9.12 | Derrick/Crane description (Number, SWL and location): |  |  |  |  |  |  | Cranes: 1 x 10.00 Tonnes Center |  |
| 9.13 | Accommodation ladder direction: |  |  |  |  |  |  | Aft |  |
|  | Does vessel have a portable gangway? If yes, state length: |  |  |  |  |  |  | Yes, 15 Metres |  |
| Single Point Mooring (SPM) Equipment |  |  |  |  |  |  |  |  |  |
| 9.14 | Does the vessel meet the recommendations in the latest edition of OCIMF 'Recommendations for Equipment Employed in the Bow Mooring of Conventional Tankers at Single Point Moorings (SPM)’:? |  |  |  |  |  |  | Yes |  |
| 9.15 | If fitted, how many chain stoppers: |  |  |  |  |  |  | 1 |  |
| 9.16 | State type/SWL of chain stopper(s): |  |  |  |  |  |  | Tongue Type | 204.00 Metric Tonnes |
| 9.17 | What is the maximum size chain diameter the bow stopper(s) can handle: |  |  |  |  |  |  | 76.00 Millimetres |  |
| 9.18 | Distance between the bow fairlead and chain stopper/bracket: |  |  |  |  |  |  | 3.44 Metres |  |
| 9.19 | Is bow chock and/or fairlead of enclosed type of OCIMF recommended size (600mm x 450mm)? If not, give details of size: |  |  |  |  |  |  | Yes NA |  |
| 10. | PROPULSION |  |  |  |  |  |  |  |  |
| 10.1 | Speed |  |  |  |  |  |  | Maximum | Economical |
|  | Ballast speed: |  |  |  |  |  |  | 14.50 Knots (WSNP) | 13.00 Knots (WSNP) |
|  | Laden speed: |  |  |  |  |  |  | 14 Knots (WSNP) | 12.50 Knots (WSNP) |
| 10.2 | What type of fuel is used for main propulsion/generating plant: |  |  |  |  |  |  |  | MARINE FUEL OIL 380 CST |
| 10.3 | Type/Capacity of bunker tanks: |  |  |  |  |  |  | Fuel Oil: 1,173.50 Cu. Metres Diesel Oil: 526.80 Cu. Metres Gas Oil: 0 Cu. Metres |  |
| 10.4 | Is vessel fitted with fixed or controllable pitch propeller(s): |  |  |  |  |  |  | Fixed |  |
| 10.5 | Engines |  |  |  |  |  | No | Capacity | Make/Type |
|  | Main engine: |  |  |  |  |  | 1 | 8,090 Kilowatt | Hyundai-B&W 6G50ME-B9.3(Tier II) |
|  | Aux engine: |  |  |  |  |  | 3 | 850 Kilowatt | Hyundai Himsen 6H21/32 |
|  | Power packs: |  |  |  |  |  | 4 | 194 Cu. Metres/Hour | Cummins KTA19-DM1 |
|  | Boilers: |  |  |  |  |  | 2 | 18 Metric Tonnes/Hour | Manf: Aalborg 1.Aalborg OL Mission 2. Aalborg OC |
| Bow/Stern Thruster |  |  |  |  |  |  |  |  |  |
| 10.6 | What is brake horse power of bow thruster (if fitted): |  |  |  |  |  |  | No, |  |
| 10.7 | What is brake horse power of stern thruster (if fitted): |  |  |  |  |  |  | No, |  |
| Emissions |  |  |  |  |  |  |  |  |  |
| 10.8 | Main engine IMO NOx emission standard: |  |  |  |  |  |  | Tier II |  |
| 10.9 | Energy Efficiency Design Index (EEDI) rating number: |  |  |  |  |  |  | 4.52 |  |
| 11. | SHIP TO SHIP TRANSFER |  |  |  |  |  |  |  |  |
| 11.1 | Does vessel comply with recommendations contained in OCIMF/ICS Ship To Ship Transfer Guide (Petroleum, Chemicals or Liquified Gas, as applicable)? |  |  |  |  |  |  | Yes |  |
| 11.2 | What is maximum outreach of cranes/derricks outboard of the ship’s side: |  |  |  |  |  |  | 7.90 Metres |  |
| 11.3 | Date/place of last STS operation: |  |  |  |  |  |  | 28 Feb 2021 ,Lagos, Nigeria |  |
| 12. | RECENT OPERATIONAL HISTORY |  |  |  |  |  |  |  |  |
| 12.1 | Last three cargoes/charterers/voyages (Last/2nd Last/3rd Last): |  |  |  |  |  |  | last : Jet A1 // Charterer // PETROCHINA INTERNATIONAL (HONG KONG) CORPORATION LIMITED // Load Port : JINZHOU, CHINA ; Disport : PAFF TERMINAL, HONG KONG 2nd Last : GASOIL / ULP / PULP ; Charterer name : AMPOL SINGAPORE ; Load port : YEOSU & Disport Name (TBA) : MOUNT MAUNGANUI, NELSON, NAPIER, , LYTTELTON, DUNEDIN 3rd Last JET A-1, ULP, PULP // Charterer // AMPOL // Load Port : KERTIH & TANJUNG LANGSAT ; Disport : MARSDEN POINT |  |
| 12.2 | Has vessel been involved in a pollution, grounding, serious casualty, unscheduled repair or collision incident during the past 12 months? If yes, provide details: |  |  |  |  |  |  | Pollution: No, NA Grounding: No, NA Casualty: N/A, NA Repair: No, Collision: No, NA |  |
| 12.3 | Date and place of last Port State Control inspection: |  |  |  |  |  |  | Feb 27, 2023 / Tianjin, China |  |
| 12.4 | Any outstanding deficiencies as reported by any Port State Control? If yes, provide details: |  |  |  |  |  |  | No NA |  |
| 12.5 | Recent Oil company inspections/screenings (To the best of owners knowledge and without guarantee of acceptance for future business)*: * "Approvals" are not given by Oil Majors and ships are accepted for the voyage on a case by case basis. |  |  |  |  |  |  | AMPOL, Equinor , Idemitsu |  |
| 12.6 | Date/Place of last SIRE inspection: |  |  |  |  |  |  | Aug 20, 2023 / Hong kong |  |
| 12.6.1 | Date/Place of last CDI inspection: |  |  |  |  |  |  | N/A |  |
| 12.7 | Additional information relating to features of the ship or operational characteristics: |  |  |  |  |  |  | None |  |

Revised 2018 (INTERTANKO/Q88.com)

Form completed on http://www.q88.com/integration.aspx  Please email support@q88.com an updated copy if this is not the latest version.