- INTERTANKO CHARTERING QUESTIONNAIRE 88 - OIL/CHEMICAL	Version 5

| 1. | GENERAL INFORMATION |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1.1 | Date updated: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 05th Dec 2023 |  |  |  |  |  |
| 1.2 | Vessel’s name (IMO number): |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | NO.2 HEUNG-A PIONEER (9415480) |  |  |  |  |  |
| 1.3 | Vessel’s previous name(s) and date(s) of change: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  |
| 1.4 | Date delivered/Builder (where built): |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 12th Nov, 2008 NOKBONG SHIPBUILDING CO., LTD. KOREA |  |  |  |  |  |
| 1.5 | Flag/Port of Registry: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | PANAMA |  |  |  |  |  |
| 1.6 | Call sign/MMSI: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 3EBD8 / 356471000 |  |  |  |  |  |
| 1.7 | Vessel’s contact details (satcom/fax/email etc.): |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Satcom : +870773212144 Fax : +870783005816 E-mail : 2hap@sksat.kr |  |  |  |  |  |
| 1.8 | Type of vessel (as described in Form A or Form B Q1.11 of the IOPPC): |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Product carrier (OIL / CHEMICAL TANKER) |  |  |  |  |  |
| 1.9 | Type of hull: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Double Hull |  |  |  |  |  |
| Ownership and Operation |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1.10 | Registered owner - Full style: |  |  |  |  |  |  |  |  |  | HA 2HAP S.A. PASEO DEL MAR AND PACIFIC AVENUES, COSTA DEL ESTE, MMG TOWER, 23rd FLOOR, PANAMA CITY, REPUBLIC OF PANAMA TEL : +82-2-3449-3296 FAX : +82-2-3449-3391 |  |  |  |  |  |  |  |  |  |  |  |
| 1.11 | Technical operator - Full style: |  |  |  |  |  |  |  |  |  | PTS Co., Ltd. 16F, 102, Jungang-Daero, Jung-Gu, Busan, Republic of Korea. TEL : +82-51-460-7240~5 FAX : +82-51-460-7288 E-MAIL : safety@pioneerts.co.kr |  |  |  |  |  |  |  |  |  |  |  |
| 1.12 | Commercial operator - Full style: |  |  |  |  |  |  |  |  |  | HEUNG-A SHIPPING CO., LTD. 5F, Premier Place, 8, Cheonggyecheon-ro, Jung-gu, Seoul, 04520, Korea TEL : +82-2-3449-3081 FAX : +82-70-7950-4675 E-MAIL : tkt@heung-a.co.kr |  |  |  |  |  |  |  |  |  |  |  |
| 1.13 | Commercial operator - Full style: |  |  |  |  |  |  |  |  |  | HEUNG-A SHIPPING CO., LTD. 5F, Premier Place, 8, Cheonggyecheon-ro, Jung-gu, Seoul, 04520, Korea TEL : +82-2-3449-3081 FAX : +82-70-7950-4675 E-MAIL : tkt@heung-a.co.kr |  |  |  |  |  |  |  |  |  |  |  |
| Insurance |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1.14 | P & I Club - Full Style: |  |  |  |  |  |  |  |  |  | Steamship Mutual Underwriting Association Limited |  |  |  |  |  |  |  |  |  |  |  |
| 1.15 | P & I Club pollution liability coverage/expiration date: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | USD1,000,000,000 |  |  |  | 20th Feb 2024 |  |
| 1.16 | Hull & Machinery insured by - Full Style: (Specify broker or leading underwriter) |  |  |  |  |  |  |  |  |  | KB Insurance Co., Ltd |  |  |  |  |  |  |  |  |  |  |  |
| 1.17 | Hull & Machinery insured value/expiration date: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | USD 17,400,000 |  |  |  | 25th Mar 2024 |  |
| Classification |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1.18 | Classification society: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Korean Register of Shipping (KRS) |  |  |  |  |  |
| 1.19 | Class notation: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | +KRS1 OIL/CHEMICAL TANKER(DOUBLE HULL) 'ESP' (FBC)PRODUCT/II 2G / 1.6SG (IBC), CLEAN1 ERS LG LI +KRM1 , BWT VEC1 |  |  |  |  |  |
| 1.20 | Is the vessel subject to any conditions of class, class extensions, outstanding memorandums or class recommendations? If yes, give details: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | No |  |  |  |  |  |
| 1.21 | If classification society changed, name of previous and date of change: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  |
| 1.22 | Does the vessel have ice class? If yes, state what level: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | No |  |  |  |  |  |
| 1.23 | Date/place of last dry-dock: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 30th NOV 2023 / Busan, Korea |  |  |  |  |  |
| 1.24 | Date next dry dock due/next annual survey due: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 30th Nov 2026 |  |  |  | 11th Nov 2024 |  |
| 1.25 | Date of last special survey/next special survey due: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 30th NOV 2023 |  |  |  | 11th Nov 2028 |  |
| 1.26 | If ship has Condition Assessment Program (CAP), what is the latest overall rating: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  |
| Dimensions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1.27 | Length overall (LOA): |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 124.50 Metres |  |  |  |  |  |
| 1.28 | Length between perpendiculars (LBP): |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 116.00 Metres |  |  |  |  |  |
| 1.29 | Extreme breadth (Beam): |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 20.40 Metres |  |  |  |  |  |
| 1.30 | Moulded depth: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 11.80 Metres |  |  |  |  |  |
| 1.31 | Keel to masthead (KTM)/ Keel to masthead (KTM) in collapsed condition, if applicable: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 39.25 Metres |  |  |  | Metres |  |
| 1.32 | Distance bridge front to center of manifold: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 33.27 Metres |  |  |  |  |  |
| 1.33 | Bow to center manifold (BCM)/Stern to center manifold (SCM): |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 62.98 Metres |  |  |  | 61.52 Metres |  |
| 1.34 | Parallel body distances |  |  |  |  |  |  |  |  |  | Lightship |  |  |  |  |  | Normal Ballast |  |  |  | Summer Dwt |  |
|  | # Forward to mid-point manifold: |  |  |  |  |  |  |  |  |  | 19.41 Metres |  |  |  |  |  | 27.02 Metres |  |  |  | 31.787 Metres |  |
|  | # Aft to mid-point manifold: |  |  |  |  |  |  |  |  |  | 23.37 Metres |  |  |  |  |  | 28.012 Metres |  |  |  | 34.873 Metres |  |
|  | # Parallel body length: |  |  |  |  |  |  |  |  |  | 42.78 Metres |  |  |  |  |  | 55.032 Metres |  |  |  | 66.66 Metres |  |
| Tonnages |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1.35 | Net Tonnage: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 3,642 Metric Tons |  |  |  |  |  |
| 1.36 | Gross Tonnage/Reduced Gross Tonnage (if applicable): |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 8,271 Metric Tons |  |  |  | N/A |  |
| 1.37 | Suez Canal Tonnage - Gross (SCGT)/Net (SCNT): |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  |
| # 1.38 | Panama Canal Net Tonnage (PCNT): |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  |
| # Loadline Information |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| # 1.39 | # Loadline |  |  |  |  | # Freeboard |  |  | # Draft |  |  |  |  |  |  | # Deadweight |  |  | # Displacement |  |  |  |
|  | # Summer: |  |  |  |  | # 3.212 Metres |  |  | # 8.614 Metres |  |  |  |  |  |  | # 12,301.555 Metric Tons |  |  | # 16,453.446 Metric Tons |  |  |  |
|  | # Winter: |  |  |  |  | # 3.391 Metres |  |  | # 8.435 Metres |  |  |  |  |  |  | # 11,903.604 Metric Tons |  |  | # 16,055.495 Metric Tons |  |  |  |
|  | # Tropical: |  |  |  |  | # 3.033 Metres |  |  | # 8.793 Metres |  |  |  |  |  |  | # 12,700.463 Metric Tons |  |  | # 16,852.354 Metric Tons |  |  |  |
|  | # Lightship: |  |  |  |  | # 9.362 Metres |  |  | # 2.482 Metres |  |  |  |  |  |  | N/A |  |  | # 4,151.891 Metric Tons |  |  |  |
|  | # Normal Ballast Condition: |  |  |  |  | # 6.426 Metres |  |  | # 5.400 Metres |  |  |  |  |  |  | 5,476.616 Metric Tons |  |  | # 9594.663 Metric Tons |  |  |  |
|  | # Segregated Ballast Condition: |  |  |  |  | # 6.426 Metres |  |  | # 5.400 Metres |  |  |  |  |  |  | 5,476.616 Metric Tons |  |  | # 9594.663 Metric Tons |  |  |  |
| # 1.40 | # FWA/TPC at summer draft: |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 185 Millimetres |  |  | 22.254 Metric Tons |  |  |  |
| # 1.41 | # Does vessel have multiple SDWT? If yes, please provide all assigned loadlines: |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes / 9,998 dwt |  |  |  |  |  |  |
| 1.42 | Constant (excluding fresh water): |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 120 Metric Tons |  |  |  |  |  |  |
| 1.43 | What is the company guidelines for Under Keel Clearance (UKC) for this vessel? |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Ocean : 50% of the current deepest draft Shallow water : 15% of the current deepest draft Port limits : 10% of the current deepest draft At or SBM/CBM mooring : 1.5% ship’s breadth or 0.6meters, Whichever is the bigger |  |  |  |  |  |  |
| 1.44 | What is the max height of mast above waterline (air draft) |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Full Mast |  |  |  |  | Collapsed Mast |  |
|  | Summer deadweight: |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 30.636 Metres |  |  |  |  | Metres |  |
|  | Normal ballast: |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 33.906 Metres |  |  |  |  | Metres |  |
|  | Lightship: |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 36.815 Metres |  |  |  |  | Metres |  |
| 2. | CERTIFICATES |  |  |  |  | Issued |  |  |  | Last Annual |  |  |  |  |  | Last Intermediate |  |  |  |  | Expires |  |
| 2.1 | Safety Equipment Certificate (SEC): |  |  |  |  | 30th Nov 2023 |  |  |  | - |  |  |  |  |  | - |  |  |  |  | 11th Nov 2028 |  |
| 2.2 | Safety Radio Certificate (SRC): |  |  |  |  | 30th Nov 2023 |  |  |  | - |  |  |  |  |  | - |  |  |  |  | 11th Nov 2028 |  |
| 2.3 | Safety Construction Certificate (SCC): |  |  |  |  | 30th Nov 2023 |  |  |  | - |  |  |  |  |  | - |  |  |  |  | 11th Nov 2028 |  |
| 2.4 | International Loadline Certificate (ILC): |  |  |  |  | 30th Nov 2023 |  |  |  | - |  |  |  |  |  | - |  |  |  |  | 11th Nov 2028 |  |
| 2.5 | International Oil Pollution Prevention Certificate (IOPPC): |  |  |  |  | 30th Nov 2023 |  |  |  | - |  |  |  |  |  | - |  |  |  |  | 11th Nov 2028 |  |
| # 2.6 | # International Ship Security Certificate (ISSC): |  |  |  |  | # 25th Aug 2021 |  |  |  | N/A |  |  |  |  |  | - |  |  |  |  | 16th Oct 2026 |  |
| # 2.7 | # Maritime Labour Certificate (MLC): |  |  |  |  | # 16th Oct 2021 |  |  |  | N/A |  |  |  |  |  | - |  |  |  |  | 26th Oct 2026 |  |
| 2.8 | ISM Safety Management Certificate (SMC): |  |  |  |  | # 23th Aug 2021 |  |  |  | N/A |  |  |  |  |  | - |  |  |  |  | 15th Oct 2026 |  |
| # 2.9 | # Document of Compliance (DOC): |  |  |  |  | # 23th May 2023 |  |  |  | 13th Apr 2023 |  |  |  |  |  | N/A |  |  |  |  | 03rd Mar 2026 |  |
| 2.10 | USCG Certificate of Compliance (USCGCOC): |  |  |  |  | N/A |  |  |  | N/A |  |  |  |  |  | N/A |  |  |  |  | N/A |  |
| 2.11 | Civil Liability Convention (CLC) 1992 Certificate: |  |  |  |  | 17th Feb 2023 |  |  |  | N/A |  |  |  |  |  | N/A |  |  |  |  | 20th Feb 2024 |  |
| 2.12 | Civil Liability for Bunker Oil Pollution Damage Convention (CLBC) Certificate: |  |  |  |  | 17th Feb 2023 |  |  |  | N/A |  |  |  |  |  | N/A |  |  |  |  | 20th Feb 2024 |  |
| # 2.13 | # Liability for the Removal of Wrecks Certificate (WRC): |  |  |  |  | 16th Feb 2023 |  |  |  | N/A |  |  |  |  |  | N/A |  |  |  |  | 20th Feb 2024 |  |
| 2.14 | U.S. Certificate of Financial Responsibility (COFR): |  |  |  |  | N/A |  |  |  | N/A |  |  |  |  |  | N/A |  |  |  |  | N/A |  |
| 2.15 | Certificate of Class (COC): |  |  |  |  | 30th Nov 2023 |  |  |  | - |  |  |  |  |  | - |  |  |  |  | 11th Nov 2028 |  |
| # 2.16 | # International Sewage Pollution Prevention Certificate (ISPPC): |  |  |  |  | 30th Nov 2023 |  |  |  | - |  |  |  |  |  | - |  |  |  |  | 11th Nov 2028 |  |
| # 2.17 | # Certificate of Fitness (COF): |  |  |  |  | 30th Nov 2023 |  |  |  | - |  |  |  |  |  | - |  |  |  |  | 11th Nov 2028 |  |
| # 2.18 | # International Energy Efficiency Certificate (IEEC): |  |  |  |  | 20th Jan 2021 |  |  |  | N/A |  |  |  |  |  | N/A |  |  |  |  | Permanence |  |
| # 2.19 | # International Air Pollution Prevention Certificate (IAPPC): |  |  |  |  | 30th Nov 2023 |  |  |  | - |  |  |  |  |  | - |  |  |  |  | 11th Nov 2028 |  |
| Documentation |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2.20 | Owner warrant that vessel is member of ITOPF and will remain so for the entire duration of this voyage/contract: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |
| 2.21 | Does vessel have in place a Drug and Alcohol Policy complying with OCIMF guidelines for Control of Drugs and Alcohol Onboard Ship? |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |
| 2.22 | Is the ITF Special Agreement on board (if applicable)? |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  |
| 2.23 | ITF Blue Card expiry date (if applicable): |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  |
| 3. | CREW |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3.1 | Nationality of Master: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Korea |  |  |  |  |  |
| 3.2 | Number and nationality of Officers: |  |  |  |  |  |  |  |  |  | 4 |  |  |  |  |  | Korea |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | 3 |  |  |  |  |  | Myanmar |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | 1 |  |  |  |  |  | Indonesia |  |  |  |  |  |
| 3.3 | Number and nationality of Crew: |  |  |  |  |  |  |  |  |  | 11 |  |  |  |  |  | Myanmar |  |  |  |  |  |
| 3.4 | What is the common working language onboard: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | English |  |  |  |  |  |
| 3.5 | Do officers speak and understand English? |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |
| 3.6 | If Officers/ratings employed by a manning agency - Full style: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | PTS Co., Ltd. 16F, 102, Jungang-Daero, Jung-Gu, Busan, Republic of Korea. TEL : +82-51-460-7250~4 FAX : +82-51-460-7288 EMAIL :marine@pioneerts.co.kr |  |  |  |  |  |
| 4. | FOR USA CALLS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4.1 | Has the vessel Operator submitted a Vessel Spill Response Plan to the US Coast Guard which has been approved by official USCG letter? |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  |
| 4.2 | Qualified individual (QI) - Full style: |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  |  |  |  |  |  |  |
| 4.3 | Oil Spill Response Organization (OSRO) - Full style: |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  |  |  |  |  |  |  |
| 4.4 | Salvage and Marine Firefighting Services (SMFF) - Full Style: |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  |  |  |  |  |  |  |
| 5. | SAFETY/HELICOPTER |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.1 | Is the vessel operated under a Quality Management System? If Yes, what type of system? (ISO9001 or IMO Resolution A.741(18) as amended): |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ISO 9001, 14001 and 45001 |  |  |  |  |  |
| 5.2 | Can the ship comply with the ICS Helicopter Guidelines? |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | No |  |  |  |  |  |
| 5.2.1 | If Yes, state whether winching or landing area provided: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  |
| 5.2.2 | If Yes, what is the diameter of the circle provided: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  |
| 6. | COATING/ANODES |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6.1 | Tank Coating |  |  |  |  | Coated |  |  |  |  | Type |  |  |  |  |  | To What Extent |  |  |  | Anodes |  |
|  | Cargo tanks: |  |  |  |  | No |  |  |  |  | SUS316L |  |  |  |  |  | All tanks |  |  |  |  |  |
|  | Ballast tanks: |  |  |  |  | Yes |  |  |  |  | EPOXY |  |  |  |  |  | All tanks |  |  |  | Zinc |  |
|  | Slop tanks: |  |  |  |  | No |  |  |  |  | SUS316L |  |  |  |  |  | All tanks |  |  |  |  |  |
| 7. | BALLAST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7.1 | Pumps |  |  |  |  | No. |  | Type |  |  |  |  |  | Capacity |  |  |  |  | At What Head (sg=1.0) |  |  |  |
|  | Ballast Pumps: |  |  |  |  | 2 |  | CENTRIFUGAL |  |  |  |  |  | 350 Cu. Metres/Hour |  |  |  |  | 25 |  |  |  |
|  | Ballast Eductors: |  |  |  |  | No |  | N/A |  |  |  |  |  | N/A |  |  |  |  | N/A |  |  |  |
| 8. | CARGO |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Double Hull Vessels |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.1 | Is vessel fitted with centerline bulkhead in all cargo tanks? If Yes, solid or perforated: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes / Solid |  |  |  |  |  |
| Cargo Tank Capacities |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.2 | Number of cargo tanks and total cubic capacity (98%): |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 16 Tanks |  |  | 13250.455 Cu.Meters |  |  |
| 8.2.1 | Capacity (98%) of each natural segregation with double valve (specify tanks): |  |  |  |  |  | Seg#1 : 611.883 m3 (NO.1 (P)), Seg#2 : 602.578 m3 (NO.1 (S)) Seg#3 : 907.620 m3 (NO.2 (P)), Seg#4 : 921.409 m3 (NO.2 (S)) Seg#5 : 941.506 m3 (NO.3 (P)), Seg#6 : 947.845 m3 (NO.3 (S)) Seg#7 : 944.089 m3 (NO.4 (P)), Seg#8 : 951.155 m3 (NO.4 (S)) Seg#9 : 1268.501 m3 (NO.5 (P)), Seg#10 : 1259.913 m3 (NO.5 (S)) Seg#11 : 944.279 m3 (NO.6 (P)), Seg#12 : 950.619 m3 (NO.6 (S)) Seg#13 : 629.970 m3 (NO.7 (P)), Seg#14 : 625.949 m3 (NO.7 (S)) Seg#15 : 372.233 m3 (SLOP (P)), Seg#16 : 370.906 m3 (SLOP (S)) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.2.2 | IMO class (Oil/Chemical Ship Type 1, 2 or 3): |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2 |  |  |  |  |  |  |
| 8.3 | Number of slop tanks and total cubic capacity (98%): |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2 Tanks |  |  |  |  | 743.139 Cu. Meters |  |
| 8.3.1 | Specify segregations which slops tanks belong to and their capacity with double valve: |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Seg#1 : 372.233 Cu. Meters (SLOP (P)) Seg#2 : 370.906 Cu. Meters (SLOP (S)) |  |  |  |  |  |  |
| 8.3.2 | Residual/retention oil tank(s) capacity (98%), if applicable: |  |  |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  |  |
| SBT Vessels |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.3.3 | What is total SBT capacity and percentage of SDWT vessel can maintain? |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 5118.221 Cu.Meters |  |  |  |  | 41.49 % |  |
| 8.3.4 | Does vessel meet the requirements of MARPOL Annex I Reg 18.2: |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |
| Cargo Handling and Pumping Systems |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.4 | How many grades/products can vessel load/discharge with double valve segregation: |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 16 |  |  |  |  |  |  |
| 8.4.1 | State type of cargo containment (integral, independent, gravity or pressure tanks): |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Gravity |  |  |  |  |  |  |
| 8.5 | Are there any cargo tank filling restrictions? If yes, specify number of slack tanks, max s.g., ullage restrictions etc.: |  |  |  |  |  |  |  |  |  |  |  |  |  |  | No |  |  |  |  |  |  |
| 8.6 | Max loading rate for homogenous cargo |  |  |  |  |  |  |  |  |  |  |  |  |  |  | With VECS |  |  |  | Without VECS |  |  |
|  | Loaded per manifold connection: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 445 Cu.Meters/Hour |  |  |
|  | Loaded simultaneously through all manifolds: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1000 Cu.Meters/Hour |  |  |
| Cargo Control Room |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.7 | Is ship fitted with a Cargo Control Room (CCR)? |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |
| 8.8 | Can tank innage/ullage be read from the CCR? |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |
| Gauging and Sampling |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.9 | Is gauging system certified and calibrated? If no, specify which ones are not calibrated: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |
|  | What type of gauging system as per IBC 13.1 is fitted (Open/Restricted/Closed )? |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Closed |  |  |  |  |  |
|  | What type of fixed closed tank gauging system is fitted: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Radar |  |  |  |  |  |
|  | Is a tank overflow control system fitted? If yes, then state if system includes automatic closing of valves? |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | No |  |  |  |  |  |
|  | Are high level alarms fitted to the cargo tanks? If Yes, indicate whether to all tanks or partial: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes, All tanks |  |  |  |  |  |
| 8.9.1 | Can cargo be transferred under closed loading conditions in accordance with ISGOTT 11.1.6.6? |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |
| 8.9.2 | Are cargo tanks fitted with multipoint gauging? If yes, specify type and locations: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | No |  |  |  |  |  |
| 8.10 | Number of portable gauging units (example- MMC) on board: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2 |  |  |  |  |  |
| Vapor Emission Control System (VECS) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.11 | Is a Vapour Emission Control System (VECS) fitted? |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |
| 8.12 | Number/size of VECS manifolds (per side): |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2 |  |  |  | 150 Millimeters |  |
| 8.13 | Number/size/type of VECS reducers: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 4 / 8inch / Concentric |  |  |  |  |  |
| Venting |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.14 | State what type of venting system is fitted: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Independent High Velocity P/V valve |  |  |  |  |  |
| Cargo Manifolds and Reducers |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.15 | Total number/size of cargo manifold connections on each side: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 16 / 150 Millimeters |  |  |  |  |  |
| 8.15.1 | Does the vessel have a Common Line Manifold connection? If yes, describe: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes, All tanks can be connected to Common Manifold. Manifold presentation flange connection is 10 inch. |  |  |  |  |  |
| 8.16 | What type of valves are fitted at manifold: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Butterfly valve |  |  |  |  |  |
| 8.17 | What is the material/rating of the manifold: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | SUS 316L Stainless Steel / JIS |  |  |  |  |  |
| 8.17.1 | Does vessel comply with the latest edition of the OCIMF 'Recommendations for Oil Tanker Manifolds and Associated Equipment’? |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |
| 8.18 | Distance between cargo manifold centers: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 700 mm |  |  |  |  |  |
| 8.19 | Distance ships rail to manifold : |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2805 mm |  |  |  |  |  |
| 8.20 | Distance ships side to manifold : |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2925 mm |  |  |  |  |  |
| 8.21 | Top of rail to center of manifold: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 600 mm |  |  |  |  |  |
| 8.22 | Distance main deck to center of manifold: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2189 mm |  |  |  |  |  |
| 8.23 | Spill tank grating to center of manifold: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 984 mm |  |  |  |  |  |
| 8.24 | Manifold height above the waterline in normal ballast/at SDWT condition: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 8.615 Meters |  |  |  | 5.401 Meters |  |
| 8.25 | Number/size/type of reducers: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Total 30 EA 01) 04inch X 05inch X 1EA (JIS X JIS) 02) 06inch X 03inch X 1EA (JIS X JIS) 03) 06inch X 04inch X 1EA (JIS X JIS) 04) 06inch X 05inch X 3EA (JIS X JIS) 05) 06inch X 06inch X 1EA (JIS X JIS) 06) 06inch X 08inch X 2EA (JIS X JIS) 07) 08inch X 08inch X 2EA (JIS X JIS) 08) 10inch X 10inch X 1EA (JIS X JIS) 09) 10inch X 12inch X 1EA (JIS X JIS) 10) 06inch X 04inch X 1EA (JIS X ASA) 11) 06inch X 08inch X 1EA (JIS X ASA) 12) 06inch X 10inch X 1EA (JIS X ASA) 13) 06inch X 12inch X 1EA (JIS X ASA) 14) 08inch X 04inch X 1EA (JIS X ASA) 15) 08inch X 06inch X 1EA (JIS X ASA) 16) 10inch X 06inch X 2EA (JIS X ASA) 17) 10inch X 08inch X 1EA (JIS X ASA) 18) 10inch X 10inch X 1EA (JIS X ASA) 19) 10inch X 12inch X 1EA (JIS X ASA) 20) 06inch X 08inch X 2EA (ASA X ASA) 21) 06inch X 10inch X 2EA (ASA X ASA) 22) 06inch X 12inch X 1EA (ASA X ASA) 23) 10inch X 12inch X 1EA (ASA X ASA) |  |  |  |  |  |
| 8.26 | Is vessel fitted with a stern manifold? If yes, state size: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | No |  |  |  |  |  |
| Heating |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.27 | Cargo/slop tanks fitted with a cargo heating system? |  |  |  |  |  |  |  |  |  | Type |  |  |  |  |  | Coiled |  |  |  | Material |  |
|  | Cargo Tanks: |  |  |  |  |  |  |  |  |  | Steam |  |  |  |  |  | Yes |  |  |  | SUS316L |  |
|  | Slop Tanks: |  |  |  |  |  |  |  |  |  | Steam |  |  |  |  |  | Yes |  |  |  | SUS316L |  |
| 8.27.1 | Is a Thermal Oil Heating system fitted? If yes, identify tanks? |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | No |  |  |  |  |  |
| 8.28 | Maximum temperature cargo can be loaded/maintained: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 80.0 deg C |  |  |  | 80.0 deg C |  |
| 8.28.1 | Minimum temperature cargo can be loaded/maintained: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 10.0 deg C |  |  |  | 10.0 deg C |  |
| Inert Gas and Crude Oil Washing |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.29 | Is an Inert Gas System (IGS) fitted/operational? |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | No / N/A |  |  |  |  |  |
| 8.29.1 | Is a Crude Oil Washing (COW) installation fitted/operational? |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | No / N/A |  |  |  |  |  |
| 8.30 | Is IGS supplied by flue gas, inert gas (IG) generator and/or nitrogen: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | No, only nitrogen bottle system |  |  |  |  |  |
| 8.30.1 | If nitrogen generator, specify the applicable flow rate for each of the designed purity modes: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  |
| Cargo Pumps |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.31 | How many cargo pumps can be run simultaneously at full capacity: |  |  |  |  |  |  |  |  |  |  |  | 6 |  |  |  |  |  |  |  |  |  |
| 8.32 | Pumps |  | No. | Type |  |  |  |  |  |  |  |  | Capacity |  |  |  |  |  | At What Head (sg=1.0) |  |  |  |
|  | Cargo Pumps: (For Cargo Tank) (For Slop Tank) |  | 14 2 | CENTRIFUGAL SUBMERGED / MARFLEX CENTRIFUGAL SUBMERGED / MARFLEX |  |  |  |  |  |  |  |  | 200 Cu. Metres/Hour 200 Cu. Metres/Hour |  |  |  |  |  | 115 115 |  |  |  |
|  | Cargo Eductors: |  | No |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Stripping: |  | No |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.33 | Is at least one emergency portable cargo pump provided? |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |
| Tank Cleaning Systems |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.34 | Is tank cleaning equipment fixed in cargo tanks? |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |
| 8.35 | Is portable tank cleaning equipment provided? |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |
| 8.36 | Tank washing pump capacity: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 120 Cu Meters/Hour |  |  |  |  |  |
| 8.37 | Is a washing water heater fitted? If yes is it operational and state max washing water temperature: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes, 130 Deg C |  |  |  |  |  |
| 8.38 | What is the maximum number of machines that can be operated at their designed max pressure? |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 4 |  |  |  |  |  |
| Other Deck Equipment |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.39 | Is vessel fitted with a remote cargo tank temperature monitoring system. If yes, is it operational? |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes / Yes |  |  |  |  |  |
| 8.40 | Is vessel fitted with a remote cargo tank pressure monitoring system. If yes, is it operational? |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes / Yes |  |  |  |  |  |
| 8.41 | Is vessel fitted with a cargo tank drier. If yes is it operational and state capacity: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | No |  |  |  |  |  |
| 8.42 | Is vessel fitted with a cargo cooling system. If yes is it operational and state tanks applicable: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | No |  |  |  |  |  |
| 8.43 | Is steam available on deck? |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |
| 9. | MOORING |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 9.1 | Wires (on drums) |  |  | No. | Diameter |  |  |  | Material |  |  |  |  |  | Length |  |  |  |  | Breaking Strength |  |  |
|  | Forecastle: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Main deck fwd: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Main deck aft: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Poop deck: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 9.2 | Wire tails |  |  | No. | Diameter |  |  |  | Material |  |  |  |  |  | Length |  |  |  |  | Breaking Strength |  |  |
|  | Forecastle: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Main deck fwd: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Main deck aft: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Poop deck: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 9.3 | Ropes (on drums) |  |  | No. | Diameter |  |  |  | Material |  |  |  |  |  | Length |  |  |  |  | Breaking Strength |  |  |
|  | Forecastle: |  |  | 4 | 65 Millimetres |  |  |  | MEGAFLEX |  |  |  |  |  | 200 Metres |  |  |  |  | 33 Metric Tonnes |  |  |
|  | Main deck fwd: |  |  |  | Millimetres |  |  |  |  |  |  |  |  |  | Metres |  |  |  |  | Metric Tonnes |  |  |
|  | Main deck aft: |  |  |  | Millimetres |  |  |  |  |  |  |  |  |  | Metres |  |  |  |  | Metric Tonnes |  |  |
|  | Poop deck: |  |  | 4 | 65 Millimetres |  |  |  | MEGAFLEX |  |  |  |  |  | 200 Metres |  |  |  |  | 33 Metric Tonnes |  |  |
| 9.4 | Other lines |  |  | No. | Diameter |  |  |  | Material |  |  |  |  |  | Length |  |  |  |  | Breaking Strength |  |  |
|  | Forecastle: |  |  | 5 | 65 Millimetres |  |  |  | MEGAFLEX |  |  |  |  |  | 200 Metres |  |  |  |  | 33 Metric Tonnes |  |  |
|  | Main deck fwd: |  |  |  | Millimetres |  |  |  |  |  |  |  |  |  | Metres |  |  |  |  | Metric Tonnes |  |  |
|  | Main deck aft: |  |  |  | Millimetres |  |  |  |  |  |  |  |  |  | Metres |  |  |  |  | Metric Tonnes |  |  |
|  | Poop deck: |  |  | 5 | 65 Millimetres |  |  |  | MEGAFLEX |  |  |  |  |  | 200 Metres |  |  |  |  | 33 Metric Tonnes |  |  |
| 9.5 | Winches |  |  | No. | No. Drums |  |  |  | Motive Power |  |  |  |  |  | Brake Capacity |  |  |  |  | Type of Brake |  |  |
|  | Forecastle: |  |  | 2 | 4 |  |  |  |  |  |  |  |  |  | 28.64 Metric Tonnes |  |  |  |  | Split Double Drums |  |  |
|  | Main deck fwd: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Main deck aft: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Poop deck: |  |  | 2 | 4 |  |  |  |  |  |  |  |  |  | 28.64 Metric Tonnes |  |  |  |  | Split Double Drums |  |  |
| 9.6 | Bitts, closed chocks/fairleads |  |  |  | No. Bitts |  |  |  | SWL Bitts |  |  |  |  |  | No. Closed Chocks |  |  |  |  | SWL Closed Chocks |  |  |
|  | Forecastle: |  |  |  | 4 |  |  |  | 33 Metric Tonnes |  |  |  |  |  | 1/6 |  |  |  |  | 200/33 Metric Tonnes |  |  |
|  | Main deck fwd: |  |  |  | 8 |  |  |  | 33 Metric Tonnes |  |  |  |  |  | 8 |  |  |  |  | 33 Metric Tonnes |  |  |
|  | Main deck aft: |  |  |  | 4 |  |  |  | 33 Metric Tonnes |  |  |  |  |  | 4 |  |  |  |  | 33 Metric Tonnes |  |  |
|  | Poop deck: |  |  |  | 6 |  |  |  | 33 Metric Tonnes |  |  |  |  |  | 11 |  |  |  |  | 33 Metric Tonnes |  |  |
| Anchors/Emergency Towing System |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 9.7 | Number of shackles on port/starboard cable: |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 10 / 10 |  |  |  |  |  |  |
| 9.8 | Type/SWL of Emergency Towing system forward: |  |  |  |  |  |  |  |  |  |  |  |  |  |  | IWRC(6X36) Type Wire |  |  |  |  |  | 31.3Metric Tonnes |
| 9.9 | Type/SWL of Emergency Towing system aft: |  |  |  |  |  |  |  |  |  |  |  |  |  |  | IWRC(6X36) Type Wire |  |  |  |  |  | 31.3Metric Tonnes |
| Escort Tug |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 9.10 | What is size/SWL of closed chock and/or fairleads of enclosed type on stern: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 9.11 | What is SWL of bollard on poop deck suitable for escort tug: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Lifting Equipment/Gangway |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 9.12 | Derrick/Crane description (Number, SWL and location): |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | CRANE / 1 / 3.0 Metric tons / Deck |  |  |  |  |  |
| 9.13 | Gangway direction & length: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Face aft |  |  |  | 7.57 M |  |
| Single Point Mooring (SPM) Equipment |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 9.14 | Does the vessel meet the recommendations in the latest edition of OCIMF 'Recommendations for Equipment Employed in the Bow Mooring of Conventional Tankers at Single Point Moorings (SPM)’:? |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | No |  |  |  |  |  |
| 9.15 | If fitted, how many chain stoppers: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 |  |  |  |  |  |
| 9.16 | State type/SWL of chain stopper(s): |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Tonque |  |  |  | 100 Metric Tonnes |  |
| 9.17 | What is the maximum size chain diameter the bow stopper(s) can handle: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 54 Millimeters |  |  |  |  |  |
| 9.18 | Distance between the bow fairlead and chain stopper/bracket: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 3000 Millimeters |  |  |  |  |  |
| 9.19 | Is bow chock and/or fairlead of enclosed type of OCIMF recommended size (600mm x 450mm)? If not, give details of size: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |
| 10. | PROPULSION |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 10.1 | Speed |  |  |  |  |  |  |  |  |  |  | Maximum |  |  |  |  |  |  |  |  | Economical |  |
|  | Ballast speed: |  |  |  |  |  |  |  |  |  |  | 14.25KTS |  |  |  |  |  |  |  |  | 13.75KTS |  |
|  | Laden speed: |  |  |  |  |  |  |  |  |  |  | 13.5KTS |  |  |  |  |  |  |  |  | 13.0 KTS |  |
| 10.2 | What type of fuel is used for main propulsion/generating plant: |  |  |  |  |  |  |  |  |  |  | MFO 380 CST |  |  |  |  |  |  |  |  | MDO |  |
| 10.3 | Type/Capacity of bunker tanks: |  |  |  |  |  |  |  |  |  |  | IFO : 631.532Cu.Meter MDO : 66.726Cu.Meter |  |  |  |  |  |  |  |  |  |  |
| 10.4 | Is vessel fitted with fixed or controllable pitch propeller(s): |  |  |  |  |  |  |  |  |  |  | Fixed Pitch |  |  |  |  |  |  |  |  |  |  |
| 10.5 | Engines |  |  |  |  |  |  |  |  | No |  | Capacity |  |  |  |  |  | Make/Type |  |  |  |  |
|  | Main engine: |  |  |  |  |  |  |  |  | 1 |  | 4440.00 Kw |  |  |  |  |  | STX MAN B&W / DIESEL |  |  |  |  |
|  | Aux engine: |  |  |  |  |  |  |  |  | 3 |  | 615 Kw |  |  |  |  |  | YANMAR / VERTICAL |  |  |  |  |
|  | Power packs: |  |  |  |  |  |  |  |  | 16 |  | 66 Kw |  |  |  |  |  | MARFLEX / PUMP HEAD TYPE |  |  |  |  |
|  | Boilers: |  |  |  |  |  |  |  |  | 1 |  | 12,000KG/H |  |  |  |  |  | MIURA / VERTICAL |  |  |  |  |
| Bow/Stern Thruster |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 10.6 | What is brake horse power of bow thruster (if fitted): |  |  |  |  |  |  |  |  |  |  |  |  |  | 612 bhp / 456.552 Kilowatt |  |  |  |  |  |  |  |
| 10.7 | What is brake horse power of stern thruster (if fitted): |  |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  |  |  |
| Emissions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 10.8 | Main engine IMO NOx emission standard: |  |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  |  |  |
| 10.9 | Energy Efficiency Design Index (EEDI) rating number: |  |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  |  |  |
| 11. | SHIP TO SHIP TRANSFER |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 11.1 | Does vessel comply with recommendations contained in OCIMF/ICS Ship To Ship Transfer Guide (Petroleum, Chemicals or Liquified Gas, as applicable)? |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |
| 11.2 | What is maximum outreach of cranes/derricks outboard of the ship’s side: |  |  |  |  |  |  |  |  |  |  |  |  |  | 4.8 Meters |  |  |  |  |  |  |  |
| 11.3 | Date/place of last STS operation: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 12. | RECENT OPERATIONAL HISTORY |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 12.1 | Last three cargoes/charterers/voyages (Last/2nd Last/3rd Last): |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 12.2 | Has vessel been involved in a pollution, grounding, serious casualty, unscheduled repair or collision incident during the past 12 months? If yes, provide details: |  |  |  |  |  |  |  |  |  |  |  |  |  | Pollution: No Grounding: No Serious casualty: No Collision: No |  |  |  |  |  |  |  |
| 12.3 | Date and place of last Port State Control inspection: |  |  |  |  |  |  |  |  |  |  |  |  |  | 18TH OCT 2023 / Port Klang, Malaysia |  |  |  |  |  |  |  |
| 12.4 | Any outstanding deficiencies as reported by any Port State Control? If yes, provide details: |  |  |  |  |  |  |  |  |  |  |  |  |  | No |  |  |  |  |  |  |  |
| 12.5 | Recent Oil company inspections/screenings (To the best of owners knowledge and without guarantee of acceptance for future business)*: * "Approvals" are not given by Oil Majors and ships are accepted for the voyage on a case by case basis. |  |  |  |  |  |  |  |  |  |  |  |  |  | IDEMITSU, SHELL, BP, CHEVRON, BASF, PETRONAS |  |  |  |  |  |  |  |
| 12.6 | Date/Place of last SIRE inspection: |  |  |  |  |  |  |  |  |  |  |  |  |  | 06th Aug 2023 / PREEM / BANGKOK, THAILAND |  |  |  |  |  |  |  |
| 12.6.1 | Date/Place of last CDI inspection: |  |  |  |  |  |  |  |  |  |  |  |  |  | 21st Jan 2023 / Mailiao, Taiwan |  |  |  |  |  |  |  |
| 12.7 | Additional information relating to features of the ship or operational characteristics: |  |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  |  |  |

Revised 2018 (INTERTANKO/Q88.com)

Form completed on http://www.q88.com/integration.aspx  Please email support@q88.com an updated copy if this is not the latest version.